丙类大副光碟版题库（共计1059题）

一．各类证书的内容，展期和更新（共计54道题）

1.______ is issued under the provision of International Convention for the Prevention of Pollution from Ships，1973，as modified by the Protocol of 1978 relating thereto．

______是根据提供的国际公约防止船舶造成污染， 1973年议定书修订的1978年有关。

A．International Oil Pollution Prevention Certificate    国际防止油污证书

B.  Cargo Ship Safety Construction Certificate        货船构造安全证书

C.  Cargo Ship Safety Equipment Certificate          货船设备安全证明书

D.  Cargo Ship Safety Radiotelephony Certificate      货船无线电话安全证书

2. A Permit to Proceed is issued by the ______．      许可证着手印发的______ 。

A. CCS

B. CCIC

C. MSA OF CHINA

D. Department of Labor                    劳工部

3. A record of the types and strengths of steels used on a ship must be included in the ______．

记录的类型和优势用钢的船舶必须列入______ 。

A. General plans                       总体规划

B．Builder's documentation

C．Certificate of Inspection               检验证

D．Construction portfolio                建设投资

4．A vessel has sustained damage in a collision with another vessel．It is necessary to have a Seaworthy Certificate before the vessel sails．Who will issue this certificate ________．

A. Diplomatic Consul of the Flag State       外交领事船旗国

B．Classification Society                  船级社

C．Captain of the Port

D．Officer in Charge of Marine Inspection    主管海洋检查

5. According to the Maritime Safety Traffic Law of the People's Republic of China，which administrative organ in China is in charge of issuing the Certificate of Vessel's Nationality ________．

A．Customs Office

B．Harbor Operation Office

C．MSA

D．Register of Shipping

据交通部海事安全法中华人民共和国的中国，这在中国的行政机关是负责签发证书的船舶国籍________ 。

6. All bulk grain carriers shall carry on board the following certificates except ______．

A．cargo information

B．document of authorization for the carriage of grain

C．decision support system for masters

D．cargo ship safety certificate

所有散粮运输局应进行下列证书除外______ 。

7. All cargo ships shall carry on board the following certificates except ______．

A．Cargo Ship Safety Construction Certificate        货船构造安全证书

B．Cargo Ship Safety Equipment Certificate          货船设备安全证书

C．Decision support system for masters

D．Cargo Ship Safety Radio Certificate              货船无线电安全证明书

所有货物的船舶须在船上进行以下证书除外______ 。

8. All oil tankers shall carry on board the following certificates except ______．

A．Enhanced survey report file

B．Condition Assessment Scheme（CAS）Statement of Compliance，CAS Final Report and Review Record Document of authorization for the carriage of grain

C．Decision support system for masters

D．Hydrostatically Balanced Loading（HBL）Operational Manual

所有油轮不得经营下列证书委员会除外______ 。

9. All passenger ships shall carry on board the following certificates except ______．

A．passenger Ship Safety Certificate         客船安全证书

B．search and rescue co-operation plan

C．decision support system for masters

D．dangerous goods manifest or stowage plan

所有客船应进行下列证书委员会除外______ 。

10.All ships shall carry on board the following certificates except ______．

A．Intact stability booklet              完整的稳定的小册子

B．Minimum safe manning document    最低安全配员文件

C．Fire safety training manual           消防安全培训手册

D．Dangerous goods manifest or stowage plan   危险货物舱单或积载计划

所有船舶应在船上进行以下证书除外______ 。

11. All ships shall carry on board the following certificates except ______．

A．Fire Control plan/booklet            消防控制计划/小册子

B．On board training and drills record      培训和演练记录

C．Fire safety operational booklet         消防安全业务的小册子

D．International Certificate of Fitness for the Carriage of INF Cargo   国际适装证书的运输辐照核燃料货物

所有船舶应在船上进行以下证书除外______ 。

12. All ships shall carry on board the following certificates except ______．

A．International Ship Security Certificate（ISSC）or Interim International

Ship Security Certificate              国际船舶保安证书（社科理事会）或临时国际

B．Ship Security Plan and associated records        船舶安全证书

C．Continuous Synopsis Record（CSR）           船舶保安计划及相关记录

D．Dangerous goods manifest or stowage plan        危险货物舱单或积载计划

所有船舶应在船上进行以下证书除外______ 。

13. An International Tonnage Certificate will be issued to a vessel when it meets several requirements，one of which is that the vessel must ______．

A．admeasure over 100 GT

B．be 79 or more feet in length

C．engage in intercoastal or international trade

D．be issued a Certificate of Inspection

国际吨位证明书将发给船只开会时数的要求，其中之一是该船只必须______ 。

14. Certificate of Fitness for the Carriage of Liquefied Gases in Bulk shall be carried on board by ______．

A．gas carriers constructed on or after 1 July 1986

B．gas carriers constructed before 1 July 1986

C．All chemical tankers                 所有化学品液货船

D．oil tankers of 150 gross tonnage and above

适装证书的运输散装液化气体应进行局______ 。

15. Certificate of insurance or other financial security in respect of civil liability for oil pollution damage shall be carried on board by ______．

A．ships carrying more than 2000 tons of oil in bulk as cargo

B．oil tankers

C．bulk chemicals carriers

D．liquefied gas carriers

证书的保险或其他财务担保方面的民事赔偿责任而造成油污损害的，应在船上进行的______ 。

16. Compliance with the terms of the load line certificate on a ship is the responsibility of the ______．

A．Ballast Control Operator            电子镇流器控制操作

B．Barge Supervisor

C．Master or Offshore Installation Manager

D．operators representative

符合条件的载重线证书的船舶的责任是______ 。

17. Damage control plans and booklets shall be carried on board by ______．

A．all ships

B．cargo ships only

C．passenger ships

D．those ships loaded with liquid dangerous chemicals in bulk

损毁控制图和小册子应进行局______ 。

18.Decision support system for masters shall be carried on board by ______．

A．Passenger ships

B．Cargo vessels

C．Liquid material carriers

D．All ships

19. Garbage Management Plan shall be carried on board by ______．

A．Every ship of 400 gross tonnage and above and every ship which is certified to carry 15 persons or more

B．Every ship of less than 400 gross tonnage and every ship which is certified to carry 15 persons or less

C．Solid cargo vessel of other than Handysize or larger

D．Liquid cargo ship of other than LNG carrier

垃圾管理计划应进行局______ 。

答：每艘400总吨及以上的每艘船舶，经证明可运载15人或以上

B .每艘小于400总吨数和每艘船舶，经证明可运载15人或更少

C .固体货船以外的小灵便型或更大的

D .液体货物的船舶以外的其他LNG船

20. Garbage Record Book shall be carried on board by ______．

A．Every ship of 400 gross tonnage and above and every ship which is certified to carry 15 persons or more

B．Every ship of less than 400 gross tonnage and every ship which is certified to carry 15 persons or less

C．Solid cargo vessel of other than Handysize or larger

D．Liquid cargo ship of other than LNG carrier

垃圾记录簿应进行局______ 。

答：A .每艘400总吨及以上的每艘船舶，经证明可运载15人或以上

B .每艘小于400总吨数和每艘船舶，经证明可运载15人或更少

C .固体货船以外的小灵便型或更大的

D .液体货物的船舶以外的其他LNG船

21. If the Certificate of Inspection of a damaged tank barge has expired，which certificate may be issued to allow its movement to a repair facility ________．

A．change of employment

B．permit to proceed

C．application for inspection

D．temporary certificate of inspection

22. If there are any Certificates expired or nearing expiry the master of a ship must apply for their __at any port．

A．inspection or issuance          检查或发行

B．requirements or renewal        要求或续期

C．renewal or extension          续期或延期

D．extension or expiring         延长或届满

如果有任何证书过期或即将过期的船舶的船长，必须申请其______在任何港口。

23. International Certificate of Fitness for the Carriage of Dangerous Chemicals in Bulk shall be carried on board by ______．

A．chemical tankers constructed on or after 1 July 1986

B．chemical tankers constructed before 1 July 1986

C．All chemical tankers

D．oil tankers of 150 gross tonnage and above

国际适装证书为运载散装危险化学品应进行局______ 。

24. International Certificate of Fitness for the Carriage of Liquefied Gases in Bulk shall be carried on board by ______．

A．gas carriers constructed on or after 1 July 1986

B．gas carriers constructed before 1 July 1986

C．All chemical tankers

D．oil tankers of 150 gross tonnage and above

25. International Load Line Certificate shall be carried on board by ______．

A．all ships

B．cargo ships only

C．passenger ships

D．those ships loaded with dangerous chemicals in bulk

国际载重线证明书须在船上的____

26. International Oil Pollution Prevention Certificate shall be carried on board by ______ which is engaged in voyages to ports or offshore terminals under the jurisdiction of other Parties to MARPOL 73/78．

A．Oil tanker of 150 gross tonnage and above

B．Bulk carrier

C．Cargo vessel

D．Passenger ship

国际防止油污证书须在船上的______这是从事航行到港口或近岸码头管辖下的其他缔约方73/78防污公约。

27.Official proof of a Chinese vessel's nationality is contained in the ______．

A．Certificate of Inspection        检验证

B．Official Log                  官方日志

C．Certificate of Documentation    船舶证书总册（船长保管）

D．Shipping Articles              航运条款

官方的证明，中国船只的国籍的情况载于______ 。

28. Oil Record Book shall be carried on board by ______ which is engaged in voyages to ports or offshore terminals under the jurisdiction of other Parties to MARPOL 73/78．

A．oil tanker of 150 gross tonnage and above

B．bulk carrier

C．container vessel

D．passenger ship

油类记录簿应进行局______这是从事航行到港口或近岸码头管辖下的其他缔约方73/78防污公约。

29. Oil Record Book shall be carried on board by ______ which is engaged in voyages to ports or offshore terminals under the jurisdiction of other Parties to MARPOL 73/78．

A．Oil tanker

B．Bulk carrier of 400 gross tonnage and above

C．Container vessel

D．Passenger ship

油类记录簿应进行局______这是从事航行到港口或近岸码头管辖下的其他缔约方73/78防污公约。

30.Please arrange the ______ of my Deratting Exemption Certificate．

A．Renewal

B．Replacement

C．Recheck

D．Review

请安排______我国灭鼠豁免证明书。

31. Shipboard Oil Pollution Emergency Plan shall be carried on board by ______．

A．Oil tanker of 150 gross tonnage and above

B．Bulk carrier

C．Container vessel

D．Passenger ship

船上油污应急计划应在船上进行的______ 。

32. Shipboard Oil Pollution Emergency Plan shall be carried on board by ______．

A．Oil tanker

B．Bulk carrier of 400 gross tonnage and above

C．Container vessel

D．Passenger ship

船上油污应急计划应在船上进行的______ 。

33. That she was entered in the REGISTER BOOK of this Society，with the Character +100A1 subject to Periodical Surveys as required by the Rules is likely to appear in ______．

A．INTERNATIONAL OIL POLLUTION PREVENTION CERTIFICATE    国际防止油类污染证书

B．FUMIGATION AND GAS FREE CERTIFICATE                     熏蒸和清除气体证书

C．CERTIFICATE OF CLEANLINESS OF NATIONAL CARGO BUREAU，INC   证书的洁净情况国家货物局，公司

D．CERTIFICATE OF CLASS                                       证书类

她入登记簿的这个社会，性质百格A1受定期检验所要求的规则很可能会出现在______ 。

34. The agency which assigns load lines and issues Load Line Certificates is the ______．

A．the Bureau of Shipping            航运局

B．Secretary of Commerce            商务部长

C．the Customs                     海关

D．MSA or Coast Guard              海事或海岸警卫队

该机构的分配问题载重线和载重线证明书是______ 。

35. The Certificate of Inspection for a containership ______．

A．is issued by the Coast Guard and is usually valid for 2 years        海岸警卫队颁发有效期一般为2年

B．must be posted under transparent material near the officers' licenses

C．lists all of the stability limitations and conditions imposed on the vessel

D．shows the due date of the quadrennial test of the cargo gear

集装箱的检验证书______ 。

36.The government of PR China ______ that the above-mentioned ship complies with the provisions of the regulations annexed to the Convention referred to above as regards radiotelegraphy and radar．

A．certifies

B．confirms

C．contains

D．construes

政府公关中国______ ，上述船舶符合规定所附条例公约上面提到的关于无线电和雷达。

37. The loadline certificate of a ship registered in China is issued by ______．

A．CCS

B．MOC

C．MSA

D．PRC Customs

在载重线证书注册的船舶在中国是由______ 。

38. The official identification of a vessel is found in the ______．

A．certificate of inspection      检验证明书

B．classification certificate       分类证书

C．loadline certificate            载重线证书

D．certificate of documentation    船舶证书总册

39.The Tonnage Certificate gives the ______ of a ship which is classified into gross tonnage and net tonnage．

A．specifications     说明书

B．descriptions      类型

C．dimensions      部分

D．prescriptions    处方

吨位证书使______的船舶分为总吨位和净吨位。

40.The Tonnage Certificate indicates ______．     吨位证书表明______

A．Deadweight tons        载重吨

B．Displacement tons       排水量吨

C．Net tons and gross tons   净吨和总吨

D．Volume tons            容积吨

41. The undersigned ______ that he is duly authorized（经正式授权） by the said （上述）Government to issue this certificate．

A．Claims      声称

B．Declares    声明

C．Complains   投诉

D．Says

42.The validity（有效） of this certificate is，in accordance with article 19（4）（a）of the Convention，______ until Jan 1 2003．

A．stretched

B．expanded

C．enlarged

D．extended   持续

此证书的有效性，是根据第19条（ 4 ） （ a ）项规定,______ 直到2003年1月1日。

43.The vessel's Load Line Certificate is always issued by ______．

A．The Register of Shipping         船舶登记

B．Customs House                 海关

C．Cargo Survey Bureau            货运公司

D．Maritime Safety Administration    海事局

船舶载重线证明书总是______发布 。

44.This certificate is issued under the authority of the said Government，and it will remain in ______ until Jan 1 2008．

A．Strength

B．Power

C．Force

D．Stress

这签发证书的权力之下说政府，而且将继续留在______直到2008年1月1日。

45. This Certificate will remain in force （在有效期中）until May 1，2009．This Sentence refers to ______ of the Certificate．

A．the validity      有效性

B．the time

C．the date

D．the issue

此证书将继续有效，直至5月1,2009 。这句话是指______证书。

46.Vessels must be ______ valid certificate of nationality or certificate of registry or license．

A．acquiring

B．with possession of

C．required for

D．in possession of      拥有

船只必须______有效国籍证书或登记证书或许可证。

47.What is official proof of a vessel's ownership ________．

A．certificate of documentation

B．bill of lading

C．transfer certificate

D．logbook

什么是官方证明该船的所有权________ 。

48.Which agency assigns an official number to a vessel ________．

A．Bureau of Shipping    航运局

B．Collector of Customs  海关征收员

C．Treasury Department  财政部

D．Coast Guard        海岸警卫队

哪个机构可以给船一个正式编号________ 。

49.Which certificate is issued by the American Bureau of Shipping ________．

A．Certificate of Inspection       检验证

B．Load Line Certificate         载重线证书

C．Safety Equipment Certificate   设备安全证书

D．Permit to Proceed for repairs

哪个证书由美国航运局签发________ 。

50.Which document is NOT required by law to be posted aboard a vessel ________．

A．Official Crew List       正式船员名单

B．Certificate of Inspection  检验证

C．Officer's licenses

D．Muster List            部署表

该文件并非法律要求张贴船上________ 。

51.Which document shows the minimum required crew a vessel must have to navigate from one port in the United States to another

A．Articles

B．Certificate of Inspection

C．Crew List

D．Register

哪个文件显示所需的最低船员船只必须航行从一个美国一个港口到另一港口________ 。

52. You are in port A，and your Certificate of Inspection has expired．You wish to go to port B in the same country for repairs and to complete the inspection．If the Officer-in-Charge Marine Inspection deems it safe，he may issue a ______．

A．certificate of seaworthiness

B．limited certificate of inspection

C．temporary certificate of inspection

D．permit to proceed

您在港口A ，而您的检验证已过期，你想到港口B在同一个国家修理和完成检查。假如主管海洋安全检查组认为安全，他可发出______

53.Your load line certificate expires on 27 May 2006．The vessel is surveyed on that date and is found satisfactory．You are sailing foreign the same day．Which statement is TRUE ________．

A．A new certificate must be issued before you sail

B．The existing certificate is endorsed as valid for a five year period commencing 27 May 2006

C．The existing certificate is extended for a period of up to 150 days      现有证书延长期限最长为150天

D．The existing certificate is extended until the first foreign port of call where a new certificate will be issued by the local surveyor

您的载重线证书到期的5月27日调查2006.The船只在该日期和发现并很满意。.You的帆船外国同day.Which说法正确________ 。

54. Your vessel has completed an inspection for certification and is issued a Temporary Certificate of Inspection．The Temporary Certificate ______．

A．Has the full force of the regular Certificate of Inspection

B．Expires six months after it is issued

C．Must be exchanged for a regular Certificate of Inspection within 3 months

D．Is retained in the custody of the Master

您的船只已经完成了检验证书，并发表了一份临时证书Inspection.The临时证书______ 。

二．各类报表（共计33道题）

55.______ General Declaration，Passenger List，Crew List，Crew's Effects Declaration，Ship's Stores Declaration，Cargo Declaration and Maritime Declaration of Health，Ammunition List，Dangerous Cargo List，Stowaway List，Walkaway List，Drug List，etc．are also necessarily to be presented by the Master to the Joint Inspection Party．

A．Besides

B．Beside

C．Except

D．Except for

56. ______ is the best name and description of the vessel referred to．

A．M/V Yu Long

B．MV Yu Long

C．M/V YuLong

D．MV YuLong

57. ______ are not to be declared in the Crew's Personal Effects Declaration．

A．Cigar，cigarettes，tobacco and snuff    雪茄，香烟，烟草和鼻烟

B．Beer，wine and spirits                啤酒，葡萄酒和烈酒

C．Mineral water and soft drink            矿泉水和软饮料

D．Shirts，trousers and clothes            恤衫，长裤和衣服

______是不被宣布船员的个人财物宣言。

58. ______ signifies that vessel has passed quarantine inspection and may enter port．

A．Freeboard    干舷

B．Freight       货运

C．Pratique      入港许可

D．Patent       专利

______标志着船只已通过检验检疫，并可能进入港口。

59.A document which has a list of names，birthplaces，and residences of persons employed on a merchant vessel bound from a PRC port on a foreign voyage and is required at every port is called the ______．

A．Certified Crew List     认证船员名单

B．Crew Manifest         船员舱单

C．Shipping Articles      船员雇佣合同

D．Station Bill

一个文件，这个文件有一个名单，出生地，和住宅的人雇用的商船约束从中华人民共和国外国港口的航程，并须在每一个港口被称为______ 。

60. A vessel sailing from Liverpool to New York puts into Boston，Mass．for emergency repairs．If no inward foreign cargo is to be discharged at that port，which of the following documents is required ________．

A．Custom Manifest        自定义舱单

B．Inward Foreign Manifest   内向外国舱单

C．Pro Forma Manifest       形势仓单

D．Traveling Manifest        旅游舱单

一艘帆船从利物浦到纽约投入波士顿， Mass.for紧急repairs.If没有内向外国货物要出院时，港口，其中下列文件需要____

61. A vessel's Derat Certificate is always issued by ______．

A．Classification Society            船级社

B．Cargo Survey Bureau            货运调查局

C．Environmental Protection Agency   环境保护局

D．Public Health                   公共卫生

船的灭鼠证书总是发出______ 。

62. At any rate ______ is liable for delay in obtaining the necessary Customs House papers for discharging，when the delay arises from the fact that special papers are required for the particular cargo carried．

A．The Charterer承租人

B．The Shipowner船东

C．The Shipper托运人

D．The Receiver

无论如何______的规定，即属迟延获得必要的海关文件，供履行，拖延时，产生的事实，特别文件所需要的特定货物。

63.Before sailing for a foreign port，what document must you get ________．

A．The Policy of insurance    保险单

B．The Report of Entrance     入口的报告

C．The Clearance        清除

D．The Pratique         实用

在航行的外国港口，什么文件都必须你________ 。

64. Customs officers，upon boarding a vessel desiring entry into one of their port，would inspect which document ________．

A．Cargo Manifest      货物舱单

B．Certified Crew List   认证船员名单

C．Stores List         店铺列表

D．All of the above

海关人员后，渴望登上船只进入其港口之一，将检查该文件________ 。

65. Dangerous Cargoes are to be declared in ______．

A．Ship's Stores Declaration

B．Cargo Declaration

C．General Declaration

D．Crew's Effects Declaration

危险货物将被宣布______ 。

66. Free pratique means that ______．

A．clearance requirements for all regulatory bodies have been met

B．health requirements have been met and no further quarantine formalities are required

C．shipment will be made at no cost

D．tonnage taxes are not required to be paid

自由无疫通行证意味着______ 。

67. General Declaration，Passenger List，Crew List，Crew's Effects Declaration，Ship's Stores Declaration，Cargo Declaration and Maritime Declaration of Health，Ammunition List，Dangerous Cargo List，Stowaway List，Walkaway List，Drug List，etc．are necessarily to be presented by the Master to the ______．

A．Joint Inspection Party

B．The Shipper

C．The Receiver

D．Customs House

总申报单，旅客名单，船员名单，船员的影响宣言，船舶的商店宣言，宣言和海上货运宣言的健康，弹药清单，危险货物清单，偷渡者名单， Walkaway名单，药物清单， etc.are不一定是所提出的硕士的______

68. In accordance with Maritime Health Declaration，______ is not an infectious disease．

A．cholera

B．typhus fever

C．yellow fever

D．venereal disease

根据海事健康宣言,______不是一种传染病。

69.In the absence of a surgeon，the master should not regard the symptoms of ______ as ground for suspecting the existence of infectious disease．

A．higher blood glucose after mealr      餐后血糖

B．fever accompanied by prostration or persisting for several days or attended with glandular swelling伴随虚脱或持续数天腺肿胀

C．any acute skin rash or eruption with or without fever    急性皮疹或火山喷发或不热

D．severe diarrhoea with symptoms of collapse         腹泻症状的崩溃

由于缺乏医生，船长不应方面的症状______作为理由怀疑存在的传染病。

70. In the absence of a surgeon，the master should not regard the symptoms of ______ as ground for suspecting the existence of infectious disease．

A．arthritis pain include swelling within the joints      疼痛肿胀的关节

B．any acute skin rash or eruption with or without fever  急性皮疹或火山喷发或不热

C．severe diarrhoea with symptoms of collapse        腹泻症状的崩溃

D．jaundice by fever

由于缺乏医生，船长不应方面的症状______作为理由怀疑存在的传染病。

71. Maritime Declaration of Health is generally prepared by the Master ______．

A．when his vessel is arriving at a foreign port

B．when his vessel is leaving a foreign port

C．after his vessel has left a foreign port

D．after his vessel has arrived at a port

海事宣言一般卫生编写的硕士______ 。

72. Marks and Nos．，numbers and kinds of packages，description of goods are to be provided in ______．

A．Passenger List

B．General Declaration

C．Cargo Declaration

D．Ship's Stores Declaration

标志和编号，数量和种类的包装，说明货物必须提供______ 。

73. My Deratization Exemption Certificate will become ______ on the 16th of September．

A．Due

B．Expire

C．Lawful

D．Non-effective

我灭鼠豁免证明书将成为______的9月16日。

74.Of the following items，______ is not to be provided in the General Declaration．

A．position of the ship in the port

B．brief particulars of the voyage

C．brief description of the cargo

D．number and kind of packages of the goods carried

下列项目,______是不能提供的总申报单。

75. Permission is kindly ______ about 1000 tons of clean sea water ballast，as and when required for trimming purposes．

A．Requested to pump out

B．Requested pumping out

C．Requesting to pump out

D．Requesting pumping out

许可和蔼______约1000吨的清洁海水镇流器，并在有需要时进行微调的目的。

76. Ship's stores declaration generally consists of ________．

A．Deck Stores List，Engine Stores List，Kitchen Stores List，Bonded Stores List and Doctor's Stores List

B．Marks and Nos．，number and kind of packages

C．Gross weight，measurement and quantity

D．Brief particulars of the voyage and brief particulars of the cargo

77. The agency most concerned with a stowaway is ______．

A．Customs

B．Immigration

C．Public Health

D．MSA

该机构最关心的偷渡是______ 。

78. The Certificate of pratique shall only have effect in ______．

A．all ports of the issuing country

B．all ports of the world

C．all ports other than a issuing country

D．all ports of ship's arrival

证书实践应具有效力，只能______ 。

79. The citizenship of a crew member of a vessel is determined solely by the ______．

A．Customs Officer

B．Immigration Officer

C．Coast Guard

D．Union Official

80. The date and place of Deratisation or Deratisation Exemption Certificate are to be provided in ______．

A．Maritime Declaration of Health

B．General Declaration

C．Passenger List

D．Cargo Declaration

日期和地点Deratisation或Deratisation豁免证明书是提供______ 。

81. The Immigration and Naturalization Service is concerned with which document on a vessel making preliminary entry into a U．S．port from a foreign port ________．

A．Cargo Manifest

B．Certified Crew List

C．Curio List

D．Shipping Articles

移民和归化局所关心的这些文件，提出初步的船只进入U.S.port从外国港口________ 。

82. The Quarantine officer told the master that the Certificate of Pratique shall become ______ if the ship calls at other foreign ports．

A．No disposal

B．Ineffective

C．No result

D．No event

检疫官员告诉船长，该证书将成为实用______如船舶停靠外国港口。

83. Uncleared crew curios remaining on board during a domestic coastwise voyage after returning from foreign should be ______．

A．listed in the Official Logbook

B．cleared prior to the next foreign voyage

C．noted in the Traveling Curio Manifest

D．retained under locked security by the owner

未船员留在船上珍玩在国内近海航行从国外回国后应该______ 。

84. What form should the master hand over to the Customs Officer at the port of arrival ________．

A．Last Port Clearance

B．Declaration of Crew's Baggages

C．Maritime Declaration of Health

D．List of Bonded Stores

什么形式的船长应移交给海关人员在到达港口________ 。

85.When clearing customs for a foreign voyage，which of the following is processed at the custom's house and returned to the vessel ________．

A．Shipping Articles

B．Traveling Curio Manifest

C．Official Logbook

D．Cargo Gear Register

当关为外国航程，下列哪项是处理自定义的家，回到船上________ 。

86. When satisfied that there are no contagious diseases on board，the quarantine officer will issue ______．

A．an outward clearance

B．a free pratique

C．landing permits

D．shore passes

当确信没有任何传染疾病，对董事会，检疫人员将问题______ 。

87. Would you please ask the ______ Officer to issue 30 sheets of landing permits for our crew members to go ashore．

A．Quarantine

B．Immigration

C．Customs

D．Harbor

该问______干事发出30张许可证，我们的目标船员上岸。

第二章第一节航路指南（共计53题）

88. A “loose” tow may cause all of the following EXCEPT ______．

A．loss of maneuverability

B．lines to part

C．damage to the towing vessel and tow

D．a saving in the transit time

89. A single vessel being towed alongside shall exhibit ______．

A．One all-round white light

B．Sidelights and a stern light

C．Only the outboard sidelight and a stern light

D．A masthead light，sidelights，and a stern light

一个单一的船只被拖曳的同时须展示______ 。

90.A towing vessel becomes tripped while towing on a hawser astern．What factor is MOST important when assessing the risk of capsizing ________．

A．Length of the towline

B．Height of the towline connection

C．Longitudinal position of the towline connection

D．Direction of opposing force

拖曳船只成为绊倒而拖着一个hawser astern.What因素最重要的是在评估风险倾覆________ 。

91. A twin-screw ship going ahead on the starboard screw only tends to move ______．

A．in a straight line

B．to port

C．from side to side

D．to starboard

双螺杆船前进的右舷螺钉往往只移动______ 。

92. A twin-screw vessel can clear the inboard propeller and maneuver off a pier best by holding a（n）______．

A．Forward spring line and going slow ahead on the inboard engine

B．After spring line and going slow astern on the outboard engine

C．Forward spring line and going slow ahead on both engines

D．Forward spring line and going slow ahead on the outboard engine

双螺杆船只可以清除机动船螺旋桨和最好的一个码头对开发生举行（ ñ )______

93. A vessel outbound from a port or pilot station within an inshore traffic zone is recommended to ______．

A．call at a port，pilot station，destination or sheltered anchorage

B．use the appropriate lane other than the adjacent traffic separation scheme

C．use the inshore traffic zone

D．join the adjacent traffic lane as soon as possible

94. A vessel that is dead in the water on an even keel with most of her superstructure forward will lay ______．

A．With the wind abaft the beam or on the stern

B．With the wind on the beam

C．With the wind on the bow

D．With the wind dead ahead

船只已经死在水中平稳大部分上层建筑着她奠定______ 。

95. A vessel which through some exceptional circumstance is unable to maneuver as required by these Rules and is therefore unable to keep out of the way of another vessel is called ______．

A．a vessel restricted in her ability to maneuver

B．a vessel not under command

C．a vessel engaged in fishing

D．a vessel constrained by her draught．

船只通过一些特殊情况下无法操作所要求的这些规则，因此是无法阻挡的方式另一艘船只被称为______ 。

96. Admiralty Sailing Directions are Kept up to date by ______．

A．Corrections

B．Publications

C．Supplements

D．Alterations

航行指南不断更新的______ 。

97. Admiralty Sailing Directions are written with the assumption that the reader ______．

A．is familiar with all information contained in UKHO publications

B．knows nothing about any sailing directions

C．is lack of particular knowledge concerning the navigation of the area

D．has the appropriate chart before him and other relevant publications to hand

98. An ideal mooring system would be ______．

A．symmetrical and in equilibrium

B．asymmetrical and in flux

C．distorted and in equilibrium

D．concentric and in flux

99. Apparent wind speed blowing across a vessel under tow can be measured by a（n）______．

A．Barometer

B．Wind vane

C．Anemometer

D．Thermometer

100. As a ship moves through the water，it drags with it a body of water called the wake．The ratio of the wake speed to the ship's speed is called ______．

A．Propeller velocity

B．Speed of advance

C．Wake distribution

D．Wake fraction

101. As the propeller turns，voids are formed on the trailing and leading edges of the propeller blades causing a loss of propulsive efficiency，pitting of the blades，and vibration．These voids are known as ______．

A．Advance

B．Cavitation

C．Edging

D．Slip

由于螺旋桨旋转，空洞形成的落后和前缘的螺旋桨桨叶造成损失的推进效率，孔的刀片，并vibration.These空隙称为______ 。

102. At last，the palm trees that fringe the natural harbor of Port Aloahoa come into view on the horizon lit by the setting sun．The meaning of this sentence is that ______．

A．A vessel arrives at a port in a evening

B．A vessel leaves a port in a evening

C．A vessel arrives at a port in a morning

D．A vessel leaves a port in a morning

最后，棕榈树，附带的天然良港的港口Aloahoa到认为地平线上点燃了设置sun.The这句话的含义是_____

103. Besides saving distance along the track line，another advantage of the Scharnow Turn over the Williamson Turn in a man overboard situation because ______．

A．it is faster

B．it can be used in both the immediate action and the delayed action situations

C．in fog，if the turn is started as soon as the man goes over，the vessel will be at the point where he went over when the turn is completed

D．it returns the vessel to the original track line on a reciprocal course

除了节约距离沿轨道线的另一个优点是Scharnow交出威廉姆森转向一个人海里的局面，因为______ 。

104. Cable tension for catenary calculations is taken at the ______．

A．chain locker

B．fairlead

C．anchor

D．contact point of chain with seabed

105. CALDOVREP is ______．

A．a pilot station

B．a port

C．a mandatory ship reporting system

D．a vessel commences its voyage from a location beyond one limit of the inshore traffic zone

106. Catenary as applied to tow lines denotes the ______．

A．Dip of the line

B．Stretch of the line

C．Strain on the line

D．Length of the line

107. Every new supplement to the Sailing Directions ______ the previous one．

A．enforces

B．cancels

C．corrects

D．replenishes

每一个新的补充航行指南______前一个。

108. For safety reasons，when assistance towing ______．

A．the disabled vessel should be towed to the nearest port

B．the disabled vessel should be inspected for flammable gases

C．passengers should always be removed from the disabled vessel

D．personnel on the disabled vessel should don PFD's

109. For the deepest water when rounding a bend in a river，you should navigate your vessel ______．

A．Toward the inside of the bend

B．Toward the outside of the bend

C．Toward the center of the river just before the bend，then change course for the river's center after the bend

D．In the river's center

110. Good anchorage may be obtained anywhere in this water area，the bottom being generally mud and sand，vessel may have ______．

A．good getting ground

B．good holding ground

C．good grabbing ground

D．good handling ground

111. How does the effect known as bank suction act on a single-screw vessel proceeding along a narrow channel _______．

A．It pulls the bow toward the bank

B．It pushes the entire vessel away from the bank

C．It pulls the stern toward the bank

D．It heels the vessel toward the bank

如何影响被称为银行吸力行为的单螺杆船只沿着狭窄的通道________ 。

112. If a mooring line should part while you are tying up at a dock，you should make a temporary eye by tying a ______．

A．becket bend

B．clove hitch

C．bowline

D．square knot

113. If a vessel under tow starts jumping on its tow line，the most appropriate action to alleviate the condition is to ______．

A．Change course

B．Slow down

C．Heave to

D．Adjust tow line length

114. In Admiralty Sailing Directions，bands is the word used to indicate ______ marking．

A．horizontal

B．vertical

C．diagonal

D．vertical or diagonal

115. In Admiralty Sailing Directions，STRIPES is the word used to indicate ______ marking．

A．horizontal

B．vertical

C．diagonal

D．vertical or diagonal

116. In securing a towing cable，consideration must be given to letting go in an emergency．The possible whip of towlines when released can be overcome by ______．

A．increasing the shaft RPM prior to release

B．using a pelican hook for quick release

C．using preventers

D．using a short chain for the lead through the stern chock

117. Navigation lights must be displayed in all weathers from sunset to sunrise．They also ______．

A．must be displayed when day signals are being used

B．must be displayed when moored to a pier

C．may be extinguished at night on open waters when no other vessels are in the area

D．may be displayed during daylight

航行灯必须显示全天候从日落到sunrise.They还______ 。

118. Place names used on board ships should be those ______．

A．specified by international authorities

B．specified by national authorities

C．on standard map

D．on the chart or sailing directions in use

119. Place names used should be those ______．

A．specified by international authorities

B．specified by national authorities

C．on standard map

D．on the chart or Sailing Directions in use

120. Sailing directions are kept up to date by ______．

A．Publications

B．Notices to mariners

C．Supplements

D．Annual Summary of Admiralty Notices to Mariners

帆船运动的方向是不断更新的__

121. Solid green arrows on the main body of a pilot chart indicate ______．

A．Prevailing wind directions

B．Prevailing ocean current directions

C．Probable surface current flow

D．Shortest great circle routes

坚实的绿色箭头的主体试点图表显示______ 。

122. The best method to secure a tow line to a cleat is to ______．

A．Take a turn around the cleat，then figure-eights，and a half-hitch

B．Make figure-eights，followed by a half-hitch，then a figure-eight knot

C．Take a turn，a half turn，and a figure-eight

D．Take several turns around the cleat only

123. The high risk of collision in Dover Straight is caused by ______．

A．the strong tidal streams in the strait

B．the density of vessel traffic on passage through the strait

C．the gales and poor visibility

D．the fact that it is shallow and narrow

124. The lateral distance between the original position where the vessel put the rudder one side and the farther lateral position where the vessel drifts away from the center of turning during the early stage of the turning defines ______．

A．Kick

B．Reach

C．Transfer

D．Advance

横向之间的距离原来的位置，船只的舵把一方和更远的横向位置，该船漂离中心把早期的转折点定义__

125. The numeral in the center of a wind rose circle on a pilot chart indicates the ______．

A．Total number of observations

B．Average wind force on the Beaufort scale

C．Average wind force in knots

D．Percentage of calms

数字的中心，风向圆圈试点图表显示的____

126. The NW limit of Dover Straight is ______．

A．the S and SE coast of England

B．a line across the English Channel joining Bognor Regis（50°46′N 0°41′E）to Cap d'Antifer（49°41'N 0°10'E）

C．the SE coast of the Netherlands

D．the River Thames

127. The Sailing Directions contain information on ______．

A．Required navigation lights

B．Lifesaving equipment standards

C．Casualty reporting procedures

D．Currents in various locations

航行指南包含的信息______ 。

128. The winds with the greatest effect on the set，drift，and depth of the equatorial currents are the ______．

A．doldrums

B．horse latitudes

C．trade winds

D．prevailing westerlies

129. There are two areas off the N coast of France，which remain dangerous because of ______．

A．the density of vessel traffic on passage through the strait

B．the fact that the passage through the strait is constricted by a series of narrow banks，running NE-SW

C．mines lied in the war of 1939-1945

D．the strong tidal streams in the strait

130. When hugging a bank in a narrow channel，you should take precautions against ______．

A．Bank suction，squat and the effects of vessels passing close aboard

B．Clogged sea chests，plugged sea strainers and overheated machinery

C．Striking underwater obstructions close to the bank

D．All of the above

131. When making up a tow connection，you should use ______．

A．safety hooks

B．plain eye hooks

C．round pin shackles

D．screw pin shackles

132. When steering by hand，which of the following may be a functional input to the steering gear as a result of turning the wheel ________．

A．Non-follow-up

B．Rudder adjustment

C．Weather adjustment

D．All the above may be activated

当转向另一方面，以下哪项可能是功能性的投入，舵机由于转动方向盘________ 。

133.Which method is used to detect rot in manila lines ________．

A．Feeling the surface of the line for broken fibers

B．Measuring the reduction in circumference of the line

C．Observing for the appearance of mildew on the outer surface

D．Opening the strands and examining the inner fibers

134. Which statement concerning the handling characteristics of a fully loaded vessel as compared with those of a light vessel is FALSE ________．

A．A fully loaded vessel will be slower to respond to the engines

B．A fully loaded vessel will maintain her headway further

C．A light vessel will be more affected by the wind

D．A light vessel loses more rudder effect in shallow water

135. While towing，bridle legs of unequal lengths may cause ______．

A．The bridle to foul

B．The shorter leg to fail

C．Chafing on the fairlead or bitts

D．A bent swivel

136. You are on a single-screw vessel with a right-handed propeller．The vessel is going full speed astern with full right rudder．The bow will swing ______．

A．Quickly to port，then more slowly to port

B．Probably to port

C．Slowly to port，then quickly to starboard

D．Probably to starboard

137. You are preparing to take another vessel in tow．Which signal indicates，Hawser is made fast ______．

A．Firing of a green star signal

B．Firing of a red star signal

C．An orange smoke signal

D．Three white star rockets at 1-minute intervals

138. You are pushing a tow ahead，at high speed，near the right hand bank of a canal．The forces affecting your towboat and tow will tend to ______．

A．Push both the head of the tow and the stern of the towboat away from the right hand bank

B．Push the head of the tow away from，and pull the stern of the towboat

into，the right hand bank

C．Pull both the head of the tow and the stern of the towboat into the right hand bank

D．Pull the head of the tow into，and push the stern of the towboat away

from，the right hand bank

139. You may BEST turn a twin-screw vessel about，to the right，in a narrow channel by using ______．

A．Both engines ahead and helm

B．One engine only

C．Port engine ahead and the starboard engine astern

D．Both engines astern and use helm

140. Your vessel goes aground in soft mud．You would have the best chance of refloating it on the next tide if it grounded at ______．

A．low water neap

B．low water spring

C．high water neap

D．high water spring

第二章第二节进港指南

141. An anchored vessel on pilotage duty must show which light（s）at night ________．

A．A sternlight only

B．Anchor lights only

C．A white light over a red light only

D．A white light over a red light and anchor lights

一个停泊船只引航责任必须显示灯（ s ）在夜间________ 。

142. An embarked Pilot ______．

A．is a specialist hired for his local navigational knowledge

B．is solely responsible for the safe navigation of the vessel

C．relieves the master of his duties

D．relieves the officer of the watch

143.Attention is called to the fact that in many cases pipelines are not ______ and charted depths may be decreased by up to 2 meters．

A．concealed

B．buried

C．existed

D．built

请注意这样一个事实，即在许多情况下，管道不会______和绘制深度可减少至2米。

144. Before a Master relieves a Pilot of the conn，the ______．

A．Master should foresee any danger to the vessel on the present course

B．Vessel must be in extremis

C．Master should agree to sign a release of liability form

D．Master must first request the Pilot to take corrective action

145. Charted depth is the ______．

A．Vertical distance from the chart sounding datum to the ocean bottom，plus the height of tide

B．Vertical distance from the chart sounding datum to the ocean bottom

C．Average height of water over a specified period of time

D．Average height of all low waters at a place

146. Control of fire should be addressed ______．

A．immediately after restoring vital services

B．immediately

C．following control of flooding

D．following establishment of fire boundaries

147. Depths of ______ are to be inserted in the chart．

A．Work

B．Wreck

C．Wake

D．Weak

148. During an abandonment or drill，the first person to arrive at the survival craft should ______．

A．pass out food and water to personnel

B．open the doors and start the sprinkler system

C．activate the emergency release handle

D．open the doors and prepare the craft for boarding

149.During the towing of a survival craft，a lookout should be on station to ______．

A．release the towline in an emergency

B．help the helmsman steer

C．look for food and water

D．check the water level in the bilge

150. Each hand portable，semi-portable and fixed fire extinguishing unit on a ship must be tested and inspected at least once every ______．

A．six weeks

B．six months

C．twelve months

D．two years

每手便携式，半便携式和固定式灭火装置的船舶，必须进行测试和检查至少每______ 。

151. Each life jacket light that has a non-replaceable power source must be replaced ______．

A．every 6 months after initial installation

B．every 12 months after initial installation

C．every 24 months after initial installation

D．on or before the expiration date of the power source

每个救生衣灯已非更换电源必须更换______ 。

152.Each life preserver must be readily accessible to the person for whom it is intended while he or she is _____．

A．at work

B．in his or her berthing area

C．BOTH at work and in his or her berthing area

D．None of the above are correct

每一个生命保护者必须易于向某人打算，而他或她是______ 。

153. Entering from sea，a daymark on the port side of the channel would be indicated on a chart by a ______．

A．Red triangle with the letter R

B．White triangle with the letters RG

C．Green square with the letter G

D．White square with the letters GR

154.Fire in an engine compartment is best extinguished with carbon dioxide gas（CO2）and by ______．

A．completely closing the compartment

B．closing the compartment except for the ventilators

C．leaving the compartment open to the air

D．increasing the air flow to the compartment by blowers

消防在一个引擎是最好的熄灭与二氧化碳气体（二氧化碳）和______ 。

155.Fire protection regulations for towing vessels require that drills be conducted on board the vessel as if there were an actual emergency．Drills include all of the following，EXCEPT ______．

A．testing all alarm and detection systems所有警报与火警探测系统

B．breaking out and using the vessel's emergency equipment使用船只的紧急设备

C．participation by selected crew members选定的机组成员

D．one person putting on protective clothing，if the vessel is so equipped

防火条例拖着渔船要求进行演练船上好像有一个实际的emergency.Drills包括以下所有，除非______ 。

156.Fires of which class would most likely occur in the engine room of a vessel ________．

A．Classes A and B

B．Classes B and C

C．Classes C and D

D．Classes A and D

火灾的阶级最有可能发生在引擎室的船只________ 。

157. For the purpose of training and drills，if reasonable and practicable，rescue boats on board must be launched with their assigned crew ______．

A．once a week

B．once a month

C．once a year

D．twice a year

为了培训和演练，如果合理和切实可行的，救援艇上必须启动其指定的船员______ 。

158. For use as protection from gas leaking from a refrigeration unit，each ship must be equipped with a ______．

A．Portable ventilation system便携式通风系统

B．Flame safety lamp

C．Self-contained breathing apparatus.Self式呼吸器

D．Gas mask

用作保护气体泄漏制冷单元，每一船舶必须具备______ 。

159. Frapping lines ______．

A．secure the lifeboat in the davits when in the stowed position

B．bring the lifeboat close alongside the rail in the embarkation position

C．give the occupants a safety line when the boat is being lowered from the embarkation level

D．reduce the swinging of the lifeboat at the embarkation level摆动救生艇在上船水平

160. Gangway ______ with pilot ladder on my starboard side．

A．Being rigged combined

B．Is rigged combining

C．Rigs combining

D．Is rigged combined

161. Hand holds or straps on the underside of an inflatable liferaft are provided ______．

A．to right the raft if it capsizes

B．to carry the raft around on deck

C．for crewmen to hang on to

D．to hang the raft for drying

162. Heavy fuel oils when spilled are ______．

A．more harmful to sea life than lighter oils

B．easier to clean up than lighter refined oils

C．less harmful to sea life than lighter oils

D．not a real threat to marine life

重燃料油泄漏时是______ 。

163.How does foam extinguish an oil fire ________．

A．By cooling the oil below the ignition temperature

B．By removing the fuel source from the fire

C．By excluding the oxygen from the fire

D．By increasing the weight of the oil

如何石油泡沫灭火消防________ 。

164. How many adult life jackets are required on board a ship ________．

A．Enough for 100 percent of the persons allowed on board足以让百分之百的人允许在船上

B．One for each work station and industrial work site每个工作站和工业工作网站

C．Enough for 150 percent of the persons allowed on board

D．Both A & B above

有多少成人救生衣须在船上________

165. If help has not arrived in 10-12 hours after you abandon a vessel in a survival craft，you should ______．

A．go in one direction until the fuel runs out

B．plot a course for the nearest land

C．take a vote on which direction you should go

D．shut down the engine（s）and set the sea anchor

如果帮助还没有到达10月12日小时后你放弃船只救生艇筏，您应该______ 。

166. If passengers are on board when an abandon ship drill is carried out，they should ______．

A．take part

B．watch

C．go to their quarters

D．stay out of the way and do what they want

如果乘客在船上时弃船演习进行的，他们应该______ 。

167. If the survival craft is not loaded to full capacity，the personnel should be ______．

A．loaded more on the port side forward

B．loaded equally on both sides with more forward

C．loaded equally on both sides with more aft

D．allowed to sit anywhere

如救生艇筏不加载全部产能，人员应当______ 。

168. If you have to abandon ship，and enter a liferaft，your main course of action should be to ______．

A．Remain in the vicinity of the sinking vessel

B．Head for the closest land

C．Head for the closest sea-lanes

D．Get a majority opinion

如果您必须弃船，并进入一个救生筏，您的主要行动方针应是______

169. If you have to jump in the water when abandoning a vessel，your legs should be ______．

A．spread apart as far as possible

B．held as tightly against your chest as possible

C．in a kneeling position

D．extended straight down and crossed at the ankles

如果您有跳在水中时，放弃船只，您的腿应该______ 。

170. If you must jump from a ship，your posture should include ______．

A．holding down the life preserver against the chest with one arm crossing

the other，covering the mouth and nose with a hand，and feet together holding的生命保存下来的胸部单臂过境另一方面，包括嘴和鼻子的手，站起来

B．knees bent and held close to the body with both arms around legs

C．body straight and arms held tightly at the sides for feet first entry into the water

D．both hands holding the life preserver below the chin，with knees bent，

and legs crossed

如果您必须跳转从船舶，您的姿态应包括______ 。

171. In Bahrain，Mina Sulman Pilots board at ______ NAVAREA I of the world-wide navigation warning service．

A．Sitra Beacon

B．ASRY Beacon

C．Alba Beacon

D．Bapco Beacon

在巴林，米纳萨勒曼阿飞行员局______ NAVAREA一世界各地航行预警服务。

172.It's all right to rig the pilot ladder on lee side ______．

A．To allow port Authorities to board

B．Allowing port Authorities to board

C．To allow port Authorities boarding

D．Allowing port Authorities to be boarded

它的所有权利钻机上的领港员梯李方______ 。

173. Passage through the Suez Canal is operated on a ______ system．

A．Convey

B．Convoy

C．Escort

D．Conform

通过苏伊士运河的运作上______系统。

174. Pilot ladder is ______ on port side．

A．Equipped

B．Rigged

C．Put on

D．Put off

领港员梯是______的左舷。

175. Temperature and moisture characteristics are modified in a warm or cold air mass due to ______．

A．pressure changes in the air mass

B．movement of the air mass

C．the heterogeneous nature of the air mass

D．upper level atmospheric changes

温度和湿度特性修改热烈或冷气团由于______ 。

176. The characteristic of a lighted cardinal mark may be ______．

A．very quick flashing

B．flashing

C．fixed

D．occulting

177. The Master of a vessel berthed，moored or anchored within the area of jurisdiction and who wishes to move his vessel to another berth，mooring or anchorage within that area ______ from‘Bahrain Port Control’prior to moving．

A．shall not request permission

B．shall request permission

C．shall，if in convenience，request permission

D．shall send his permission to the Harbour Master

178.The Master of the inward bound vessels shall on coming within VHF radio range of Bahrain Port Control give the vessel's ETA at ______．

A．Alba

B．Bapco

C．ASRY

D．Sitra Beacon

船长的船只抵港的约束范围内，须于未来甚高频无线电一系列巴林港口管制使船只的埃塔在______

179. The pilot motor launches are ______ blue，with the PILOT in white．

A．furnished

B．painted

C．prepared

D．provided

试点机动发射是______蓝色，与白色的试点。

180.The pilot said he ______ our vessel directly to the berth．

A．Will take

B．Took

C．Is taking

D．Would take

181. The pilot station is ______ W of the breakwater．

A．To

B．Near

C．Distant

D．On

182. There shall be two principal ______ through the harbor for the passage of vessels of over 60 tons burden．

A．straits

B．channels

C．canals

D．roadsteads

183. When a tug is pulling on a hawser at right angles to the ship，and the pilot wants to come ahead or astern on the ship's engine，care must be taken that the pilot ______．

A．Does not break the towline

B．Does not get too much way on the vessel

C．Keeps a steady course so the towline will remain tight

D．Turns the ship toward the direction of pull

184. When the movement of engine is no longer required the pilot usually gives the order like this ______．

A．Stop engine

B．Nothing to port

C．Finish engines

D．Finished with engines

185. Which vessel may show three lights in a vertical line，the top and bottom being red and the middle being white ________．

A．A vessel engaged in diving operations

B．A pilot vessel

C．A vessel trawling

D．All of the above

这些船只可能会显示三个灯在一条垂直线，顶部和底部的红色和白色的中间被___

186. You are on a large merchant vessel entering a port．There is a Pilot on board and he has the conn．Which statement is TRUE ________．

A．The Master is responsible for the safe navigation of the ship and the Pilot is employed for his local knowledge

B．The Pilot is solely responsible for the safe maneuvering of the ship only if he is required to be on board by law

C．The Pilot is solely responsible for the internal working of the ship

D．The Pilot becomes solely responsible for the safe navigation of the vessel only if the Master relinquishes the conn

187. You are stopped with no way upon your vessel at the pilot station．Your vessel is a large twin-screw ship．You must come around 180° to board your Pilot．How should you use the engines and rudder to turn the ship fastest in the least amount of space ________．

A．Full ahead on the engines and hard over rudder

B．Full ahead on one engine，full astern on the other

C．Half ahead with hard over rudder，then full astern on inboard engine

D．Slow ahead with hard over rudder

你是没有办法阻止对你的船只在试点station.Your船只是一家大型双螺杆ship.You必须在180 ° ，以局的Pilot.How应该使用的发动机和舵把最快的船最少的空间________ 。

第三章 航行日志填写（31题）

188. Which______ need NOT be entered in the Official Logbook?

A．The testing of the internal combustion engine driven emergency generators at least once each month the vessel is navigated测试内燃机驱动的应急发电机，至少每月一次的船只航行

B．The testing of storage batteries for emergency lighting and power systems once each 6 months the vessel is navigated测试蓄电池的应急照明系统和电力系统一次6个月的船只航行

C．The actual draft when the vessel arrives in salt water after departing a port

D．The testing of the line-throwing appliance once every 3 months

189. A seaman assaults the Second Mate and injures him with a beer bottle while the ship is at sea．The incident is logged in the Official Logbook．In subsequent suspension and revocation proceedings against the seaman，according to the regulations，______．

A．the Second Mate and the Master must testify as to the facts of the assault

B．the case will be dismissed if the logbook entries are improperly made

C．the logbook entry is prima facie evidence of the facts if it complies with the law日志条目的初步证据的事实是否符合法律

D．the logbook is inadmissible if the logbook entries do not conform to the law

一个海员的第二次攻击伤害马特格和他的啤酒瓶，而船舶在sea.The事件记录在随后的正式Logbook.In暂时吊销和撤销诉讼海员，根据该条例,______.

190.A seaman leaves a vessel before it sails from a foreign port．He informs the Chief Officer that he won't return．After the vessel sails，the Chief Officer finds the seaman's work clothes in his locker．How should the Master handle this matter ________．

A．Log the seaman as a deserter

B．Log the seaman as a fail to join

C．Log the seaman for misconduct

D．Take no action

191. A wise Captain gives clear direction in his ______ for the officers on watch to call him if in any doubt whatsoever

A．Deck Logbook

B．Master's Note

C．Bell Book

D．Master's Standing Orders

聪明的船长提供了明确的方向，他______的人员要求看他是否在任何疑问

192. All events relating to the voyage，such as ship's position，speed and details of the weather，are recorded in ______．

A．Logbook

B．Bell Book

C．Oil Record Book

D．Compass Error Book

所有有关的事件的航程，如船舶的位置，速度和细节的天气，都记录在______ 。

193. All of the following records are usually maintained by the watch-standing officers aboard a vessel EXCEPT the ______．

A．deck logbook

B．official logbook

C．compass record book

D．chronometer error book

以下所有记录通常保持的手表常设人员乘坐的船只除外______ 。

194.As per the standing orders，you should carry out all the following operations except ______ upon having received words that a person has fallen overboard．

A．call the Master immediately

B．stop the engines

C．sound the “Man Overboard” signal of 3 prolonged blasts on the whistle and 3 prolonged blasts on the general alarm

D．prepare，as many as possible，rescue facilities，including warm clothes尽多救援设施包括御寒的衣服

按常规，你应该执行所有以下操作时，除非收到了______的话，一个人已经下降海里。

195. Entrance Light Vessel beamed port，stopped eng．& Tug Ever took pilot off．From this remark，the ship is likely ______．

A．making inward operation

B．making outward operation

C．making cargo operation

D．at anchor

196. Every entry required to be made in the Official Logbook shall be signed by the ______．

A．Mate on watch

B．Master and Chief Mate or other member of the crew

C．Master only

D．Purser，one of the Mates，and some other member of the crew

197. Give all traffic a good safe berth obeying the Int．Rules．This wording is likely to appear in ______．

A．Decklogs

B．Night Orders

C．Wheel Orders

D．B/L

198. If you carry packaged hazardous cargoes on a break bulk vessel bound foreign，you must ______．

A．stow the hazardous cargoes on deck available for jettisoning if necessary

B．remove the hazardous cargo labels from a portable tank after the tank is emptied

C．have the shipping papers indicate the proper shipping name and the technical name of n.o.s．cargoes

D．log the receipt of hazardous cargoes in the Official Logbook

如果您携带包装危险货物的散装船打破约束外国，您必须______ 。

199. In writing up the log book at the end of your watch，you make an error．Which of the following is the way to correct the error ________．

A．Carefully and neatly erase the entry and rewrite it correctly

B．Remove this page of the log book and rewrite all entries on a clean page

C．Blot out the error completely and rewrite the entry correctly

D．Cross out the error with a single line and rewrite the entry correctly

200. In writing up the logbook at the end of your watch，you make an error in writing an entry．What is the proper means of correcting this error ________．

A．Cross out the error with a single line，and write the correct entry，then initial it

B．Carefully and neatly erase the entry and rewrite it correctly

C．Remove this page of the log book，and rewrite all entries on a clean page

D．Blot out the error completely and rewrite the entry correctly

以书面的日志结束时，您看，您做出了错误的书面entry.What是一个适当的手段，纠正这种错误________ 。

201. Instructions for training of new seamen are usually found in ______．

A．Decklogs

B．Night Orders

C．Standing orders

D．Muster List

202. Never make comment，estimate or guess in your Logbook，but ______ only．

A．the specifications

B．the facts

C．the descriptions

D．the details

从来没有发表评论，估计或猜测在您的日志，但只有______

203. Tested & inspected the steering systems & other navigational equipments & found them in good condition．L'dg & unl'dg finished．This remark is likely made by the OOW ______．

A．at the beginning of his watch

B．during his watch

C．at the end of his watch

D．in his cargo watch on deck

测试及检查转向系统与其他导航设备及发现他们在良好的condition.L '危险品及unl'dg finished.This的话很可能作出的OOW ______ 。

204. That REMOVE ANY LIST ON THE V/L AFTER EACH WATCH END means that ______．

A．any list，no matter where it is posted，shall be removed prior to ending his watch by the OOW

B．the vessel should not be removed

C．any and all lists on board the vessel should be taken off

D．the inclination of the vessel should be corrected prior to ending his watch by the OOW

该名单上删除任何的V / L的每次值班完意味着______ 。

205. The entries in the Official Logbook must be signed by the Master and ______．

A．the Chief Engineer

B．the person about whom the entry concerns

C．one other crew member

D．no other signature is required

中的条目正式签署日志必须由船长和______ 。

206. The Master or person in charge of a ship is required to log ______．

A．the names of all persons on board

B．only the names of the crew members on board

C．only the names of passengers on board

D．information on emergency training drills    应急培训演练

207. Took tug Satex on St'd bow，using eng．and tug var'ly at Capt．and Plt orders．This wording is likely to appear in ______．

A．Deck logs

B．Night Orders

C．Captain's Standing Orders

D．B/L

208. What is NOT accepted as the required shipping papers on a manned and loaded tank barge ________．

A．Bill of Lading

B．Manifest

C．Shipping document

D．Logbook entry

什么是不能接受为所需要的航运文件载人和载箱驳船________ 。

209. What is required to be entered into the Official Logbook ________．

A．Opening a sideport at sea to renew a gasket

B．The annual required stripping and cleaning of the lifeboats

C．The biennial weight test of the lifeboats and falls

D．The drafts on entering port

210. When a vessel is entering or leaving a port，a record of engine speeds is kept in the ______．

A．Bell book

B．Deck rough log

C．Official Logbook

D．Engine rough log

当船只进入或离开港口，记录发动机转速保持在______ 。

211. When must the Master of a vessel log the position of load line marks in relation to the surface of the water in the Official Logbook ________．

A．Once a day

B．At the change of every watch

C．Only when in fresh or brackish water

D．Prior to getting underway

当必须船只的船长日志的立场，载重线标志与水面的官方日志________ 。

212. When recording the wind direction in the weather log，you would report the ______．

A．direction the wind is blowing toward

B．direction the wind is blowing from

C．duration of the maximum gust of wind

D．wind chill factor

当录制风向的天气记录，你会报告______ 。

213. Where must the master or person in charge of a ship record the date of each test of emergency lighting and power systems and the condition and performance of the equipment ________．

A．On the Certificate of Inspection

B．On the station bill

C．In the Operations Manual

D．In either the official or unofficial log

凡必须的船长或负责人对船舶的日期记录每个测试的紧急照明和电力系统的条件和性能的设备________

214. Where must you record the date of each emergency training drill conducted on a ship ________．

A．In the logbook

B．In the Operations Manual

C．On the Certificate of Inspection

D．On the muster list

如果你必须记录每一个日期进行了紧急培训演习的船舶________ 。

215.Which is NOT a required entry in the ship's Official Logbook ________．

A．Sale of effects of a deceased crew member

B．Medical treatment of an injury

C．Inspections of cargo gear

D．Drydocking of the vessel

这不是需要进入船舶的正式日志________ 。

216. Which is supplied to the vessel by the MSA ________．

A．Bell book

B．Cargo gear register

C．Official Logbook

D．Rough Logbook

这是提供给该船只的生活津贴________ 。

217.Which log includes a statement of the conduct，ability，and character of each crew member on the completion of a voyage ________．

A．Official Logbook

B．Department Logbook

C．Crew Logbook

D．Smooth Logbook

日志包括一项声明的行为，能力和品格的每个船员的完成航程________ 。

218.Which statement pertaining to log keeping is TRUE ________．

A．Entries relating to pre-voyage，pre-departure and daily tests are required

B．Both A and C

C．All distress，urgent and safety communications must be logged

D．Routine daily MF-HF and INMARSAT-C transmissions do not have to be logged

该报表相关的日志保存为TRUE ________ 。

第四章  船舶安全管理（39题）

219. ______ has the ship?

A．How much hatches

B．How many hatch

C．How much hatch

D．How many hatches

220. ______ in this area is prohibited．______在这一领域，是禁止的。

A．Fish

B．Fishing

C．Fisher

D．Fishery

221. ______ is not a kind of package of goods．

A．Demijohn

B．Hogshead

C．Castwheel

D．Diarrhoea

222. ______ is not suitable for bagged cargo．______不适合袋装货物。

A．The hydraulic crab

B．The rope sling

C．The canvas sling

D．The cargo tray

223. ______ of arresting the ship is to secure her continued presence and to prevent her from slipping away．

A．Project

B．Subject

C．Object

D．Inject

______逮捕的船舶，以确保其继续存在，并阻止她滑了。

224. ______ the survivors known more of how to cope with cold water in the sinking of MV Titanic in 1912 countless lives ______．

A．Have/could have been saved

B．Have/could have saved

C．Had/could has saved

D．Had/could have been saved本来可以挽救

______已知的幸存者更如何应付冷水在泰坦尼克号沉没的MV在1912年无数人的生命______ 。

225. ______ using the derrick at once，the gooseneck is bent．

A．Continue

B．Start

C．Commence

D．Stop

226. “How do you read?”“I read you with signal strength one”______．

A．poor

B．bad

C．fair

D．good

“你怎么看？ ” “我看你的信号强度一"______.

227. ______ tankers over 100 meters long must not proceed beyond Dongbei Buoy．

A．Loading

B．Load

C．Lade

D．Laden

______油轮100多米长的绝不能跨越东北浮标。

228. ______ we have worked out the loading plan，we shall start doing more revision work．

A．For now

B．Ever since

C．Now that

D．By now

______我们已制定的装载计划，我们将开始做更多的修改工作。

229. A large angle of list can be created by which of the following ________．① Negative initial stability；② Off center weight．

A．① only

B．② only

C．Either ① or ②

D．Neither ① nor ②

大角度的清单可以建立由下列哪些________ 。 ①负初步稳定; ②体重偏离中心。

230.  A bad separation makes the cargo ______ at its destination．

A．Easier to discharge

B．More difficult to discharge

C．Easier to load

D．More difficult to load

231.A cargo that has a stowage factor over 40 CBF per M/T is known as a ______．

A．Hygroscopic cargo

B．Measurement cargo

C．Stowage cargo

D．Weight cargo

货物的积载因素有超过40脑血流量每平方米/吨被称为______ 。

232. A Class C EPIRB ______．

A．Should be used on board vessels operating not more than 200 miles offshore

B．Transmits an alerting signal on channel 16 followed by a homing signal on channel 15

C．Is automatically activated when it floats free of a sinking vessel

D．Transmits a swept tone signal on the aviation distress frequency

属C类应急无线电示位标______ 。

233. A message by flashing light consists of the call，the identity，the ending，and the ______．

A．reply

B．text

C．time

D．answer

234. A message giving warning of a hurricane should have which prefix when sent by radiotelephone ________．

A．Pan-Pan（3 times）

B．Securite Securite Securite

C．TTT TTT TTT

D．No special prefix

一个消息让预警的飓风应该有它的前缀时发出的无线电话________ 。

235. A person who observes an individual fall overboard from an OSV should ______．

A．immediately jump into the water to assist the individual

B．call for help and keep the individual in sight

C．run to the radio room to send an emergency message

D．go to the control room for the distress flares

236. A predictable result of a vessel nearing a bank or edge of a channel is that the ______．

A．stern is drawn to the bank as the bow sheers off

B．bow sheers toward the bank

C．vessel continues in a straight line，but with greatly reduced maneuverability

D．vessel will be drawn bodily into the bank unless the engines are stopped

237. A radiotelegraph message commencing with XXX has the same meaning as a message sent by the radiotelephone commencing with ______．

A．TTT

B．SOS

C．The spoken word URGENCY

D．The spoken word PAN

无线电讯息开始与三十具有相同的含义为发出的信息无线电话开始与______ 。

238. A vessel has a strong wind on the port beam．This has the same affect on stability as ______．

A．Weight that is off-center to starboard

B．Increasing the draft

C．Reducing the freeboard

D．Increasing the trim

船只曾强风的港口beam.This有相同的影响稳定的______ 。

239. A yellow signal，floating in the air from a parachute，about 100 meter above the water，indicates that a submarine ______．

A．will be coming to periscope depth

B．will be coming to the surface

C．is on the bottom in distress

D．is in distress and will try to surface

黄色信号，漂浮在空中的降落伞，大约100米以上的水，表明潜艇______ 。

240. According to the “Vessel Bridge-to-Bridge Radiotelephone Act” ，your radiotelephone log must contain _．

A．a record of all routine calls

B．a record of your transmissions only

C．the home address of the vessel's Master or owner

D．a summary of all distress calls and messages

根据“船舶大桥至大桥无线电话法” ，您的无线电话日志必须包含______ 。

241. Any person maintaining a listening watch on a bridge-to-bridge radiotelephone must be able to ______．

A．speak English

B．repair the unit

C．send Morse Code

D．speak a language the vessel's crew will understand

任何人保持听值班的桥梁，以桥无线电话必须能够______ 。

242. Argon is classified as a ______．

A．Corrosive腐蚀

B．Flammable gas易燃气体

C．Flammable liquid易燃液体

D．Nonflammable gas不燃气体

氩气被列为______ 。

243.Don't ______ things too seriously．

A．take

B．think

C．believe

D．consider

不要______的事情看得太重了。

244. I can not ______ you on my radar．

A．locate

B．look for

C．watch

D．look out

245. If you are transmitting a distress message by radiotelephone you should ______．

A．Use English language

B．Always use the International Code

C．Preface it by the word SOS

D．Follow the transmission with the radio alarm signal

如果您是转递无线电话遇险电文您应该______ 。

246. If your bridge-to-bridge radiotelephone ceases to operate，you must ______．

A．immediately anchor your vessel and arrange for repairs to the system

B．moor your vessel at the nearest dock available and arrange for repairs to the system

C．arrange for the repair of the system to be completed within 48 hours

D．exercise due diligence to restore the system at the earliest practicable time

如果您的桥梁，以桥无线电话停止运作，您必须______ 。

247. Messages concerning weather conditions transmitted by radiotelephone are preceded by ______．

A．MAYDAY

B．PAN-PAN

C．SECURITE

D．SOS

关于气候条件的邮件转发无线电话前面______ 。

248. PAN-PAN repeated three times over the radiotelephone indicates which type of message will follow _____．

A．Distress

B．Safety

C．All clear

D．Urgency

盼盼重复三次无线电话表明哪种类型的信息将遵循________ 。

249. Persons engaged in the transport of dangerous goods shall ______ training in the contents of dangerous goods requirements commensurate with their responsibilities．

A．have

B．make

C．take

D．receive

从事危险货物运输的应______培训内容的危险货物的要求与他们的责任。

250.The following ______ is a standard phrase．

A．May I enter fairway

B．Should I enter fairway

C．Could I enter fairway

D．Is it permitted to enter fairway

以下______是一个标准的短语。

251.The following ______ is a standard phrase．

A．You should anchor in anchorage B 3

B．Anchor in anchorage B 3

C．You could anchor in anchorage B 3

D．You could，I think，anchor in anchorage B 3

252. We will ______ special attention to the dangerous cargo．

A．Put

B．Make

C．Take

D．Pay

我们将______特别注意危险货物。

253. Which type of hull damage should be repaired FIRST ________．

A．Damage below the waterline

B．Damage to interior watertight boundaries

C．Damage in way of machinery rooms

D．Damage at or just above the waterline损伤或略高于水线

哪种类型的船体损坏应修复第一________ 。

254. Who should be notified prior to starting up a crane ________．

A．The pumpman

B．The stevedore foreman

C．The engine room

D．All the above

谁前，应先通知启动起重机________ 。

255. Would you mind ______ some dunnage materials for us ?

A．order

B．to order

C．to ordering

D．ordering

你介意______一些充气材料吗？

256. You have the “conn” at the time the helmsman who is steering by hand reports that the rudder is not responding to the wheel．Your FIRST action should be to ______．

A．shift the selector switch to the other steering pump

B．signal the engineroom to stop the engines

C．call the engineroom

D．call the Master

你有“康恩”当时谁的舵手是指导手报告说，舵没有回应的wheel.Your第一个行动应该是______ 。

257.  You may be able to avoid loss of suction in a pump by ______．

A．Using a full tank to keep the pump primed

B．Opening the vent cock on the pump

C．Closing down on the tank valve

D．All of the above

您可以以避免损失的吸力泵的______ 。

第七章 航行技术（第一节船舶操纵基本知识100题）

258. 258. ______ amplify charted detail and contain information needed for safe navigation．

A．sailing directions

B．list of lights

C．list of signals

D．notices to mariners

______放大绘制详细，包含所需的信息安全航行。

259. ______ are changes on charts due to changes in hydrography，topography，construction，removal or addition of dangers to navigation etc．

A．revised edition

B．new edition

C．small corrections

D．source data

______是更改图表由于变化水文，地形，建筑，拆除或增加危险，导航

260. ______ are clouds having their base between sea level and 2 km and 6 km height．

A．Low clouds

B．High clouds

C．Special low clouds

D．Medium clouds

______是云彩之间有自己的基地和海平面2公里和6公里的高度。

261. ______ are mainly caused by the gravitational pull of the moon．

A．tides

B．sea

C．swell

D．wind

_____是主要原因的引力月球。

262. ______ generally indicates the gale or storm is approaching．

A．Falling barometric pressure

B．Rising barometric pressure

C．The circulations of winds

D．The change of wind direction

______普遍表示烈风或暴风正在到来。

263.  ______ is a casing used for exhaust pipes from the engines．

A．the funnel

B．the messroom

C．the galley

D．the satellite antenna

______是套管用于排气管的发动机。

264. ______ is a high layer of cloud in the form of small flakes or cauliflower，white in colour with no dark              shadows  in between．

A．cirrostratus

B．cirrus

C．stratus

D．altostratus

______是一个高度层的云形式的小片或菜花，白颜色，没有阴影之间。

265. ______ is a low，even layer of dark grey clouds，with light and dark patches．

A．Cirrostratus

B．Cirrus

C．Stratus

D．Altostratus

______是一个低，甚至一层深灰色云彩，配有轻型和黑补丁。

266. ______ is a point midway between the forward and the after perpendicular．

A．Length overall

B．After perpendicular

C．Amidships

D．Extreme breadth

______是一个中间点的前瞻性和后垂直。

267. ______ is an indication that the weather is about to clear．

A．rising clouds

B．lowering clouds

C．thickening clouds

D．scattering clouds

268. ______ is formed when the droplets or ice crystals in clouds have become sufficiently large to fall to the ground．

A．Humidity

B．Temperature

C．Precipitation

D．Wind

______是何时形成的水滴或冰晶的云层变得足够大，对倒在地上。

269. ______ is normally responsible for maintaining records of corrections to navigational charts and nautical publications．

A．the chief officer

B．the second officer

C．the third officer

D．the PSC officer

______通常是负责维护记录更正航海图和航海出版物。

270. ______ is not suitable for bagged cargo．

A．The hydraulic crab

B．The rope sling

C．The canvas sling

D．The cargo tray

______不适合袋装货物。

271.______ is placed in the front of the ship under the water，which eases berthing or manoeuvering sideways at slow speed．

A．the bulbous bow

B．the anchor

C．the bow thruster

D．the propeller

______是放在前面的船下的水，从而简化停泊或操纵一边在速度缓慢。

272. ______ is the depth of the vessel at ship's side from the upper deck to the lowest point of the keel．

A．extreme draft

B．molded draft．

C．extreme depth

D．molded depth

______是深入船只在船舷从上层到最低点的龙骨。

273. ______ is the head of the deck department．

A．the chief officer

B．the second officer

C．the third officer

D．the master

______的首长是甲板部。

274. ______ is the length of ship measured from extreme points of the ship forward and aft．

A．length overall

B．after perpendicular

C．amidships

D．extreme breadth

______是船舶长度测量极值点的船舶推进和船尾。

275. ______ is the most visible signal of distress from a lifeboat at sea in the daytime．

A．Smoke signal

B．Flash signal

C．Fire signal

D．Radio emergency transmission

______是最明显的信号从一个遇险救生艇在海上白天。

276. ______ is used for distress and urgency traffic，and may be used by aircraft for safety purpose．

A．channel 6

B．channel 13

C．channel 16

D．channel 70

______用于遇险和紧迫性的流量，并可能使用的飞机安全的目的。

277. ______ is used for providing homing signals from survival craft for detection by 9 GHz radar．

A．NAVETEX receiver

B．search and rescue transponder

C．emergency position indicating radio beacon

D．digital selective calling

______用于提供寻信号救生艇筏检测了9千兆赫雷达。

278. ______ is used for safety of navigation ship-to-ship．

A．channel 6

B．channel 13

C．channel 16

D．channel 70

279. ______ is used to moor the ship to the seabed to keep it stationary．（C）

A．the bulbous bow

B．the anchor

C．the bow thruster

D．the propeller

280. ______ is where delicious food is cooked．(B)

A．the funnel

B．the messroom

C．the galley

D．the satellite antenna

281. ______ is where the crew eat their meals．(B)

A．the funnel

B．the messroom

C．the galley

D．the satellite antenna

282. ______ may be a warning for an approaching storm．(A)

A．the presence of cirrus clouds

B．the low-hanging stratus clouds

C．the altocumulus clouds

D．the massive cumulus clouds

283. ______ means rudder to be held in the fore and aft position．   (C)

A．steady

B．meet her

C．midships

D．stop engines

284. ______ of this deratting exemption certificate is hereby extended until 4th Oct．2008 as the vessel is laden with cargo．(A)

A．validity

B．valid

C．validation

D．the coming into force

______这一灭鼠豁免证明书现延至第四Oct.2008作为该船满载货物。

285. ______ provides a paper trace of course steered by the vessel against time．(C)

A．the echo sounder

B．the radar．

C．the course recorder

D．the direction finder

______提供了一份文件，追踪当然导向的船只与时间。

286. ______ provides strength and storage space for fuel，lubricating oil，fresh water，salt（ballast）water and potable water．(A)

A．The double bottom

B．The cofferdam

C．The navigating bridge

D．The superstructure

______提供力量和存储空间的燃料，润滑油，淡水，盐（压）的水和饮用水。

287. ______ refers to the amount of water vapour in the air．(A)

A．humidity

B．temperature

C．precipitation

D．wind

______是指水蒸汽量的空气中。

288.______ sends a radio signal from the bottom of the ship to the sea-bed，from which it is reflected．(C)

A．The marine sextant

B．The chronometer

C．The echo sounder

D．The radar

______发送无线电信号从底部的船舶，海床，从它的反映。

289. ______ will be broadcast every one hour ion VHF channel 6．(D)

A．VHF news

B．Channel rules

C．Visibility time

D．Navigational warnings

290. ______，as the chemical extinguisher agent，should be used for an electric fire．(D)

A．Dry chemical or foam

B．Foam or soda acid

C．Carbon dioxide on foam

D．Carbon dioxide or dry chemical

______,的化学灭火剂，应使用的电气火灾。

291. “Nimbus”means ______．(A)

A．rain

B．high

C．layer

D．cirrus

292.“the ship is exempted from compliance with the following provisions of the code”means ______．(B)

A．the ship must comply with the provisions

B．the ship need not comply with the provisions

C．the ship is not able to comply with the provisions

D．the ship mustn't comply with the provisions

“船舶免于遵守下列守则的规定”是指______ 。

293.A ______ is announced by“SECURITE-SECURITE-SECURITE-ALL    (C)

STATIONS/VSSELS-ALL STATIONS/VESSEL-ALL

STATIONS/VESSELS；THIS IS MV…CALL SIGN…（3X）'．

A．Distress message

B．urgency message

C．safety message

D．routine message

294. A bad separation makes the cargo ______ at its destination．   (B)

A．easier to discharge

B．more difficult to discharge

C．easier to load

D．more difficult to load

不良分离使货物______在其目的地。

295. A charter party is an agreement reached for ______．   (B)

A．the conveyance of goods

B．the hiring of a ship

C．the replenishment of fuels

D．the payment of freight

租船合同达成的协议是为______ 。

296. A common occurrence when a vessel is running into shallow water is that ______．(C)

A．The wake is less pronounced

B．The vessel is more responsive to the rudder

C．Squat will cause a decrease in bottom clearance and an increase in draft

D．All of the above

司空见惯的船只时，运行到浅水是______

297. A ship is said to drag if the anchor ______．   (D)

A．will hold her in position

B．will prevent the movement of the ship

C．will not move over the bottom

D．will not hold her in position

船舶拖如果说锚______ 。

298. A（n）______ indicates that there is serious and immediate danger for vessel，crew and passengers．(A)

A．distress alert

B．urgency message

C．safety message

D．routine message

299. According to the rules，a vessel's length is her ______．(C)

A．length between the perpendiculars

B．length along the waterline

C．length overall

D．registered length

根据规则，该船的长度是她______ 。

300. ADVISE YOU MAKE COURSE 036 means that your should ______．(C)

A．alter your course of 036

B．keep your present course of 036

C．change your course to 036

D．change your course by 036

301. AFTER DROPPING THE PILOT means ______．(B)

A．after boarding the pilot

B．after the pilot leaving the ship

C．after the pilot going on board

D．after taking the pilot

302. Altostratus belongs to ______．(B)

A．low clouds

B．high clouds

C．special low clouds

D．medium clouds

303. ANCHOR IS ACROSS means that ______．(C)

A．anchor is cross the bow

B．anchor is in the ground．

C．anchor is foul

D．anchor is dragging

304. Changing direction by bringing the stern of the vessel through the eye of the wind is known as ______．(A)

A．Jibing

B．Running before the wind

C．Tacking

D．Reefing

改变方向，使船尾船只通过眼风被称为______ 。

306. How does the effect known as“bank suction”act on a single-screw vessel proceeding along a narrow channel ________．(A)

A．It pulls the stern toward the bank

B．It heels the vessel toward the bank

C．It pushes the entire vessel away from the bank

D．It pulls the bow toward the bank

如何影响被称为“银行吸力”行为的单螺杆船只沿着狭窄的通道________ 。

307. If a seaplane is not used as a means of transpiration on water but is capable of being used so，it shall ______．(C)

A．be regarded as a vessel not under command

B．be regarded as a vessel underway

C．be regarded as a vessel

D．be regarded as a sailing vessel

如果水上飞机不被用作一种手段，蒸腾的水，但能够被用来做的，应当

308. If a towing vessel and her tow are severely restricted in their ability to change course，they may show lights in addition to their towing identification lights．These additional lights may be shown if the tow is ______．(D)

A．pushed ahead

B．towed alongside

C．towed astern

D．Any of the above

如果船只和她的拖车拖走被严格限制在其有能力改变当然，他们可能会显示灯除了他们拖着鉴定lights.These额外的灯可以显示，如果拖车是______ 。

309. If the towline parts，you should ______．   (C)

A．Start towing by pushing ahead

B．Abandon the towing vessel

C．Retrieve the tow bridle

D．Relieve strain on the retrieving line

如果拖缆部分，您应该______ 。

310. In towing，chocks are used to ______．   (A)

A．protect the towline from chafing

B．secure the end of the towline on the tug

C．stop off the towline while retrieving it

D．absorb shock loading on the towline

311. It is easier to dock a right-hand，single-screw vessel ______．(C)

A．Starboard side to the wharf

B．Either side to the wharf

C．Port side to the wharf

D．Stern to the wharf

很容易码头一右手，单螺杆船只______

312. One major advantage of the round turn maneuver in a man overboard situation is that it ______．(A)

A．Is the fastest method

B．Is easy for a single-screw vessel to perform

C．Requires the least shiphandling skills to perform

D．Can be used in reduced visibility

一个主要的优势又一轮军事演习中的一个人海里的情况是，它______ 。

313. One nautical mile is the length of an arc of great circle which subtends an angle of ______ at the centre of the earth．  (B)

A．1 degree

B．1 minute

C．1 second

D．1.5 degree

一海里的长度是一个弧形的大圈的subtends ______角的中心，地球。

314. Speed must be regulated ______ no damage is done to the wharf．(C)

A．In order to

B．So as

C．So that

D．Such that

速度必须管制______没有损害的是码头。

315. The “Mode” selector switch can be positioned and select all of the following EXCEPT ______．(A)

A．weather adjustments

B．hand-electric steering

C．automatic gyro

D．non-follow-up control

316. The biggest problem you generally encounter while towing a single tow astern is ______．(D)

A．The catenary dragging on the bottom

B．Swamping of the tow

C．The tow tending to dive

D．Yaw亚乌奥

最大的问题，而你一般遇到一个拖车拖走后退是______ 。

317. The catenary ______．(D)

A．acts as a reserve length of towing hawser when the tug applies more power，and it dampens the surge effect of the tow

B．gives an approximation of the amount of strain on the towing hawser

C．is the dip in the towing hawser between the tug and the tow

D．All of the above

悬链______ 。

318.The condition where a ship on the end of a tow line is riding a wave crest at the same time as its tug rides a wave crest is known as riding in ______．(D)

A．synchronism

B．harmony

C．check

D．step

条件凡船舶的结束纤绳是骑波峰的同时，其拖船骑波峰被称为骑在______ 。

319.The distance from the still water level（corrected for tides and storm surge）to the bottom of the hull of a ship is the ______．(A)

A．Air gap

B．Freeboard

C．Penetration

D．Water depth

距离仍然水位（更正为潮汐和风暴潮）的底部船体船舶是______ 。

320. The effect known as bank cushion acts in which of the following ways on a single-screw vessel proceeding along a narrow channel ________．(A)

A．It forces the bow away from the bank

B．It forces the stern away from the bank

C．It forces the entire vessel away from the bank

D．It heels the vessel toward the bank

效果称为银行垫行为，其中以下列方式对单螺杆船只沿着狭窄的通道________ 。

321. The horizontal fore-and-aft movement of a vessel is called ______．(D)

A．yaw

B．sway

C．heave

D．surge

横向前和纵向运动的船只被称为______ 。

322. The longitudinal distance between the original position where the vessel put the rudder one side and the position where the vessel turned 90 degrees from her original course defines ______．(D)

A．Kick

B．Reach

C．Transfer

D．Advance

纵向之间的距离原来的位置，船只的舵把一方的立场，并在该船转向90度从原来的过程定义______ 。

323. The major components which determine the length of catenary in a deployed anchor cable are water depth，cable tension，and ______．(C)

A．Environmental forces

B．Bottom conditions

C．Cable weight

D．Water density

其中的主要组成部分的长度确定在接触网部署锚索的水深，电缆紧张， ______ 。

324. The maneuvering information required to be posted in the wheelhouse must be based on certain conditions．Which of the following is NOT one of these conditions ________．(D)

A．The hull must be clean

B．There must be calm weather-wind 10 knots or less and a calm sea

C．There must be no current

D．The depth of the water must be at least one and one-half times the draft

机动所需的信息张贴在驾驶室必须基于一定的conditions.Which以下是不是这些条件________ 。

325. The term LEE SHORE refers to the ______．(A)

A．Shore on the lee side of the vessel

B．Shore that is in the lee

C．Western shore of the Lesser Channel

D．Shore in a harbor

326. The tow makeup that is designed to keep the catenary of the tow hawser to a minimum is called the ______．

A．Christmas tree tow

B．Tandem tow                         (A)

C．British tow

D．Tandem tug tow

拖车化妆，目的是使接触网的牵引hawser到最低限度称为______ 。

327. To obtain better steering control when you are towing alongside，your vessel should be positioned with its ______．(C)

A．Bow extending forward of the tow

B．Stern amidships of the tow

C．Stern extending aft of the tow

D．Bow even with the bow of the tow

获得更好的转向控制当您拖着同时，您的船只应保持其______ 。

328. Tripping defects in anchors frequently occur in ______．(D)

A．deep water

B．shallow water

C．stiff soils

D．soft soils

跳闸缺陷主播经常发生______ 。

329. Usually the most gentle way of riding out a severe storm on a larger vessel is ______．(C)

A．head on at slow speeds

B．hove to

C．running before the seas

D．to rig a sea anchor

通常是最温和的方式帮助纾解民困严重风暴较大船只______ 。

330.  Water may boil up around the stern of a vessel in a channel due to ______．(B)

A．slack water when upbound

B．shallow water

C．a cross current

D．a head current

水烧开了船尾附近船只渠道由于______ 。

331.What is most likely to happen when you push a multiple tow into a countercurrent ________．(C)

A．Going upstream you will make better speed with no danger involved

B．Going downstream you will be slowed down but will keep control of the tow

C．There is a good chance you will break up the tow

D．No danger exists as long as you steer a straight course through the eddy

什么是最可能发生当您把一个多拖成逆流________ 。

332. What is required to be posted in the pilothouse of a vessel ________．(D)

A．Certificate of Inspection

B．Officer's licenses

C．Muster List

D．Vessel's Maneuvering Characteristics

什么是必须张贴在操舵船只________ 。

333. What is the ______ of the turning circle in manoeuvring．(C)

A．distance

B．largeness

C．diameter

D．measurement

什么是______的转折点循环操纵。

334. What is the greatest danger of an overriding tow ________．(D)

A．Fouling of the towing hawser

B．Loss of steering

C．Tripping

D．Collision between the tow and the stern of the towing vessel

什么是最大的危险是压倒一切的拖车________ 。

335. What is the normal length of anchor cable used to anchor a vessel ________．(D)

A．An amount equal to the depth of the water

B．Two times the depth of water

C．Three to four times the depth of water

D．Five to seven times the depth of water

什么是正常的长度锚索用于锚船只________ 。

336. When a tug is in irons，she ______．(C)

A．Is made fast to the dock with engines secured

B．Is in dry dock

C．May be in danger of being overrun by her tow

D．Should pay out more towline

当拖船是铁杆，她______ 。

337. When a vessel is on autopilot steering，the“weather”control is adjusted to compensate for which severe weather effect on a vessel ________．(A)

A．Yaw

B．Roll

C．Pitch

D．Leeway

当该船的自动转向， “天气”的控制调整，以弥补这恶劣天气影响船只________ 。

338. When being towed，a fairlead is a ______．(A)

A．Fabricated shape used to change the direction of a flexible member of the tow hookup材料形状来改变方向的一种灵活的成员丝束联播

B．Fabricated shape used to secure the tow hookup to the towed vessel

C．Line connecting the fishplate to the bridle legs

D．Line connecting the tow bridle to the towed vessel

当被拖走，一个fairlead是______ 。

339. When comparing twin screw tug to single-screw tugs，which statement about a twin-screw tug is FALSE ________．(C)

A．If one engine fails，you do not lose control of the tow

B．It is more maneuverable

C．It develops more bollard pull for the same horsepower

D．It is generally subject to more propeller damage from debris in the water

在比较双螺杆拖船单螺杆拖船，这说明了双螺杆拖船是FALSE ________ 。

340. When hip towing，a line led from the stern of the towboat forward to the barge provides the towing pull when ______．(D)

A．going ahead

B．dead in the water

C．in a following current

D．backing

当髋关节牵引，一条线从斯特恩为首的拖轮着驳船提供拖车拉时______ 。

341. When making a Scharnow turn，the ______．(C)

A．Rudder must be put over towards the side the man went over

B．Initial turn direction is away from the side the man went over

C．Rudder is put hard over and the initial turn is maintained until about 240° from the original course

D．Man overboard must be not more than 300 feet astern when starting the turn

当作出Scharnow反过来， ______ 。

342. When maneuvering a heavy barge up a wide channel with a tug，the tow may be most closely controlled by making up to the barge ______．(D)

A．with a short tow astern

B．nearly bow to bow，at a small angle

C．amidships，parallel to the barge

D．nearly stern to stern，at a small angle to the barge

当沉重的机动驳船了广泛的渠道与一艘拖船，拖车可最严密控制，使到驳船______ 。

343. When towing astern，one way to reduce yawing of the tow is to ______．(A)

A．trim the tow by the stern

B．trim the tow by the head

C．have the tow on an even keel

D．list the tow on the side it is yawing

当拖着后退，一个途径，以减少yawing的拖车是______ 。

344. When turning a vessel in shallow water，which statement is TRUE ________．(C)

A．The rate of turn is increased

B．The rate of turn is decreased

C．The turning diameter increases

D．The turning diameter remains the same

当谈到船舶在浅水中，哪种说法正确________ 。

345. When you enter shallow water，you would expect your rudder response to ______．(A)

A．be sluggish and your speed to decrease

B．be sluggish and your speed to increase

C．improve and your speed to decrease

D．improve and your speed to increase

当您进入浅水，您希望您的方向舵响应______ 。

346. Where does the look-out man stand ________．(A)

A．In positions in which the noises in the ship are least likely to interfere with the hearing of a fog signal and the man can see clearly

B．In positions in which the noises in the ship are much likely to interfere with the hearing of a fog signal

C．In positions in which the hearing of a signal is reduced to a minimum

D．In a position in which the man can see clearly

347. Which may ignite fuel vapors ________．(D)

A．Static electricity

B．An open and running motor

C．Loose wiring

D．All of the above

这可能点燃燃料蒸汽________ 。

348. Which statement about the Navigation Rules is TRUE ________．(D)

A．The rules require vessels to comply with Traffic Separation Scheme regulations规则要求船只遵守分道通航制条例

B．The rules use the term safe speed规则使用的术语安全速度

C．The Rules permit a stand-on vessel to take action prior to being in extremis.在规则允许一个独立的船只采取行动之前，在极端情况

D．All of the above are correct

下列哪项关于航行规则是TRUE ________ 。

349. Which statement about the Williamson turn is FALSE ________．(A)

A．It requires the highest degree of shiphandling skills to accomplish这需要高度的技巧来完成船舶

B．It is the slowest of the methods used in turning the vessel

C．It is the best turn to use when the victim is not in sight due to reduced visibility

D．It returns the vessel to the original trackline on a reciprocal course

下列哪项关于威廉姆森又是FALSE ________ 。

350. Which towing method maintains the most control over the tow ________．(D)

A．Tandem towing

B．Honolulu towing

C．Tandem tug towing

D．Breasted tug towing

其中牵引方法保持最控制丝束________ 。

351.While towing，what is the principal danger in attempting to swing a barge on a hawser in order to slow the barge's speed ________．(B)

A．The barge may swing too quickly and run over the tug

B．The barge may pass under the hawser and capsize the tug驳船可置于hawser和倾覆拖船

C．Free surface affect of liquid inside the barge may rupture the barge bulkheads when turning too quickly

D．Dangerous wakes may result from the swinging barge and capsize the tug

虽然拖着，什么是主要危险在试图挥杆驳船上hawser以缓慢的驳船的速度________ 。

答：驳船可摆动太快和辗过拖船

352. You are on a single-screw vessel with a right-handed propeller，and you are making headway．When you enter shallow water，______．(C)

A．You will have better rudder response

B．Your speed will increase without a change in your throttle

C．Your rudder response will become sluggish

D．Your vessel will tend to ride higher

您是在单螺杆船只用右手螺旋桨，你正在headway.When输入浅水,______.

353. You are operating a twin-screw vessel and lose your port engine．You continue to operate on your starboard engine only．Which action would you take to move your vessel ahead in a straight line ________．(A)

A．Compensate with right rudder

B．Compensate with left rudder

C．Surge the starboard engine

D．Rudder amidships - no compensation is necessary on a twin-screw vessel

您正在经营双螺杆船只和港口失去您engine.You继续经营的右舷引擎only.Which行动将您将您的船只提前直线________ 。

354. You have a large，broken-down vessel in tow with a wire rope and anchor cable towline．Both vessels have made provision for slipping the tow in an emergency; however，unless there are special circumstances _____．(A)

A．the towing vessel should slip first

B．the vessel towed should slip first

C．they should slip simultaneously

D．either vessel may slip first

您有大量的，破旧船只拖了钢丝绳和锚索towline.Both船只已作出了规定滑拖车在紧急情况下;但是，除非有特殊情况______ 。

355. You have taken another vessel in tow．You can tell that the towing speed is too fast when the ______．(C)

A．Vessels are not in step

B．Tow line feels like it is jumping when touched

C．Catenary comes clear of the water

D．Towed vessel goes in irons

你采取另一艘船只在tow.You可以告诉说，拖曳速度太快时， ______ 。

356. Your ship is dead in the water with the rudder amidships．As the right-handed screw starts to turn ahead，the bow will tend to go ______．(B)

A．To starboard

B．To port

C．Straight ahead

D．As influenced by the tide and sea

您的船是在水中死亡的舵amidships.As的右手螺旋开始转向未来，船头会去______ 。

357.Your twin-screw vessel is moving ASTERN with rudders amidships．The starboard screw suddenly stops turning．Your vessel's head will ______．(A)

A．Go to port

B．Go to starboard

C．Remain stationary

D．Suddenly drop down

您的双螺杆船只正在倒车的方向舵amidships.The右舷螺丝突然停止turning.Your船的负责人将______ 。

358. Your vessel is broken down and rolling in heavy seas．You can reduce the possibility of capsizing by __．(B)

A．Constantly shifting the rudder

B．Rigging a sea anchor

C．Moving all personnel aft

D．Moving all personnel forward and low

您的船只细分和轧制重seas.You可以减少的可能性倾覆的______ 。

第七章第六节（海上避碰规则共124题）

359. A vessel is approaching from dead ahead．Both of her sidelights are visible and her range lights are in line．Which of the following could you do first ________．（B）

A．Sound one blast of the whistle

B．Alter course to starboard

C．Construct a radar plot

D．Sound the danger signal

该船接近死亡ahead.Both她侧记可见和她的各种灯光的line.Which下列可能你第一次________ 。

360. A vessel is considered to be restricted in her ability to maneuver under the Rules if she is ______．(B)

A．at anchor

B．mine-clearing

C．engaged in fishing

D．engaged in towing

该船被认为是限制她的能力演习根据规则，如果她是______ 。

362. A vessel is in sight of another vessel when ______．(B)

A．She can be observed by radar

B．She can be observed visually from the other vessel可以看到视力从其他船只

C．She can be plotted on radar well enough to determine her heading

D．Her fog signal can be heard

该船在看到另一艘船只时， ______ 。

363. A vessel may exhibit lights other than those prescribed by the Rules as long as the additional lights ___．(A)

A．do not interfere with the keeping of a proper look-out不会干扰保持适当瞭望

B．are not the color of either sidelight

C．have a lesser range than the prescribed lights

D．All of the above

船只可能会灯以外的其他规则所订明，只要额外灯______ 。

364. A vessel must proceed at a safe speed ______．(D)

A．in restricted visibility

B．in congested waters

C．during darkness

D．at all times

船只必须以安全速度______ 。

365. A vessel shall，so far as practicable，______ traffic lanes．(D)

A．avoid cross

B．avoid to cross

C．avoid crossly

D．avoid crossing

船只须在切实可行范围内尽量,______行车线。

366. A vessel showing a yellow light over a white light at night is a vessel ______．(B)

A．engaged in piloting

B．towing astern

C．engaged in fishing

D．in distress

一艘显示黄灯了白光夜间船只______ 。

367. A vessel sounding a fog signal of one short，one prolonged，and one short blast is indicating that the vessel is ______．(C)

A．fishing

B．in distress

C．at anchor

D．not under command

船只发出的一个信号雾短，一个长期的，一个是短期爆炸表明，该船是______ 。

368.  A vessel sounds one short blast．This signal indicates the vessel ______．(C)

A．intends to alter course to starboard

B．intends to pass starboard to starboard

C．is altering course to starboard

D．intends to pass port to port

一艘声音短期blast.This一个信号表明，船只______ 。

369. A vessel towing astern in an operation which severely restricts the towing vessel and her tow in their ability to change course shall，when making way，exhibit ______．(D)

A．the masthead lights for a towing vessel

B．the lights for a vessel restricted in its ability to maneuver

C．sidelights，sternlight and towing light

D．All of the above

一艘拖后退的一项行动中，这严重地限制了她的拖车拖走船只的能力，以改变方向时，须决策的方式，展示______ 。

370. A vessel will NOT show sidelights when ______．(C)

A．underway but not making way

B．making way，not under command

C．not under command，not making way

D．trolling underway

一艘不会显示侧记时______ 。

371. A vessel，which does not normally engage in towing operations，is towing a vessel in distress．She ____．(A)

A．need not show the lights for a vessel engaged in towing，if it is impractical to do so不会显示灯船只从事拖车，如果是这样做不切实际

B．may show the lights for a vessel not under command

C．must show a yellow light above the stern light

D．must show the lights for a vessel towing

船只，其中通常不从事拖带作业，是拖着一艘船只在distress.She ______ 。

372. A vessel's Classification Certificate is issued by the ______．(A)

A．American Bureau of Shipping美国航运局

B．National Cargo Bureau

C．United States Coast Guard

D．United States Customs

一艘分类凭证发行的______ 。

373.A yellow signal，floating in the air from a parachute，about 300 feet above the water，indicates that a submarine ______．(A)

A．will be coming to periscope depth

B．will be coming to the surface

C．is on the bottom in distress

D．is in distress and will try to surface

黄色信号，漂浮在空中的降落伞，大约300英尺以上的水，表明潜艇______ 。

374. According to the Rules，when should lights be displayed ________．(C)

A．During the hours of darkness

B．At all times when underway

C．From sunset to sunrise and restricted visibility

D．Sunrise to sunset

根据规则，应该在什么时候灯显示________ 。

375. Advise ______ your fishing gear．(B)

A．you recovery

B．you recover

C．you recovering

D．you recovered

376. All of the following are practices of good seamanship EXCEPT ______．(A)

A．When meeting，altering course to the left to increase sea room

B．Maintaining an alert radar watch in reduced visibility

C．Showing a flare-up light to attract attention when you are not under command

D．Maintaining a proper lookout from sunrise to sunset

以下所有的做法很好航海除非______ 。

377. Any alteration of course and/or speed to avoid collision shall，if the circumstances of the case admit，______ to another vessel observing visually or by radar．   (A)

A．Be large enough to be readily apparent足够大，是显而易见的

B．Be a succession of small alterations

C．Be with due regard to the power and speed of the vessel

D．Leave sufficient room for the other vessel to take action

当然，任何改动和/或速度以避免碰撞，应在有关个案的情况下承认,______到另一艘船只观测视觉或雷达。

378. Approaching an anchorage in fog，you hear one short，one prolonged，and one short blast in that sequence on a ship's whistle．This indicates ______．(C)

A．A vessel towed

B．A vessel not under command and unable to maneuver

C．A vessel anchored giving warning of her position

D．A vessel stopped dead in the water

接近停泊在雾，你听到一个短，一个长期的，和一个短的爆炸在该序列船舶whistle.This表明______ 。

379. As defined in the Rules，a towing light is a yellow light having the same characteristics as a（n）______．（D）

A．masthead light

B．all-round light

C．sidelight

D．sternlight

所界定的规则，一个拖着轻是黄灯具有相同特性的（ ñ )______.

380. At night，a vessel shall indicate that she is restricted in her ability to maneuver by showing in a vertical line two ______．（C）

A．red lights

B．red lights and two white lights

C．red lights with a white light in between

D．white lights with a red light in between

到了晚上，船只应表明，她被限制在她的能力，演习展示在一条垂直线，两个______ 。

381. By night，you sight the lights of a vessel engaged in underwater operations．If an obstruction exists on the port side of the vessel，it will be marked by ______．(B)

A．a floodlight

B．two red lights in a vertical line

C．a single red light

D．any visible lights

晚上，你看到的灯光船只从事水下障碍物存在operations.If在港口方面的船只，它的特点将是______ 。

382. Day-shapes must be displayed ______．(C)

A．Between sunset and sunrise

B．Only between 8 AM and 4 PM

C．During daylight hours in any visibility

D．During daylight hours in unrestricted visibility only

日间形状必须显示______ 。

383. During a period of “whiteout”，you should expect which of the following ________．(B)

A．Snowfall or blowing snow

B．Lack of ability to estimate distance

C．Harsh contrast between sun-illuminated snow cover and the background

D．Hazy horizons with extensive mirage effects

期间“ whiteout ” ，你应该期望下列哪些________ 。

384. During the day，a vessel picking up a submarine cable shall carry ______．(D)

A．Three shapes，the highest and lowest shall be red balls，and the middle shall be a white diamond

B．Two black balls

C．Three shapes；the highest and lowest shall be black balls，and the middle shall be a red diamond

D．Three shapes；the highest and lowest shall be black balls and the middle shall be a black diamond

在白天，船只拿起海底电缆应履行______ 。

385. During the day，a vessel with a tow over 200 meters in length will show ______．(B)

A．a black ball

B．a diamond shape

C．two cones，apexes together

D．one cone，apex upward

在这一天，一艘船的拖200多米长会显示______ 。

386. Entering from seaward，triangular-shaped daymarks are used to mark ______．(A)

A．the starboard side of the channel

B．the centerline of the channel

C．an obstruction where the preferred channel is to starboard

D．special purpose areas

进入从海，三角形daymarks是用来标记______ 。

387. Every vessel shall ______ maintain a proper look-out by sight and hearing as well as by all available means appropriate in prevailing circumstances and conditions so as to make a full appraisal of the situation and of the risk of collision．(D)

A．in fog

B．at night

C．in narrow fairway

D．at all times

每艘船应______保持适当瞭望了视力和听力以及用一切可用的手段适当的情况和条件，以便充分评估的情况和相撞的危险。

388. Every vessel should at all times proceed at a safe speed．Safe speed is

defined as that speed where ______．(B)

A．You can stop within your visibility range

B．You can take proper and effective action to avoid collision您可以采取适当和有效的行动，以避免碰撞

C．You are traveling slower than surrounding vessels

D．No wake comes from your vessel

每艘船必须在任何时候都以安全speed.Safe速度定义为速度在______ 。

389. Every vessel which is ______ to keep out of the way of another vessel shall，so far as possible，take early and substantial action to keep well clear．(C)

A．detected

B．decided

C．directed

D．deceased

每艘船是______保持了道路另一艘船只，应尽可能及早采取实质性的行动，以保持和明确的。

390. Every vessel which is directed by these Rules to keep out of the way of another vessel shall，if the circumstances of the case admit，avoid ______．(A)

A．Crossing ahead of the other

B．Crossing astern of the other

C．Passing port to port

D．Passing starboard to starboard

每艘船是由这些规则保持了道路的另一艘船只，应在有关个案的情况下承认，避免______ 。

391. Five or more short blasts on a vessel's whistle indicates that she is ______．(A)

A．In doubt that another vessel is taking sufficient action to avoid a collision在疑问，另一艘船正在采取足够的行动，以避免碰撞

B．Altering course to starboard

C．Altering course to port

D．The stand-on vessel and will maintain course and speed

五年或五年以上短期爆炸船只的汽笛表明，她是______ 。

392. If a vessel of special construction cannot fully comply with the Rules，her degree of compliance shall be determined by ______．(C)

A．IMO

B．the owners of the vessel

C．the government

D．the builder

如果船只的专项建设不能完全遵守规则，她的遵守程度应由______ 。

393. If it becomes necessary for a stand-on vessel to take action to avoid collision，she shall NOT，if possible，______．(C)

A．Decrease speed

B．Increase speed

C．Turn to port for a vessel on her own port side

D．Turn to starboard for a vessel on her own port side

如果有必要为一个独立的船只采取行动，以避免碰撞，她不得，如果可能的话,______.

394. If necessary to avoid collision or allow more time to ______ the situation，a vessel shall slacken her speed or take all way off by stopping or reversing her means of propulsion．(A)

A．access

B．process

C．profess

D．recess

如果有必要，以避免碰撞或允许更多的时间来______的情况下，她的船只不得放松速度或采取一切遥远停止或扭转她推进。

395.If there is any doubt as to the proper operation of a radar，which statement is TRUE ________．(C)

A．Only a radar expert can determine if the radar is operating

B．All radars have indicator lights and alarms to signal improper operation

C．A radar range compared to the actual range of a known object can be used to check the operation of the radar雷达范围相比，实际的各种已知的对象可以用来检查的运作雷达

D．The radar resolution detector must be energized to check the radar

如果有任何疑问，正常运作的雷达，哪种说法正确________ 。

396. If you are the stand-on vessel in a crossing situation，you may take action to avoid collision by your maneuver alone．When may this action be taken ________．(D)

A．At any time you feel it is appropriate

B．Only when you have reached extremis

C．When you determine that your present course will cross ahead of the other vessel

D．When it becomes apparent to you that the give-way vessel is not taking appropriate action当它显然你，给予单程船没有采取适当的行动

如果您是独立的船只过境情况下，您可能采取行动，以避免碰撞你的动作alone.When今年5月采取行动________ 。

397.If your vessel is the stand-on vessel in a crossing situation ______．（A)

A．You must keep your course and speed

B．You may change course and speed as the other vessel must keep clear

C．The other vessel must keep her course and speed

D．Both vessels must keep their course and speed

如果您的船只待命的船只过境情况______ 。

398. If your vessel is underway in fog and you hear one prolonged and three short blasts，this is a ______．（C）

A．vessel not under command

B．sailing vessel

C．vessel being towed（manned）

D．vessel being towed（unmanned）

399. In a crossing situation on open waters，a sailing vessel shall keep out of the way of all the following vessels EXCEPT a vessel ______．（C）

A．Not under command

B．Restricted in her ability to maneuver

C．Engaged in towing

D．Fishing

在过境点开放水域状况，一个帆船应保存的方式进行的所有下面的船只除外船只______ 。

400. In a crossing situation，a stand-on vessel which is forced to take action in order to avoid collision with a vessel on her own port side shall，if possible，avoid ______．（A）

A．turning to port

B．turning to starboard

C．decreasing speed

D．increasing speed

在过境情况下，一个独立的船只被迫采取行动，以避免船只碰撞她自己港口方面，应尽可能避免______ 。

401. In complying with the Rules，of what must the mariner take due regard ________．(D)

A．Limited backing power of his vessel

B．Radar information about nearby vessels

C．The occupation of the other vessel，if known

D．All of the above

在遵守规则，是必须的水手采取适当顾及________ 。

402. In dense fog a vessel without operational radar may not be justified ______ at all but should anchor if it is safe and practicable for her to do so．（D）

A．to be underway

B．on under way condition

C．to stop engine

D．in being under way

在浓雾的船只没有业务雷达可能是不值得的______所有，而应定位如果是安全和可行的，她这样做。

403.In determining a safe speed ______ shall not be among those taken into account．(C)

A．the presence of the background light at night

B．the state of wind，sea and current

C．the number of crew on board

D．the draught in relation to the available depth of water

在确定安全速度______不得在这些考虑进去。

404.In determining if risk of collision exists，if there is any doubt，such risk shall be deemed to ______．(A)

A．exist

B．exists

C．existing

D．to be existed

在确定是否相撞的危险存在，如果有任何疑问，这种风险应被视为______ 。

405. In determining safe speed，all of the following must be taken into account EXCEPT the ______．(A)

A．maximum horsepower of your vessel

B．presence of background lights at night

C．draft of your vessel

D．maneuverability of your vessel

在确定安全速度，以下所有必须考虑到除______ 。

406.In determining safe speed，the Rules list all of the following as factors which must be taken into account EXCEPT the ______．(C)

A．Limitations of radar equipment

B．Presence of background lights at night

C．Maximum horsepower of your vessel

D．Maneuverability of your vessel

在确定安全速度，规则列出所有下列因素必须考虑到除______ 。

407. In order for a stand-on vessel to take action in a situation，she must determine that the other vessel ____．(C)

A．Is restricted in her ability to maneuver

B．Has sounded the danger signal

C．Is not taking appropriate action

D．Has not changed course since risk of collision was determined

为了让一个独立的船只采取行动，在这种情况下，她必须确定的其他船只______ 。

408. In order for a vessel to be engaged in fishing she must be ______．(D)

A．underway

B．using gear which extends more than 50 meters outboard

C．using a seine of some type

D．using gear which restricts her maneuverability

为了让船只从事捕鱼，她必须______ 。

409. In restricted visibility a towed vessel must sound a fog signal when it is ______．(B)

A．the last vessel in the tow

B．the last vessel in the tow and it is carrying a crew

C．manned，regardless of its position in the tow

D．None of the above are correct

能见度拖曳船只必须健全雾信号时， ______ 。

410. In restricted visibility the speed of a vessel without operational radar may be ______ enable effective avoiding action to be taken on sighting another ship．(A)

A．too low to

B．too high to

C．too high for

D．too low for

在能见度低的速度，没有业务的船只可能是______雷达能够有效避免采取的行动对发现的另一船舶。

411. In the daytime，you see a large sailing vessel on the beam．You know that she is also propelled by machinery if she shows ______．(C)

A．A basket

B．A black ball

C．A black cone

D．Two black cones

在白天，你会看到一个大帆船的beam.You知道她也是推进的机制，如果她表明______ 。

412. SAFE SPEED is defined as that speed where ______．(B)

A．you can stop within your visibility range

B．you can take proper and effective action to avoid collision可以采取适当和有效的行动，以避免碰撞

C．you are traveling slower than surrounding vessels

D．no wake comes from your vessel

安全速度是指速度在______ 。

413. Sailing vessels are stand-on over power-driven vessels except ______．(C)

A．In a crossing situation

B．In a meeting situation

C．When they are the overtaking vessel

D．On the inland waters of the PR China

帆船是备用的电力驱动的船只除外______ 。

414. She（the vessel）shall if necessary  ______ and in any event navigate with extreme caution until danger of collision is over．(A)

A．take all her way off

B．take her way all off

C．take her way off

D．take off her all way

她（船）应在必要时______和在任何情况下浏览极为谨慎，直到碰撞的危险已经过去。

415.The duration of a prolonged blast of the whistle is ______．(B)

A．2 to 4 seconds

B．4 to 6 seconds

C．6 to 8 seconds

D．8 to 10 seconds

期限延长高炉的哨子是______ 。

416.  The effectiveness of the action shall be carefully checked until the other vessel is finally ______．(D)

A．Past

B．Clear

C．Past or clear

D．Past and clear

效力的行动，应仔细检查，直到其他船只终于______ 。

417. The effectiveness of the action shall be carefully checked until the other vessel is ______．(D)

A．finally clear and past

B．clear and past finally

C．past and clear finally

D．finally past and clear

效力的行动，应仔细检查，直到其他船只是______ 。

418. The look-out must be able ______ full attention to the ______ of a proper look-out．(D)

A．to give/keep

B．giving/keeping

C．giving/keep

D．to give/keeping

留意必须能够充分注意______的______的适当瞭望。

419. The Navigation Rules state that a vessel shall be operated at a safe speed at all times so that she can be stopped within ______．(C)

A．The distance of visibility

B．1/2 the distance of visibility

C．A distance appropriate to the existing circumstances and conditions

D．The distance that it would require for the propeller to go from full ahead to full astern

航行规则的国家的船只，应在安全操作的速度在任何时候，这样她可以停止在______ 。

420. The risk of collision shall be deemed to exist if the compass bearing of an approaching vessel ______．(B)

A．did not appreciably change

B．does not appreciably change

C．do not appreciably change

D．not appreciably change

相撞的危险应视为存在如果携带一个指南针接近船只______ 。

421. The rules require that a stand-on vessel SHALL take action to avoid collision when she determines that ______．(D)

A．risk of collision exists

B．the other vessel will cross ahead of her

C．the other vessel is not taking appropriate action

D．collision cannot be avoided by the give-way vessel's maneuver alone

该规则要求，一个独立的船只应采取行动，以避免碰撞时，她断定______ 。

422. The rules require which factor to be taken into account when determining safe speed ________．(C)

A．The construction of the vessel

B．The experience of the vessel's crew

C．The location of vessels detected by radar

D．All of the above

该规则要求的因素，必须考虑到在确定安全速度________ 。

423. The Rules state that a seaplane shall ______．(B)

A．not be regarded as a vessel

B．in general，keep well clear of all vessels

C．proceed at a slower speed than surrounding vessels

D．when making way，show the lights for a vessel not under command

规则指出，水上飞机应______ 。

424. The Rules state that certain factors are to be taken into account when determining safe speed．One of the factors is the ______．(D)

A．radio communications that are available

B．maximum speed of your vessel

C．temperature

D．current

该规则规定，某些因素是必须考虑到安全speed.One时确定的因素是______ 。

425.The Rules state that certain factors are to be taken into account when determining safe speed．Those factors include ______．(A)

A．state of wind，sea，and current，and the proximity of navigational hazards

B．maximum attainable speed of your vessel

C．temperature

D．aids to navigation that are available

该规则规定，某些因素是必须考虑到在确定安全speed.Those因素包括______ 。

426. The Rules state that risk of collision shall be deemed to exist ______．(B)

A．Whenever two vessels approach from opposite directions

B．If the bearing of an approaching vessel does not appreciably change.如果携带一个接近船只没有明显变化

C．Whenever a vessel crosses ahead of the intended track of another vessel

D．If one vessel approaches another so as to be overtaking

该规则规定，相撞的危险应视为存在______ 。

427. The term power-driven vessel means ______ in these Rules．(B)

A．Any sailing vessel with propelling machinery

B．Any vessel propelled by machinery

C．Any sailing vessel with or without machinery for propelling

D．Any sailing vessel with propelling machinery not in use

任期电力驱动的船只是指______在这些规则。

428. The term PROLONGED BLAST refers to ______．(C)

A．a blast of from five to seven seconds' duration

B．a blast of from five to six seconds' duration

C．a blast of from four to six seconds' duration

D．a blast of from six to seven seconds' duration

429. The term restricted visibility，when used in the Rules，refers to ______．(C)

A．Situations when you can see vessels on radar that you cannot see visually

B．Visibility of less than half a mile

C．Any condition where visibility is restricted

D．Visibility where you cannot see shore

任期有限能见度，当用在规则，是指______ 。

430. The two vessels underway may collide with ______，if they don't take immediate measures．(D)

A．One to another

B．One the other

C．Each the other

D．Each other

该两艘船只进行可能碰撞______,如果不立即采取措施。

431. The type of fog that occurs on clear nights with very light breezes and forms when the earth cools rapidly by radiation is known as ______．(A)

A．Radiation fog辐射雾

B．Frontal fog正面雾

C．Convection fog对流雾

D．Advection fog平流雾

雾的类型发生了明确的夜很轻的微风和形式时，地球辐射降温迅速被称为______ 。

432. Traffic separation schemes may be ______ by the Organization for the purpose of these Rules．(A)

A．adopted

B．adapted

C．adjusted

D．admitted

分道通航制可能是______由本组织的目的是这些规则。

433. Two barges are being pushed ahead by a tugboat．Which statement is TRUE concerning lights on the barges ________．(D)

A．Each vessel should show sidelights

B．Each vessel should show at least one white light

C．The barges should be lighted as separate units

D．The barges should be lighted as one vessel

两艘驳船正在推进的tugboat.Which说法正确关于灯光的驳船________ 。

434. Two short blasts of the whistle have all of the following meanings EXCEPT ______．(B)

A．I intend to meet you starboard to starboard

B．I do not intend to hold course and speed

C．I intend to overtake you on your port side

D．I intend to alter my course to port

短短的两个爆炸的哨子都下列含义除非______ 。

435. Two vessels are in an overtaking situation．Which of the lights on the overtaken vessel will the overtaking vessel see ________．(D)

A．Two masthead lights

B．One masthead light and a sidelight

C．Both sidelights

D．Stern light only

这两艘船是在超车situation.Which的灯就超过船只将在超车船只见________ 。

436. Two vessels meeting in a “head-on” situation are directed by the Rules to ______．(A)

A．alter course to starboard and pass port to port

B．alter course to port and pass starboard to starboard

C．decide on which side the passage will occur by matching whistle signals

D．slow to bare steerageway

两艘船只上的“正面”的局面是由规则，以______ 。

437. Under what condition are you allowed to depart from the rules of the road ________．(A)

A．To avoid immediate danger

B．When authorized by the rig superintendent

C．To comply with an operator's requirement

D．Under no conditions

在什么条件下你可以偏离道路规则________ 。

438.Underway at night，a vessel displaying the lights shown is ______．(D)

A．Engaged in fishing

B．Mine sweeping

C．A pilot boat

D．Under sail

在晚上进行，船只显示灯显示是______ 。

439. Vessel engaged in the launching or recovery of aircraft is called ______．(A)

A．A vessel restricted in her ability to maneuver

B．A vessel not under command

C．A vessel constrained by her draught

D．A vessel not under way

船只从事发射或回收的飞机被称为______ 。

440. Vessel underway means that a vessel is ______．(D)

A．On the way

B．At anchor

C．Made fast to a terminal

D．Moving against water

船只正在进行意味着该船______ 。

441. Vessels shall be deemed to be in sight of one another only when one ______ from the other．(A)

A．Can be observed visually

B．Can be observed by radar

C．Can be located on the radar

D．Can be heard

船只须当作是在看到一个只有当一个______其他。

442. Vessels should maintain a sharp lookout，especially during December through March，when navigating the right whale's only known calving grounds which lie off the coasts of ______．(C)

A．Nova Scotia

B．Maine and Massachusetts

C．Georgia and NE Florida

D．California and Mexico

船只应保持戒心，尤其是在12月至3月，当航行的露脊鲸的唯一已知的产仔理由在于海岸的______ 。

443. What determines if a vessel is restricted in her ability to maneuver ________．(C)

A．Whether or not all of the vessel's control equipment is in working order

B．The vessel's draft in relation to the available depth of water

C．Whether the nature of the vessel's work limits maneuverability required by the Rules性质是否船只的机动性工作限制规则所规定

D．Whether or not the vessel is the give-way vessel in a meeting situation

哪些因素决定，如果该船的限制，她的能力，机动________ 。

444. When a vessel is in any doubt as to whether such a situation exists she shall  ______ that it does exist and act accordingly．(B)

A．consume

B．assume

C．perfume

D．resume

当船只在任何疑问，是否存在这种情况，她应______ ，它确实存在，并据此采取行动。

445. When a vessel navigates in an area with a small underkeel clearance but with adequate space to take avoiding action she ______．(A)

A．Should not be regarded as a vessel constrained by her draught不应该被视为限制船只她吃水

B．Should be regarded as a vessel constrained by her draught

C．Shall be regarded as a vessel restricted in her ability to maneuver

D．Should be regarded as a non-displacement vessel

当某船只导航的一个地区小underkeel清除，但有足够的空间采取行动，避免她______ 。

446. When a vessel sees the other ahead or nearly ahead，by night she could see the masthead lights of the other in a line or nearly in a line or both side lights，______ shall be deemed to exist．(A)

A．head-on situation

B．end-situation

C．crossing situation

D．close-quarters situation

当看到其他船只提前或近未来，在夜间可以看到她的头灯光中的其他行或近线或在两侧灯,______应视为存在。

447. When in sight of another vessel，any action taken to avoid collision must ______．(B)

A．be accompanied by sound signals

B．not result in another close quarters situation

C．include a speed change

D．All of the above

当看到另一艘船只，采取的任何行动，以避免碰撞必须______ 。

448. When making landfall at night，the light from a powerful lighthouse may sometimes be seen before the lantern breaks the horizon．This light is called the ______．(C)

A．diffusion

B．backscatter

C．loom

D．elevation

登陆时，在晚上，光线从一个强大的灯塔有时会看到面前的花灯打破了horizon.This轻称为______ 。

449.When one of two vessels ______ is required to keep out of the way the other must keep her course and speed．(D)

A．In sight of another

B．Sights the another

C．Not in sight of the other

D．In sight of one another

当一个人的两艘船只______是必须保持了其他的方式，必须让她当然和速度。

450. When one upbound vessel is overtaking another vessel and both are pushing a tow ahead，what reaction may you expect ________．(C)

A．Both towheads will tend to drift apart，and the overtaking vessel will be slowed down

B．Both towheads will tend to drift together，and the overtaking vessel will be slowed down

C．Both towheads will tend to drift apart，and the overtaken vessel will be slowed down

D．Both towheads will tend to drift together，and the overtaken vessel will be slowed down

当一个upbound船只超车另一艘船只，都推着拖车未来，您有何反应可能期望________ 。

451. When shall the stand-on vessel in a crossing situation take action to avoid the other vessel ________．(B)

A．When a risk of collision exists

B．When action by the give-way vessel alone will not prevent a collision当采取的行动给予单程船只不会单独防止碰撞

C．When the bearing to give-way vessel becomes steady

D．When the vessels become less than 1/2 mile apart

当应待命的船只过境局势采取行动，以避免其他船只________ 。

452. When ship at anchor she shall be deemed to be ______．(A)

A．Not under-way

B．Not under command

C．Restricted in her ability to maneuver

D．A non-displacement ship

当船舶停泊她应被视为______ 。

453. When taking action to avoid collision，you should ______．(A)

A．Make sure the action is taken in enough time请确认采取行动，有足够的时间

B．Not make any large course changes

C．Not make any large speed changes

D．All of the above

当采取行动，以避免碰撞，您应该______ 。

454. When two vessels are in sight of one another and NOT in or near an area of restricted visibility，any of the following signals may be given EXCEPT ______．(C)

A．a light signal of at least five short and rapid flashes

B．one prolonged，one short，one prolonged，and one short whistle blasts

C．four short whistle blasts

D．two short whistle blasts

当两艘船是在看到彼此，而不是在一个地区或附近的能见度，下列任何一种信号，可考虑除______ 。

455. When underway in a channel，you should if safe and practicable ______．(D)

A．stay near the middle of the channel

B．keep to the starboard side of any vessels you meet

C．exchange whistle signals with any other vessels in the channel

D．keep to the side of the channel which lies to your starboard

当正在进行一个渠道，你应该如果安全和切实可行的______ 。

456. When underway in a channel，you should keep to the ______．(B)

A．Middle of the channel

B．Starboard side of the channel

C．Port side of the channel

D．Side of the channel that has the widest turns

当正在进行一个渠道，你应该坚持______ 。

457. When visibility is restricted ______，we can say it is restricted visibility．(D)

A．by long distance

B．by the darkness at night

C．by a vessel in front

D．by fog or falling snow

当能见度限制______,我们可以说这是受限制的能见度。

458. When you doubt the existence of risk of collision，______．(A)

A．such risk shall be deemed to exist风险应被视为存在

B．such risk shall not be deemed to exist

C．you needn't take any action to avoid collision with any vessel

D．you should accelerate your speed ahead

当您怀疑存在碰撞危险,______. 。

468. Which vessel，when anchored at night，is NOT required to show anchor lights ________．(C)

A．A power-driven vessel

B．A vessel engaged on pilotage duty

C．A vessel dredging

D．A vessel restricted in her ability to maneuver

这些船只，停泊在晚上的时候，并不需要查看锚灯________ 。

469. You are aboard the give-way vessel in a crossing situation．What should you NOT do in obeying the Rules ________．(A)

A．Cross ahead of the stand-on vessel

B．Make a large course change to starboard

C．Slow your vessel

D．Back your vessel

您正在上给予单程船只过境situation.What您应该不会在服从规则________ 。

470.You are approaching another vessel and are not sure whether danger of collision exists．You must assume ______．(A)

A．there is a risk of collision

B．you are the give-way vessel

C．the other vessel is the give-way vessel

D．there is no risk of collision

您已接近另一艘船只，并不能确认自己是否危险碰撞exists.You必须承担______ 。

471. You are approaching another vessel and will pass starboard to starboard without danger if no course changes are made．You should ______．(B)

A．hold course and sound a two blast whistle signal

B．hold course and sound no whistle signal

C．change course to the right and sound one blast

D．hold course and sound two prolonged and two short blasts

您已接近另一艘船只，并会把右舷向右舷没有危险，如果没有当然变化是made.You应该______ 。

472. You are Master of a towing vessel engaged in towing three barges astern．The middle barge of the tow would be required to sound which of the following during restricted visibility ________．(A)

A．No fog signal

B．A prolonged blast at intervals never to exceed more than two minutes

C．A prolonged blast followed by two short blasts at intervals never to exceed more than two minutes

D．A prolonged blast followed by three short blasts at intervals never to exceed more than one minute

你是船长，船舶从事拖拖驳船astern.The三个中东的拖驳船将需要健全下列哪些在有限能见度________ 。

473. You are on a vessel that cannot comply with the spacing requirement for masthead lights．What is required in this situation ________．(B)

A．The vessel must carry only the lights that comply with the rules; the others may be omitted

B．The vessel's lights must comply as closely as possible，as determined by her government

C．The vessel must be altered to permit full compliance with the rules

D．An all-round light should be substituted for the after masthead light and the stern light

您正处于一个容器，可以不符合间距要求的磁lights.What需要在这种情况下________ 。

474. You are under sail and overtaking a tug and tow．Which action is correct ________．(B)

A．The power-driven tug must maneuver to avoid collision

B．You must maneuver to avoid the tug and tow您必须动作，以避免拖船和丝束

C．You must maneuver to avoid collision only if the tug is to leeward and the wind is on your port side

D．Both vessels are required to maneuver to avoid collision

您正在帆和超车一艘拖船和tow.Which行动是正确的________ 。

475. You are underway and approaching a bend in the channel where vessels approaching from the opposite direction cannot be seen．You should sound ______．(A)

A．one blast，4 to 6 seconds in duration

B．three blasts，4 to 6 seconds in duration

C．one continuous blast until you are able to see around the bend

D．one blast，8 to 10 seconds in duration

你正在和接近一个弯道的渠道驶近从相反的方向不能seen.You应当健全______ 。

476. You are underway in reduced visibility．You hear the fog signal of another vessel about 20° on your starboard bow．Risk of collision may exist．You should ______．(B)

A．alter course to starboard to pass around the other vessel

B．reduce your speed to bare steerageway

C．slow your engines and let the other vessel pass ahead of you

D．alter course to port to pass the other vessel on its portside

您正在减少visibility.You听到雾信号的另一艘船只约20 °的右舷bow.Risk碰撞可能exist.You应该_____。

477. You hear the fog signal of another vessel forward of your beam．Risk of collision may exist．You MUST ______．(A)

A．Reduce speed to bare steerageway

B．Stop your engines

C．Begin a radar plot

D．All of the above

你听到雾信号另一船只着您的beam.Risk的碰撞可能exist.You必须______ 。

478. You see a vessel carrying a black diamond which indicates a ______．(C)

A．vessel towing astern

B．barge pushed ahead

C．vessel towing astern only when the length of her tow exceeds 200 meters

D．this signal does not exist in international waters

您会看到一艘船手持一个黑色钻石这说明______ 。

479. You see a vessel displaying three lights in a vertical line．The highest and lowest lights are red and the middle light is white．She is also showing a white light at the stern，which is lower than the forward light．It could be a ______．(A)

A．Survey vessel

B．Vessel not under command

C．Vessel aground

D．Pilot vessel with port side to you

你可以看到船只上展示了三个灯在一个垂直的line.The最高和最低的灯是红色和中东光white.She也显示白光在船尾，低于远期light.It可能是______ 。

480. You see another vessel approaching，and its compass bearing does not significantly change．This would indicate that ______．(B)

A．You are the stand-on vessel

B．Risk of collision exists

C．A special circumstances situation exists

D．The other vessel is dead in the water

你可以看到另一艘船只接近，它的指南针同时不显着change.This表明______ 。

481.Your vessel is NOT making way，but is not in any way disabled．Another vessel is approaching you on your starboard beam．Which statement is TRUE ________．(A)

A．Your vessel is obligated to stay out of the way您的船只有义务远离方式

B．The other vessel must give way，since your vessel is stopped

C．You should be showing the lights or shapes for a vessel not under command

D．You should be showing the lights or shapes for a vessel restricted in her ability to maneuver

您的船只是没有办法，但不以任何方式disabled.Another船只接近你右舷beam.Which说法正确________ 。

482. Your vessel is not underway when ______．(D)

A．Her anchor is dragging

B．Her anchor is used in docking

C．She is dredging her anchor

D．Her anchor holds fast while she is swinging

您的船只不能进行时， ______ 。

第七章 ：第七节航海学基础知识（共计89题）

483. ______ is the distance east or west measured in degrees，minutes and seconds of arc on the equator between the prime meridian and the meridian passing through the place．（A）

A．Longitude

B．Latitude

C．Height

D．Bearing

______是东方或西方的距离来衡量度，分，秒的弧赤道之间的本初子午线，并通过经络的地方。

484.______ refers to the angle between true North and the heading line of the ship．(C)

A．True position

B．True meridian

C．True course

D．True bearing

______是指真正的角和北方之间的标题行的船。

485. A bluff bar is a bar ______．(C)

A．extending out from a bluff alongside the river

B．that tends to give a false indication of its position

C．that has a sharp drop off into deep water

D．that is perpendicular to the current

486. A correction for augmentation is included in the Nautical Almanac corrections for ______．(B)

A．the Sun

B．the Moon

C．Venus

D．None of the above

包括在航海天文历更正为______ 。

487. A first magnitude star is ______．(A)

A．2.5 times as bright as a second magnitude star

B．3 times as bright as a second magnitude star

C．5 times as bright as a second magnitude star

D．10 times as bright as a second magnitude star

488. A flashing light is a light that ______．(A)

A．flashes at regular intervals at a frequency of 120 flashes or more per minute

B．is yellow in color

C．is visible over an arc of the horizon of not less than 180° nor more than 225°

D．All of the above

489. A lateral system buoy displaying a quick light ______．(B)

A．should be passed close aboard on either side

B．indicates that special caution is required

C．is used at a channel bifurcation or junction

D．is painted with red and white vertical stripes

横向系统浮标显示快速轻______ 。

490. A light having characteristics which include color variations is defined as ______．(B)

A．switching

B．alternating

C．oscillating

D．fluctuating

491. A light used to signal passing intentions must be an ______．(D)

A．alternating red and yellow light

B．alternating white and yellow light

C．all-round white or yellow light

D．all-round white light only

一光年用于信号传递的意图必须是______ 。

492. A nun buoy will ______．(B)

A．be green in color

B．have an even number

C．be left to port when entering from seaward

D．be cylindrical in shape

493. A red pillar light-buoy，Fl（2）5s，is to be substituted for the green conical light-buoy close S．W．of the stranded wreck．The above sentence mainly refers to ______．(B)

A．A red buoy is to be replaced by a green buoy

B．A green buoy is to be replaced by a red buoy A绿色浮标要取而代之的是一个红色的浮标

C．Either a red buoy or a green buoy is to be replaced

D．Neither a red buoy nor a green buoy is to be replaced

一个红色的支柱轻浮标，楼（ 2 ）五，要取代绿色锥形光浮标密切S.W.of滞留wreck.The上述一句主要是指______ 。

494. A rock and sand structure extending from the bank of the river toward the channel is known as a ______．(A)

A．Wingdam

B．Towhead

C．Cutoff

D．Landwall

岩石和沙子结构扩大从河岸向通道被称为______ 。

495. A SEAMARK，I．E． A BUOY，INDICATING THE NORTH，EAST，SOUTH OR WEST FROM A FIXED POINT，E．G． A WRECK is ______．(D)

A．Entrance buoy

B．New buoy

C．Port buoy

D．Cardinal buoy

甲波线，即一个浮标，表明北，东，南或西从固定点，如破坏是______ 。

496. A section of the river that is narrower than usual and is often navigable from bank to bank is a ______．(A)

A．chute

B．stabilized channel

C．slough

D．navigable pass

一节河流较窄较平时往往通航银行的是______ 。

497. A sextant having an index error that is off the arc has a ______．(A)

A．positive correction

B．dip error

C．negative correction

D．semidiameter error

六分仪具有指数的错误，已关闭弧有______ 。

498. A sidereal day is shorter than a solar day．This difference is due to ______．(D)

A．irregularities in the daily rotational rate of the Sun

B．the space motion of the solar system

C．the precession of the equinoxes

D．the use of different reference points

恒星日短于一个太阳能day.This差别是由于______ 。

499. A sidereal day is shorter than a solar day．This difference is due to ______．(D)

A．irregularities in the daily rotational rate of the Sun

B．the space motion of the solar system

C．the precession of the equinoxes

D．the use of different reference points

恒星日短于一个太阳能day.This差别是由于______ 。

500. A V-shaped ripple with the point of the V pointing upstream in a river may indicate a ______．(C)

A．Submerged rock，not dangerous to navigation

B．Sunken wreck，not dangerous to navigation

C．Towed-under buoy

D．All of the above

V形纹波与点的V指着河中上游可能说明______ 。

501. A white buoy with a blue band is ______．(C)

A．An isolated danger mark

B．A hydrographic data collection buoy

C．A mooring buoy

D．Marking a restricted area

502. A white buoy with an open-faced orange diamond on it indicates ______．(A)

A．danger

B．vessels are excluded from the area

C．the buoy is a mooring buoy

D．operating restrictions are in effect

白色浮标的开放面临着橙色钻石它表明______ 。

503. A yellow buoy may exhibit a（n）______．(B)

A．fixed red light

B．flashing light

C．white light

D．occulting light

504. A yellow buoy may mark a（n）______．(C)

A．wreck

B．shoal area

C．anchorage area

D．middle ground

505. Astronomical refraction causes a celestial body to appear ______．(C)

A．To the left of its position in the Northern Hemisphere and to the right in

the Southern Hemisphere

B．To the right of its position in the Northern Hemisphere and to the left in

the Southern Hemisphere

C．Higher than its actual position

D．Lower than its actual position

天文大气折射原因天体出现______ 。

506. Based on your running fix，you ______．(A)

A．Have a following current

B．Have a head current

C．Are being set to the north

D．Are not affected by a current

根据您运行修复程序，您______ 。

507. Branford reef is ______．(B)

A．A hard sand shoal marked with a light

B．Completely submerged at all stages of the tide

C．Surrounded by rocks awash at low water spring tides

D．A small，low，sandy islet surrounded by shoal water

508. Buoys are marked with reflective material to assist in their detection by searchlight．Which statement is TRUE ________．(D)

A．A safe-water buoy will display red and white vertical stripes of reflective material

B．All reflective material is white because it is the most visible at night

C．A special-purpose mark will display either red or green reflective material to agree with its shape

D．A preferred-channel buoy displays either red or green reflective material to agree with the top band of color首选通道浮标显示红色或绿色，或者反光材料同意顶端带颜色

浮标标有反光材料，以帮助他们检测searchlight.Which说法正确________ 。

509. Buoys which mark dredging areas are painted ______．(B)

A．Black

B．Yellow

C．Green

D．Red

浮标标志着疏浚领域是画______ 。

510. Buoys which mark isolated dangers are painted with alternating ______．(A)

A．Red and black bands

B．Green and black bands

C．Red and white stripes

D．Green and white bands

浮标标志着孤立的危险是画交替______ 。

511. Concerning a celestial observation，the azimuth angle is measured from the principal vertical circle to the ______．(D)

A．Greenwich celestial meridian

B．hour circle of the body

C．local celestial meridian

D．vertical circle of the body

关于天体观测，方位角测量从主要垂直循环的______ 。

512. Daylight savings time is a form of zone time that adopts the time ______．(A)

A．one zone to the east

B．one zone to the west

C．two zones to the east

D．two zones to the west

日光节约时间是一种形式的区时，采用时间______ 。

513. During daylight savings time the meridian used for determining the time is located farther ______．(D)

A．West in west longitude and east in east longitude

B．East in west longitude and west in east longitude

C．West

D．East

在日光节约时间经络用于确定时间位于更远______ 。

514.During daylight savings time the meridian used for determining the time is located farther ______．(A)

A．east

B．west

C．east in west longitude and west in east longitude

D．west in west longitude and east in east longitude

在日光节约时间经络用于确定时间位于更远______ 。

515. During the month of October the Sun's declination is ______．(C)

A．North and increasing

B．North and decreasing

C．South and increasing

D．South and decreasing

在10月份的太阳赤纬是______ 。

516. During which condition should the operator of a pedestal crane shutdown operations ________．(D)

A．Lightning

B．High winds

C．Fire aboard

D．All of the above

在这期间条件的经营者应趁低起重机关机行动________ 。

517.During which condition should the operator of a pedestal crane shutdown operations ________．(D)

A．Lightning in the vicinity

B．An approaching squall line

C．A fire on the pier

D．All of the above

在这期间条件的经营者应趁低起重机关机行动________ 。

518. Environmental loading consists of the forces caused by wind，waves，and ______．(D)

A．drilling

B．soil reactions

C．temperature

D．current

环境负荷由部队造成的风，浪，和______ 。

519. Fishing stake，the position of which frequently ______，exist in the area covered by this chart．(B)

A．Removed

B．Altered

C．Corrected

D．Converted

520. For a well made and well maintained sextant，the maximum value of which correction is usually so small that it can be ignored ________．(B)

A．Personal correction

B．Instrument correction

C．Phase

D．Dip correction

在良好的进展和良好的维护六分仪，最高价值校正通常是如此之小，它可以忽略________ 。

521. How is the intensity of a light expressed in the Light Lists ________．(C)

A．Luminous range

B．Geographic range

C．Nominal range

D．Meteorological range

522. In ______ the time of tidal current varies across the channel from shore to shore．(A)

A．inland tidal estuaries

B．sea port

C．coastal port

D．rivers

在______的时间潮流而有所不同渠道从海岸到海岸。

523.In order to remove index error from a sextant，you should adjust the ______．(B)

A．Index mirror to make it parallel to the horizon glass with the index set at zero

B．Horizon glass to make it parallel to the index mirror with the index set at zero

C．Horizon glass to make it parallel to the sextant frame

D．Telescope to make it perpendicular to the sextant frame

为了消除指数从六分仪错误，你应该调整______ 。

524. In the celestial equator system of coordinates what is equivalent to the colatitude of the Earth system of coordinates ________．(C)

A．Coaltitude

B．Zenith distance

C．Polar distance

D．Declination

在天体赤道坐标系统是什么相当于colatitude地球系统的坐标________ 。

525. In the horizon system of coordinates what is equivalent to latitude on the Earth ________．(A)

A．Altitude

B．Zenith

C．Declination

D．Zenith distance

视野中的坐标系统是什么纬度相当于地球上的________ 。

526. In the horizon system of coordinates what is equivalent to the equator on the Earth ________．(D)

A．Prime vertical circle

B．Principal vertical circle

C．Parallels of altitude

D．Horizon

视野中的坐标系统是什么相当于地球上的赤道________ 。

527. In waters where the cardinal system is used you would expect to find danger ______．(B)

A．Lying to the south of an eastern quadrant buoy

B．Lying to the south of a northern quadrant buoy

C．Lying to the east of an eastern quadrant buoy

D．Beneath or directly adjacent to the buoy

水域的基本系统是用来你希望找到的危险______ 。

528. In which voyage，between two points，is the rhumb line distance NOT approximately the same as the great circle distance ________．(B)

A．The two points are in low latitudes in the same hemisphere

B．The two points are in high latitudes in the same hemisphere两分是在高纬度地区在同一半球

C．The two points are near the equator，but in different hemispheres

D．One point is near the equator，one point is in a high latitude，and both are near the 180th meridian

其中航程，两点之间，是距离不恒线大致相同的大圆距离________ 。

529. In working out a local apparent noon sight for your latitude，you subtract the Ho from 90°．The 90° represents the angular distance from ______．(B)

A．the equator to the elevated pole

B．your horizon to your zenith

C．your zenith to the elevated pole

D．the geographical position of the Sun to the elevated pole

在当地工作了明显的中午看到您的纬度，您减去何90 ° 。代表90 °的角距离______ 。

530. Index error of a sextant is primarily caused by ______．(C)

A．Improperly correcting the other errors in a sextant

B．The horizon glass not being parallel to the horizon mirror

C．The horizon glass not being parallel to the index mirror

D．Human error in taking a celestial observation

指数误差主要是六分仪所造成______ 。

531.Lateral movement of the vessel to leeward of its course defines ______．(C)

A．Leeway

B．Leeside

C．Leeward

D．Lee moving

横向移动的船只当然顺风其定义______ 。

532. Leeway is the ______．(C)

A．Difference between the true course and the compass course

B．Momentum of a vessel after her engines have been stopped

C．Lateral movement of a vessel downwind of her intended course

D．Displacement of a vessel multiplied by her speed

533. My position has been obtained by ______．(A)

A．Astronomical observation

B．Hydrographic survey

C．Weather prediction

D．Current sailing

534. Opposition occurs when ______．(D)

A．the Sun，Earth，and Moon are at right angles

B．the Sun's declination is 0° and is moving south

C．an inferior planet is at the maximum angle to the line of sight to the Sun

D．the Earth is between a planet and the Sun

535. Perihelion is the point where the Sun ______．(A)

A．is nearest to the Earth

B．is farthest from the Earth

C．is on the opposite side of the Earth from the Moon

D．and Moon and Earth are in line

536. Retrograde motion is the ______．（B）

A．Movement of the points of intersection of the planes of the ecliptic and the equator

B．Apparent westerly motion of a planet with respect to stars

C．Movement of a superior planet in its orbit about the Sun

D．Movement of the celestial north pole in an elliptical pattern in space

537. The altitude at LAN may be observed by starting several minutes in advance and continuing until a maximum altitude occurs．This procedure should not be used ______．(D)

A．when the declination and latitude are of different names

B．when the declination is greater than and the same name as the latitude

C．if the vessel is stopped or making bare steerageway

D．on a fast vessel on northerly or southerly headings

海拔在局域网可能会看到开始几分钟提前持续到最高海拔occurs.This程序不应当被用来______ 。

538. The angle measured eastward from the vernal equinox along the celestial equator often expressed in time units is the ______．(B)

A．Greenwich sidereal time

B．Right ascension

C．Local sidereal time

D．Sidereal hour angle

角度测量向东从春分天体沿赤道往往表现在时间单位是______ 。

539. The arc of a great circle which passes through the body and celestial poles is part of the ______．(A)

A．Hour circle

B．Diurnal circle

C．Observer's meridian

D．Altitude circle

圆弧的大圆穿过身体和天体极是______ 。

540. The arc of an hour circle between the celestial equator and a point on the celestial sphere，measured northward or southward through 90°，is the ______．(B)

A．altitude

B．declination

C．latitude

D．azimuth angle

弧圈的一个小时之间的天体赤道上的一个点天球，衡量向北或向南通过90 ° ，是______ 。

541. The design of a spring buoy helps ______．(B)

A．To retrieve the anchor

B．To prevent chafing at the lower end of the pendant

C．To mark the rig's approach to the location

D．An anchor handling vessel back toward the rig

542. The error in a sextant altitude caused by refraction is greatest when the celestial body is ______．(B)

A．high in the sky

B．near the horizon

C．rising

D．at or near transit

错误在六分仪高度所造成的折射是最大的天体时，是______ 。

543.The expression FIRST MAGNITUDE is usually used to refer only to bodies of magnitude ______．(A)

A．1．5 and greater

B．1.25 and greater

C．1.0 and greater

D．0.5 and greater

544. The GHA of a star ______．(A)

A．increases at a rate of approximately 15° per hour

B．increases at a rate of approximately 4° per hour

C．decreases at a rate of approximately 15° per hour

D．decreases at a rate of approximately 4° per hour

545. The height of a light is measured from which reference plane ________．(B)

A．Mean low water

B．Mean high water

C．Average water level

D．Geographical sea level

高度来衡量光从参考平面________ 。

546. The height of tide is the ______．(B)

A．Depth of water at a specific time due to tidal effect

B．Difference between the depth of the water and the area's tidal datum

C．Difference between the depth of the water and the high water tidal level

D．Difference between the depth of the water at high tide and the depth of the water at low tide

547. The horizon glass of a sextant is ______．(A)

A．Silvered on its half nearer the frame

B．Mounted on the index arm

C．Between the horizon and the shade glasses

D．All of the above

地平线玻璃的六分仪是______ 。

548. The horizontal plane，perpendicular to the zenith-nadir axis，that intersects with the celestial sphere and is tangent to the earth is called the ______．(D)

A．Celestial horizon

B．Sensible horizon

C．Visible horizon

D．Geoidal horizon

该平面垂直于天顶-奈迪尔轴，即交织在一起天球和相切的地球被称为______ 。

549. The largest of the navigational planets is ______．(C)

A．Mars

B．Venus

C．Jupiter

D．Saturn

最大的航行行星是______ 。

550. The light characteristic of composite group flashing（2 + 1）is used in the Aids to Navigation System on the Western Rivers for lights on ______．(C)

A．the right descending bank

B．the left descending bank

C．preferred-channel buoys

D．daymarks with no lateral significance

根据特点，综合组闪动（ 2 + 1 ）中使用的航路系统对西部河流的灯______ 。

551. The navigational triangle uses parts of two systems of coordinates，one of which is the celestial equator system，the other system is the ______．(B)

A．terrestrial system

B．horizon system

C．astronomical system

D．ecliptic system

使用的导航三角地区的两个系统的协调，其中之一是天体赤道系统，其他系统是______ 。

552. The navigator is concerned with three systems of coordinates．Which system is not of major concern __．(B)

A．Terrestrial

B．Ecliptic

C．Celestial horizon

D．Celestial equator

航海家关注的是三个系统的coordinates.Which系统并非主要关切________ 。

553. The parallel of latitude at 23°27'N is the ______．(A)

A．Tropic of Cancer

B．Tropic of Capricorn

C．Arctic Circle

D．Ecliptic

平行纬度在23 ° 27'N是______ 。

554. The period at high or low tide during which there is no change in the height of the water is called the __．(C)

A．range of the tide

B．plane of the tide

C．stand of the tide

D．reversing of the tide

这一时期在高或低潮期间，没有任何变化的高度，水被称为______ 。

555. The point where the vertical rise or fall of tide has stopped is referred to as ______．(C)

A．Slack water

B．The rip tide

C．The stand of the tide

D．The reverse of the tide

点的垂直兴衰浪潮已经停止被称为______ 。

556. The prime vertical is the reference point from which the angle of which type of observation is measured ________．(C)

A．Sextant angle

B．Azimuth

C．Amplitude

D．Local apparent noon

总理垂直的参考点是从这一角度看哪种类型的观察测量________ 。

557. The spinning of a celestial body about its axis is known as ______．(A)

A．rotation

B．revolution

C．space motion

D．nutation

旋转的天体其轴被称为______ 。

558. The Sun's center may be coincident with both the celestial equator and the observer's prime vertical circle when ______．(D)

A．It crosses the December solstitial point

B．It crosses the June solstitial point

C．It is in upper transit

D．Its declination is zero

太阳的中心可能是一致的天体赤道和观察员总理垂直循环时______ 。

559. The tropical year differs from which year by 20 minutes ________．(D)

A．Astronomical year

B．Natural year

C．Equinoctial year

D．Sidereal year

热带一年不同于这一年的20分钟________ 。

560. Universal time（UTI）is another name for ______．(B)

A．Sidereal time

B．Greenwich mean time

C．Ephemeris time

D．Atomic time

通用时间（丁）的另一个名称是______ 。

561. Vessels must not ______ a speed of 8 knots in this area．(D)

A．Sail at

B．Expect

C．Proceed at

D．Steam over

562. What causes geocentric parallax ________．(B)

A．The varying distance between the Earth and Moon

B．The change in the Moon's position relative to the stars when viewed from the Earth's surface，as compared to the Earth's center变化，月球的相对位置时的星级，从地球表面的，相比之下，地球的中心

C．The rapid change in declination of the Moon causes a rotational oscillation of its axis

D．The nearness of the Moon causes an apparent increase in diameter as its altitude increases

是什么原因导致地心视差________ 。

563. What is a characteristic of a rhumb line ________．(C)

A．It is the shortest distance between two points on the Earth

B．It plots as a straight line on a Lambert conformal chart

C．It cuts each meridian at the same angle

D．The course angle constantly changes to form the loxodromic curve

什么是典型的恒线________ 。

564. When a light is first seen on the horizon it will disappear again if the height of eye is immediately lowered several feet．When the eye is raised to its former height the light will again be visible．This process is called ______．(D)

A．Checking a light

B．Raising a light

C．Obscuring a light

D．Bobbing a light

当光线是第一次看到地平线上，将再次消失，如果眼睛的高度是立即降低几个feet.When的眼睛是前提出其高度的光将再次visible.This过程被称为______ 。

565. When correcting apparent altitude to observed altitude，you do NOT apply a correction for ______．(D)

A．the equivalent reading to the center of the body

B．the equivalent reading from the center of the Earth

C．the bending of the rays of light from the body

D．inaccuracies in the reference level

当纠正明显的高度，观察高度，你不改正的______ 。

566. When is the rhumb line distance the same as the great circle distance ________．(B)

A．Course 090°T in high latitudes

B．Course 180°T when you cross the equator

C．Course 045°T in low latitudes

D．The rhumb line distance is always longer than the great circle distance．

当是恒线距离相同的大圆距离________ 。

567. When using a mechanical（windup type）marine chronometer，how often should it be reset ________．(A)

A．Only when it is overhauled

B．Whenever the chronometer error exceeds approximately four minutes

C．At the start of each voyage

C．At the start of each voyage

D．If the chronometer rate changes from gaining to losing or vice versa

当使用机械（挥臂型）海洋计，应该多久才能重置________ 。

568. Which is an inferior planet ________．(B)

A．Mars

B．Venus

C．Neptune

D．Pluto

这是一个低劣的星球________ 。

569. Which of the four adjustable errors in the sextant causes side error ________．(A)

A．Horizon glass not being perpendicular to the frame

B．Index mirror not being perpendicular to the frame

C．Telescope not being parallel to the frame

D．Elliptical centering error

这四个可调中的错误原因六分仪端错误________ 。

570. Which of the four adjustable errors in the sextant is the principle cause of index error ________．(B)

A．Telescope not being parallel to the frame

B．Index mirror and horizon glass not being parallel

C．Index mirror not being perpendicular to the frame

D．Horizon glass not being perpendicular to the frame

这四个可调错误六分仪的原则是错误的原因指数________ 。

571. Υ on Admiralty Chart represents ______．(D)

A．kelp

B．ruin

C．dyke

D．spring

Υ对金钟图代表______ 。

第七章第九节潮汐（共计38题）

572. ______ is not a type of tide．（D）

A．SEMIDIURNAL

B．DIURNAL

C．MIXED

D．OSCILLATING

______不是一个类型的潮流。

573. A wave entering ______ is likely to produce bores．(A)

A．shallow water

B．deep water

C．river

D．tide

574. In which month will the equatorial counter current be strongest ________．(C)

A．January

B．April

C．August

D．October

在这一个月将赤道逆流是最强________ 。

575. Neap tides occur ______．(C)

A．At the start of spring，when the Sun is nearly over the equator

B．Only when the Sun and Moon are on the same sides of the Earth and are nearly in line

C．When the Sun and Moon are at approximately 90° to each other，as seen from the Earth当太阳和月亮是在大约90 °对方，从地球

D．When the Sun，Moon，and Earth are nearly in line，regardless of

alignment order

小潮涨潮发生______ 。

576. Neap tides occur only ______．(D)

A．at a new or full Moon

B．when the Sun，Moon，and Earth are in line

C．at approximately 28-day intervals

D．when the Moon is at quadrature

赤潮发生的大小只有______ 。

577. Neap tides occur when the ______．(A)

A．Moon is in its first quarter and third quarter phases

B．Sun and Moon are on opposite sides of the Earth

C．Moon's declination is maximum and opposite to that of the Sun

D．Sun and Moon are in conjunction

小潮涨潮时发生的______ 。

578. Priming of the tides occurs ______．(D)

A．at times of new and full Moon

B．when the Earth，Moon，and Sun are lying approximately on the same line

C．when the Moon is between first quarter and full and between third quarter and new

D．when the Moon is between new and first quarter and between full and third quarter

引发的赤潮发生______ 。

579. River currents tend to ______．(D)

A．Pick up speed where the channel widens

B．Run slower in the center of the channel

C．Hug the inside of a bend

D．Cause the greatest depth of water to be along the outside of a bend

580. Spring tides are tides that ______．(A)

A．have lows lower than normal and highs higher than normal低点低于正常和高高于正常

B．have lows higher than normal and highs lower than normal

C．are unpredictable

D．occur in the spring of the year

大潮的潮汐是______ 。

581. Stand of the tide is that time when ______．(A)

A．the vertical rise or fall of the tide has stopped

B．slack water occurs

C．tidal current is at a maximum

D．the actual depth of the water equals the charted depth

立场潮流的是，当时______ 。

582. Swell is the rise and fall of the ocean's surface due to ______．(B)

A．Fetch

B．Distant winds

C．Local storms

D．The pull of the moon

胀大的上升和下降的海洋表面由于______ 。

583. The Benguela Current flows in a ______．(C)

A．SW'ly direction along the NW coast of Africa

B．S'ly direction off the East Coast of Australia

C．NW'ly direction along the SW coast of Africa

D．SW'ly direction along the SE coast of Greenland

本格拉洋流流动的______ 。

584. The Brazil Current flows in which general direction ________．(B)

A．Northwesterly

B．Southwesterly

C．Southerly

D．Northerly

巴西目前流入的大方向________ 。

585. The distance between the surface of the water and the tidal datum is the ______．(B)

A．range of tide

B．height of tide

C．charted depth

D．actual water depth

之间的距离水面和潮汐基准面是______ 。

586. The easiest way to anchor a vessel in a current is to ______．（B）

A．Stem the current and make very slow headway when the anchor is dropped

B．Stem the current and be falling aft very slowly when the anchor is dropped

C．Stem the current and endeavor to make neither headway nor sternway when the anchor is dropped

D．Stop all headway through the water and keep the current astern when the anchor is dropped

最简单的方法锚船只目前是______ 。

587. The Humboldt Current flows in which direction ________．(A)

A．North

B．South

C．East

D．West

洪堡洋流流动的方向________ 。

588. The interval of the average elapsed time from the meridian transit of the moon until the next high tide is called the ______．(B)

A．harmonic constant

B．establishment of the port

C．half-tide level

D．tide cycle

间隔的平均时间从过去的经络过境月球，直至下一次高潮称为______ 。

589. The movement of water away from the shore or downstream is called a（n）______．(B)

A．reversing current

B．ebb current

C．flood current

D．slack current

流动的水远离岸边或下游被称为（ ñ )______.

590. The period at high or low water during which there is no apparent change of level is called ______．(C)

A．HEIGHT

B．RANGE

C．STAND

D．FALL

这一时期在高或低水在这期间没有明显变化的水平被称为______ 。

591. The set of the current is the ______．(D)

A．speed of the current at a particular time

B．maximum speed of the current

C．direction from which the current flows

D．direction in which the current flows

这套目前是______ 。

592. The set of the ocean current is caused by ______．(D)

A．density differences of the water

B．rotation of the earth

C．direction of primary circulation of air

D．All of the above

这套海流所造成的______ 。

593. The term flood current refers to that time when the water ______．(A)

A．is flowing towards the land

B．is moving towards the ocean

C．level is not changing

D．level is rising because of heavy rains

这个词是指洪水目前的时候，水______ 。

594. The two most effective generating forces of surface ocean currents are ______．(B)

A．temperature and salinity differences in the water

B．wind and density differences in the water

C．water depth and underwater topography

D．rotation of the Earth and continental interference

这两个最有效的创造力量的表面洋流是______ 。

595. The type of current which will have the greatest effect on the course made good for your vessel is ____．(C)

A．one flowing in the same direction as your course steered

B．one flowing in the opposite direction as your course steered

C．one that flows at nearly right angles to your course steered

D．a rotary current in which the direction of current flow constantly changes

该类型的电流将产生最大影响的过程取得了良好的船只是______ 。

596. The velocity of a rotary tidal current will be decreased when the Moon is ______．(A)

A．at apogee

B．new

C．full

D．All of the above

速度旋转潮流将下降时，月球是______ 。

597. The velocity of the current in large coastal harbors is ______．(A)

A．predicted in Tidal Current Tables

B．unpredictable

C．generally constant

D．generally too weak to be of concern

速度目前在大型沿海港口是______ 。

598. The velocity of the wind，its steady direction，and the amount of time it has blown determines a wind driven current's ______．(D)

A．Temperature

B．Density

C．Deflection

D．Speed

速度的风，稳定的方向，以及所需的时间已确定风力吹驱动电流的______ 。

599. Tide is ______ in direction ENE．(B)

A．Getting

B．Setting

C．Being

D．Having

退潮______偏东方向。

600. Tides are superimposed on nontidal rising and falling water levels，caused by ______．(D)

A．freshets

B．floods

C．river flows

D．seismic events

潮汐是叠加在非潮汐升降水位，造成______ 。

601. When daylight savings time is kept，the time of tide and current calculations must be adjusted．One way of doing this is to ______．(A)

A．Add one hour to the times listed under the reference stations

B．Subtract one hour from the time differences listed for the subordinate stations

C．Apply no correction as the times in the reference stations are adjusted for daylight savings time

D．Add 15° to the standard meridian when calculating the time difference

当日光节约时间保存的时间力挽狂澜，目前的计算结果必须adjusted.One的做法是，以______ 。

602. When there are small differences between the heights of two successive high tides or two successive low tides，the tides are called ______．(B)

A．Diurnal

B．Semi-diurnal

C．Solar

D．Mixed

当有小的分歧的高度连续两次高潮或连续两次退潮，潮汐称为____

603. Which current is responsible for the movement of icebergs into the North Atlantic shipping lanes _____ .(C)

A．Iceland Current

B．Baltic Current

C．Labrador Current

D．Baffin Current

目前的负责流动冰山到北大西洋航道________ 。

604. Which current would you encounter on a direct passage from London，England，to Capetown，South Africa ________．(D)

A．Falkland Current

B．Brazil Current

C．Norway Current

D．Benguela Current

目前的你会遇到的一个直接通道从英国伦敦，开普敦，南非________ 。

605. Which current would you encounter on a direct passage from southern Africa to Argentina，South America ________．(A)

A．South Atlantic

B．South Equatorial

C．Agulhas

D．Guinea

目前的你会遇到的一个直接通道从阿根廷南部非洲，南美洲________ 。

606. Which statement is TRUE concerning apogean tides ________．(D)

A．They occur only at quadrature

B．They occur when the Moon is nearest the Earth

C．They cause diurnal tides to become mixed

D．They have a decreased range from normal他们有一个正常的范围内减少

607. Which statement is TRUE concerning equatorial tides ________．(C)

A．They occur when the Sun is at minimum declination north or south

B．They occur when the Moon is at maximum declination north or south

C．The difference in height between consecutive high or low tides is at a minimum在不同高度之间的连续高或低潮汐是在最低限度

D．They are used as the basis for the vulgar establishment of the port

哪种说法正确的赤道潮汐________ 。

608. You are anchoring in a river where the current is from one direction only．The best way to lay out two anchors is to have them ______．(C)

A．Directly in line with the bow

B．Side by side，with their lines on the port and starboard side

C．So that their lines form an angle

D．On top of one another

您正在锚泊在河里那里目前是从一个方向only.The最佳途径奠定了两个锚点是让他们______ 。

609. You are docking a vessel．Wind and current are most favorable when they are ______．(C)

A．crossing your course in the same direction

B．crossing your course in opposite directions

C．parallel to the pier from ahead

D．setting you on the pier

您正在对接一vessel.Wind和当前最有利时， ______ 。

第八章 第一节船舶货运基本知识（共计109题）

610.At all angles of inclination，the metacenter is ______．(A)

A．vertically above the center of buoyancy

B．vertically above the center of gravity

C．at the intersection of the upright vertical centerline and the line of action of the buoyant force

D．at the geometric center of the underwater volume

在各个角度的倾斜，在metacenter是______ 。

611. Deballasting a double bottom has what affect on KG _________．(A)

A．KG is increased

B．KG is decreased

C．KG is not affected

D．KG increases at light drafts and decreases at deep drafts

减压双层底有何影响公斤_________ 。

612. During counterflooding to correct a severe list or trim aggravated by an off-center load，a ship suddenly takes a list or trim to the opposite side．You should ______．(C)

A．Continue counterflooding in the same direction

B．Continue counterflooding，but in the opposite direction

C．Immediately stop counterflooding

D．Deballast from the low side

在counterflooding ，纠正了严重的清单或削减加剧了场外中心负荷，船舶突然采取列表或修剪对面side.You应该______ 。

613. Fighting a fire in the galley poses the additional threat of ______．(D)

A．contaminating food with extinguishing agent

B．spreading through the engineering space

C．loss of stability

D．a grease fire in the ventilation system

战斗发生火灾，厨房构成威胁的______ 。

614. For a floating vessel，the result of subtracting KG from KM is the ______．(D)

A．height of the metacenter

B．height of the righting arm

C．height of the center of buoyancy

D．metacentric height

对于浮动船只，结果减去公斤知识是______ 。

615. For a given displacement，the righting arm has its maximum value when ______．(A)

A．KG is minimum

B．angle of inclination is a maximum

C．small-angle stability applies

D．KM is a minimum

对于给定的流离失所，翻ARM公司最大值时______ 。

616. For a ship with longitudinal inclination，an increase in GML causes ______．(D)

A．list to stabilize at an angle of loll

B．trim to stabilize at an angle of loll

C．trim to increase

D．trim to decrease

对于船舶纵倾角，增加GML的原因______ 。

617. For a vessel inclined by the wind，multiplying the buoyant force by the horizontal distance between the lines of action of the buoyant and gravity forces gives the ______．(A)

A．righting moment

B．vertical moment

C．longitudinal moment

D．transverse moment

为船只倾斜的风，乘以浮力的水平之间的距离的行动方针的浮力和重力部队给予______ 。

618. For an upright vessel，draft is the vertical distance between the keel and the ______．(A)

A．Waterline

B．Freeboard deck

C．Plimsoll mark

D．Amidships section

一个正直的船只，草案之间的垂直距离龙骨和______ 。

619. For optimum stability during jacking operations，mat ballast tanks or spud cans should be ______．(B)

A．empty

B．fully ballasted

C．partially ballasted

D．sealed

为获得最佳稳定顶行动期间，垫压载舱或土豆罐应当______ 。

620.Forces within a vessel have caused a difference between the starboard and port drafts．This difference is called ______．(A)

A．list

B．heel

C．trim

D．flotation

部队的船只造成了差异右舷和港口drafts.This的差异被称为______

621. GM cannot be used as an indicator of stability at all angles of inclination because ______．(A)

A．M is not fixed at large angles

B．there is no M at large angles

C．G is not fixed at large angles

D．there is no G at large angles

通用汽车公司不能作为一个指标稳定在各个角度的倾斜，因为______ 。

622. GM is a measure of ______．① The initial stability of a vessel；② The rightingness of the vessel．(A)

A．① only

B．② only

C．Both ① and ②

D．Neither ① nor ②

通用汽车是衡量______ 。 ①初步稳定的船只; ②在rightingness的船只

623. Horizontal fore or aft motion of a vessel is known as ______．(B)

A．pitch

B．surge

C．sway

D．roll

横向或纵向运动前的船只被称为______ 。

624. How do you determine the weight of the vessel that is supported by the ground when a vessel has run aground _________．(C)

A．This requires extensive calculation and is usually performed only by a naval architect not by a ship's officer

B．Determine the point where aground and the draft at that point，then calculate it using the grounding formula

C．Use the hydrostatic tables and enter with the mean draft before grounding and the mean draft after grounding

D．Use the inclining experiment formula and substitute the change of trim for the angle of list

你怎么确定的重量船只所支持的地面时，船只搁浅_________ 。

625. I hereby declare that my vessel has a ______ 15 400 metric tons．(D)

A．bale capacity

B．grain capacity

C．cargo capacity

D．DWTC

我在此宣布，我的船有______ 15 400吨

626. If dangerous goods have been thrown overboard by the Shipowner or the master，______ can be made liable for their loss．(D)

A．either

B．the master

C．the Shipowner

D．neither

如果危险品已经扔到海里的船东或船长,______可以承担他们的损失。

627. If the cause of severe list or trim is off-center ballast，counterflooding into empty tanks will ______．(D)

A．Increase the righting moment

B．Increase the righting arm

C．Increase list or trim

D．Decrease list or trim

如果造成严重的清单或修剪是偏离中心镇流器， counterflooding成空坦克将______

628. If the metacentric height is large，a floating vessel will ______．(C)

A．Be tender

B．Have a slow and easy motion

C．Be stiff

D．Have a tendency to yaw

如果稳心高度大，浮船将______ 。

629. If your vessel will list with equal readiness to either side，the list is most likely caused by ______．(A)

A．Negative GM

B．Off-center weight

C．Pocketing of free surface

D．Excessive freeboard

如果您的船只将列出平等愿意与任何一方，这个名单很可能是由______ 。

630. In order to determine the trimming arm for your vessel at any particular draft，which of the following would you compare _________．(A)

A．Longitudinal center of buoyancy and longitudinal center of gravity

B．Longitudinal center of floatation and longitudinal center of gravity

C．Longitudinal center of floatation and longitudinal center of buoyancy

D．Longitudinal tipping center and longitudinal center of gravity

为了确定微调手臂的船只在任何特定的草案，以下你比较_________ 。

631. In the absence of external forces，adding weight on one side of a floating vessel causes the vessel to ______．(B)

A．heel until the angle of loll is reached

B．list until the center of buoyancy is aligned vertically with the center of gravity

C．trim to the side opposite TCG until all moments are equal

D．decrease draft at the center of flotation

在没有外部力量，增加体重的一面浮动船只造成船只______ 。

632. In the absence of external forces，the center of buoyancy of an inclined vessel is vertically aligned directly below the ______．(A)

A．Center of gravity

B．Amidships station

C．Center of flotation

D．Geometric center of the waterplane area

在没有外部势力，该中心的浮力倾斜船只直接垂直对齐低于______ 。

633. Inclination of the vessel to port side or starboard side defines ______．(C)

A．Tip

B．Capsize

C．List

D．Trim

倾角的船只左舷或右舷定义______ 。

634. Increasing free surfaces has the effect of raising the ______．(B)

A．uncorrected KG

B．virtual height of the center of gravity

C．metacenter

D．metacentric height

增加自由曲面的效果，提高______ 。

635. Intact buoyancy is a term used to describe ______．(B)

A．The volume of all intact spaces above the waterline

B．An intact space below the surface of a flooded area

C．An intact space which can be flooded without causing a ship to sink

D．The space at which all the vertical upward forces of buoyancy are considered to be concentrated

完整的浮力是一个用来描述______ 。

636. It would be possible that part of the cargo got ______ as great change in the weather during the voyage caused heavy ______ in the hold．(C)

A．damp/water

B．dampness/sweater

C．wet/condensation

D．wetness/dew

有可能的那部分货物______了巨大变化，在航行期间的天气造成了重大的______举行。

637. Keeping the draft at or below the load line mark will insure that the vessel has adequate ______．(B)

A．Ballast

B．Reserve buoyancy

C．Displacement

D．Rolling periods

维持草案或以下的载重线标记将保证该船有足够的______ 。

638. Load line regulations are designed to insure that a vessel has adequate structural strength and sufficient ______．(B)

A．lifesaving equipment

B．stability

C．mooring tension

D．riser tension

载重线法规的目的是确保船舶有足够的结构强度和足够的______ 。

639. Metacentric height is an indication of a vessel's stability ______．(C)

A．for all angles of inclination

B．for large angles of inclination

C．for small angles of inclination

D．in no case

稳心高度，这表明该船的稳定______ 。

640. MV Utopia may conduct the following operations to stop listing except to __________．(B)

A．Jettison cargo

B．Immobilize the engine

C．Transfer cargo

D．Transfer bunkers

压乌托邦可以进行以下操作停止上市的除外__________ 。

641. My drafts are 28 feet ______ and 29 feet aft．(A)

A．fore

B．advance

C．before

D．beforehand

我的草稿二十八英尺______和二十九英尺船尾。

642. On a semisubmersible ship，reserve buoyancy is increased by the presence of void spaces above the waterline in the ______．(C)

A．Submerged hulls

B．Quarters

C．Columns and upper structure

D．Submerged connecting structures

在semisubmersible船舶，储备浮力增加了在场的情况下空舱水线以上的______

643. On a ship，the deck stringer is the outboard most deck ______．(A)

A．Plating

B．Beam

C．Stiffener

D．Stanchion

在船上，在甲板上斯丁格是舷外最甲板______ 。

644. Owing to the big draft，the ship is not permitted to go alongside until the time of ______．(C)

A．ebb tide

B．flood tide

C．spring tide

D．slack water

由于大的草案，该船舶不得进入到一起时， _____

645. Pitching is angular motion of the vessel about what axis _________．(B)

A．Longitudinal

B．Transverse

C．Vertical

D．Centerline

俯仰角运动是该船什么轴_________

646. Reducing the liquid free surfaces in a vessel reduces the ______．(A)

A．Roll period

B．Metacentric height

C．Waterplane area

D．Vessel's draft

减少液体自由表面的船只减少了______ 。

647. Reserve buoyancy is the ______．(B)

A．Unoccupied space below the waterline

B．Volume of intact space above the waterline

C．Excess of the buoyant force over the gravity force

D．Difference in the buoyant force in salt and fresh waters

储备浮力是______ 。

648. Semisubmersibles A and B are identical．However，A is more tender than B．This means that A relative to B has a ______．(B)

A．Lower KG

B．Smaller GM

C．Smaller roll angle

D．Larger GZ

Semisubmersibles A和B是identical.However ， A是更招标比湾这意味着，相对于B的______ 。

649. Signs of racking stresses generally appear at the ______．(B)

A．Bow and stern shell frames and plating

B．Junction of the frames with the beams and floors

C．Garboard strake，at each side of the keel

D．Thrust bearing of the main shaft

迹象货架强调一般出现在______ 。

650. That center around which a vessel trims is called the ______．(A)

A．tipping center

B．center of buoyancy

C．center of gravity

D．turning center

该中心围绕着这些船只装饰称为______ 。

651. The angle to which a floating ship，with a negative initial metacentric height，lies while at rest in still water is the angle of ______．(D)

A．Trim

B．Yaw

C．Heel

D．Loll

角的浮动船舶，以消极的初步稳心高度，谎言，而其余的仍是水的角度_____

652. The average of the forward and after drafts is the ______．(A)

A．mean draft

B．true mean draft

C．mean of the calculated drafts

D．draft at the center of flotation

平均后的草案提出的是______ 。

653. The center of buoyancy is located at the ______．(D)

A．geometric center of the waterplane area

B．intersection of the vertical centerline and line of action of the buoyant force

C．center of gravity of the vessel corrected for free surface effects

D．geometric center of the displaced volume

该中心的浮力位于______ 。

654. The center of flotation of a vessel is ______．(B)

A．The center of volume of the immersed portion of the vessel

B．The center of gravity of the water plane

C．That point at which all the vertical downward forces of weight are considered to be concentrated

D．That point at which all the vertical upward forces of buoyancy are considered to be concentrated

该中心的浮选的船只是______ 。

655. The center of flotation of a vessel is the point in the waterplane ______．(A)

A．About which the vessel lists and trims

B．Which coincides with the center of buoyancy

C．Which，in the absence of external forces，is always vertically aligned with the center of gravity

D．Which is shown in the hydrostatic tables as VCB

该中心的浮选的船只是点waterplane ______ 。

656. The center of the underwater volume of a floating vessel is the ______．(A)

A．Center of buoyancy

B．Center of flotation

C．Uncorrected height of the center of gravity of the vessel

D．Center of gravity of the vessel corrected for free surface effects

中心的水下货量浮动船只是______ 。

657. The center of volume of the immersed portion of the hull is called the ______．(A)

A．center of buoyancy

B．center of flotation

C．center of gravity

D．tipping center

该中心的体积浸入部分船体被称为______ 。

658. The controlling depth of the river is ______．(D)

A．the minimum depth of the river prescribed in the channel maintenance program

B．the edge of a dredged channel

C．the highest level to which the river may rise without flooding

D．the least available water in a channel which limits the draft of boats and tows

控制深度河是______ 。

659. The damaged cargo is not allowed to export without ______．(C)

A．Repairing or replacing

B．To repair or to replace

C．Being repaired or replaced

D．To be repaired or replaced

损坏的货物，不准出口没有______ 。

660. The difference between the height of the metacenter and the height of the center of gravity is known as the ______．(A)

A．metacentric height

B．height of the righting arm

C．fore and aft perpendicular

D．height of the center of buoyancy

之间的区别的高度metacenter和高度的重心被称为______ 。

661. The downflooding angle for a ship is the minimum angle at which ______．(B)

A．The load line becomes submerged

B．A non-watertight opening is at still water level

C．Offset becomes excessive

D．Reserve buoyancy is a maximum

该downflooding角船舶最低角上______ 。

662. The existence of liquids in partially full tanks or compartments of a ship causes a virtual rise in the height of the ______．(D)

A．metacenter

B．center of buoyancy

C．center of flotation

D．center of gravity

存在的液体部分充分坦克或舱室船舶造成虚拟上升的高度______ 。

663. The free surface effects of a partially-full tank in a floating ship increase with the ______．(A)

A．surface area of the fluid in the tank

B．displacement volume of the ship

C．draft of the ship

D．height of the tank above the keel

自由表面效应的一个部分，充分池浮船随着______ 。

664. The geometric center of the waterplane area is called the ______．(D)

A．center of buoyancy

B．center of gravity

C．metacenter

D．center of flotation

几何中心waterplane领域被称为______ 。

665.The harbor master ordered that foreign ship to ______ a safe area．(B)

A．Drift to

B．Shift to

C．Draft to

D．Lift to

港口船长下令向外国船舶______安全区。

666. The height of the metacenter above the keel will vary depending on the ______．(A)

A．draft and beam of the ship

B．displacement and deadweight of the ship

C．buoyancy and trim of the ship

D．tonnage and deadweight of the ship

高度metacenter以上龙骨将取决于______ 。

667. The International Load Line Certificate is a document indicating the ______ of the various load lines of the vessel．(D)

A．Decks

B．Cabins

C．Holds

D．Positions

国际载重线证书是一份文件，说明了______的各种载重线的船舶。

668. The jettisoning of topside weight from a ship serves what purpose _________．(C)

A．It returns the ship to an even keel

B．It reduces free surface effect

C．It lowers the center of gravity

D．It raises the center of gravity

该摈弃的上部重量从船舶提供何种目的_________ 。

669. The KM for a vessel may be determined by which of the following _________．(A)

A．Adding the KB and the BM

B．Subtracting the KB from the BM

C．Subtracting the GM from the KB

D．Adding the GM and the KB

知识的船只可能取决于下列哪些_________ 。

670. The maximum draft to which a ship can legally be submerged is indicated by the ______．(A)

A．Load Line mark

B．Certificate of Inspection

C．Muster List

D．Tonnage mark

草案的最大的船舶可以合法地被淹没所表明______ 。

671.The original equilibrium position is stable when ______．(A)

A．Metacentric height is positive

B．Metacentric radius is positive

C．KG exceeds maximum allowable limits

D．Free surfaces are excessive

原来的平衡位置是稳定时______ 。

672. The period of roll is the time difference between ______．(C)

A．Zero inclination to full inclination on one side

B．Full inclination on one side to full inclination on the other side

C．Full inclination on one side to the next full inclination on the same side

D．Zero inclination to the next zero inclination

期间推出的时差______ 。

673. The Plimsoll mark on a vessel is used to ______．(C)

A．align the vessel's tailshaft

B．determine the vessel's trim

C．determine the vessel's freeboard

D．locate the vessel's centerline

普利姆索尔标记的船只是用来______ 。

674. The purpose of the inclining experiment is to ______．(B)

A．Determine the location of the metacenter

B．Determine the lightweight center of gravity location

C．Verify the hydrostatic data

D．Verify data in the vessel's operating manual

目的倾斜实验是______ 。

675. The result of multiplying a weight by a distance is a ______．(A)

A．moment

B．force

C．couple

D．center of gravity location

结果乘以重量的距离是______ 。

676. The shearing stresses on a ship's structure are usually greatest at ______．(D)

A．The bow

B．The stern

C．Midships

D．The ship's quarter-length points

剪切应力对船舶的结构通常是最大的_____

677. The ship has ______ of more than 10 meters at full load．(D)

A．an engine

B．an overall length

C．a displacement

D．a draft

船舶______超过10米满负荷。

678. The ship should have a slightly deeper draft ______．(D)

A．on port side

B．on starboard side

C．at the port quarter

D．at the stern

该船应该有一个更深入略有草案______ 。

679. The ship's crew observed the vessel to be riding low in the water．This indicates that the vessel has a ______ GM．(B)

A．larger

B．smaller

C．proper

D．negative

船员观察船只将骑低water.This表明，该船具有______通用。

680. The stability of a semisubmersible ship would be seriously reduced if flooding occurred in the ______．(B)

A．Submerged hulls

B．Column void spaces

C．Dry mud tanks

D．Ballast tanks

稳定semisubmersible船舶将严重减少，如果洪水发生在______ 。

681. The term LENGTH OF A VESSEL in International Regulations for Preventing Collision at Sea 1972 means ______．(C)

A．the length between perpendiculars

B．the registered length

C．the length overall

D．the length of waterplane on summer draft

任期长度船国际避碰规则1972年海上手段______ 。

682. The time required to incline from port to starboard and back to port again is called ______．(D)

A．Initial stability

B．Range of stability

C．Inclining moment

D．Rolling period

所需要的时间倾斜从港口向右舷和回港又被称为______ 。

683. The weight of the liquid displaced by a vessel floating in sea water is equal to the ______．(B)

A．Weight required to sink the vessel

B．Total weight of the vessel

C．Displaced volume

D．Reserve buoyancy

重量的液体流离失所船只漂浮在海水等于______ 。

684. Timber can be stowed ______．(D)

A．Only on deck

B．Only in holds

C．In forecastle，bridge and poop

D．Both on deck and in holds

木材可存放______ 。

685. To ensure the ship's general stability，______．(D)

A．she's loaded a little deeper forward

B．she's loaded a little deeper aft

C．she has not any ballast at all

D．the cargo is properly distributed throughout the ship

为了确保船舶的整体稳定,______.

686. To increase the extent of flooding your vessel can suffer without sinking，you could ______．(B)

A．ballast the vessel

B．increase reserve buoyancy

C．lower the center of gravity

D．raise the center of gravity

增加的程度，洪水可能遭受的船只没有沉没，您可以______ 。

687. Topside icing decreases vessel stability because it increases ______．(D)

A．displacement

B．free surface

C．draft

D．KG

上部除冰减少船只稳定，因为它增加______ 。

688. Unstable equilibrium exists at small angles of inclination when ______．(A)

A．G is above M

B．G is off the centerline

C．B is off the centerline

D．B is above

不稳定平衡存在于小角度倾斜时， ______ 。

689. Vinyl chloride reacts dangerously with ______．(B)

A．alkalies

B．concentrated nitric acid

C．saltwater

D．organic acids

氯乙烯反应危险与______ 。

690. Weight concentration in which area will cause a vessel's bottom to be subjected to tension stresses _________．(B)

A．Aft

B．Amidships

C．At both ends

D．Forward

重量集中在哪些领域将导致船的底部受到张力强调_________

691. What DOCS SALT WATER DRAFT means _________．(D)

A．It means summer draft

B．It means Maximum draft

C．It means Fresh Water draft

D．It means Seawater Draft

什么文件草案咸水手段_________ 。

692. What does MBDD mean on the load waterline mark _________．(D)

A．Timber tropical load waterline

B．Timber summer load waterline

C．Timber winter load waterline

D．Timber winter North Atlantic load waterline

意味着什么MBDD的载重水线标志_________ 。

693. What does NOT affect the value of the free surface correction _________．(C)

A．Width of the tank

B．Length of the tank

C．Registered tonnage

D．Specific gravity of the liquid in the tank

什么不影响价值的自由曲面校正_________ 。

694. What is MTI _________．(A)

A．The foot/tons needed to trim the vessel by 1 inch

B．The foot/tons needed to trim the vessel by 1 foot

C．The moments needed to trim the vessel by 1 centimeter

D．The amount of vessel will trim when loading 100 tons 100 feet from the tipping Center

695. What is the purpose of a centerboard when sailing on a tack _________．(C)

A．To reduce heeling of the vessel

B．To add weight stability

C．To reduce side slip of the vessel

D．To prevent the vessel from jibing

目的是什么时，一个centerboard帆船上钉_________ 。

696. When a vessel is aground，______．(B)

A．She is underway

B．She is not underway

C．Whether she is underway cannot be determined

D．She might be or not be underway

697. When considering a vessel's stability，which spaces in a general cargo vessel are the best locations for the carriage of bulk grain _________．(A)

A．Deep tanks

B．Lower holds

C．Lower holds at the ends of the vessel

D．Tween-decks

当考虑船只的稳定，这在一般的空间货船是最好的地点，运输散粮_________ 。

698. When the height of the metacenter has the same value as the height of the center of gravity，the metacentric height is equal to ______．(D)

A．The height of the metacenter

B．The height of the center of gravity

C．The same as half the height of the metacenter

D．Zero

当高度metacenter具有相同的价值，因为高度的重心，在稳心高度等于______ 。

699. When using a stabilogauge，unless the density correction is applied，the center of gravity of a loaded compartment is assumed to be located ______．(C)

A．On the deck of the compartment

B．At one-third the height of the compartment

C．At the geometric center of the compartment

D．At one-half the height of the compartment

当使用stabilogauge ，除非密度校正应用，重心一个装舱假定位于______ 。

700. Which action will affect the trim of a vessel _________．(C)

A．Moving high weights lower

B．Adding weight at the tipping center

C．Moving a weight forward

D．All of the above

该行动将影响到装饰船只_________ 。

701. Which condition would NOT entitle a vessel to carry goods on deck _________．(B)

A．The shipper agrees to deck stowage

B．The cargo will damage other cargoes by tainting or contamination

C．Deck stowage is required by law or regulation

D．Deck stowage is customary in the trade （i．e．timber cargoes）

这些条件将不会有权将货物船只甲板上_________ 。

702. Which information is NOT required to be posted in or near the wheelhouse _________．(B)

A．Stopping time and distance from full speed while maintaining course with minimum rudder

B．A diagram of advance and transfer for turns of 30°，60°，90° and 120° at full speed with maximum rudder and constant power

C．For vessels with a fixed propeller，a table of shaft RPMs for a representative range of speeds

D．Operating instructions for change-over procedures for remote steering gear systems

这些信息并不需要张贴或附近的驾驶室_________ 。

703. Which is an advantage of using watertight longitudinal divisions in double bottom tanks _________．(A)

A．Cuts down free surface effect降低自由表面效应

B．Increases the rolling period

C．Decreases weight because extra stiffeners are unneeded

D．Lowers the center of buoyancy without decreasing GM

这是一个利用使用水密纵向司双层底液_________

704. Which is an indication of reserve buoyancy _________．(D)

A．Metacentric height

B．Righting moment

C．Rolling period

D．Freeboard

这表明储备浮力_________ 。

705. Which is the summer load waterline on the load waterline mark _________．(C)

A．The first line at the bottom

B．The second line from the bottom

C．The third line from the bottom

D．The fourth line from the bottom

这是夏季载重水线的载重水线标志_________ 。

706. Which of the following is the reason that the metacentric height is a valid measure of only the initial stability _________．(A)

A．The metacenter does not remain in the same position for large angles of inclination

B．The center of gravity shifts due to inclination

C．The center of gravity shifts above the metacenter as the vessel is inclined

D．The mean draft will change as the vessel is inclined

下列哪个是原因，稳心高度是一个有效的措施，只是初步稳定_________ 。

707. Which of the following will increase the height of the center of buoyancy for your vessel _________．(B)

A．Discharging weight from lower decks

B．Loading weight in upper decks

C．Shifting weight from lower to upper decks

D．Shifting weight from upper to lower decks

下列哪项将增加的高度中心浮力的船只_________ 。

708. Which statement is FALSE，concerning the Williamson turn _________．(D)

A．In a large vessel（VLCC）much of the headway will be lost thereby requiring little astern maneuvering

B．When the turn is completed，the vessel will be on a reciprocal course and nearly on the original track line

C．The initial actions are taken at well defined points and reduce the need for individual judgment

D．The turn will return the vessel to the man's location in the shortest possible time反过来将返回该船只的人的位置在尽可能短的时间

该声明是FALSE ，关于威廉姆森反过来_________ 。

709. Which type of ballast is most commonly used in barges and ships _________．(A)

A．Water

B．Oil

C．Concrete and barite

D．Sand，rock and gravel

哪种类型的镇流器是最常用的驳船和船舶________

710. While steering by autopilot you notice that the vessel has deviated 15 degrees from course and there is no corrective rudder being applied．As a standard operating procedure you should first ______．(C)

A．Check the setting of the rudder adjustment

B．Check the setting of the weather adjustment

C．Switch to hand steering

D．Immediately engage the trick wheel

虽然方向盘的自动驾驶仪您发现该船偏离了15度的过程，也没有纠正舵正在applied.As标准作业程序应首先______ 。

711. With damaged floating vessels，the most important consideration is the preservation of ______．(B)

A．Bilge pumping capacity

B．Reserve buoyancy

C．Level attitude

D．Instability

浮动船只受损，最重要的考虑是维护______ 。

712. You are loading in the winter in Albany，N．Y．，for a voyage to a port governed by the tropical load line mark．Which of the following statements is TRUE _________．（Hydrometer reading in Albany is 1.000）   (B)

A．You may not exceed the winter load line mark when you finish loading except for the burnout to sea

B．The freshwater allowance and burnout to sea may be subtracted from the required freeboard in Albany

C．You may calculate the burnout necessary to reach the tropical zone and load extra cargo to compensate

D．You may load to the winter mark less the fresh water allowance if you will be at the tropical mark upon arrival in the tropical zone

您正在载入中，冬季在奥尔巴尼，纽约州的一个港口航行到所管辖的热带载重线mark.Which下列报表为TRUE _________.(比重读奥尔巴尼是1.000 ）

713. You are on a cargo vessel carrying portable tanks of dangerous cargoes in bulk．Which statement is TRUE _________．(D)

A．If the tanks are pumped out while remaining on board，each hose connection must have a minimum of three bolts

B．If the tanks are off loaded，no more than two tanks may be lifted together in a hoist

C．When transferring sulfuric acid，you must display a red flag by day whether at anchor or moored

D．All electrical equipment within 10 feet horizontally must be explosion proof or intrinsically safe

您正处于一个货船运载便携式罐体的危险货物在bulk.Which说法正确_________ 。

714. You receive a package，for shipment aboard your vessel，containing Class 1 explosives．The package is damp，moldy and stained．You must ______．(A)

A．Refuse to accept the package

B．Note the exception（s）on the Bill of Lading

C．Replace the packaging material before stowage

D．Seek the approval of the USCG Captain of the Port

您会收到一个包裹，船上装运的船只，载有第1类explosives.The包潮湿，发霉和stained.You必须______ 。

715. Your vessel has been in a collision．After assessing the damage，you begin down flooding．This will cause the KB to do what _________．(C)

A．Fall

B．Remain stationary

C．Rise

D．Shift to the high side

您的船只一直在collision.After评估损害时，您就会开始下降flooding.This将导致或做_________ 。

716. Your vessel has taken a slight list from off-center loading of material on deck．The ______．(A)

A．List should be easily removed

B．Mean draft is affected

C．Vessel may flop

D．Vessel is trimmed

您的船只采取了轻微名单偏离中心装载材料deck.The ______ 。

717. Your vessel is damaged and is listing to port．The rolling period is short．There is sufficient freeboard so that deck edge submersion is not a problem．What corrective action should be taken FIRST in regard to the vessel's stability _________．(D)

A．Press up any slack double-bottom tanks to eliminate free surface

B．Flood any empty double-bottom tanks to add weight low and down

C．Jettison topside weights to reduce KG and KB

D．Shift any off-center weights from port to starboard移位任何偏离中心重量从港口向右舷

您的船只被损坏，是上市port.The滚动期short.There是足够的干舷甲板边缘，使淹没不是problem.What纠正应采取行动的第一部分，关于船只的稳定_________ 。

718. Your vessel is damaged and listing to port．There is a short rolling period around the angle of list．The port side freeboard is reduced to 1 foot．There is no trim and the weather is calm．You should FIRST ______．(C)

A．press up a slack double bottom tank on the port side

B．fill an empty centerline double bottom tank

C．pump out a slack marine portable tank located on the portside amidships

D．jettison the anchors and anchor cables

您的船只被损坏和物品port.There是一种短时期的滚动角list.The左舷舷减少到1 foot.There没有修剪和天气calm.You应首先______ 。

第九章第一节航海气象基本知识（共计72题）

719. ______ is a device used to measure atmospheric pressure．（A）

A．Barometer

B．Thermometer

C．Chronometer

D．Speedometer

______是一种装置用于测量大气压力。

720. A cloud sequence of cirrus，cirrostratus，and altostratus clouds followed by rain usually signifies the approach of a（n）______．(C)

A．Occluded front

B．Stationary front

C．Warm front

D．Cold front

云序列卷云，卷，和高层云云彩其次是雨水的办法通常标志着一个（ ñ )______.

721. A cyclone in its final stage of development is called a（n）______．(C)

A．Tornado

B．Anticyclone

C．Occluded cyclone or occluded front

D．Polar cyclone

飓风在其最后的发展阶段，被称为（ ñ )______.

722. A line of clouds，sharp changes in wind direction，and squalls are most frequently associated with a（n）______．(C)

A．Occluded front

B．Warm front

C．Cold front

D．Warm sector

一排云，急剧变化，风向，和狂风最常关联（ ñ )______.

723. A mercurial barometer at sea is subject to rapid variations in height（”pumping”）due to the pitch and roll of the vessel．To avoid this error，measurements of atmospheric pressure at sea are usually measured with a（n）______．(C)

A．Syphon barometer

B．Cistern barometer

C．Aneroid barometer

D．Fortin barometer

是个怪异的晴雨表海上受到迅速变化的高度（ “抽水” ） ，这是因为在球场和轧辊的vessel.To避免此错误，测量大气压力通常是在海上测量了（ ñ )______.

724. A rapid rise or fall of the barometer indicates ______．(C)

A．Heavy rain within six hours

B．A decrease in wind velocity

C．A change in the present weather conditions

D．That fog will soon set in

迅速上升或下降的晴雨表显示______ 。

725. A slow rise in the barometric pressure forecasts ______．(C)

A．Rainy weather for the next 48 hours

B．High seas

C．Improving weather conditions

D．Deteriorating weather conditions

缓慢上升，气压预测______ 。

726. A slow，gradual fall of the barometer indicates approaching ______．(C)

A．gale force winds within 12 hours

B．blizzard conditions

C．deteriorating or unsettled weather

D．heavy，wind driven rain

缓慢而逐渐下降的晴雨表显示接近______ 。

727. A source of an air mass labeled mTw is ______．(B)

A．The equator

B．The Gulf of Mexico

C．Alaska

D．Canada

来源的空气质量标记mTw是______ 。

728. A steep barometric gradient indicates ______．(C)

A．Calms

B．Light winds

C．Strong winds

D．Precipitation

陡峭的气压梯度表明______ 。

729. A storm is forecast for the area where your vessel is moored．For its safety you should put ______．(C)

A．more slack in the mooring lines

B．a strain on the mooring lines

C．chafing gear on the mooring lines

D．grease on the mooring lines

风暴预报的地区，您的船只的安全moored.For您应该______ 。

730. According to Buys Ballot's law，when an observer in the Northern Hemisphere experiences a northwest wind，the center of low pressure is located to the ______．(A)

A．northeast

B．west-southwest

C．northwest

D．south-southeast

根据购买选票的法律，当一名观察员在北半球经历西北风，该中心的低压位于______ 。

731. Advection fog holds longest over which portions of the lakes ________．(A)

A．Northwest

B．Southeast

C．Northeast

D．Southwest

平流雾持有时间最长的超过该部分的湖泊________ 。

732. After the passage of a cold front the visibility ______．(B)

A．does not change

B．improves rapidly

C．improves only slightly

D．becomes poor

通过后的冷锋的知名度______ 。

733. After the passage of a cold front the visibility ______．(B)

A．Does not change

B．Improves rapidly

C．Improves only slightly

D．Becomes poor

通过后的冷锋的知名度_____

734. Air temperature varies with ______．(D)

A．The altitude above sea level

B．The season of the year

C．The latitude or distance from the equator

D．All of the above

空气温度随______ 。

735. An anemometer on a moving vessel measures ______．(A)

A．apparent wind speed only

B．true wind speed and true wind direction

C．true wind speed only

D．apparent wind speed and true wind direction

一个风速在移动船只措施______ 。

736. An instrument useful in predicting fog is the ______．(A)

A．sling psychrometer

B．microbarograph

C．anemometer

D．aneroid barometer

一个工具有助于预测雾是______ 。

737. An occluded front is caused by a（n）______．(D)

A．low pressure area

B．high pressure area

C．area of calm air

D．cold front overtaking a warm front

前面是一个闭塞造成的（ ñ )______.

738. An occluded front is usually caused by a ______．(C)

A．Cold front becoming stationary

B．Warm front becoming stationary

C．Cold front overtaking a warm front

D．Warm front dissipating

一个闭塞前通常是由______ 。

739. Atmospheric pressure may be measured with a（n）______．(D)

A．barograph

B．aneroid barometer

C．mercurial barometer

D．All of the above

常压可衡量的（ ñ )______.

740. Brief，violent showers frequently accompanied by thunder and lightning are usually associated with ______．(B)

A．Passage of a warm front

B．Passage of a cold front

C．Winds shifting counterclockwise in the Northern Hemisphere

D．Stationary high pressure systems

简言之，暴力阵雨往往伴随雷电通常与______ 。

741. Cirrus clouds followed by cirrostratus then altostratus，stratus，and occasionally nimbostratus indicate the approach of a（n）______．(B)

A．cold front

B．warm front

C．tropical front

D．occluded front

Cirrus的云彩其次卷然后高层云，层云，有时雨层云的做法表明了（ ñ )______.

742. Clouds are classified according to their ______．(C)

A．Size

B．Moisture content

C．Altitude and how they were formed

D．Location in a front

云分类根据自己的______ 。

743. Clouds that form as small white flakes or scaly globular masses covering either small or large portions of the sky are ______．(D)

A．Cirrus

B．Cirrostratus

C．Altostratus

D．Cirrocumulus

云层形式的白色小片或鳞片覆盖要么球状群众小型或大型的部分天空是______ 。

744. Compared to a low pressure system，generally the air in a high is ______．(B)

A．Warmer，less dense，and less stable

B．Cool，more dense，and drier

C．Muggy and cloudy

D．Extremely moist with high relative humidity

相比，低气压系统，一般空气中高是______ 。

745. Considering the general circulation of the atmosphere，the wind system between latitudes 30°N and 60°N is commonly called the ______．(A)

A．Prevailing westerlies

B．Horse latitudes

C．Trade winds

D．Subpolar low pressure belts

考虑到一般循环的气氛，风系统北纬30 ° N和60 ° N是通常称为______ 。

746. During the winter months，the southeast trade winds are ______．(A)

A．stronger than during the summer months

B．weaker than during the summer months

C．drier than during the summer months

D．wetter than during the summer months

在冬季，东南信风是______ 。

747. Fog is formed when ______．（D）

A．The moisture in the air is condensed into small droplets

B．Air is cooled to its dew point

C．The base of a cloud is on the ground

D．All of the above

雾形成时， ______ 。

748. From which type of cloud can a tornado or waterspout develop ________．(C)

A．Nimbostratus

B．Altostratus

C．Cumulonimbus

D．Cirrus

从哪种类型的云可以龙卷风或水龙卷发展________ 。

749. High pressure and low pressure alarms must be tested ______．(A)

A．no earlier than 24 hours prior to loading

B．no later than 24 hours prior to loading

C．anytime prior to loading

D．weekly

高压和低压警报必须测试______ 。

750. Hot air can hold ______．(B)

A．less moisture than cold air

B．more moisture than cold air

C．the same amount of moisture as cold air

D．moisture independent of air temperature

热空气可以容纳______ 。

751. How can you estimate the position of a tropical storm's center ________．(D)

A．With a radio weather bulletin or weather fax

B．Using shipboard radar

C．Observe the wind direction and apply Buys Ballot's law

D．All of the above

你怎么估计的立场，一个热带风暴的中心________ 。

752. If an observer in the Northern Hemisphere faces the surface wind，the center of low pressure is to his______．(B)

A．Left，slightly behind him

B．Right，slightly behind him

C．Left，slightly in front of him

D．Right，slightly in front of him

如果一个观察员在北半球面临的地面风，该中心的低气压是his______ 。

753. In the Northern Hemisphere，an area of counterclockwise wind circulation surrounded by higher pressure is a ______．(A)

A．Low

B．High

C．Warm front

D．Cold front

在北半球，一个地区的逆时针循环风所包围高压力是______ 。

754. In the Northern Hemisphere，gusty winds shifting clockwise，a rapid drop in temperature，thunderstorms or rain squalls in summer（frequent rain/snow squalls in winter）then a rise in pressure followed by clearing skies，indicate the passage of a（n）______(D)

A．Warm front

B．Tropical cyclone

C．Anticyclone

D．Cold front

在北半球，大风转向顺时针，迅速降温，雷雨或暴雨狂风夏季（频繁降雨/雪飑冬季）再增加压力之后，结算天空，表明通过一项（ ñ )______

755. In the Northern Hemisphere，the largest waves or swells created by a typhoon or hurricane will be located ______．(C)

A．in the southeast quadrant of the storm

B．directly behind the storm center

C．forward and to the right of its course

D．behind and to the left of its course

在北半球，最大波或膨胀所造成的台风或飓风将位于______ 。

756. In the Northern Hemisphere，what type of cloud formations would you expect to see to the west of an approaching tropical wave ________．(A)

A．Cumulus clouds lined up in rows extending in a northeast to southwest direction

B．High altostratus clouds in the morning hours

C．Cirrostratus clouds lined up in rows extending in a northeast to southwest direction

D．Cirrostratus clouds lined up in rows extending in a north to south direction

在北半球，什么类型的云编队将您期望看到以西的一个接近热带波________ 。

757. In the Northern Hemisphere，winds veering sharply to the west or northwest with increasing speed are indications that a ______．(A)

A．Cold front has passed

B．Low pressure center is approaching

C．Stationary front exists

D．High pressure center has passed

在北半球，风转向大幅以西或西北的增加速度是有迹象表明， ______ 。

758. On the approach of a warm front the barometric pressure usually ______．(A)

A．falls

B．rises

C．is steady

D．is unreliable

办法，一个暖锋的气压通常______ 。

759. Pressure gradient is a measure of ______．(B)

A．a high-pressure area

B．pressure difference over horizontal distance

C．pressure difference over time

D．vertical pressure variation

压力梯度是衡量______ 。

760. Prior to reading an aneroid barometer，you should tap the face lightly with your finger to ______．(C)

A．expose any loose connections

B．demagnetize the metal elements

C．bring the pointer to its true position

D．contract and expand the glass face

之前读了无液气压表晴雨表，你应该挖掘面对轻轻用手指向______ 。

761. SHOWERY WIND MAINLY LIGHT NORTH WESTERLY SEA SLIGHT．This phrase is likely extracted from ______．(A)

A．forecast

B．synopsis

C．general Situation

D．warnings

风骤雨为主灯海SLIGHT.This西风北语可能是从______ 。

762. The barometer is an instrument for measuring the ______．(D)

A．Temperature

B．Relative humidity

C．Dew point

D．Atmospheric pressure

该指数是一个测量仪的______ 。

763. The best estimate of the wind direction at sea level can be obtained from observing the direction of the ______．(C)

A．cloud movement

B．vessel heading

C．waves

D．swells

最佳估计数风向海平面可观测的方向______ 。

764. The consistent winds blowing from the horse latitudes to the doldrums are called the ______．(C)

A．Prevailing westerlies

B．Polar easterlies

C．Trade winds

D．Roaring forties

一贯吹向从马纬度的低迷被称为______ 。

765. The fog produced by warm moist air passing over a cold surface is called ______．(D)

A．conduction fog

B．radiation fog

C．frontal fog

D．advection fog

雾产生温暖潮湿的空气通过一个冰冷的表面被称为______ 。

766.The form of cloud often known as mackerel sky which is generally associated with fair weather is ______．(C)

A．nimbostratus

B．stratus

C．cirrocumulus

D．cumulonimbus

云的形式常常被称为鲭天空通常是与公平的天气是______ 。

767. The most important information to be obtained from a barometer is the ______．(C)

A．difference between the reading of the two pointers，which shows wind direction

B．last two figures of the reading of the pointer，such as 87，76，or 92

C．present reading of the pressure，combined with the changes in pressure observed in the recent past

D．weather indications printed on the dial（such as cold，wet，etc．）under the pointer

最重要的信息，得到的晴雨表是______ 。

768. The navigable semicircle of a tropical storm in the South Indian Ocean is located on which side of the storm's track ________．(D)

A．Rear

B．Front

C．Left

D．Right

通航半圆形的热带风暴在南印度洋位于哪一方风暴的跟踪________

769. The purpose of the set” and on an aneroid barometer is to ______．(B)

A．Adjust the barometer

B．Indicate any change in the reading of the barometer

C．Provide a correction for height above sea level

D．Provide a correction for temperature changes

宗旨，设置了“和对无液气压表晴雨表是______ 。

770. The speed at which a storm centre moves is the ______．(B)

A．speed of movement

B．speed of advance

C．speed of storm

D．storm movement speed

速度，在风暴中心的举动是______ 。

771. The speed of an ocean current is dependent on ______．(A)

A．the density of the water

B．the air temperature

C．the presence of a high pressure area near it

D．underwater soil conditions

速度的一个海流依赖于______

772. The temperature at which the air is saturated with water vapor and below which condensation of water vapor will occur is referred to as the ______．(C)

A．precipitation point

B．vapor point

C．dew point

D．absolute humidity

温度在空气饱和的水蒸汽和以下的水汽凝结会出现被称为______ 。

773. The warm front is usually lifted up from the surface by ______．(A)

A．the cold front

B．heavy wind

C．high pressure

D．hurricane

在暖锋通常抬起从表面______ 。

774. To avoid error you should read the scale of an aneroid barometer with your eye placed ______．(C)

A．To the right of the pointer

B．To the left of the pointer

C．Directly in front of the pointer

D．Slightly above the meniscus

为了避免错误，您应该阅读规模的无液气压表晴雨表与你的眼睛放在______ 。

775. Tropic tides are caused by the ______．(A)

A．Moon being at its maximum declination

B．Moon crossing the equator

C．Sun and Moon both being near 0° declination

D．Moon being at perigee

热带风暴潮造成的______ 。

776. What generally occurs when the land is cooler than the nearby water ________．(A)

A．A land breeze

B．A sea breeze

C．A norther

D．A prevailing westerly

一般发生在什么土地温度低于附近水________ 。

777. What is a common unit of measure for atmospheric pressure ________．(B)

A．Centimeters

B．Inches

C．Degrees

D．Feet

什么是共同的单位为大气压________ 。

778. What occurs when rising air cools to the dew point ________．(D)

A．Advection fog forms

B．Humidity decreases

C．Winds increase

D．Clouds form

发生什么事时，上升的空气冷却到露点________ 。

779. When compared to air temperature，which factor is most useful in predicting fog ________．(B)

A．Vapor pressure

B．Dew point

C．Barometric pressure

D．Absolute humidity

当空气温度相比，这些因素是最有用的预测雾________ 。

780. When crossing a front isobars tend to ______．(A)

A．Change from smooth curves within the air mass to sharp bends at the front

B．Change from sharp bends within the air mass to smooth curves at the front

C．Pass smoothly across the front with no change

D．Become closer together at the front and pass through in straight lines

过境时前共振态往往______ 。

781. Which condition would most likely result in fog ________．(A)

A．Warm moist air blowing over cold water温暖潮湿的空气吹过冷水

B．Airborne dust particles

C．Warm moist air blowing over warm water

D．Dew point falling below the air temperature

其中条件最有可能导致雾________ 。

782. Which correction（s）must be applied to an aneroid barometer ________．(A)

A．Instrument error and height error仪器误差，高度误差

B．Instrument error only

C．Height error only

D．Instrument error and latitude correction

更正（县）必须适用于一个无液气压表晴雨表________ 。

783. Which data can NOT be found on a moisture equilibrium chart ________．(D)

A．Dry bulb temperature

B．Moisture content

C．Vapor pressure

D．Wet bulb temperature

数据不能被发现的水分平衡图表________ 。

784. Which of the following is associated with consistently high barometric pressure ________．(A)

A．The horse latitudes

B．The doldrums

C．The prevailing westerlies

D．The trade winds

以下哪项与稳定的高气压______

785. Which of the following is TRUE of advection fog ________．(D)

A．It commonly occurs on coastal waters during cold seasons

B．It moves in a bank or dense cloud

C．It is caused by warmer air moving to a cooler location

D．All of the above

下列哪项是如此平流雾________ 。

786. Which of the following statements is FALSE ________．(D)

A．An anemometer measures wind speed

B．A barometer measures atmospheric pressure

C．A thermometer measures temperature

D．A psychrometer measures wind pressure

以下哪项陈述是FALSE ________ 。

787. Which of the listed properties does warm air possess ________．(D)

A．It rises above cooler air and cools as it rises高于空气冷却器和冷却，因为它上升

B．Atmospheric pressure drops as warm air rises大气压力下降的暖空气上升

C．Moisture in warm air condenses as the air is cooled水分在温暖的空气凝结的空气冷却

D．All of the above

其中所列出的属性不暖空气拥有________ 。

788.Which scale is used to estimate wind speed by observing sea conditions ______．(D)

A．Metric scale

B．Wind scale

C．Coriolis scale

D．Beaufort scale

该表是用来估计风速观察海况______ 。

789. Which type of front forms when a cold front overtakes and forces a warm front upwards ________．(B)

A．A cold front

B．An occluded front

C．A warm front

D．A stationary front

哪种类型的前端形式冷锋时超越和部队的热烈前端向上________ 。

790. You can expect frontal activity when two air masses collide and ______．(D)

A．their barometric pressures and temperatures are the same

B．there are differences in how they track along the jet stream

C．there are no significant differences between their temperatures and moisture content

D．there are significant differences between the temperature of each air mass

您可以预期额叶活动时两个气团碰撞和______ 。

第十章船舶结构与设备 船舶结构基础知识（共计52题）

791. ______ of my ship's propeller is very strong．（B）

A．The running reflection

B．The turning effect

C．The moving speed

D．The changing sensibility

______我船螺旋桨很强。

792. A“check”line is ______．(C)

A．a safety line attached to a man working over the side

B．used to measure water depth

C．used to slow the headway of a barge

D．used to measure the overhead height of a bridge

793. A continuous watertight bulkhead on a ship may also be a/an ______．(A)

A．structural bulkhead

B．exterior bulkhead

C．centerline bulkhead

D．joiner bulkhead

连续水密舱壁船舶上也可能是/一个______ 。

794. A liner in riveted construction of a vessel is a（n）______．(A)

A．Small plate which fills the aperture between riveted strakes and the vessel framing

B．Backing plate which is used to level the strakes while riveting，and then removed

C．Internal frame to which the side shell is riveted

D．Seam that is welded after riveting is completed

班轮在铆接建造船只是（ ñ )______.

795. Allowances may be made for the expansion and contraction in piping by the use of expansion joints or ______．(D)

A．Unions

B．Retractable flanges

C．Union bulkhead fittings

D．Bends or loops in the line

津贴可用于扩大和收缩管道的使用伸缩缝或______ 。

796.Before entering the chain locker，you should ______．(D)

A．have someone standing by

B．make sure there is sufficient air within the locker

C．de-energize the windlass

D．All of the above

797.BHP stands for the ______ of the main engine．(C)

A．Breadth dimension

B．Builder's name

C．Brake horsepower

D．Revolutions per minute

必和必拓主张______的主要动力。

798. Bulkheads forming part of the tanks on a ship are stiffened to withstand ______．(C)

A．Deck loads from above

B．Dynamic forces while afloat

C．Hydrostatic pressure

D．Overpressurization

舱壁的组成部分坦克船舶上的加筋承受______ 。

799. By definition，a“spar deck”s the ______．(C)

A．lower most continuous deck not broken by water tight bulkheads

B．after most weather deck above the main strength deck

C．upper or weather deck above the main strength deck

D．deck of light construction below the main or strength deck

根据定义，一个“石甲板”第二张______ 。

800. Deck beams on a ship are generally spaced at equal intervals and run ______．(C)

A．longitudinally

B．vertically

C．transversely

D．intermittently

桥面梁船舶上一般间距相等的间隔和运行______ 。

801. Deep-sea liners are __________  (D)

A．Ferries

B．Passenger liners

C．Cruise ships

D．Ocean going vessels

深海强硬是__________

802.Deficient oxygen content inside a chain locker can be detected with ______．(D)

A．litmus paper

B．a combustible gas indicator

C．an oxygen breathing apparatus

D．an oxygen indicator

氧气含量不足链更衣室内可检测______ 。

803. Freeboard is measured from the upper edge of the ______．(B)

A．bulwark

B．deck line

C．gunwale bar

D．sheer strake

舷测量从上部边缘______ 。

804. Freeing ports on a vessel with solid bulwarks ______．(D)

A．prevent stress concentration in the bulwark

B．permit easy jettison of deck cargo in an emergency

C．provide openings through the bulwarks for mooring lines

D．allow water shipped on deck to flow off rapidly

舷船舶上的固体壁垒______ 。

805. If the buoyant force on a ship's hull is equal to the displacement tonnage，the ship will ______．(D)

A．require ballast

B．be down by the head

C．sink

D．float

如果浮力对船体等于位移吨位，船舶将______ 。

806.In ship construction，a large number of watertight bulkheads results in ______．(A)

A．increased capacity to set flooding boundaries

B．decreased capacity to set flooding boundaries

C．reduced compartmentation

D．greater deck load capacity

船舶建造，有大量的水密舱壁结果______ 。

807. In ship construction，beams are transverse girders which provide support to ______．(C)

A．bulkheads

B．deckhouse structures

C．decks

D．vertical frames

在船舶建造，梁的横向梁提供支持______ 。

808. In ship construction，bulkheads in the quarters are generally ______．(C)

A．Structural

B．Watertight

C．Non-structural

D．Continuous

在船舶建造，舱壁的季度通常是______

809. Many vessels are provided with flume tanks，which also have a dump tank located under the flume tanks． In the event the ship is damaged，you could dump the flume tanks into the dump tank which would ______．(C)

A．Reduce the free surface effect and raise the KG

B．Not have any effect on free surface and raise the KG

C．Reduce the free surface effect and lower the KG

D．Not have any effect on free surface and lower the KG

许多船只提供水槽坦克，其中也有一个垃圾罐位于水槽下的坦克。遇有船舶已损坏，您可以转储水槽坦克进入转储坦克将______ 。

810. Merchant vessels can operate in three basic ways，but ______ is not one of them．(D)

A．liners

B．tramps

C．specialized vessels

D．passenger ships

商船可以在三种基本方法，但______是不是其中之一。

811. Metal plates that cover the top of the hawsepipe are called ______．(C)

A．footings

B．plugs

C．buckler plates

D．stop waters

金属板，覆盖顶部的锚链孔被称为______ 。

812. Molded depth is measured from the ______．(A)

A．inside of the shell

B．outside of the shell

C．top of the center vertical keel

D．top of the garboard stake

模塑深度测量从______ 。

813.No deduction is allowed for the ______ mess-room in ships having passenger accommodation，which are not also provided with a passengers' mess-room．(D)

A．Officer

B．Officers

C．Officer's

D．Officers’

没有扣除允许______混乱室的船舶有乘客起居，这是不是也提供了乘客的混乱房间。

814. Panting frames are located in the ______．(C)

A．After double bottoms

B．Centerline tanks on tankers

C．Fore and after peaks

D．Forward double bottoms

索潘廷帧位于______ 。

815. Penetrations and openings in watertight bulkheads in a vessel of less than 100 gross tons must ______．(D)

A．Only be placed in transverse watertight bulkheads that extend to the bulwark deck

B．Be provided with non-packed slip joints for expansion to permit passage of piping or electric cable

C．Incorporate approved sluice valves

D．Be kept as high and as far inboard as practicable

穿透和水密舱壁的开口船只不到100总吨必须______ 。

816. Shell plating that has curvature in two directions and must be heated and hammered to shape over specially prepared forms is called ______．(B)

A．compound plate

B．furnaced plate

C．flat plate

D．rolled plate

壳板已经在两个方向弯曲，必须加热和敲定形成了专门准备的形式被称为______ 。

817. Ships are designed by __________  (C)

A．Shipowners

B．Cargoowners

C．Naval architects

D．Shipping agents

船舶的设计是_________

818. Stanchions prevent the entire deck load on a ship from being carried by the ______．(C)

A．Bulkheads

B．Stringers

C．Frames and beam brackets

D．Deck longitudinals

支柱防止整个甲板负荷船舶从正在开展的_____

819. Support of ship side plating is provided primarily by transverse ______．（C）

A．beams

B．girders

C．frames

D．bulkheads

支持船舷电镀提供主要由横向______ 。

820. The collision bulkhead shall be of ______．(A)

A．water-tight

B．air-tight

C．fire-tight

D．oil-tight

防撞舱壁应______ 。

821. The construction portfolio may be included as part of the ship ______．(B)

A．general plans

B．operating manual

C．builders documentation

D．CCS file

建设投资组合可包括在船舶______ 。

822. The distance between the bottom of the hull and the waterline is called ______．(C)

A．tonnage

B．reserve buoyancy

C．draft

D．freeboard

之间的距离底部船体和水线被称为______ 。

823.The half-breadth plan is ______．(D)

A．An endwise view of the ship's molded form

B．A plan with the forebody to right of centerline and afterbody to the left of centerline

C．A longitudinal side elevation

D．Usually drawn for the port side only

半广度计划是______ 。

824. The inner bottom is the ______．(A)

A．tank top

B．compartment between the tank top and shell of the vessel

C．inner side of the vessel's shell

D．space between two transverse bottom frames

内层底部是______ 。

825. The objective of shoring a damaged bulkhead is to ______．(B)

A．force the warped，bulged，or deformed sections back into place

B．support and hold the area in the damaged position

C．withstand subsequent additional damage

D．make a watertight seal at the damaged area

其目标是支撑损坏舱壁是______ 。

826. The percentage of the total surface area or volume of a flooded compartment that can be occupied by water caused by damage is known as ______．(C)

A．one compartment standard

B．center of flotation

C．permeability

D．form gain

总数的百分比面积或体积的洪水舱室，可被水所造成的损失被称为______ 。

827. The point that is halfway between the forward and after perpendicular and is a reference point for vessel construction is the ______．(D)

A．Half length

B．Mid-body

C．Center line

D．Amidships

一点是中间向前后垂直，是一个参考点船只建设是______ 。

828. The purpose of swash bulkheads is to ______．(D)

A．Minimize the effect of a listing condition

B．Restrict flooding within a tank

C．Separate cargoes in a common tank

D．Reduce liquid movement and surging within a tank

目的是吓唬舱壁是______ 。

829. The term gross tonnage refers to ______．(D)

A．The weight of the vessel measured in long tons

B．The weight of a vessel with all tanks full

C．The weight of a grossly overloaded vessel

D．The vessel's approximate volume including all enclosed spaces less certain exempt spaces

总吨位一词是指______ 。

830. The terms“cant frame”and“counter”are associated with the vessel's  ______．(D)

A．Cargo hatch

B．Forecastle

C．Steering engine

D．Stern

术语“不能帧”和“反”的相关船只的

831. The word “breadth of a vessel” in Intentional Regulations for Preventing Collision at Sea 1972 means ______．(D)

A．the molded breadth

B．the registered breadth

C．the breadth amidships

D．the maximum breadth

单词“广度船只”的意向条例海上避碰意味着1972年______ 。

832. Vertical partitions providing strength and compartmentation on a ship are called ______．(B)

A．decks

B．bulkheads

C．joiner work

D．walls

垂直分割提供力量和分隔船舶上被称为______ 。

833. What does the word length refer to ________．(B)

A．Length between the perpendiculars

B．Length overall

C．Waterline length

D．Register length

什么字长指________ 。

834. What is the purpose of a striker plate ________．(B)

A．Provides surface for applying force on machinery

B．Provides landing surface for the sounding bob

C．Absorbs machinery vibration

D．Prevents valve stem over-travel

的目的是什么前锋板________ 。

835. What is the purpose of cant frames in steel vessels ________．(B)

A．To support the overhang of the stern

B．To provide strength to shell plating at the stern

C．To add strength to the deck beams which support the weather decks

D．To support the plating of a cylindrical tank

的目的是什么不能张钢船只________ 。

836. What term is used to describe a river barge designed to carry coal or any similar cargo not requiring weather protection ________．(C)

A．Single skin

B．Double skin

C．Open hopper

D．Deck barge

什么长期用来描述河流驳船设计携带煤或任何类似的货物不需要天气保护________ 。

837. When the ship's bulkheads are reinforced against bending and bulging，the bulkheads are provided with ______．(C)

A．Stanchions

B．Girders

C．Stiffeners

D．Rabbits

当船舶的舱壁加强对弯曲和胀，舱壁提供______ 。

838. When towing，the least amount of tension will be on each bridle leg when the two legs ______．(B)

A．form a large angle with each other

B．form a small angle with each other

C．are of unequal length

D．are joined by a fishplate

当拖车，最少的紧张局势将在每个缰绳时腿两条腿______ 。

839. When using the hand crank on gravity davits，you should ALWAYS __________．(B)

A．make sure the hand brake is disengaged

B．put the emergency disconnect switch in the off position

C．make sure the crank is in the locked position

D．disconnect the limit switch

当使用手摇重力吊杆，你应该始终__________ 。

840. Which document shows details of a tank vessel，cargoes it may carry，manning and safety equipment required ________．(C)

A．Safety Construction Certificate

B．Cargo Gear Certificate

C．Certificate of Inspection

D．Declaration of Inspection

该文件详列坦克船只，它可携带货物，人员配备和安全设备需要________ 。

841. Which space（s）is（are）NOT exempt when measuring gross tonnage ________．(C)

A．Auxiliary machinery spaces above the deck

B．Steering gear room

C．Cargo holds

D．Galley in a deckhouse

空间（ s ）是（是）不能免除测量总吨________ 。

842. Which statement is TRUE concerning the vessel's slipstream ________．(D)

A．It has no effect on the steering of the vessel

B．It has no effect on the rudder when the helm is amidships

C．Its velocity is the same as that of the wake

D．The propeller gives it a helical motion

哪种说法正确的船只汇集________ 。

第十一章 航海仪器 （第一节航海仪器的基本使用方法 共计46题）

843.______ 20 targets can be tracked at one time．When maximum tracking capacity is reached，no further acquisitions are possible．（A）

A．As many as

B．So many as

C．As much as

D．As more as

______ 20个指标可以跟踪一个time.When最大跟踪能力达到，没有进一步的并购是可能的。

844. A Doppler log in the bottom return mode indicates the ______．(D)

A．Velocity of the current

B．Bottom characteristics

C．Depth of the water

D．Speed over the ground

多普勒日志底部返回模式表明______ 。

845. A Doppler speed log indicates speed over ground ______．(B)

A．at all times

B．in the bottom return mode

C．in the volume reverberation mode

D．only when there is no current

多普勒速度记录显示速度超过地面______ 。

846. A radar contact will remain stationary on a relative motion radar display only when it is ______．(C)

A．On the same course as your vessel

B．At the same speed as your vessel

C．On the same course and speed as your vessel

D．On a reciprocal course at the same speed as your vessel

雷达将保持固定联系的相对运动时，雷达显示器不仅是______ 。

847. A radar display in which North is always at the top of the screen is a（n）______．(B)

A．Unstabilized display

B．Stabilized display

C．Composition display

D．Relative display

雷达显示，其中北总是在屏幕的顶端是一个（ ñ )______.

848. A radar range to a small，charted object such as a light will provide a line of position in which form ________．(B)

A．straight line

B．arc

C．parabola

D．hyperbola

雷达的一个小范围，绘制对象，如轻将提供一个在线的立场形成________ 。

849. All self-propelled vessels on an international voyage must be equipped with how many Emergency Position Indicating Radio beacons（EPIRB）________．(A)

A．One approved Category I EPIRB

B．Three approved Category I EPIRBs

C．One approved Class B EPIRB

D．Two approved Class B EPIRBs

所有自行船只国际航程必须配备多少紧急位置指示无线电信标（应急无线电示位标)________.

850. An electronic depth finder operates on the principle that ______．(B)

A．radio signals reflect from a solid surface

B．sound waves travel at a constant speed through water

C．radar signals travel at a constant speed through water

D．pressure increases with depth

电子测深仪经营的原则是______ 。

851. An indirect radar echo is caused by a reflection of the main lobe of the radar beam off the observer's vessel．Which of the following is NOT a characteristic of indirect echoes ________．(B)

A．Their bearing is almost constant，even when the true bearing of the contact changes appreciably

B．They always appear on a bearing of 90℉rom the true bearing of the contact

C．The indirect echoes usually appear in shadow sectors

D．When plotted，their movements are usually abnormal

雷达回波的间接原因是反映了主瓣雷达波束从观察员的vessel.Which下列并非特点间接回声________ 。

852. Areas which cannot be scanned by radar of the vessel because they are shielded by parts of its superstructure，masts，etc means ______．(A)

A．blind sectors

B．sheltered area

C．unseen area

D．unscanable area

地区不能扫描雷达的船只，因为它们是屏蔽其部分上层建筑，桅杆等手段______ 。

853. By radar alone，you detect a vessel ahead on a collision course，about 3 miles distant．Your radar plot shows this to be a meeting situation．You should ______．(B)

A．turn to port

B．turn to starboard

C．maintain course and speed and sound the danger signal

D．maintain course and speed and sound no signal

雷达本身，你发现前面的船只上的碰撞过程，大约3英里distant.Your雷达图显示，这是一个会议situation.You应该______ 。

854. Chart information details to be used in ECDIS should be the latest edition of information originated by a government-authorized hydrographic office and conform to the standards of（the）______．(B)

A．International Maritime Organization

B．International Hydrographic Organization

C．NASA

D．US Coast Guard

图表信息的细节，以便用于电子海图系统应该是最新版的资料来源于由政府授权的水文局和符合标准的（在)______.

855. ECDIS must give an alarm for which of the following cases ________．(D)

A．When the specified limit for deviation from the planned route is exceeded

B．If the ship，within a specified time set by the watch officer，is going to cross a safety contour

C．If the ship，within a specified time set by the watch officer，is going to cross the boundary of a prohibited area

D．All of the above

电子海图系统发出警报，必须对下列哪些案件______

856. Error may be introduced into a magnetic compass by ______．(D)

A．Making a structural change to the vessel

B．A short circuit near the compass

C．Belt buckles

D．All of the above

错误可能是引入磁罗盘的______ 。

857. If ______ is shut down for an extended time optimum position fixing will take up to 2 hours after power is reapplied．(C)

A．Radar

B．DF

C．Satellite Navigator

D．Automatic Omega Navigator

如果______关闭延长时间最佳定位将长达2小时后重新权力

858. If a magnetic compass is not affected by any magnetic field other than the Earth's，which statement is TRUE ________．(A)

A．Compass error and variation are equal

B．Compass north will be true north

C．Variation will equal deviation

D．There will be no compass error

如果磁罗盘不会受到任何磁场以外的地球，哪种说法正确________ 。

859. If the acquisition symbol flashes three times when acquisition is attempted，it may be possible that the acquisition symbol ______ over the navigation mark．(A)

A．is not positioned properly

B．is not a fixed position

C．is not at a position fixed

D．is a wrong position

如果收购符号闪烁三次试图收购时，它可能会被收购象征______的航标。

860. If the gyrocompass error is east，what describes the error and the correction to be made to gyrocompass headings to obtain true headings ________．(A)

A．The readings are too low（small numerically）and the amount of the error must be added to the compass to obtain true

B．The readings are too low and the amount of the error must be subtracted from the compass to obtain true

C．The readings are too high（large numerically）and the amount of the error must be added to the compass to obtain true

D．The readings are too high and the amount of the error must be subtracted from the compass to obtain true

如果罗经错误是东，说明了什么错误和更正，以罗经标题获得真正的标题________ 。

答：A.读数过低（小数值）和数额的错误必须加入指南针获得真正的

B.的读数过低和数额的错误，必须减去指南针获得真正的

c.该读数过高（大型数控）和数额的错误必须加入指南针获得真正的

d.在读数过高和数额的错误，必须减去指南针获得真正的

861. In order to insure that the racon signal is visible on your 3 cm radar，the ______．(D)

A．10 cm radar should be placed on standby or turned off

B．gain control should be turned to maximum

C．radar should be stabilized，head up

D．rain clutter control should be off but，if necessary，may be on low

为了保证信号的racon上看到您的3厘米雷达， ______ 。

862. My gyro-compass ______ is two degrees east．(B)

A．trouble

B．error

C．wrong

D．mistake

我的陀螺罗盘______是两个学位东。

863. My gyro-compass error is ______．(B)

A．east two degrees

B．two degrees east

C．two east degrees

D．two-degrees east

我的陀螺罗盘错误是______ 。

864. The abbreviation PPI in the Sperry Collision Avoidance System means ______．(A)

A．Plan position indicator

B．Plain position indicator

C．Plane position indication

D．Policy proof of interest

生产者价格指数的缩写在斯佩里防撞系统是指______ 。

865. The database information that should be shown when a chart is first displayed on ECDIS is the ______．(B)

A．Display base information

B．Standard display information

C．System electronic nautical chart

D．Chart display information

该数据库的信息应该显示在图表上显示的电子海图系统首先是______ 。

866. The index error is determined by adjusting the ______．(D)

A．Sextant frame

B．Horizon glass

C．Index mirror

D．Micrometer drum

该指数的错误决定调整______ 。

867. The microsecond unit of time is used in radar applications． The microsecond represents which of the following ________．(A)

A．One-millionth part of a second

B．One-thousandth part of a second

C．One-hundredth part of a second

D．One-tenth part of a second

在微秒的时间单位中使用雷达的应用。微秒代表的下列哪些________ 。

868. The minimum range at which a target can be detected is basically determined by which of the following ________．(C)

A．Persistence

B．Pulse repetition rate

C．Pulse length

D．Rotation rate

最低范围内的目标时可以发现，基本上取决于下列哪些________ 。

869. The radar component that produces the synchronizing signal and triggers the indicator sweep is which of the following ________．(D)

A．Transmitter

B．Antenna system

C．Indicator

D．Modulator

该雷达组件生产同步信号，触发指标扫是以下哪个________ 。

870. The reception of weak radio signals may be improved by“opening up”the squelch control．What is the normal setting of the squelch control ________．(A)

A．Just past the point where background noise is cut off

B．Completely closed with the volume at the highest level

C．Completely open with the volume at the lowest level

D．None of the above

接收无线电信号弱可改善“开放”的压制control.What是正常设置的静噪控制________ 。

871. The signal from a ramark will show on the PPI as a ______．(C)

A．Coded signal on the same bearing and at a greater range then the transponder

B．Circle surrounding the transponder

C．Radial line from the transponder to the center of the PPI

D．Dashed circle at the same range as the transponder

信号从一个ramark将显示在生产者价格指数作为______ 。

872. The speed of sound through ocean water is nearly always ______．(A)

A．Faster than the speed of calibration for the fathometer

B．The same speed as the speed of calibration for the fathometer

C．Slower than the speed of calibration for the fathometer

D．Faster than the speed of calibration for the fathometer，unless the water is very warm

音速通过海水几乎总是______ 。

873. The system will accept a harbor map which is incorrect，therefore the operator must ______ in harbor selection．(B)

A．be considered

B．be prudent

C．be bold enough

D．be carelessness

该系统将接受港地图这是不正确的，因此，经营者必须______在港的选择。

874. The transmission time duration of a single pulse of radio-frequency energy is a function of the ______．(B)

A．persistence

B．pulse repetition rate

C．pulse length

D．resolution

传输延迟时间的单脉冲的射频能量职能之一，是______ 。

875. What will NOT induce errors into a doppler sonar log ________．(A)

A．Increased draft

B．Pitch

C．Roll

D．Change in trim

什么不会引起错误变成多普勒声纳记录________ 。

876. When navigating using GPS，what is an indicator of the geometry of the satellites that your receiver is locked onto ________．(A)

A．Horizontal Dilution of Precision

B．Selective Availability

C．Doppler Shifting

D．Precision Coding

当航行使用全球定位系统，什么是指标的几何形状的卫星，你的接收机是锁住________ 。

877. When sending and receiving messages on the marine radio，confusion over unusual words can be avoided by using the ______．(B)

A．Delimiter switch

B．Standard phonetic alphabet

C．Low power switch

D．High power switch

当传送和接收邮件的海洋电台，混淆了不同寻常的话，可避免使用______ 。

878. Which factor has the greatest effect on the amount of gain required to obtain a fathometer reading ________．(D)

A．Salinity of water

B．Temperature of water

C．Atmospheric pressure

D．Type of bottom

这些因素影响最大的数额获得需获得测深仪阅读________ 。

879. Which of the following is true when navigation with radar during restricted visibility ________．① Radar may not indicate small vessels and icebergs；② Radar may mean that safe speed is even slower due to the indication of other vessels on the scope．(C)

A．① only

B．② only

C．Both ① and ②

D．Neither ① nor ②

以下哪个是真正的导航雷达时，在有限能见度________ 。 ①雷达可能不会显示小型船只和冰山; ②雷达可能意味着安全速度慢，甚至由于说明其他船只的范围。

快闪记忆体，什么类型的轴承生产________ 。

第十二章 海上通信(共计91题)

第一节 海上通信基础知识（共计57题）

889.Chief，I see that ship ______ to her port．（B）

A．turns

B．turning

C．to turn

D．be turning

890. DO NOT，REPEAT，NOT OVER-TAKE means that you should ______．(B)

A．overtake

B．not overtake

C．overtake or not

D．decide if you should overtake

不这样做，重复，不超过取意味着您应该______ 。

891. I will wait for Utopia ______ before getting underway．(B)

A．clear

B．to clear

C．clearing

D．cleared

892. I would be very much ______ if you could arrange for a radio technician to come on board as soon as possible for my radar repair．(A)

A．obliged

B．obliging

C．oblige

D．being obliged

我将非常______如果你能安排一个无线电技师来船上尽快对我国雷达修理。

893. Ice situation expected to ______ in your position．(A)

A．deteriorate

B．bad

C．heavy

D．serous

冰情况预计______在您的立场。

894. If a davit-launched life raft aboard a vessel cannot be launched because of damage to the davit，you should ______．(B)

A．inflate the life raft on deck

B．roll the life raft over the side

C．go to another life raft station

D．get a saw and cut the life raft free

如果吊发起救生筏船上无法启动，因为损失的吊，您应该______ 。

895. If we had stayed at that port much longer，we ______ the tropical storm．(D)

A．will meet with

B．met with

C．would meet with

D．would have met with

如果我们停留在港口多长时间，我们______的热带风暴。

896. If you are transmitting a distress message by radiotelephone you should ______．(A)

A．use English language

B．always use the International Code

C．preface it by the word“Interco”

D．follow the transmission with the radio alarm signal

如果您是转递无线电话遇险电文您应该______ 。

897. If you wished to transmit a message by voice concerning the safety of navigation，you would preface it by the word ______．(C)

A．Mayday

B．Pan-pan

C．Securite

D．Safety

如果你想传递一个信息通过语音关于航行安全的，你会前言它的字______

898. If your vessel is equipped with a radiotelephone，what must also be aboard ________．(D)

A．Certificate of Inspection

B．List of ship stations

C．Copy of ship to shore channels

D．Radio station license

如果您的船只配备了无线电话，什么也必须乘坐________ 。

899. In radiotelephone communications，the prefix PAN-PAN indicates that ______．(B)

A．a ship is threatened by grave and imminent danger and requests immediate assistance

B．a calling station has an urgent message about the safety of a person

C．the message following the prefix will be about the safety of navigation

D．the message following is a meteorological warning

在无线电话通信，前缀盼盼表明______ 。

900. In the International Code of signals，a group of three letters indicates ______．(D)

A．urgency or an emergency

B．the vessel's national identity signal

C．a group from the general signal code

D．a group from the medical signal code

在国际行为守则的信号，一组三个字母表示______ 。

901. Life jackets should be marked with the ______．(D)

A．maximum weight allowed

B．stowage space assigned

C．vessel's home port

D．vessel's name

救生衣应标有______ 。

902. Life preservers should be stowed in ______．(C)

A．the forepeak

B．messrooms

C．convenient protected topside locations

D．locked watertight containers

救生具应存放在______ 。

903. My vessel ______ bad weather with winds reaching force 9 on her way home．(A)

A．encountered

B．faced

C．found

D．met

我国船只______天气恶劣，风力达到9级回家

904. Navigate with ______．Small fishing boats are within 0.8 mile of me．(D)

A．Careful

B．Skill

C．Concern

D．Caution

浏览与______.Small渔船都在0.8英里我

905. PAN-PAN repeated three times over the radiotelephone indicates which type of message will follow ________．(D)

A．Distress

B．Safety

C．All clear

D．Urgency

盼盼重复三次无线电话表明哪种类型的信息将遵循________ 。

906. Sorry to have kept you ______ for a long time．(A)

A．waiting

B．wait

C．waited

D．to wait

对不起，让您______了很长时间。

907. Take tug's towing line to ______．(B)

A．captain

B．capstan

C．capital

D．carpenter

采取拖船的牵引线______ 。

908. The command MEET HER means the helmsman should ______．(C)

A．Decrease the rudder angle

B．Steer more carefully

C．Use rudder to slow the vessel's swing

D．Note the course and steady on that heading

见到她的指挥手段的舵手应该______ 。

909. The date and time kept in the radiotelephone log shall commence at ______．(A)

A．midnight

B．noon

C．beginning of the watch

D．any convenient time

日期和时间保存在无线电话记录应在______ 。

910. The following ______ is a standard phrase．(C)

A．I may enter fairway

B．I might enter fairway

C．I will enter fairway

D．I will probably enter fairway

911. The following ______ is a standard phrase．(C)

A．You may enter fairway

B．You should enter fairway

C．It is permitted to enter fairway

D．You could enter fairway

912. The purpose of fuses in electric wiring is to ______．(B)

A．allow for cutting out branch circuits

B．prevent overloading the circuits

C．reduce voltage to the branch circuits

D．permit the use of smaller wiring for lighting circuits

目的，引信在电线是______ 。

913.The radiotelephone safety message urgently concerned with safety of a person would be prefixed by the word ______．(B)

A．Mayday

B．Pan

C．Safety

D．Interco

无线电话安全信息急需关注安全的人将这个词的开头______

914. The terminology “able to understand any order spoken by the officers” refers to all of the following situations EXCEPT ______．(C)

A．in response to a fire

B．directing the use of lifesaving equipment

C．for deck department crew members to understand the terminology used in the engine room

D．in response to a man overboard

术语“能够理解所讲的任何命令的军官”是指所有下列情况除外______ 。

915. The unit of duration of a dash in Morse Code is ______．(C)

A．one and one-half times the length of a dot

B．twice the length of a dot

C．three times the length of a dot

D．four times the length of a dot

该单位的期限破折号在摩尔斯电码是______ 。

916. The urgency signal concerning the illness of persons on board ship is ______．(B)

A．MAYDAY

B．PAN

C．SECURITE

D．ATTENTION

紧迫的信号有关的疾病的人在船上是______ 。

917. The Vessel Bridge-to-Bridge Radiotelephone Act applies to ______．(D)

A．every towing vessel of 16 feet or over in length while navigating

B．every vessel of 50 GT and upward，carrying one or more persons for hire

C．all aircraft operating on the water

D．every power-driven vessel of 20 meters and upward while navigating

该船桥到桥无线电话法案适用于______ 。

918. The Vessel Bridge-to-Bridge Radiotelephone Act applies to which towboat ________．(C)

A．A 100 GT towboat，24 feet in length

B．A 90-foot towboat tied to the pier

C．A 60-foot towboat towing by pushing ahead

D．A 400 GT towboat anchored

该船桥到桥无线电话法适用于该拖轮________ 。

919. The vessel's Emergency Position Indicating Radio beacon（EPIRB）must be tested ______．(B)

A．weekly

B．monthly

C．every 2 months

D．every 3 months

该船的紧急位置指示无线电信标（应急无线电示位标）必须测试______ 。

920. The VHF radiotelephone calling/safety/distress frequency is ______．(A)

A．156.8 MHz（channel 16）

B．156.7 MHz（channel 14）

C．156.65 MHz（channel 13）

D．156.6 MHz（channel 12）

在甚高频无线电话呼叫/安全/遇险频率是______ 。

921. There is not ______ depth of water around the area centered at the lighthouse．(B)

A．ample

B．sufficient

C．enormous

D．incredible

没有______水深周围地区为中心的灯塔。

922. These vessels are all ______ for Shanghai．(B)

A．arriving

B．bound

C．loading

D．discharging

这些船只都是______的上海。

923. What does the term TO NAVIGATE WITH CAUTION mean ________．(C)

A．To navigate intentionally

B．To navigate intensely

C．To navigate carefully

D．To navigate completely

924. What is NOT a form used by ice support services to disseminate information ________．(D)

A．Ice Analyses

B．Ice Forecasts

C．Ice Outlooks

D．Ice Bulletins

什么是不是一种利用冰的支持服务，以传播信息________

925. What is NOT required in the radiotelephone log ________．(C)

A．Distress and alarm signals transmitted or intercepted

B．Times of beginning and end of watch period

C．Routine navigational traffic

D．Daily statement about the condition of the required radiotelephone equipment

什么是不需要无线电话登入________ 。

926. What is the primary equipment for receiving MSI ________．(C)

A．SART

B．EPIRB

C．NAVTEX

D．INMARSAT-B

什么是主要设备，用于接收微星________ 。

927. What would NOT be a distress signal ________．(C)

A．MAYDAY sent by radiotelephone

B．Continuous sounding of fog horn

C．Green star shells fired from a launcher

D．Square flag and ball in a vertical line

928. What would NOT be a distress signal ________．(C)

A．MAYDAY sent by radiotelephone

B．Continuous sounding of fog horn

C．Green star shells fired from a launcher

D．Square flag and ball in a vertical line

929. When joining a vessel，do not forget personal documents and your spectacles，and ______ if necessary．(C)

A．drug

B．remedy

C．medicine

D．healer

930. When testing a Class A EPIRB you will know the EPIRB is working properly if you hear on 99.5 Mhz FM or any vacant frequency on a nearby AM radio______．(A)

A．a distinctive signal

B．the Morse Code signal for SOS

C．a computerized voice announcement of Mayday

D．your vessel's International Radio Call Sign

当测试A级应急无线电示位标你就会知道了应急无线电示位标工作正常，如果你听到的99.5赫兹调频或任何空缺频率附近的一个上午radio______

931. Which devices may be used as part of an internal communication system on a towing vessel ________．(D)

A．Sound-powered telephones

B．Portable radios

C．Either fixed or portable equipment

D．All of the above

该装置可作为一部分内部通信系统的拖曳船________ 。

932.Which EPIRB transmits a distress alert that is received and relayed by an INMARSAT satellite _______．(C)

A．Class A EPIRBs

B．Class B EPIRBs

C．L-Band EPIRBs

D．Category I EPIRBs

其中应急无线电示位标传送遇险警报说，在收到并转达了国际海事卫星组织卫星________ 。

933. Which information must automatic identification systems（AIS）automatically provide to appropriately equipped shore stations，vessels and aircraft ________．(D)

A．Vessel's type

B．Vessel's course

C．Navigational status

D．All of the above

这些信息必须自动识别系统（ AIS公司）自动提供适当的装备岸站，船舶和航空器________ 。

934. Which installed equipment must be tested and logged when a new Master assumes command ________．(D)

A．Internal communications，including sound-powered telephones

B．Navigational lights and search lights

C．Vessel control alarms

D．All of the above must be tested

其中安装的设备必须是测试和记录时，一个新的主假设命令________ 。 \

935. Which instrument is used to measure the relative humidity of the air ________．(B)

A．A hydrometer

B．A hygrometer

C．A spectrometer

D．A barograph

该仪器是用来衡量相对湿度空气________ 。

936. Which of the following statements is TRUE regarding automatic identification systems（AIS）________．(B)

A．AIS is designed to replace ARPA，maneuvering boards，and visual

bearings as a means to ascertain the risk of collision

B．AIS provides near real-time information regarding another vessel's speed over ground and heading regardless of visibility

C．AIS will not provide information on another vessel if that vessel is indistinguishable in radar sea clutter

D．AIS can be relied upon as the sole means to determine course changes due to other AIS equipped traffic

以下哪项陈述是TRUE就自动识别系统（ AIS公司)________.

答： AIS公司的目的是取代美国国防部，操纵板，与视觉

轴承作为一种手段，以确定风险的碰撞

AIS提供近实时资料，另一艘船的速度，地面和标题不论能见度

AIS将不提供资料，另一艘船，如果该船在雷达区分海杂波 AIS公司可以依靠的唯一手段，以确定过程的变化，由于其他交通设备的AIS

937. Which one of the following is the safety signal ________．(B)

A．SSS

B．TTT

C．XXX

D．FFF

938. Which radiotelephone signal indicates receipt of a distress message ________．(B)

A．Roger wilco

B．Romeo，romeo，romeo

C．SOS acknowledged

D．Mayday roger

其中无线电话信号表明收到求救讯息________ 。

939. Which signal is detected as originating from a SART ________．(B)

A．The Morse code distress series S-O-S repeated 3 times followed by DE

and the vessel's call sign

B．A line of blip code on a radar screen outward from the SART's position along its line of bearing

C．A line of blip code on a radar screen inward from the SART's position to its own ship along its line of bearing

D．None of these

这些信号检测为来自从萨尔________ 。

940. Which word is an international distress signal when transmitted by radiotelephone ________．(B)

A．Securite

B．Mayday

C．Breaker

D．Pan

这些字是一个国际求救信号时，转交无线电话________ 。

941. You are communicating on the radiotelephone using plain English．Which procedural word（proword）indicates that you have received another vessel's transmission ________．(C)

A．Out

B．Over

C．Roger

D．Wilco

你正在传输的无线电话使用纯English.Which程序字（ proword ）表示您已收到另一船只的传输___

942. You are to ______ the convoy at 1745 hours．(C)

A．Leave

B．Sail for

C．Take part in

D．Make

您正在以______车队于17时45分。

943. You hear on the radiotelephone the word“Securite”spoken three times．This indicates that ______．(A)

A．A message about the safety of navigation will follow

B．A message of an urgent nature about the safety of a ship will follow

C．The sender is in distress and requests immediate assistance

D．You should secure your radiotelephone

你听到的无线电话单词“的安全”三讲times.This表明______ 。

944. You may not act as person-in-charge of oil transfer operations on more than one vessel at a time ______．(B)

A．Under any circumstances

B．Unless authorized by the Captain of the Port

C．Unless radio communication is set up between the vessels

D．Unless the vessels are moored clear of all docks

您不得作为负责人的石油转让业务在多个船只的时间______ 。

第十二章  第三节 VHF通信（共计34题）

946. The control of distress traffic is the responsibility of _____．（D）

A．the owners

B．the charterers

C．the shipper

D．the vessel in distress or of the station that relays the distress message

遇险的控制交通的责任是_____ 。

947.______ the jumbo everything is ready for loading．(A)

A．Except

B．Excepting

C．Except for

D．Except that

948. ______ will be broadcast every one hour on VHF Channel 6．(D)

A．VHF News

B．Channel Rules

C．Visibility Time

D．Navigational Warnings

______将播出的每一个小时的甚高频频道6 。

949. ______ both anchors．(C)

A．Drag

B．Dredge

C．Weigh

D．Lie

950. A squeeze-grip type carbon dioxide portable fire extinguisher has been partially discharged．It should be (C)

A．replaced in its proper location if weight loss is no more than 15%

B．replaced in its proper location if weight loss is no more than 25%

C．labeled empty and recharged as soon as possible

D．labeled empty and replaced in its proper location regardless of weight

挤压握型二氧化碳手提式灭火器已部分discharged.It应当______ 。

951. Advise ______ your fishing gear．(B)

A．You recovery

B．You recover

C．You recovering

D．You recovered

952.Advise you ______ engines．(A)

A．stop

B．to stop

C．stopping

D．stopped

953. All VHF marine band radios operate in the simplex mode，which means that ______．(A)

A．Only one person may talk at a time

B．Only two persons may talk at the same time

C．The radio only transmits

D．The radio only receives

所有的甚高频无线电海洋波段运作模式的单纯，这意味着______ 。

954. Channel 13（156．65 MHz），the designated bridge-to-bridge channel，may NOT be used to ______．(D)

A．exchange navigational information between vessels

B．exchange navigational information between a vessel and a shore station

C．conduct necessary tests

D．exchange operating schedules with company dispatcher

第13频道（ 156.65兆赫） ，指定的桥梁，以桥通道，不得用于______ 。

955. Do you require any further assistance ? My ship is safe now and no assistance is necessary．This VHF communication generally takes place during ______．(A)

A．salvage operation

B．cargo discharging

C．bunkering

D．towing operation

你需要的任何进一步的援助？我的船是安全的现在，也没有提供援助necessary.This VHF通信通常发生

956. How can the SART's audible tone monitor be used ________．(A)

A．It informs survivors that assistance may be nearby

B．It informs survivors when the battery's charge condition has weakened

C．It informs survivors when the SART switches to the standby mode

D．It informs survivors that a nearby vessel is signaling on DSC

957. How can vessel personnel detect the operation of a SART in its vicinity ________．(A)

A．A unique radar signal consisting of a blip code radiating outward from a SART's position along its line of bearing

B．A unique two tone“warbling”signal heard on VHF-FM ch-70

C．A unique two tone alarm signal heard upon the automatic unmuting of the 2182 kHz radiotelephone automatic watch receiver

D．The SART signal appears as a target which comes and goes; the effect of heavy swells on the SART

958. I am ready ______ tow line．(D)

A．cast

B．to cast of

C．cast off

D．to cast off

959. I have been instructed that on my voyage ______，I shall call at Maderra to pick up some cargo．(A)

A．Home

B．Family

C．China

D．Returning

我已接到指示，在我的航程______,我将请在Maderra拿起一些货物。

960. If you know that the vessel you are about to call on the VHF radio maintains a radio watch on both the working and the calling frequencies，which frequency should you call on ________．(D)

A．Calling frequency

B．Distress frequency

C．Urgency frequency

D．Working frequency

如果您知道该船只您要呼叫的甚高频无线电保持无线电值班的工作都和呼叫频率，这频率应您要求______

961. In radiotelephony，the spoken word for distress is ______．(D)

A．Securite

B．Safety

C．Emergency

D．MAYDAY

962. International distress signal in VHF calling for help is ______．(B)

A．Help，Help，Help

B．Mayday，Mayday，Mayday

C．Save，Save，Save

D．Rescue，Rescue，Rescue

国际求救信号的甚高频呼救是______ 。

963. Please ______ all the lines fore and aft，the vessel is just in her position of the berth now．(B)

A．pay out

B．make fast

C．shorten

D．hold on

请______所有线路向前或向后，该船是在她的立场，现在泊位。

964. The generators on your ship have shut down，leaving you without navigation lights．Which emergency signal would you transmit over the VHF radio to alert vessels in the area of your predicament ________．(C)

A．Mayday，Mayday，Mayday

B．Pan，Pan，Pan

C．Security，Security，Security

D．Lights out，Lights out，Lights out

发电机的船舶已经关闭了，让你没有导航lights.Which紧急信号将传送的甚高频无线电，提醒船只在该地区的困境________ 。

965. The present voyage of my ship began at the port of Shanghai after loading 8 000 tons of Cargo on board and ended after discharging the Cargo at the port of Hong Kong，which was ______．(A)

A．the port of destination

B．the next port of call

C．the port of arrival

D．the last port of call

本航次开始，我国船舶在港口装货后的上海8 000吨货物在船上和结束后的货物在履行香港的港口，这是

966. There is a ______ mine in position 50°N，30°W．(A)

A．drifting

B．drifted

C．drift

D．draft

967. There is a drifting mine ______ in position 21 degrees 31 minutes North 124 degrees 20 minutes East．(C)

A．Report

B．Reports

C．Reported

D．Reporting

有漂流地雷的位置______ 21度31分北124度20分钟东。

968. VHF Radio is one of the very important navigational instruments on board，______ messages can be sent．(B)

A．by which

B．through which

C．by that

D．through that

甚高频无线电是一个非常重要的航海仪器上,______邮件可以被发送

969. What does the abbreviation VHF stand for ________．(D)

A．Vessel's Hoisting Flag

B．Very High Safety

C．Vessel's Homing Frequency

D．Very High Frequency

970. What is meant by the term“radio silence”________．(A)

A．Stations not directly involved with the on-going distress communications may not transmit on the distress frequency or channel

B．Stations remaining off the air to safeguard proprietary information

C．Two three-minute silent periods，at 15 and 45 minutes after the hour，that provide a transmitting“window”for distressed vessels to transmit distress alerts using J3E

D．Communications on a distress frequency or channel is banned for 24 hours following the cessation of the distress traffic

971. What is meant by veering the anchor chain ________．(D)

A．Bringing the anchor to short stay

B．Heaving in all the chain

C．Locking the windlass to prevent more chain from running out

D．Paying out more chain

972. What is the spoken emergency signal for a“man overboard”on the VHF radio ________．(D)

A．Man Overboard

B．Security

C．Mayday

D．Pan-Pan

什么是紧急信号讲的“人海里”的甚高频无线电________ 。

973. When a call is complete，and subsequently during an exchange of messages，a station invites a reply by saying ______．(A)

A．“Over”

B．“Out”

C．“OK”

D．“Roger”

974. When another calling channel/frequency is available，do not use ______ or other safety frequencies to make a transmission．(A)

A．2182 kHz or VHF Channel 16

B．VHF Channel 76

C．FAX

D．CABLES

当另一位致电频道/频率的情况下，不使用______或其他安全的频率进行传输。

975.When it is accepted to remain on the frequency indicated，you should say ______．(A)

A．standing by on VHF frequency

B．coming to VHF frequency

C．VHF frequency．is the best place for you to stay by

D．remaining in frequency．and do not change

接受后留在频率表示，你应该说， ______ 。

976.When it is advisable to change to another VHF channel，for example channel 15，you should say：______(B)

A．Advise（you）change to VHF frequency 15

B．Advise（you）change to VHF channel 15

C．Change to VHF channel 15 is advised

D．Change to VHF frequency 15 is advised

当最好改用另一种甚高频频道，例如频道15 ，你应该说:______

977. When it is advisable to remain on VHF channel 14 you should say ______．(A)

A．stand by on VHF channel 14

B．come to VHF channel 14

C．VHF channel 14 is the best place for you to stay by

D．remain in channel 14 and do not change

当最好留在甚高频频道， 14你应该说______ 。

978. You are making a telephone call ship-to-shore using the VHF-FM service．You can tell that the working channel is busy if you hear ______．(D)

A．Speech

B．Signaling tones

C．A busy signal

D．All of the above

您正在打电话船岸使用甚高频调频service.You可以告诉说，工作是繁忙的通道，如果你听到了______

第十四章 第一节海上应急基本知识（共计79题）

980. A“fifteen-pound”CO2 extinguisher is so called because ______．（A）

A．there are fifteen pounds of CO2 in the container

B．the container，when full，weighs fifteen pounds

C．the pressure at the discharge nozzle is l5 psi

D．the empty container weighs fifteen pounds

一个“ 15磅的”二氧化碳灭火器所谓因为______ 。

981. An ECDIS is required to display which information ________．(C)

A．Radar targets

B．ARPA vectors

C．Hydrographic data

D．All of the above

需要一个电子海图显示信息________ 。

982. Any person maintaining a listening watch on a bridge to bridge radio-telephone must be able to ______．(A)

A．Speak English

B．Repair the unit

C．Speak Spanish

D．Send Morse code

任何人保持听值班的桥梁，桥无线电话必须能够______ 。

983. As Chief Officer of a vessel underway，it comes to your attention that the vessel is，in some manner，unseaworthy．Under such circumstances the Master is required to take action upon receiving ______．(B)

A．Information of such condition from yourself

B．Notification of such condition from yourself and the Second Officer

C．Notification of such condition from yourself and any other member of the crew

D．Notification of such condition from yourself or the Second Officer

作为主任的船只进行，涉及到你注意该船，以某种方式， unseaworthy.Under这种情况下，船长必须采取行动，在收到______ 。

984. AS SOON AS I HAVE FURTHER INFORMATION I WILL MAKE ANOTHER ANNOUNCEMENT．I ASK YOU KINDLY TO REMAIN CALM．THERE IS NO DANGER AT THIS TIME．These words are likely from __________．(A)

A．the captain of a vessel in distress

B．representative of ship owners

C．an officer from port authorities

D．the pilot of a vessel entering a port in foreign country

当我进一步了解我会进行另一次ANNOUNCEMENT.I请将继续CALM.THERE什么危险在此TIME.These话很可能从__________ 。

985. By regulation，orange smoke distress signals will expire not more than how many months from the date of manufacture ________．(C)

A．24 months

B．36 months

C．42 months

D．54 months

条例，橙色烟雾求救信号就会过期不超过多少个月内制造________ 。

986. ECDIS must be able to perform all of the following EXCEPT ______．(B)

A．Determine true bearing and distance between two geographical points

B．Determine magnetic compass deviation

C．Transform a local datum to the WGS-'84 datum

D．Convert “graphical coordinates” to “display coordinates”

电子海图系统必须能够执行所有下列除外______ 。

987. Epilepsy is a chronic nervous disorder characterized by ______．(B)

A．Severe nausea and cramps

B．Muscular convulsions with partial or complete loss of consciousness

C．Sudden thirst and craving for candy

D．Severe agitation and desire to get out of closed spaces

癫痫是一种慢性中枢神经紊乱特点是______ 。

988. EXCEPT when suffering from a head or chest injury a patient in shock should be placed in which position ________．(B)

A．Head up and feet down

B．Head down and feet up

C．Flat on back with head and feet elevated

D．Arms above the head

除非痛苦的头部或胸部损伤病人休克应放在什么立场________ 。

989. Frapping lines are fitted to lifeboat davits to ______．(D)

A．secure the lifeboat in the davits when in the stowed position

B．bring the lifeboat close along the rail in the embarkation position

C．give the occupants a safety line when the boat is being lowered from the embarkation level

D．reduce the swinging of the lifeboat at the embarkation level

Frapping线安装在救生艇吊艇架，以______ 。

990. General requirements for a vessel's radiotelephone station log are that ______．(D)

A．logs must be kept in an orderly manner

B．erasures are not allowed

C．it must identify the vessel's name and official number

D．All of the above

一般要求的船舶电台日志是______ 。

991.Generally，when lifting an inflatable liferaft back aboard ship，you would use the ______．(A)

A．towing bridle

B．main weather cover

C．external lifelines

D．righting strap

一般而言，当解除了气胀式救生筏回到船上，你会使用______ 。

992. Given the same water depth and line tension，the holding power of a 19 pound/foot wire rope mooring system in comparison to the holding power of a 90 pound/foot chain mooring system will be ______．(A)

A．Weaker

B．Equal

C．Greater

D．Indeterminable

给予同样的水深和线路紧张，举行权力十九磅/英尺钢丝绳系泊系统相比，掌握权力的九十○英镑/英尺链系泊系统将____

993. High concentrations of H2S gas are most dangerous to personnel because they can ______．(B)

A．Cause involuntary muscle contractions

B．Paralyze your breathing system

C．Cause eye inflammation

D．Cause dizziness

高浓度的硫化氢气体是最危险的人员，因为他们可以______ 。

994. How do you determine the weight of the vessel that is supported by the ground when a vessel has run aground ________．(C)

A．This requires extensive calculation and is usually performed only by a naval architect not by a ship's officer

B．Determine the point where aground and the draft at that point，then calculate it using the grounding formula

C．Use the hydrostatic tables and enter with the mean draft before grounding and the mean draft after grounding

D．Use the inclining experiment formula and substitute the change of trim for the angle of list

你怎么确定的重量船只所支持的地面时，船只搁浅________ 。 \

995. How many life buoys ______ on board with self-igniting light ________．(D)

A．there exist

B．there is

C．there are

D．are there

996. If a man falls overboard from a rig under tow，you should FIRST ______．(B)

A．notify the tug

B．deploy life buoys

C．launch a boat

D．sound the general alarm

如果一个人属于从钻井船拖走，您应首先______ 。

997. If a person is unconscious from electric shock，you should first remove him from the electrical source and then ______．(C)

A．administer ammonia smelling salts

B．check for serious burns on the body

C．determine if he is breathing

D．massage vigorously to restore circulation

如果一个人昏迷不醒触电，应首先消除他的电源，然后______ 。

998. If a victim is unconscious，you should first look for evidence of ______．(D)

A．high fever

B．head injury

C．broken limbs

D．irregular breathing

如果受害人是无意识的，应首先寻找证据______ 。

999. If you hear the firing of a gun at one minute intervals from another vessel，this indicates that ______．(B)

A．the gun is being used to sound passing signals

B．the vessel is in distress

C．all vessels are to clear the area

D．all is clear and it is safe to pass

如果你能听到发射枪在一分钟的间隔从另一艘船，这表明______ 。

1000. If you log a distress message，it must include the ______．(C)

A．sea state

B．names of witnesses

C．time of its occurrence

D．wind direction and velocity

1001. If you see someone fall overboard from a ship，you should ______．(B)

A．immediately jump in the water to help the individual

B．call for help and keep the individual in sight

C．run to the radio room to send an emergency message

D．go to the control room for the distress flare

1002. If you see someone fall overboard from a vessel，you should ______．(B)

A．immediately jump in the water to help the individual

B．call for help and keep the individual in sight

C．run to the radio room to send an emergency message

D．go to the control room for the distress flares

如果你看到有人从秋季海里的船只，您应该______ 。

1003.If your vessel is underway in fog and you hear one prolonged and three short blasts，this indicates a ___(D)

A．vessel not under command

B．sailing vessel

C．vessel in distress

D．vessel being towed

如果您的船只是在大雾之中，你听到一个长期和短期三个爆炸，这表明______ 。

1004. In an emergency，the electro-hydraulic steering units can be directly controlled by the ______．(A)

A．Trick wheel

B．Rapson slide

C．Follow-up gear

D．Receiver unit

在紧急情况下，电液指导单位可直接控制的______ 。

1005. In order to retrieve an inflatable life raft and place it on deck，you should heave on the ______．(D)

A．lifelines

B．righting strap

C．sea anchor

D．towing bridle

为了检索救生筏，并将其放置在甲板上，你应该鼓的______ 。

1006. Injuries resulting in loss of life or incapacitation，aboard vessels，must be reported to the ______．(C)

A．MOC

B．CCS

C．MSA OF CHINA

D．International Association of Navigation

损伤导致生命损失或失常，乘坐的船只，必须向______

1007. Lines passed around the falls to hold the boat while passengers are boarding are ______．(B)

A．life lines

B．frapping lines

C．tricing lines

D．tripping lines

线路通过围绕瀑布举行，而该船的乘客登机是______ 。

1008. On a ship，when may a work vest be substituted for a required life preserver ________．(D)

A．To replace a damaged life preserver

B．For use during fire drills

C．For use during boat drills

D．At no time

船舶上，工作时可背心取代必要的救生衣________

1009. On every vessel，distress signals must be stowed ______．(A)

A．on or near the navigating bridge

B．on the flying bridge not closer than 15 feet to any bulkhead

C．above the freeboard deck away from heat

D．in an enclosed space below the freeboard deck away from heat

对每艘船只，遇险信号必须存放______ 。

1010. Overhauling a fire in the living quarters on a vessel must include ______．(A)

A．opening dead spaces to check for heat or fire

B．evacuation of the vessel

C．sounding the “all clear” signal

D．operation of the emergency generator

检修的火灾中宿舍的船只必须包括____

1011. Persons who have swallowed a non-petroleum based poison are given large quantities of warm soapy water or warm salt water to ______．(A)

A．induce vomiting

B．absorb the poison from the blood

C．neutralize the poison in the blood

D．increase the digestive process and eliminate the poison

人谁也吞下了非石油基的毒药，给出了大量的热情温暖的肥皂水或盐水，以______ 。

1012. Severe airway burns can cause ______．(C)

A．nausea

B．reddening of cheeks

C．complete obstruction of respiratory passages

D．nosebleed

严重呼吸道烧伤可造成______ 。

1014. The color of rockets，shells，or rocket parachute flares used to indicate that the vessel is in distress and requires immediate assistance is ______．(C)

A．white

B．green

C．red

D．yellow

颜色的火箭，炮弹或火箭弹的降落伞用来表明该船只遇险，需要紧急援助是______

1015.. The distress message of a ship should include considerable information which might facilitate the rescue．This information should ______．(C)

A．ALWAYS be included in the initial distress message

B．be sent to a Coast Guard station FIRST

C．be transmitted as a series of short messages，if time allows

D．include the vessel's draft

遇险船舶的信息应包括大量的信息，可能有助于rescue.This资料应当______ 。

1016. The distress message of a ship should include considerable information which might facilitate the rescue．This information should ______．(C)

A．ALWAYS be included in the initial distress message

B．be sent to a MSA station FIRST

C．be transmitted as a series of short messages，if time allows

D．include the vessel's draft

遇险船舶的信息应包括大量的信息，可能有助于rescue.This资料应当______ 。

1017. The highest level of commercial navigational accuracy is provided by ______．(A)

A．DGPS，within a coverage area

B．SPS，without selective availability

C．PPS，without selective availability

D．NAVSAT，using the Doppler-shift

最高级别的商业航行的准确性提供了______ 。

1018. The rated operating time of a self-contained breathing device may be reduced in actual use because of ______．(C)

A．pressure differences in the atmosphere

B．the length of the hose attached to the facepiece

C．the physical exertion of the person wearing the device

D．spaces containing poisonous vapors

额定工作时间，自行呼吸设备可能会减少，因为在实际使用的______ 。

1019. The regulations require that inspected vessels on an international voyage，other than small passenger vessels，must carry which of the following distress signals on or near the navigating bridge ________．(B)

A．12 hand red flares

B．12 rocket parachute flares

C．12 hand combination flares and orange smoke signals

D．6 hand red flares，and 6 hand orange smoke signals

该条例规定，检查船只的国际航程，小客车以外的其他船只，必须履行下列哪些遇险信号或附近航

1021. Thirty-five percent of the breaking strength of an anchor cable is generally accepted as the ______．(A)

A．Safe operating load

B．Normal operating tension

C．Emergency working load

D．Allowable storm load

百分之五的断裂强度的锚索被普遍接受的______ 。

1022. Three short blasts of the whistle means ______．(D)

A．danger

B．I am in distress

C．the vessel is not under command（broken down）

D．I am operating astern propulsion

1023. Unless there is danger of further injury，a person with a compound fracture should not be moved until bleeding is controlled and ______．(B)

A．The bone has been set

B．The fracture is immobilized

C．Radio advice has been obtained

D．The wound has been washed

除非有进一步的损伤的危险，一个人的复杂骨折不应该转移到出血控制和______

1024. What can be determined about an injury from examining the condition of a victim's pupils ________．(C)

A．The degree of pain being suffered

B．The degree of vision impairment

C．Whether or not the brain is functioning properly

D．Whether or not the victim's blood pressure is normal

如何才能确定一个伤害审查的条件的受害者的学生________

1025. What is a convenient and effective system of examining the body of an injury victim ________．(A)

A．Check the corresponding（left versus right）parts of the body

B．Watch the patient's eyes as you probe parts of the body

C．Look for discoloration of the patient's skin

D．Look for uncontrolled vibration or twitching of parts of the body

什么是一个便捷有效的制度，检查身体的伤害受害者________ 。

1026. What is a FALSE statement concerning the line throwing appliance on a vessel ________．(C)

A．A drill on its use must be held once every three months

B．The actual firing is at the discretion of the Master

C．The auxiliary line must be of a light color

D．The auxiliary line must be at least 500m long

什么是虚假陈述有关投掷线设备的船只________ 。

1027. What is NOT a distress signal ________．(C)

A．A continuous sounding of the fog horn

B．Firing a gun every minute

C．Five or more short rapid blasts on the whistle

D．A square flag and ball flown from the mast

1028. What is NOT a distress signal ________．(D)

A．Red flares or red rockets

B．Continuous sounding of fog signaling apparatus

C．International Code Flags “November” and “Charlie”

D．Basket hanging in the rigging

1029. What is NOT generally a characteristic of inhaling light naphtha ________．(C)

A．Dizziness

B．Headache

C．Vomiting

D．Coughing

什么是一般没有的一个特点吸入轻石脑油______

1030. What is required to launch a boat stowed in a crescent davit ________．(D)

A．Hoist the boat clear of the cradle

B．Release the outboard part of the cradle

C．Rig the tricing lines

D．Crank the crescent out

所需要的发射艇存放在月牙吊________ 。

1031. What must be accurately determined to assess the potential for progressive flooding after a vessel has been damaged ________．(A)

A．The integrity of the water tight boundaries

B．The capacity of the water sprinkler systems

C．The operation of the machinery space bilge level alarms

D．All of the above

什么必须准确地确定评估的潜力，逐步水浸后，船只被损坏________ 。

1032. What should you do with your emergency position indicating radio beacon if you are in a lifeboat during storm conditions ________．(A)

A．Bring it inside the liferaft and leave it on

B．Bring it inside the liferaft and turn it off until the storm passes

C．Leave it outside the liferaft and leave it on．

D．Leave it outside the liferaft and turn it off

您应该做些什么您的紧急位置指示无线电信标如果您是在救生艇在风暴条件________ 。

1033. When a vessel signals her distress by means of a gun or other explosive signal，the firing should be at intervals of approximately ______．(B)

A．10 minutes

B．1 minute

C．1 hour

D．3 minutes

当某船只求救信号，她通过一个枪或其他爆炸信号，发射的时间间隔应约______

1034. When hoisting personnel from a vessel，with the ship cranes，the crane operator must assure that ______(C)

A．The personnel carrier remains directly over the vessel

B．All personnel are inside the net type carrier

C．The personnel carrier remains directly over water

D．He does not swing the load until it is above the landing area

当悬挂人员从一个容器，与船用起重机，起重机操作员必须保证______ 。

1035. When it is necessary to remove a victim from a life threatening situation，the person giving first aid must ______．(B)

A．Pull the victim by the feet

B．Avoid subjecting the victim to any unnecessary disturbance

C．Carry the victim to a location where injuries can be assessed

D．Place the victim on a stretcher before attempting removal

在有需要时移除受害者从生命威胁的情况下，给急救人员必须______ 。

1037. When sending a DSC call，the vessel's ______．(B)

A．position will automatically be sent

B．position will only be sent if the vessel is making a distress call

C．MMSI will indicate its ocean region

D．None of the above

1038. When should distress flares and rockets be used ________．(B)

A．Immediately upon abandoning the vessel

B．Only when there is a chance of their being seen by a rescue vessel

C．At one hour intervals

D．At half hour intervals

当应遇险火箭弹和使用________ 。

1039. When the transmissions of a radio station or a Decca chain，etc．，have broken down，switched off or suspended，it is ______(D)

A．Unfunctional

B．Break down

C．Off power

D．Off air

当传输的广播电台或迪卡链等，已遭到破坏，关闭或暂停，这是______

1040. When treating a chemical burn，you should flood the burned area for at least ______．(A)

A．Five minutes

B．Ten minutes

C．Fifteen minutes

D．Twenty minutes

当治疗化学烧伤，你应该防洪烧伤面积至少______ 。

1041. When using the marine radiotelephone，what frequency is not commonly used for transmission of distress signals ________．(D)

A．2182 kHz

B．156.8 MHz

C．Channel 16

D．Channel 66

当使用海洋无线电话，什么频率不常用的传送遇险信号________ 。

1042. Which data must ECDIS be able to record at one-minute intervals ________．(A)

A．Course made good history

B．Estimated time of arrival

C．Speed through the water

D．Shaft RPM

电子海图系统的数据必须能够记录在一分钟的间隔________ 。

1043. Which is a distress signal ________．(D)

A．A triangular flag above or below a ball

B．The International Code Signal of distress indicated by“JV”

C．A green smoke signal

D．Flames on the vessel as from a burning tar barrel

1044. Which is NOT a distress signal ________．(D)

A．A continuous sounding with any fog signal apparatus

B．A signal sent by radiotelephone consisting of the spoken word Mayday

C．An International Code Signal of N．C．

D．The firing of green star rockets or shells

1045. Which lifesaving equipment must be tested monthly?(B)

A．Inflatable PFD's

B．EPIRB

C．Hydrostatic releases

D．Dated batteries

其中救生设备测试必须每月？

1046. Which of the following statements regarding low expansion foam and its application is INCORRECT ________．(D)

A．Foam should not be used on electrical fires

B．One kilo of low expansion foam solution produces much more foam

C．Foam is only efficient when it covers the top of burning combustibles

D．A stream of foam should be deflected off the deck in order to best agitate the fire

以下哪项陈述低膨胀泡沫及其应用不正确________ 。

1047. Which procedure should be followed when individuals are rescued in cold climates and suffer from hypothermia ________．(B)

A．Give them brandy or other alcoholic stimulation to promote a return to an acceptable body temperature

B．Move them to a warm room to gradually raise their body temperature

C．Keep them moving to stimulate circulation to raise their body temperature

D．Warm them under an electric blanket to rapidly regain normal body temperature

这些程序时应遵循个人救出在寒冷的气候和遭受低温________ 。

1048. Which signal is recognized as a distress signal ________．(B)

A．Directing the beam of a searchlight at another vessel：指导束探照灯在另一艘船只

B．A smoke signal giving off orange colored smoke烟雾信号发出橙色彩色烟雾

C．A whistle signal of one prolonged and three short blasts

D．International Code Signal PAN spoken over the radiotelephone

这些信号被认为是一种求救信号________ 。

1049. Which statement about transmitting distress messages by radiotelephone is INCORRECT ________．(B)

A．Distress messages should first be transmitted on 2182 kHz

B．It is advisable to follow a distress message on 2182 kHz by two dashes of 10 to 15 seconds duration

C．If no answer is received on the designated distress frequencies，repeat the distress call on any frequency available

D．Channel 16（156.8 MHz）may be used for distress messages

该声明转递无线电话遇险的讯息不正确________ 。

1051. Without ______ to the foregoing，every such servant，agent and subcontractor shall have the benefit of all provisions contained herein benefiting the carrier as if such provisions were expressly for their benefit；and in entering into this contract，the carrier，to the extent of those provisions does so not only on its own behalf，but also as agent and trustee for such servants，agents and sub-contractors．(D)

A．Damage

B．Injury

C．Break

D．Prejudice

如果______前述，每个这样的受雇人，代理人和分包商应具有的利益，在此包含的所有条款有利于承运人如果这些规定被明确为他们的利益;并在进入本合同，承运人的范围内，这些规定这样做，不仅以自己的名义，而且还作为代理人和委托人为这些人员，代理人和分包商。

1052. You are calling another vessel by radiotelephone using the International Code of Signals．He responds with the words Alpha Sierra．This indicates that ______．(A)

A．he cannot accept traffic immediately

B．you should proceed with your message

C．you should send your message in International Code

D．you should send your message in plain language

您正在呼吁另一船只的无线电话使用国际行为守则的Signals.He回应的话阿尔法Sierra.This表明______ 。

1053. You are on anchor watch．As an aid to preventing thievery on the vessel you should ______．(D)

A．Show running lights，anchor lights and deck lights

B．Only show the required anchor lights and keep the rest of the vessel darkened

C．Maintain water on deck with fire hoses led out and all-purpose nozzles attached

D．Show anchor lights，deck lights and cargo lights hung over the vessel's side

您正在对锚watch.As援助防止偷窃船上您应该______ 。

1054.You receive a call from the MSA OF CHINA addressed to all stations．The call begins with the words Pan-Pan（3 times）．Which type of emergency signal would this be ________．(B)

A．Safety signal

B．Urgency signal

C．Distress signal

D．Red alert signal

您收到的电话的生活津贴的中国给所有stations.The要求开始改为盼盼（ 3倍） 。哪种类型的紧急信号将这是________ 。

1055. You should FIRST treat a simple fracture by ______．(B)

A．Attempting to set the fracture

B．Preventing further movement of the bone

C．Applying a tourniquet

D．Alternately applying hot and cold compresses

您应该首先处理一个简单的骨折______ 。

1056. You should NOT use a power tool if ______．(B)

A．It has a three-prong plug

B．The insulation of the power wires is worn

C．Hand tools can be used instead

D．The power source is alternating current

你不应该使用权力的工具，如果______ 。

1057. Your explanation for eruption of the disease ______ all right．(A)

A．Sounds

B．Have sounded

C．Is sounding

D．Is sounded

1058. Your vessel has lifeboats on both sides．Lifeboat No.2 is located ______．(B)

A．forward of lifeboat No.4 on the starboard side

B．forward of lifeboat No.4 on the port side

C．aft of lifeboat No.1 on the starboard side

D．All of the above