# 航海英语三副新题库

02.3 航海出版物（灯标表、天文历、海员手册、大洋航路等）

[201]The Mariner's Handbook ______ general information affecting navigation and is complementary to the Sailing Directions．

A.　giving

B.　give

C.　gives

D.　given

KEY:　C

[202]Information on the operating times and characteristics of foreign radiobeacons can be found in which publication? ______.

A.　List of Lights

B.　Coast Pilot

C.　Sailing Directions

D.　List of Radiobeacons

KEY:　A

[203]A lashing used to secure two barges side by side，lashed in an "X" fashion，is called a ______.

A.　scissor wire

B.　towing wire

C.　breast wire

D.　cross wire

KEY:　A

[204]A lashing used to secure two barges side by side，lashed in an "X" fashion，is called a ______.

A.　quarter line

B.　back line

C.　peg line

D.　jockey line

KEY:　D

[205]A latitude line will be obtained by observing a body ______.

A.　on the prime vertical

B.　on the celestial horizon

C.　at lower transit

D.　on the Greenwich meridian

KEY:　C

[206]A life float on a fishing vessel must be equipped with ______.

A.　a painter

B.　red smoke flares

C.　a jackknife

D.　a signal mirror

KEY:　A

[207]A life preserver or buoyant work vest is required to be worn on a ship when a person is ______.

A.　working on the rig floor

B.　working over water

C.　working on the pipe racks

D.　operating line throwing equipment

KEY:　B

[208]A lifeline must be connected to the liferaft ______.

A.　at the bow

B.　at the stern

C.　in the middle

D.　all around

KEY:　D

[209]A liferaft which has inflated bottom-up on the water ______.

A.　should be righted by standing on the carbon dioxide cylinder，holding the righting straps and leaning backwards

B.　should be righted by standing on the life line，holding the righting straps leaning backwards

C.　will right itself when the canopy tube inflates

D.　must be cleared of the buoyant equipment before it will right itself

KEY:　A

[210]A light signal consisting of three flashes means ______.

A.　I am in doubt as to your actions

B.　My engines are full speed astern

C.　I desire to overtake you

D.　I am operating astern propulsion

KEY:　D

[211]A light used to signal passing intentions must be an ______.

A.　all-round yellow light only

B.　all-round white light only

C.　all-round blue light only

D.　alternating red and yellow light

KEY:　B

[212]A line of position derived from a loran reading is a section of a(n) ______.

A.　straight line

B.　arc

C.　parabola

D.　hyperbola

KEY:　D

[213]A line of position from a celestial observation is a segment of a ______.

A.　circle of equal altitude

B.　parallel of declination

C.　parallel of altitude

D.　vertical circle

KEY:　A

[214]A list man would expect to see any of the following light signals EXCEPT ______.

A.　two red lights

B.　one green light

C.　one white light

D.　one amber light

KEY:　D

[215]A list signal of one white light indicates that the vessel is ______.

A.　on an even keel

B.　listing about 1 to the side the light is displayed on

C.　listing over 5With the light displayed on the high side

D.　listing，and the chutes should be directed to the side where the light is displayed

KEY:　B

[216]A low HDOP (Horizontal Dilution of Precision) number such as 2 indicates a ______.

A.　poor fix

B.　good fix

C.　poor signal quality

D.　good signal quality

KEY:　B

[217]A low pressure annular preventer which is used to direct flow of kick fluids away from the rig floor is called a ______.

A.　dump valve

B.　deflector

C.　separator

D.　diverter

KEY:　D

[218]A major health hazard of the product tert-butylamine is that it ______.

A.　can be absorbed through the skin

B.　causes irreversible damage to eye tissue

C.　is a very unstable product

D.　All of the above

KEY:　B

[219]A marine chemist issues gas free certificates and is certified by which organization?

A.　Mine Safety Appliance Association

B.　American Chemical Society

C.　Marine Chemists Association

D.　National Fire Protection Association

KEY:　D

[220]A mean sun is used as the reference for solar time for three reasons. Which reason is NOT a cause for use of a mean sun?

A.　The motion of the apparent sun is along the ecliptic.

B.　Measurement of time is along the celestial equator.

C.　The speed of the Earth's revolution is not constant.

D.　There are variations in the Earth's rotational speed.

KEY:　B

[221]A message by flashing light consists of the call，the identity，the text，and the ______.

A.　destination

B.　answer

C.　time

D.　ending

KEY:　D

[222]A mobile offshore drilling unit must display obstruction lights when it is on the waters over the Outer Continental Shelf and is ______.

A.　under tow

B.　fixed to the seabed

C.　underway

D.　All of the above

KEY:　B

[223]A mobile offshore drilling unit under tow is approaching a fog bank. When should fog signals be started?

A.　When the towing vessel enters the fog

B.　When the drilling rig enters the fog

C.　When the towing vessel can no longer be seen from the rig

D.　Immediately

KEY:　D

[224]A mobile offshore drilling unit will show the day-shape in DIAGRAM 10 to indicate that it is ______.

A.　at anchor

B.　being towed

C.　aground

D.　drilling

KEY:　B

[225]A monkey fist is found on a ______.

A.　heaving line

B.　lead line

C.　manrope

D.　mooring line

KEY:　A

[226]A mooring line leading 45 to the keel，used to check forward or astern movement of a vessel，is called a ______.

A.　spring line

B.　warp line

C.　bow line

D.　breast line

KEY:　A

[227]A mooring line leading at nearly right angles to the keel is a ______.

A.　spring line

B.　bow line

C.　stern line

D.　breast line

KEY:　D

[228]A natural fiber rope can be ruined by dampness because it may ______.

A.　rot

B.　shrink

C.　stretch

D.　unlay

KEY:　A

[229]A navigator fixing a vessel's position by radar ______.

A.　can use radar information from one object to fix the position

B.　should never use radar bearings

C.　should only use radar bearings when the range exceeds the distance to the horizon

D.　must use information from targets forward of the beam

KEY:　A

[230]A negative metacentric height ______.

A.　should always be immediately corrected

B.　will always cause a vessel to capsize

C.　always results from off-center weights

D.　All of the above are correct

KEY:　A

[231]A petroleum liquid has a flash point of 85F. This is a grade ______.

A.　B flammable liquid

B.　C flammable liquid

C.　D combustible liquid

D.　E combustible liquid

KEY:　C

[232]A pilot chart does NOT contain information about ______.

A.　average wind conditions

B.　tidal currents

C.　magnetic variation

D.　average limits of field ice

KEY:　B

[233]A pilot vessel may continue to sound an identity signal if she is ______.

A.　aground

B.　at anchor

C.　not under command

D.　being towed

KEY:　B

[234]A pilot vessel on pilotage duty at night will show sidelights and a stern light ______.

A.　when at anchor

B.　only when making way

C.　at any time when underway

D.　only when the identifying lights are not being shown

KEY:　C

[235]A polyconic projection is based on a ______.

A.　plane tangent at one point

B.　cylinder tangent at one parallel

C.　cone tangent at one parallel

D.　series of cones tangent at selected parallels

KEY:　D

[236]A position obtained by applying ONLY your vessel's course and speed to a known position is a ______.

A.　fix

B.　dead-reckoning position

C.　running fix

D.　probable position

KEY:　B

[237]A position obtained by crossing lines of position taken at different times and advanced to a common time is a(n) ______.

A.　dead-reckoning position

B.　running fix

C.　estimated position

D.　fix

KEY:　B

[238]A position that is obtained by applying estimated current and wind to your vessel's course and speed is a(n) ______.

A.　estimated position

B.　dead reckoning position

C.　fix

D.　None of the above

KEY:　A

[239]A position that is obtained by using two or more intersecting lines of position taken at nearly the same time，is a(n) ______.

A.　fix

B.　running fix

C.　estimated position

D.　dead-reckoning position

KEY:　A

[240]A precaution you should take before bunkering is to ______

A.　plug the vents

B.　plug the sounding pipes

C.　plug the scuppers

D.　close the lids on the vents

KEY:　C

[241]A predictable result of a vessel nearing a bank or edge of a channel is that the ______.

A.　stern is drawn to the bank as the bow sheers off

B.　bow sheers toward the bank

C.　vessel continues in a straight line，but with greatly reduced maneuverability

D.　vessel will be drawn bodily into the bank unless the engines are stopped

KEY:　A

[242]A psychrometer has two thermometers that provide dry bulb and wet bulb temperatures. By comparing these two temperature readings with a set of tables you can determine the ______.

A.　atmospheric pressure

B.　wind speed

C.　relative humidity and dew point

D.　wind chill factor

KEY:　C

[243]A pumproom is suspected of accumulating gases after a ventilation machinery breakdown. Where should the combustible gas indicator case be placed when testing the pumproom atmosphere for combustible gases?

A.　In the lower level of the pumproom

B.　In the middle level of the pumproom

C.　In the upper level of the pumproom

D.　On the deck outside the pumproom

KEY:　D

[244]A quick and rapid motion of a ship in a seaway is an indication of a(n) ______.

A.　large GM

B.　high center of gravity

C.　excessive free surface

D.　small GZ

KEY:　A

[245]A radar range to a small，charted object such as a light will provide a line of position in which form?

A.　Straight line

B.　Arc

C.　Parabola

D.　Hyperbola

KEY:　B

[246]A rapid rise or fall of the barometer indicates ______.

A.　heavy rain within six hours

B.　a decrease in wind velocity

C.　a change in the present weather conditions

D.　that fog will soon set in

KEY:　C

[247]A red triangular daymark marks ______.

A.　the centerline of a navigable channel

B.　the starboard side of a channel

C.　a prominent object of navigational interest that has no lateral significance

D.　an area of a channel where passing another vessel is permitted

KEY:　B

[248]A report of casualty to a mobile offshore drilling unit must be made in writing to which office?

A.　Commandant of the Coast Guard

B.　Owner of the unit

C.　Nearest Coast Guard Marine Safety Office

D.　Captain of the nearest port

KEY:　C

[249]A report of casualty to a mobile offshore drilling unit must include ______.

A.　the estimated cost of damage to the unit

B.　an evaluation of who was at fault

C.　the amount of ballast on board

D.　the name of the owner or agent of the unit

KEY:　D

[250]A rigid replica of the International Code flag "A" may be shown by a vessel ______.

A.　pulling a submarine cable

B.　engaged in diving operations

C.　engaged in underway replenishment

D.　transferring explosives

KEY:　B

02.4 海图及海图作业

[251]To agree with larger scale chart，latitudes taken from this chart should be increased ______ about 8 seconds．

A.　with

B.　in

C.　by

D.　to

KEY:　C

[252]On this chart only the principle ______ to navigation are shown．

A.　equipment's

B.　tools

C.　apparatus

D.　aids

KEY:　D

[253]Many of the soundings shown on the chart are derived from ______.Undue reliance should not be placed upon them．

A.　complete and often very poor surveys

B.　correct and often very good surveys

C.　inadequate and often very old surveys

D.　adequate and present surveys

KEY:　C

[254]Lines on a chart which connect points of equal magnetic variation are called ______.

A.　magnetic latitudes

B.　magnetic declinations

C.　dip

D.　isogonic lines

KEY:　D

[255]Isogonic lines are lines on a chart indicating ______.

A.　points of equal variation

B.　points of zero variation

C.　the magnetic latitude

D.　magnetic dip

KEY:　A

[256]In which source could you find the number of a chart for a certain geographic area? ______.

A.　Chart No. 1

B.　Catalog of Charts

C.　IMO Practical Navigator

D.　IMO Light List

KEY:　B

[257]How is the annual rate of change for magnetic variation shown on a pilot chart? ______.

A.　Gray lines on the uppermost inset chart

B.　Red lines on the main body of the chart

C.　In parenthesis on the lines of equal magnetic variation

D.　Annual rate of change is not shown.

KEY:　A

[258]Charted depth is the ______.

A.　vertical distance from the chart sounding datum to the ocean bottom，plus the height of tide

B.　vertical distance from the chart sounding datum to the ocean bottom

C.　average height of water over a specified period of time

D.　average height of all low waters at a place

KEY:　B

[259]Chart correction information is NOT disseminated through the ______.

A.　Summary of Corrections

B.　Local Notice to Mariners

C.　Daily Memorandum

D.　Chart Correction Card

KEY:　D

[260]All straight lines represent great circle tracks on a chart based on a(n) ______.

A.　Mercator projection

B.　polyconic projection

C.　orthographic projection

D.　gnomonic projection

KEY:　D

[261]A revised print of a chart is made ______.

A.　after every major hydrographic survey of the area covered by the chart

B.　when there are numerous corrections to be made or the corrections are extensive

C.　when a low-stock situation occurs and minor corrections are made

D.　every two years to update the magnetic variation information

KEY:　C

[262]A Mercator chart is a ______.

A.　cylindrical projection

B.　simple conic projection

C.　polyconic projection

D.　rectangular projection

KEY:　A

[263]______ chart 3994，positions read from this chart should be shifted 0．03 minutes Northward．

A.　To consider

B.　To check

C.　To examine

D.　To agree with

KEY:　D

[264]The changes in the channel's sands and buoys on this coast are ______ this chart can not be considered as a safe guide of the channel．

A.　frequent

B.　so frequent

C.　frequent that

D.　so frequent that

KEY:　D

[265]______ is a term denoting the determination of a ship's position by observations of Celestial or terrestrial objects，or by a combination of both．

A.　Course-planning

B.　Position-fixing

C.　Radar-plotting

D.　Stowage-plan making

KEY:　B

[266]A chart position enclosed by a semi-circle is a(n) ______.

A.　fix

B.　estimated position

C.　dead reckoning position

D.　running fix

KEY:　C

[267]A true bearing of a charted object，when plotted on a chart，will establish a ______.

A.　fix

B.　line of position

C.　relative bearing

D.　range

KEY:　B

[268]Admiralty Charts are published in ______.

A.　U．S．A

B.　China

C.　U．K

D.　Japan

KEY:　C

[269]Attention is ______ the advice on the use of charts in The Mariner's Handbook Chapter 3，Section 1．

A.　fixed to

B.　needed for

C.　pushed to

D.　drawn to

KEY:　D

[270]Distance along a track line is measured on a Mercator chart by using the ______.

A.　latitude scale near the middle of the track line

B.　longitude scale near the middle of the track line

C.　latitude scale at the midlatitude of the chart

D.　latitude or longitude scale at the middle of the scale

KEY:　A

[271]Entering from sea，a daymark on the port side of the channel would be indicated on a chart by a ______.

A.　red triangle with the letter R

B.　white triangle with the letters RG

C.　green square with the letter G

D.　white square with the letters GR

KEY:　C

[272]Magnetic information on a chart may be ______.

A.　found in the center(s) of the compass rose(s)

B.　indicated by isogonic lines

C.　found in a note on the chart

D.　All of the above

KEY:　D

[273]My position has been obtained ______ astronomical observation．

A.　with

B.　from

C.　on

D.　by

KEY:　D

[274]Navigational charts are ______ frequent changes，the important one of which are promulgated by Admiralty Notices to Mariners．

A.　published with

B.　combined with

C.　in connection with

D.　subject to

KEY:　D

[275]On the south and the east coasts of Block Island are circles with a dot in the center and labeled CUP. This is a ______.

A.　conspicuous object

B.　steep depression in the surrounding hills that resembles a cup

C.　domed structure useful for navigation

D.　calling-up-point used for traffic control

KEY:　C

[276]Solid green arrows on the main body of a pilot chart indicate ______.

A.　prevailing wind directions

B.　prevailing ocean current directions

C.　probable surface current flow

D.　shortest great circle routes

KEY:　B

[277]The agonic line on an isomagnetic chart indicates the ______.

A.　magnetic equator

B.　magnetic longitude reference line

C.　points where there is no variation

D.　points where there is no annual change in variation

KEY:　C

[278]The buoy symbol printed on your chart is leaning to the northeast. This indicates ______.

A.　you should stay to the north or east of the buoy

B.　you should stay to the west or south of the buoy

C.　the buoy is a major lighted buoy

D.　nothing special for navigational purposes

KEY:　D

[279]The chart of a beach area shows a very flat slope to the underwater beach bottom. What type of breakers can be expected when trying to land a boat on this beach?

A.　Surging

B.　Spilling

C.　Plunging

D.　Converging

KEY:　B

[280]The chart symbol indicating that the bottom is coral is ______.

A.　C

B.　Cl

C.　Co

D.　c

KEY:　C

[281]The charts sold are of ______.

A.　newly edition with up to date correction and in reasonable prices．

B.　the current edition and incorporate the last Notices to Mariners correction

C.　the current edition and incorporate the latest Notices to Mariners correction at the time of sale．

D.　brand-new one with up to date correction and clean writing

KEY:　C

[282]The compass rose on a nautical chart indicates both variation and ______.

A.　deviation

B.　annual rate of variation change

C.　precession

D.　compass error

KEY:　B

[283]The description Racon beside an illustration on a chart would mean a ______.

A.　radar conspicuous beacon

B.　circular radiobeacon

C.　radar transponder beacon

D.　radar calibration beacon

KEY:　C

[284]The maximum difference between mean time and apparent time is ______.

A.　equal to the longitude expressed in time units

B.　about 16 minutes

C.　the difference between the GHA of mean sun and the first point of Aries

D.　15of arc

KEY:　B

[285]The numeral in the center of a wind rose circle on a pilot chart indicates the ______.

A.　total number of observations

B.　average wind force on the Beaufort scale

C.　average wind force in knots

D.　percentage of calms

KEY:　D

[286]The only cylindrical chart projection widely used for navigation is the ______.

A.　Lambert conformal

B.　Mercator

C.　azimuthal

D.　gnomonic

KEY:　B

[287]The path that a vessel is expected to follow，represented on a chart by a line drawn from the point of departure to the point of arrival，is the ______.

A.　DR plot

B.　track line

C.　heading

D.　estimated course

KEY:　B

[288]The revision date of a chart is printed on which area of the chart?

A.　Top center

B.　Lower-left corner

C.　Part of the chart title

D.　Any clear area around the neat line

KEY:　B

[289]The soundings on the chart are based on the depth of water available at ______.

A.　mean low water

B.　mean lower low water

C.　mean high water

D.　mean high water springs

KEY:　B

[290]The survey information upon which a chart is based is found ______.

A.　at the top center of the next line

B.　near the chart title

C.　at the lower left corner

D.　at any convenient location

KEY:　B

[291]The symbol which appears beside a light on a chart reads Gp Fl R (2) 10 sec 160 ft 19M. Which characteristic describes the light?

A.　It is visible 10 miles.

B.　Its distinguishing number is 19M.

C.　It has a radar reflector.

D.　None of the above

KEY:　D

[292]What defines a great circle?

A.　A curved line drawn on a Mercator Chart

B.　A course line that inscribes a loxodromic curve

C.　The shortest distance between any two points on the earth

D.　The smallest circle that can be drawn on the face of a sphere

KEY:　C

[293]What information is found in the chart title?

A.　Date of the first edition

B.　Date of the edition and，if applicable，the revision

C.　Information on the sounding datum

D.　Information on which IALA buoyage system applies

KEY:　C

[294]What information is NOT found in the chart title?

A.　Survey information

B.　Scale

C.　Date of first edition

D.　Projection

KEY:　C

[295]What is NOT an advantage of the rhumb line track over a great circle track?

A.　Easily plotted on a Mercator chart

B.　Negligible increase in distance on east-west courses near the equator

C.　Does not require constant course changes

D.　Plots as a straight line on Lambert conformal charts

KEY:　D

[296]What is the major advantage of a rhumb line track?

A.　The vessel can steam on a constant heading (disregarding wind，current，etc.).

B.　The rhumb line is the shortest distance between the arrival and departure points.

C.　It is easily plotted on a gnomonic chart for comparison with a great circle course.

D.　It approximates a great circle on east-west courses in high latitudes.

KEY:

[297]When a buoy is in position only during a certain period of the year，where may the dates when the buoy is in position be found?

A.　Light List

B.　Notice to Mariners

C.　On the chart

D.　Coast Pilot

KEY:　A

[298]When navigating in high latitudes and using a chart based on a Lambert conformal projection，______.

A.　a straight line drawn on the chart approximates a great circle

B.　the chart should not be used outside of the standard parallels

C.　the course angle is measured at the mid-longitude of the track line

D.　distance cannot be measured directly from the chart

KEY:　A

[299]When using a Lambert conformal chart in high latitudes，angles such as bearings are measured in reference to ______.

A.　the meridian through the object of the bearing

B.　the meridian through the ship's position

C.　the meridian midway between the ship and the object

D.　any meridian

KEY:　B

[300]Which aid is NOT marked on a chart with a magenta circle?

A.　Aero light

B.　Radar station

C.　Radar transponder beacon

D.　Radiobeacon

KEY:　A

02.5 航海通告与警告

[301]To ______ Admiralty Charts 438，latitudes taken from this chart should be increased by about 5 seconds．

A.　convert to

B.　come to

C.　look at

D.　agree with

KEY:　D

[302]The chart you supplied was anything ______ up to date．

A.　but

B.　except

C.　beside

D.　apart from

KEY:　A

[303]The Admiralty Notices to Mariners can be obtained ______ by Masters of vessels from any Admiralty Chart Agent．

A.　free of charge

B.　with no responsibility

C.　without limitations of distribution

D.　with little charges

KEY:　A

[304]Positions read from this chart should，to agree with chart 3949，be ______ 0．03 minutes Northwards and 0．22 minutes Westwards．

A.　shifted

B.　removed

C.　changed

D.　altered

KEY:　A

[305]Positions read from this chart should be moved 0．08 minutes Southward and 0．13 minutes Eastward so as ______ adjoining chart 344，．

A.　to agree upon

B.　in accordance with

C.　to be same as

D.　to agree with

KEY:　D

[306]Owing to old data of the survey the positions and number of the beacons shown on this chart are not to be ______.

A.　considered

B.　insured

C.　relied upon

D.　suspected

KEY:　C

[307]No navigational aids are shown and the chart is not kept corrected for alterations in depths inside the pecked lines． For more detailed information，the larger scale charts must be ______.

A.　referred to

B.　appreciated

C.　met with

D.　concerned

KEY:　A

[308]Navigational charts are subject to frequent changes，the important ones of which are promulgated by ______.

A.　Admiralty Notices to Mariners

B.　Mariner's Handbook

C.　Catalogue

D.　Sailing Directions

KEY:　A

[309]Contour elevations on this chart refer to heights in feet above mean ______.

A.　lower low water

B.　high water

C.　low water

D.　sea level

KEY:　D

[310]Charts are subject to frequent correction according to ______.

A.　Notices to Mariners

B.　Mariner's handbook

C.　Sailing Directions

D.　Guide to Port Entry

KEY:　A

[311]A chart has extensive corrections to be made to it. When these are made and the chart is again printed，the chart issue is a ______.

A.　first edition

B.　new edition

C.　revised edition

D.　reprint

KEY:　B

[312]______ are published for the correction of Admiralty Charts．

A.　Admiralty Sailing Directions

B.　Admiralty List of Signals

C.　Mariner's Handbook

D.　Admiralty Notices to Mariners

KEY:　D

[313]______ is not contained in the NM Weekly.

A.　Amendments to Admiralty Sailing Directions

B.　Amendments to Admiralty List of Lights and Fog Signals

C.　Amendments to Admiralty List of Radio Signals

D.　Supplement to Guide to Port Entry

KEY:　D

[314]The heights and soundings of Admiralty chart are expressed ______ in Fathoms ______ in Meters．

A.　either，or

B.　neither，nor

C.　another，nor

D.　other，or

KEY:　A

[315]______ day，pilot flag should be displayed from main mast．

A.　From

B.　By

C.　on

D.　With

KEY:　B

[316]______ objects are those which stand out clearly from the background or other objects and are easily identifiable from a few miles offshore in normal visibility．

A.　Conspicuous

B.　Permanent

C.　Preliminary

D.　Temporary

KEY:　A

[317]A marker pole，with a horseshoe buoy and a sea anchor attached，should be used to ______.

A.　mark the position of a lost mooring

B.　determine your vessel's sideslip underway

C.　determine your speed through the water

D.　indicate location of a man overboard

KEY:　D

[318]A stand-on vessel in a crossing situation is allowed to take action when ______.

A.　on a collision course

B.　the vessels will pass within one mile

C.　it becomes apparent to her that the give-way vessel is not taking appropriate action

D.　the relative speed of the vessels indicates collision in less than six minutes

KEY:　C

[319]A steep barometric gradient indicates ______.

A.　calms

B.　light winds

C.　strong winds

D.　precipitation

KEY:　C

[320]A storm is forecast for the area where your vessel is moored. For its safety you should put ______.

A.　more slack in the mooring lines

B.　a strain on the mooring lines

C.　chafing gear on the mooring lines

D.　grease on the mooring lines

KEY:　C

[321]Admiralty Notices to Mariners are published ______.

A.　every day

B.　every week

C.　every month

D.　every year

KEY:　B

[322]Admiralty Notices to Mariners，containing important information for the mariners and enabling them to keep their charts and books corrected for the latest information，are issued daily and published in ______.

A.　Weekly Editions

B.　Quarterly Editions

C.　Annual Editions

D.　Monthly Editions

KEY:　A

[323]Advise ______ your fishing gear．

A.　you recovery

B.　you recover

C.　you recovering

D.　you recovered

KEY:　B

[324]Advise you ______ engines．

A.　stop

B.　to stop

C.　stopping

D.　stopped

KEY:　A

[325]After the passage of a cold front the visibility ______.

A.　does not change

B.　improves rapidly

C.　improves only slightly

D.　becomes poor

KEY:　B

[326]Chart legends printed in capital letters show that the associated landmark is ______.

A.　conspicuous

B.　inconspicuous

C.　a government facility or station

D.　a radio transmitter

KEY:　A

[327]Chart legends which indicate a conspicuous landmark are printed in ______.

A.　capital letters

B.　italics

C.　boldface print

D.　underlined letters

KEY:　A

[328]Charted depth is the ______.

A.　vertical distance from the tidal datum to the ocean bottom plus the height of the tide．

B.　vertical distance from the tidal datum to the ocean bottom

C.　average height of water over a specified period of time

D.　average height of all low waters at a place

KEY:　B

[329]Charted depths ______ by 2 meters due to state of the winds．

A.　is decreased

B.　decreased

C.　decreases

D.　are decreased

KEY:　D

[330]Charted depths are decreased ______ 2 meters．

A.　in

B.　on

C.　with

D.　by

KEY:　D

[331]Fog is most commonly associated with a(n) ______.

A.　warm front at night

B.　low pressure area

C.　anticyclone

D.　lack of frontal activity

KEY:　A

[332]For what purpose would using a Lambert conformal chart be more convenient than using a Mercator?

A.　Plotting radio bearings over a long distance

B.　Determining latitude and longitude of a fix

C.　Measuring rhumb line distances

D.　Measuring rhumb line directions

KEY:　A

[333]Generally，______ are used as the track charts on the high sea，______ are used for harbors，inland waters and coastal areas．

A.　Mercator charts／Loran charts

B.　Admiralty charts／Routeing charts

C.　large scale charts／small scale charts

D.　small scale charts／large scale charts

KEY:　D

[334]In revised editions of Admiralty charts the ______ are corrected first．

A.　smaller scales

B.　larger scales

C.　smaller scales and larger scales

D.　moderate scales

KEY:　B

[335]Information for updating nautical charts is primarily found in the ______.

A.　Notice to Mariners

B.　Coast Pilots

C.　nautical chart catalogs

D.　Sailing Directions

KEY:　A

[336]LOW ABOUT 300 MILES SOUTHWEST OF LISBON 1004 MOVING SLWLY SOUTHEAST COMMA FILLING 1006 BY MIDNIGHT．This phrase is likely to be under the heading of ______.

A.　GALE WARNING

B.　24 HOURS FORECAST

C.　GENERAL SITUATION

D.　TROPICAL CYCLONE WARNING

KEY:　C

[337]LOW AT 34N 135E ESLY SLWLY INTST NC．This description is most likely to be under the heading of ______.

A.　GL WNG

B.　GEN SYN

C.　STM WNG

D.　T Y WNG

KEY:　B

[338]Many navigational warnings are of a temporary nature，but others remain in force for several months or may be succeeded by ______.

A.　Notices to Mariners

B.　Sailing Directions

C.　List of Lights and Signals

D.　Guide to Port Entries

KEY:　A

[339]Navigational warnings are published according to ______.

A.　the designed area

B.　the designated area

C.　the indicated area

D.　all area over the world

KEY:　B

[340]On a nautical chart，the inner ring of a compass rose indicates ______.

A.　true directions

B.　compass error

C.　deviation

D.　magnetic directions

KEY:　D

[341]On the approach of a warm front the barometric pressure usually ______.

A.　falls

B.　rises

C.　is steady

D.　is unreliable

KEY:　A

[342]On the high seas，a fog signal consisting of one prolonged blast followed by four short blasts would indicate the presence of a ______.

A.　vessel being towed．

B.　fishing vessel engaged in trawling．

C.　vessel at anchor warning you of her location．

D.　power-driven pilot vessel on station underway．

KEY:　D

[343]Periodic publications notifying changes in，or additions to，previously published navigational data are ______.

A.　Supplements

B.　Annual Summary

C.　Navigational Warnings

D.　Notices to Mariners

KEY:　D

[344]RYUKYU S TO SE 6 BEC 4－5．ISOL SHWRS．SEA 3 M BEC 2 M．SWELL SE 3－4M．This description is likely to be under the heading of ______.

A.　FORECAST

B.　GALE WARNING

C.　SYNOPSIS

D.　STORM WARNING

KEY:　A

[345]Steam smoke will occur when ______.

A.　extremely cold air from shore passes over warmer water

B.　warm dry air from shore passes over cooler water

C.　cold ocean water evaporates into warm air

D.　cool rain passes through a warm air mass

KEY:　A

[346]The best estimate of the wind direction at sea level can be obtained from observing the direction of the ______.

A.　cloud movement

B.　vessel heading

C.　waves

D.　swells

KEY:　C

[347]The broken magenta line which runs parallel to the shorebetween Roanoke Point and Mattituck Inlet marks a ______.

A.　fish trap area

B.　pipeline

C.　demarcation line

D.　cable area

KEY:　A

[348]The charts show two dashed lines crossing the river justsouth of St. Catherine Bar Light. What does this indicate?

A.　Overhead power lines

B.　Louisiana-Mississippi ferry crossings

C.　Two railroad trestles

D.　Two submerged oil pipelines

KEY:　D

[349]The Daily Memorandum contains information on ______.

A.　active weather disturbances such as hurricanes or tropical storms

B.　the latest navigational warnings

C.　scheduled vessel arrivals and departures for a 24-hour period

D.　water levels at river ports where run-off affects tidal heights

KEY:　B

[350]The fog most commonly encountered at sea is called ______.

A.　conduction fog

B.　radiation fog

C.　frontal fog

D.　advection fog

KEY:　D

三　　航海英文函电

03.1 航海日志的填写

[351]The entries in the Official Logbook must be signed by the Master and ______.

A.　the Chief Engineer

B.　the person about whom the entry concerns

C.　one other crew member

D.　No other signature is required.

KEY:　C

[352]If a spill occurs，where would you log it？ ______.

A.　Oil Record Book

B.　Bell book

C.　Smooth logbook

D.　Official Logbook

KEY:　D

[353]All events relating to the voyage，such as ship's position，speed and details of the weather，are recorded in ______.

A.　Logbook

B.　Bell Book

C.　Oil Record Book

D.　Compass Error Book

KEY:　A

[354]A journal kept by the officer of the watch in which day to day happenings are recorded regarding the deck department is the ______.

A.　cargo record book

B.　deck rough log

C.　bell book

D.　Official Logbook

KEY:　B

[355]______ of the following statements is INCORRECT regarding a fire and boat drill on board your cargo vessel.

A.　At least one fire and boat drill shall be held within 24 hours of leaving a port where more than 25 percent of the crew has been replaced．

B.　The Master is responsible in seeing that each lifeboat is lowered to the water at least once in each 6 months．

C.　Lifeboat equipment shall be examined at least once each month to insure that it is complete．

D.　An entry shall be made in the vessel's Official Logbook relative to each fire and boat drill

KEY:　B

[356]______ of the following need NOT be entered in the Official Logbook？ _______.

A.　The testing of the internal combustion engine driven emergency generators at least once each month the vessel is navigated．

B.　The testing of storage batteries for emergency lighting and power systems once each 6 months the vessel is navigated．

C.　The actual draft when the vessel arrives in salt water after departing a port．

D.　The testing of the line-throwing appliance once every 3 months

KEY:　A

[357]If you carry packaged hazardous cargoes on a break bulk vessel bound foreign，you must ______.

A.　stow the hazardous cargoes on deck available for jettisoning if necessary

B.　remove the hazardous cargo labels from a portable tank after the tank is emptied

C.　have the shipping papers indicate the proper shipping name and the technical name of n.o.s. cargoes

D.　log the receipt of hazardous cargoes in the Official Logbook

KEY:　C

[358]What will be accepted as the shipping papers for an unmanned tank barge but will not be accepted if the tank barge is manned? ______.

A.　Bill of Lading

B.　Logbook entry

C.　Manifest

D.　Shipping document

KEY:　B

[359]What is NOT accepted as the required shipping papers on a manned and loaded tank barge? ______.

A.　Bill of Lading

B.　Manifest

C.　Shipping document

D.　Logbook entry

KEY:　D

[360]Every entry required to be made in the Official Logbook shall be signed by the ______.

A.　Mate on watch

B.　Master and Chief Mate or other member of the crew

C.　Master only

D.　Purser，one of the Mates，and some other member of the crew

KEY:　B

[361]A wise Captain gives clear direction in his ______ for the officers on watch to call him if in any doubt whatsoever．

A.　Deck Logbook

B.　Master's Note

C.　Bell Book

D.　Master's Standing Orders

KEY:　D

[362]______ the abstracts of deck logbook covering the said accident．

A.　Attached herewith are

B.　Attached is herewith

C.　Attached are hereby

D.　Hereby are attached

KEY:　A

[363]Which is NOT a required entry in the ship's Official Logbook?

A.　Sale of effects of a deceased crew member

B.　Medical treatment of an injury

C.　Inspections of cargo gear

D.　Drydocking of the vessel

KEY:　D

[364]A journal kept by the officer of the watch in which day to day happenings are recorded regarding the deck department is the ______.

A.　cargo record book

B.　deck rough log

C.　bell book

D.　Official Logbook

KEY:　B

[365]A seaman assaults the Second Mate and injures him with a beer bottle while the ship is at sea. The incident is logged in the Official Logbook. In subsequent suspension and revocation proceedings against the seaman，according to the regulations，______

A.　the Second Mate and the Master must testify as to the facts of the assault

B.　the case will be dismissed if the logbook entries are improperly made

C.　the logbook entry is prima facie evidence of the facts if it complies with the law

D.　the logbook is inadmissible if the logbook entries do not conform to the law

KEY:　C

[366]After your vessel has been involved in a casualty，you are required to make your logbooks，bell books，etc.，available to ______.

A.　attorneys for opposition parties

B.　marine surveyors

C.　MSA officials

D.　All of the above

KEY:　C

[367]All entries in Logbook，______ made，must not be erased or amended．

A.　once

B.　whether

C.　previous

D.　just

KEY:

[368]All entries in the Official Logbook must be signed by the Master and ______.

A.　the Union Representative

B.　the person about whom the entry concerns

C.　no one else

D.　one other crew member

KEY:　D

[369]All entries in the Official Logbook must be signed by the Master and ______.

A.　the Chief Engineer

B.　the person about whom the entry concerns

C.　one other crew member

D.　No other signature is required.

KEY:　C

[370]All of the following records are usually maintained by the watch-standing officers aboard a vessel EXCEPT the ______.

A.　deck logbook

B.　official logbook

C.　compass record book

D.　chronometer error book

KEY:　B

[371]In writing up the log book at the end of your watch，you make an error．which of the following is the way to correct the error？ ______.

A.　Carefully and neatly erase the entry and rewrite it correctly

B.　Remove this page of the log book and rewrite all entries on a clean page

C.　Blot out the error completely and rewrite the entry correctly

D.　Cross out the error with a single line and rewrite the entry correctly

KEY:　D

[372]In writing up the logbook at the end of your watch，you make an error in writing an entry. What is the proper means of correcting this error?

A.　Cross out the error with a single line，and write the correct entry，then initial it.

B.　Carefully and neatly erase the entry and rewrite it correctly.

C.　Remove this page of the log book，and rewrite all entries on a clean page.

D.　Blot out the error completely and rewrite the entry correctly.

KEY:　A

[373]Lifeboat winches on a ship are required to be inspected and an entry made in the logbook. What should this entry include?

A.　The time required to lower a lifeboat

B.　The time required to raise a lifeboat

C.　The date of inspection and condition of the winch

D.　All of the above

KEY:　C

[374]Logbooks are used to record the events occurring when ______.

A.　the ship is in a harbor

B.　the ship is at anchorage

C.　the ship is underway

D.　all of the above

KEY:　D

[375]Never make comment，estimate or guess in your Logbook，but ______ only．

A.　the specifications

B.　the facts

C.　the descriptions

D.　the details

KEY:　B

[376]Official proof of a Chinese vessel's nationality is contained in the ______.

A.　Certificate of Inspection

B.　Official Log

C.　Certificate of Documentation

D.　Shipping Articles

KEY:　C

[377]Prior to burning or welding on a fuel tank on a ship，regulations require that an inspection be made. An entry in the unofficial logbook is required if this inspection is made by ______.

A.　a marine chemist

B.　the Officer in Charge，Marine Inspection

C.　the Master or person in charge of the ship

D.　the National Fire Protection Association

KEY:　C

[378]Prior to getting underway in fresh or brackish water，the Master must ______.

A.　log the density of the water

B.　secure all overboard discharges

C.　take on fresh water ballast

D.　clean the sides with fresh water

KEY:　A

[379]The Master or person in charge of a ship is required to log ______.

A.　the names of all persons on board

B.　only the names of the crew members on board

C.　only the names of passengers on board

D.　information on emergency training drills

KEY:　D

[380]The proper way to correct a mistake in the logbook is to ______.

A.　erase the entry and rewrite

B.　draw several lines through the entry，rewrite，and initial the correction

C.　completely black out the entry，rewrite，and initial the correction

D.　draw one line through the entry，rewrite，and initial the correction

KEY:　D

[381]The proper way to correct a mistake in the logbook is to ______.

A.　erase the entry and rewrite

B.　draw several lines through the entry，rewrite，and initiate the correction

C.　completely black out the entry，rewrite，and initiate the correction

D.　draw one line through the entry，rewrite，and initiate the correction

KEY:　D

[382]The responsibility for maintaining the Official Logbook on voyages between the Atlantic and Pacific coasts of the United States rests with the ______.

A.　Chief Mate of the vessel

B.　Master of the vessel

C.　Deck Officer of the watch at the time of the occurrence

D.　Purser of the vessel

KEY:　B

[383]Uncleared crew curios remaining on board during a domestic coastwise voyage after returning from foreign should be ______.

A.　listed in the Official Logbook

B.　cleared prior to the next foreign voyage

C.　noted in the Traveling Curio Manifest

D.　retained under locked security by the owner

KEY:　C

[384]Under the Pollution Regulations，when you dump garbage in to the sea you must ______.

A.　notify Coast Guard or MSA

B.　make an entry in the Official Logbook

C.　keep a record for two years

D.　No action is required if you are more than 25 miles fromland and no plastic materials are dumped.

KEY:　C

[385]What is NOT a requirement for testing the line throwing appliance on a vessel?

A.　The appliance should be tested every three months.

B.　A regular service line should be used when testing.

C.　A regular projectile should be used when testing.

D.　An entry about the test must be made in the Official Log Book.

KEY:　B

[386]What is NOT accepted as the required shipping papers on a manned and loaded tank barge?

A.　Bill of Lading

B.　Manifest

C.　Shipping document

D.　Logbook entry

KEY:　D

[387]What is official proof of a vessel's ownership?

A.　Certificate of Documentation

B.　Bill of Lading

C.　Transfer Certificate

D.　Logbook

KEY:

[388]What is required to be entered into the Official Logbook?

A.　Opening a sideport at sea to renew a gasket

B.　The annual required stripping and cleaning of the lifeboats

C.　The biennial weight test of the lifeboats and falls

D.　The drafts on entering port

KEY:　A

[389]What must the Master or person in charge of a ship enter in the logbook after conducting a boat drill?

A.　Which survival craft was used in the drill

B.　The number of each lifeboat not lowered during each drill

C.　Only the number of each motor propelled lifeboat that is lowered

D.　The length of time an oar propelled lifeboat is rowed

KEY:　A

[390]What must the Master or person in charge of a ship enter in the logbook after conducting a boat drill?

A.　Which survival craft was used in the drill

B.　The number of each lifeboat not lowered during each drill

C.　Only the number of each motor propelled lifeboat that is lowered

D.　The length of time an oar propelled lifeboat is rowed

KEY:　A

[391]What must the Master or person in charge of a ship enter in the logbook after conducting an abandonment drill?

A.　The sea condition at the time of the drill

B.　Which survival craft was used

C.　The names of all crew members participating in the drill

D.　The name of the lifeboatman in charge of each boat

KEY:　B

[392]What must the Master or person in charge of a ship enter in the logbook after conducting an abandonment drill?

A.　The sea condition at the time of the drill

B.　Which survival craft was used

C.　The names of all crew members participating in the drill

D.　The name of the lifeboatman in charge of each boat

KEY:　B

[393]What will be accepted as the shipping papers for an unmanned tank barge but will not be accepted if the tank barge is manned?

A.　Bill of Lading

B.　Logbook entry

C.　Manifest

D.　Shipping document

KEY:　B

[394]When a vessel is entering or leaving a port，a record of engine speeds is kept in the ______.

A.　bell book

B.　deck rough log

C.　Official Logbook

D.　engine rough log

KEY:　A

[395]When an azimuth of the Sun has been taken and the deviation of the standard magnetic compass computed，the watch officer should record the results ______.

A.　in the vessel's Official Logbook

B.　on the compass deviation card

C.　in the compass deviation log

D.　on a Napier diagram

KEY:　C

[396]When are fore and aft draft readings required to be entered in the unofficial logbook of a ship?

A.　Once a day

B.　Once a week

C.　Prior to getting underway

D.　Only when entering waters of different density

KEY:　C

[397]When is the density of the water required to be logged in the logbook of a ship?

A.　Any time the vessel moves from water of one density into waters of a different density

B.　Prior to getting underway when the vessel is floating in fresh or brackish water

C.　Only when the vessel moves from fresh water into salt water

D.　The density of the water is not required to be logged in the unofficial logbook

KEY:　B

[398]When must the Master of a vessel log the position of load line marks in relation to the surface of the water in the Official Logbook?

A.　Once a day

B.　At the change of every watch

C.　Only when in fresh or brackish water

D.　Prior to getting underway

KEY:　D

[399]When oil is discharged overboard，an entry is required in the ______.

A.　engine rough log

B.　Oil Record Book

C.　Official Logbook

D.　deck rough log

KEY:　B

[400]When oily ballast has been pumped overboard，an entry must be made in the ______.

A.　Oil Record Book

B.　Official Logbook

C.　deck rough log

D.　engine rough log

KEY:　A

03.6 海上事故报告

（碰撞、搁浅、火灾、溢油、货损货差等）

[601]A deck-stowed 40-foot container is giving off smoke，and one end is discolored from heat. The cargo is valuable and easily damaged by water. You want to extinguish the fire without further damage if possible. What action should you take? ______.

A.　Connect a portable line from the ship's fixed system and discharge CO2 into the container.

B.　Flood the container with water and disregard any cargo damage as the fire threatens the entire vessel.

C.　Pierce the container and discharge 6 or more portable CO2's then add more CO2 hourly.

D.　Cool the exterior of the container with water and close all vents； then keep it cooled until it can be off-loaded.

KEY:　C

[602]How do you determine the weight of the vessel that is supported by the ground when a vessel has run aground? ______.

A.　This requires extensive calculation and is usually performed only by a naval architect not by a ship's officer.

B.　Determine the point where aground and the draft at that point，then calculate it using the grounding formula.

C.　Use the hydrostatic tables and enter with the mean draft before grounding and the mean draft after grounding.

D.　Use the inclining experiment formula and substitute the change of trim for the angle of list.

KEY:　C

[603]A situation has occurred where it becomes necessary for you to be towed. What action should be taken to prevent your vessel from yawing? ______.

A.　Shift weight to the bow

B.　Shift weight to the center of the boat

C.　Shift weight to the stern

D.　Throw excess weight overboard

KEY:　C

[604]A wooden float placed between a ship and a dock to prevent damage to both is called a ______.

A.　camel

B.　dolphin

C.　rat guard

D.　wedge

KEY:　A

[605]According ______ the report，the ship hit the wharf when berthing．

A.　of

B.　to

C.　for

D.　on

KEY:　B

[606]After a collision or accident involving an uninspected vessel，regulations require that the operator assist persons affected，but only if the operator can do so without ______.

A.　further damaging the other vessel

B.　undue delay

C.　serious danger to his or her own vessel

D.　creating a panic on either vessel

KEY:　C

[607]After an accident the victim may go into shock and die. What should be done to help prevent shock?

A.　Slightly elevate the head and feet.

B.　Keep the person awake.

C.　Keep the person lying down and at a comfortable temperature.

D.　Give the person a stimulant to increase blood flow.

KEY:　C

[608]After being rescued from the vessel accident，the people agreed that they had much to ______.

A.　thank

B.　be thankful

C.　be thanked

D.　be thankful for

KEY:　A

[609]After oil pollution occurs in Chinese ports，what action shall be first taken by ship's Captain and Chief Engineer？ ______.

A.　to use the oil dispersant

B.　to organize the persons to collect the oil

C.　to report the matter to MSA and ask for instructions

D.　to put forward an accident report within 24 hours

KEY:　C

[610]An accident came ______ my mind when I saw the broken case．

A.　in

B.　on

C.　to

D.　with

KEY:　C

[611]Apart from special circumstances，the value of the goods for which compensation must be made，if they have been lost or damaged，______ that which they would have had at the time and place at which they ought to have been delivered in proper condition．

A.　contains

B.　remains

C.　has

D.　is

KEY:　D

[612]As matter of fact，the damage to the winches was due to ______.

A.　insufficiency of packaging

B.　inherent vice of the cargo

C.　improper stowage

D.　rough handling

KEY:　D

[613]Cargo that is highly susceptible to damage by tainting from odorous cargo is called ______.

A.　clean cargo

B.　delicate cargo

C.　dry cargo

D.　immune cargo

KEY:　B

[614]Damage to cargo caused by dust is known as ______.

A.　contamination

B.　oxidation

C.　tainting

D.　vaporization

KEY:　A

[615]Damage to cargo caused by fumes or vapors from liquids，gases，or solids is known as ______.

A.　contamination

B.　oxidation

C.　tainting

D.　vaporization

KEY:　C

[616]Dry cargoes are those commodities that ______.

A.　never leak but may be spoiled by leakage

B.　sometimes leak and damage themselves

C.　often leak and damage other cargo

D.　neither leak nor will be damaged by leakage

KEY:　A

[617]Dunnage may be used to protect a cargo from loss or damage by ______.

A.　ship's sweat

B.　inherent vice

C.　tainting

D.　hygroscopic absorption

KEY:　A

[618]Fracture damage to the end links of the anchor cable，or to the jews' harp may be eliminated by ______.

A.　using a small diameter connecting shackle

B.　ensuring the swivel is well lubricated and free to turn

C.　installing the connecting shackle with the bow towards the anchor

D.　securing a piece of wood to the jews' harp

KEY:　D

[619]Has any person ______ on board during the voyage otherwise than as a result of accident？

A.　die

B.　dying

C.　dyed

D.　died

KEY:　D

[620]He must have had an accident，or he ______ then．

A.　would have been here

B.　should be here

C.　had to be here

D.　would be here

KEY:　A

[621]I did find a lot of damage ______ respect to the inward cargo．

A.　on

B.　by

C.　for

D.　with

KEY:　D

[622]I have ______ damage below water line.

A.　large

B.　big

C.　important

D.　major

KEY:　D

[623]If the sling bumps ______ the beams，serious accidents may follow．

A.　at

B.　on

C.　into

D.　against

KEY:　D

[624]If you do not wear goggles and helmet，your chances of being ______ will be greater．

A.　beaten

B.　damaged

C.　hurt

D.　stricken

KEY:　C

[625]In case of accidents ______ the risk of sinking，all effective measures shall be taken to steer clear of the fairway to avoid ______ the traffic．

A.　involving／impeding

B.　involved／to impede

C.　involving／to impede

D.　involved／impeding

KEY:　A

[626]In reference to accidental oil pollution，the most critical time during bunkering is when ______.

A.　you first start to receive fuel

B.　hoses are being blown down

C.　final topping off is occurring

D.　hoses are being disconnected

KEY:　C

[627]is ______.

A.　the average distance steamed per day over the duration of the voyage

B.　charges against all parties in a marine venture to pay for damages

C.　loss sustained by only one party

D.　claimed after all liens against the vessel are settled

KEY:　C

[628]It is no defense to a claim for freight to show that the goods ______ damaged．

A.　to be

B.　are

C.　had

D.　have

KEY:　B

[629]It's ______ that the cargo was damaged by your stevedores．

A.　bright

B.　distinct

C.　plain

D.　evident

KEY:　D

[630]It's also essential that we can not ______ any marks mixed or damage to the cargo．

A.　agree

B.　adopt

C.　check

D.　accept

KEY:　D

[631]Let ______ the real fact of the accident．

A.　I tell you

B.　me to tell you

C.　I to tell you

D.　me tell you

KEY:　D

[632]Making water rapidly in all holds，all passengers and crew were ordered on deck ______ and all boats were lowered to rail．

A.　in life jackets

B.　with life jackets

C.　with life jackets on

D.　putting on life jackets

KEY:　C

[633]My vessel maneuvering ______ difficulty．Please keep well clear ______ me．

A.　with，of

B.　in，of

C.　at，apart

D.　with，from

KEY:　A

[634]No damage of any kind ______ aids to navigation or navigation facilities．

A.　should be done to

B.　should not be done to

C.　must be done with

D.　must not be done with

KEY:　A

[635]OAP IF ANY TO BE CHTRS ACCT BUT OWNERS CONTRIBUTION USD 0.3/MT. What does OAP stand for?

A.　Old Age Pension

B.　Overage Additional Premium

C.　Organization of Arab Petroleum countries

D.　Optimum amplitude and phase modulation

KEY:　B

[636]Protection of cargo against tainting damage can best be obtained by ______.

A.　Ventilating the space．

B.　Not ventilating the space．

C.　Proper use of paper separation and dunnage．

D.　Segregation of cargo by using different hatches

KEY:　D

[637]Segregation of cargoes refers to ______.

A.　separating cargoes so that the inherent characteristics of one cannot damage the other

B.　separating cargoes by destination

C.　classifying cargoes according to their toxicity

D.　listing the cargoes in order of their flammability

KEY:　A

[638]Speed must be regulated ______ no damage is done to the wharf．

A.　in order to

B.　so as

C.　so that

D.　such that

KEY:　C

[639]Sweat damage will occur in a cargo hold containing machinery parts when ______.

A.　the outside air is colder than the cargo．

B.　the air temperature of the hold is higher than the outside air temperature．

C.　the dew point of the hold is higher than the temperature of the cargo．

D.　the dew point of the outside air is lower than the temperature of the cargo

KEY:　C

[640]The {said} accident was caused by your ship．

A.　sailing

B.　damaged

C.　sounded

D.　abovementioned

KEY:　D

[641]The accident ______ caused damage to my hull．

A.　have

B.　is

C.　will

D.　has

KEY:　D

[642]The accident has caused damage ______ the ship's hull．

A.　on

B.　in

C.　at

D.　to

KEY:　D

[643]The annual change in ______ is 0．2 degree．

A.　Magnetic Variation

B.　Marine Insurance

C.　Maritime Accident

D.　Mean High Water Spring

KEY:　A

[644]The best treatment for preventing traumatic shock after an accident is to ______.

A.　have the victim exercise to increase circulation

B.　keep the victim from electrical equipment

C.　keep the victim warm and dry while lying down

D.　apply ice packs and avoid excitement

KEY:　C

[645]The damage arose ______ the accident．

A.　against

B.　of

C.　for

D.　from

KEY:　D

[646]The damage was caused ______ the stevedores．

A.　for

B.　by

C.　at

D.　in

KEY:　B

[647]The damage was caused ______ the stevedores．

A.　with

B.　from

C.　on

D.　by

KEY:　D

[648]The extent of the damage could not be ______ though I inspected it with the Chief Stevedore in charge.

A.　sure

B.　contained

C.　ascertained

D.　applied

KEY:　C

[649]The Greek ship drifted and struck the portside of a Chinese ship，which ______ the bulwalk and Stanchions．

A.　caused damage to

B.　had collision with

C.　was damaging

D.　was causing damage to

KEY:　A

[650]The indemnity for damage to cargo shall be determined on the basis of the {difference} between the value of the goods before and after the damage．

A.　profit

B.　surplus

C.　interest

D.　balance

KEY:　D

四　　船舶管理

04.1 船舶安全管理（ISM）

[801]A survey of refrigerated cargo，to certify that proper methods of stowage were utilized，can be conducted by the ______.

A.　ABS

B.　Occupational Safety and Health Administration

C.　MSA

D.　National Cargo Bureau

KEY:　D

[802]Before leaving port on an ocean tow，a tug captain should assure himself of all the following EXCEPT ______.

A.　the towing hawser can be released quickly in an emergency

B.　the correct navigation lights are rigged and operable on the tug and tow

C.　an insurance underwriter has prepared a pre-sailing survey

D.　a pick-up wire has been rigged on the tow in case of a breakaway.

KEY:　C

[803]Refrigeration machinery is often surveyed before loading reefer cargo. This survey is usually performed by the ______.

A.　MSA

B.　CCS

C.　CCIQ

D.　local port authority

KEY:　B

[804]When a marine surveyor is employed to assist in the move of a mobile offshore drilling unit，he ______.

A.　becomes responsible for the safe movement of the rig

B.　relieves the Offshore Installation Manager at the moment the rig is free from the bottom

C.　must be designated as the person in charge of the rig

D.　is a specialist who is hired to provide advice and guidance on aspects of the move

KEY:　D

[805]Which organization would conduct a survey of the insulation in a reefer compartment prior to loading cargo?

A.　PRC Customs Service

B.　OSHA

C.　CCS

D.　CCIQ

KEY:　C

[806]Which vessel when anchored at night，would not be required to show anchor lights?

A.　A vessel engaged in underwater operations

B.　A vessel engaged in mine clearance

C.　A vessel engaged on pilotage duty

D.　A vessel engaged in survey operations

KEY:　A

[807]A gas-free certificate would usually be issued by a(n) ______.

A.　ABS marine surveyor

B.　certified marine chemist

C.　port engineer

D.　Coast Guard marine inspector

KEY:　B

[808]A sail plan，including the vessel's itinerary，name，number，and persons aboard，should be filed with ______.

A.　the FAA

B.　MSA

C.　the Customs

D.　a responsible person

KEY:　D

[809]An ocean towing bridle whose legs are of equal length，but too short，may ______.

A.　fail to provide spring in the hawser

B.　cause unequal distribution of the load to one leg

C.　cause the bridle legs to jump clear of the chocks or fairleads

D.　None of the above

KEY:　A

[810]Finally，when your ship is alongside，don't forget ______.

A.　having rat guards properly mounted on your moorings

B.　to have rat guards properly mounted on your moorings

C.　to have rat guards properly mounting at your moorings

D.　having rat guards proper mounted on your moorings

KEY:　B

[811]Get everything ready ______ mooring．

A.　of

B.　on

C.　to

D.　for

KEY:　D

[812]Get the searchlight ______ for transiting the canal at night．

A.　turned off

B.　already

C.　almost

D.　ready

KEY:　D

[813]Individuals who have consumed alcohol within 24 hours of exposure to H2S can tolerate ______.

A.　unusually large concentrations of H2S

B.　smaller than normal concentrations of H2S

C.　moderate concentrations of H2S without the usual reactions

D.　longer exposure to H2S concentrations

KEY:　B

[814]Mooring ______ are placed on both banks of the Suez Canal about 200m apart．

A.　bollards

B.　ballasts

C.　bands

D.　benefits

KEY:　A

[815]Please charge the expenses ______ the ship's account．

A.　for

B.　by

C.　at

D.　into

KEY:　D

[816]Please get everything ready prior ______ shifting．

A.　on

B.　to

C.　at

D.　for

KEY:　B

[817]Please instruct the stevedores that on no account ______ to exceed this limit．

A.　they are

B.　are they

C.　they were

D.　were they

KEY:　B

[818]Please let me ______ a set of import manifests．

A.　to have

B.　have

C.　having

D.　to get

KEY:　B

[819]Please mark (out) one of the most important register of shipping in the world． ______.

A.　HK MARDEP

B.　COSCO

C.　Lloyd's Society

D.　Liberia Shipping Registry

KEY:　C

[820]That the ______ showed that the ship complied with the requirements of the said Convention.

A.　looking

B.　seeing

C.　sightseeing

D.　inspection

KEY:　D

[821]The capacity of any liferaft on board a vessel can be determined by ______.

A.　examining the Certificate of Inspection

B.　examining the plate on the outside of the raft container

C.　referring to the station bill

D.　referring to the shipping articles

KEY:　B

[822]The major components which determine the length of catenary in a deployed anchor cable are water depth，cable tension，and ______.

A.　environmental forces

B.　bottom conditions

C.　cable weight

D.　water density

KEY:　C

[823]The material used extensively as electrolytic protection around bronze propellers is ______.

A.　Brass

B.　Copper

C.　Lead

D.　Zinc

KEY:　D

[824]The Muster List ("Station Bill") of a ship must be signed by the ______.

A.　person in charge

B.　Coast Guard marine inspection officer

C.　company safety director

D.　entire crew

KEY:　A

[825]The net productivity per gang-hour can be improved through a decrease of ______.

A.　cranes／derricks

B.　crane／derrick cycles

C.　crane／derrick cycle time

D.　weight per package

KEY:　C

[826]The owner or Master of a towing vessel shall ensure that each person that directs and controls the movement of the vessel can accomplish all of the following EXCEPT ______.

A.　evaluate the danger of each closing visual or radar contact

B.　adjust speed with due regard for the weather and visibility

C.　reduce speed only where local speed limits are posted

D.　enter all required test and inspection results in the vessel's log or other record carried on board

KEY:　C

[827]The patches where rust have been removed should be wiped clean before paint is ______.

A.　applied

B.　supplied

C.　replied

D.　complied

KEY:　A

[828]The productivity of working shifts can be improved through a decrease of ______.

A.　working hours

B.　gangs

C.　idle time

D.　weight per set

KEY:　C

[829]The ship can leave the port ______ the joint inspection．

A.　before

B.　after

C.　at

D.　from

KEY:　B

[830]The SMS should provide for specific measures aimed at promoting the _________of equipment or systems the sudden operational failure of which may result in hazardous situations.

A.　visibility

B.　ability

C.　reliability

D.　capability

KEY:　C

[831]The term LEE SHORE refers to the ______.

A.　shore on the lee side of the vessel

B.　shore that is in the lee

C.　western shore of the Lesser Antilles

D.　shore in a harbor of refuge

KEY:　A

[832]The test of a towing vessel's steering gear control system includes each item EXCEPT ______.

A.　a test from the alternative power supply，if installed

B.　timing the movement of the rudder from hard over to hard over

C.　verification that the rudder angle indicator shows the actual position of the rudder(s)

D.　visual inspection of the steering linkage

KEY:　B

[833]The ventilation system of your ship has fire dampers restrained by fusible links. Which statement is TRUE?

A.　Fusible links must be replaced if a damper is activated.

B.　Fusible links are tested by applying a source of heat to them.

C.　Fusible links must be replaced at every inspection for certification.

D.　A fusible link will automatically open after a fire is extinguished and reset the damper.

KEY:　A

[834]To determine the pressure and temperature limitations under which LFG is required to be transported on a barge，you should look at the ______.

A.　Certificate of Inspection

B.　loading order

C.　rules and regulations for tank vessels

D.　tankerman's document

KEY:　A

[835]To determine which grades of cargo a tank vessel is permitted to carry you should ______.

A.　refer to the vessel's Certificate of Inspection

B.　examine the cargo tanks and fittings

C.　ask the terminal supervisor or his representative

D.　check the loading order

KEY:　A

[836]Under the Pollution Regulations，garbage disposal records must be kept ______.

A.　two years

B.　one year

C.　until the next Coast Guard inspection

D.　until the end of the voyage

KEY:　A

[837]What is NOT a requirement of the reinspection for a tank barge with a certificate of inspection valid for two years?

A.　The reinspection will be made between the fourteenth and sixteenth months.

B.　The inspector shall examine all accessible parts of the vessels's hull.

C.　The inspector shall examine the vessel's machinery as well as equipment.

D.　The scope of the reinspection shall be the same as for the inspection for certification，but in less detail.

KEY:　A

[838]What is NOT required on an uninspected towing vessel?

A.　Certificate of Documentation

B.　Certificate of Inspection

C.　Operators Merchant Marine license

D.　FCC Station License

KEY:　B

[839]What is required to be posted in the pilothouse of a vessel?

A.　Certificate of Inspection

B.　Officer's licenses

C.　Muster List ("Station Bill")

D.　Vessel's Maneuvering Characteristics

KEY:　D

[840]What is required to be posted in the pilothouse of a vessel?

A.　Certificate of Inspection

B.　Officer's licenses

C.　Station Bill

D.　Vessel's Maneuvering Characteristics

KEY:　D

[841]What is TRUE of pressure/vacuum valves?

A.　They are designed to provide for the flow of small volumes of tank atmospheres caused by thermal variations in a cargo tank.

B.　They should operate in advance of the pressure/vacuum breakers.

C.　They should be kept in good working order by regular inspection and cleaning.

D.　All of the above

KEY:　D

[842]What is(are) standard operating procedure(s) used on a Ro-Ro vessel?

A.　Periodic inspection and retensioning of lashings as required during voyage

B.　Lashings may be completed after the vessel leaves port

C.　One QMED assigned to oversee the initial loading and lashing

D.　All of the above

KEY:　A

[843]What will NOT be found on the Certificate of Inspection of an ocean going tanker?

A.　Manning requirements

B.　Minimum freeboard permitted

C.　Grade(s) of cargoes that the vessel may carry

D.　Waters upon which the vessel may be operated

KEY:　B

[844]When backing down with sternway，the pivot point of a vessel is ______.

A.　about one-quarter of the vessel's length from the stern

B.　at the bow

C.　about one-third of the vessel's length from the bow

D.　aft of the propellers

KEY:　A

[845]When inspecting a tank barge to see that it has all the required fire extinguishers and other safety items aboard，which of the following is the best source for determining what is required?

A.　The supervisor in charge of the dock to which the barge is tied up

B.　Part 38 of the Rules and Regulations for Tank Vessels

C.　The Certificate of Inspection of the barge

D.　The local fire marshal

KEY:　C

[846]When should you conduct a visual inspection of your towline?

A.　Whenever its serviceability is in doubt.

B.　In accordance with the manufacturer's recommendation.

C.　At least once a month.

D.　All of the above

KEY:　D

[847]Where are the test certificates，for wire rope used as cargo runners，and loose gear certificates usually maintained?

A.　With the Cargo Gear Register on the ship

B.　In the Official Log

C.　At the Coast Guard Marine Safety Office with the vessel's inspection records

D.　At the central records center of the agency testing the cargo gear

KEY:　A

[848]Where would you find a list of the lifesaving equipment onboard your supply boat?

A.　Ship's Articles

B.　Muster List ("Station Bill")

C.　Certificate of Inspection

D.　Coast Guard Regulations

KEY:　C

[849]Where would you find the "call sign" or "call letters" of the radio station on your rig?

A.　In the rig safety manual

B.　On the Certificate of Inspection

C.　On the Ship Station License

D.　On the rig Watch Bill

KEY:　C

[850]Which action is routinely performed at the annual servicing and inspection of a dry-chemical cartridge-operated portable fire extinguisher?

A.　Test the pressure gauge for correct reading.

B.　Weigh the cartridge.

C.　Replace the dry chemical.

D.　Pressure test the discharge hose.

KEY:　B

04.2 港口国检查文件及报告（PSC）

[851]A chain stripper is used to ______.

A.　prevent chain from clinging to the wildcat

B.　clean the marine debris from the chain

C.　flake chain from a boat's chain locker

D.　clean chain prior to an x-ray inspection

KEY:　A

[852]A load line for a ship is assigned by the ______.

A.　Minerals Management Service

B.　Department of Energy

C.　Corps of Engineers

D.　A recognized classification society approved by the Coast Guard

KEY:　D

[853]A ship must report a collision with an aid to navigation maintained by the Coast Guard to which office?

A.　Nearest Oceanographic office

B.　The Marine Safety Center at Coast Guard Headquarters

C.　National Ocean Service

D.　Nearest Officer in Charge，Marine Inspection

KEY:　D

[854]According to the regulations for mobile offshore drilling units，"industrial personnel" are considered to be all persons carried on the ship for the sole purpose of carrying out the industrial business of the unit，except for ______.

A.　the operator's representative

B.　the crew required by the Certificate of Inspection

C.　the galley personnel

D.　the designated person in charge

KEY:　B

[855]According to the regulations，the capacity of a liferaft is required to be marked ______.

A.　on the Muster List ("Station Bill")

B.　at the liferaft stowage location

C.　on the Certificate of Inspection

D.　in the Operations Manual

KEY:　B

[856]An offshore drilling unit is required to carry on board an operations manual approved by the ______.

A.　Coast Guard

B.　National Cargo Bureau

C.　Minerals Management Service

D.　builder

KEY:　A

[857]An offshore drilling unit must be equipped with a first aid kit approved by the ______.

A.　Coast Guard

B.　American Bureau of Shipping

C.　Minerals Management Service

D.　Mine Safety and Health Administration

KEY:　D

[858]As operator of a 199 GT towing vessel sailing foreign，it shall be your duty to enter in the Official Logbook or other permanent record ______.

A.　all information contained on the barge's Certificate of Inspection

B.　the condition of all towing gear to be used

C.　all navigational aids to be used

D.　the barges load line and draft readings

KEY:　D

[859]As soon as the ship is berthed，you should lower the gangway，stretch a net ______ it，and adjust it to the rise and fall of the tides so that personnel may embark or disembark in safety．

A.　over

B.　besides

C.　near

D.　underneath

KEY:　D

[860]Before being certified by the American Bureau of Shipping，anchor chain must undergo ______.

A.　USCG inspection

B.　a breaking test

C.　x-ray inspection

D.　spectroanalysis

KEY:　B

[861]e Certificate of Freeboard is the ______.

A.　Loadline Certificate

B.　Certificate of Inspection

C.　Admeasurer's Certificate

D.　Forecastle Card

KEY:　T

[862]Fire protection regulations for towing vessels require that all crew members participate in drills and receive instruction at least once a month. Who is responsible for ensuring that this takes place?

A.　The Officer in Charge，Marine Inspection

B.　The Master，or person in charge of the vessel

C.　The vessel's owner or manager

D.　The company's port captain or port engineer

KEY:　B

[863]If a fixed foam firefighting system on a ship is not of the premix type，a sample of the foam liquid must be tested by ______.

A.　a Coast Guard inspection officer

B.　the safety man aboard the unit

C.　the designated person in charge of the unit

D.　the manufacturer or his authorized representative

KEY:　D

[864]If you are the Master of a towing vessel whose only working radar no longer functions，what must you do?

A.　Notify the Captain of the Port (COTP) if you cannot repair it within 96 hours.

B.　Notify the Officer in Charge of Marine Inspection (OCMI) if you cannot repair it within 48 hours.

C.　Moor the vessel until it is repaired.

D.　Anchor the vessel until it is repaired.

KEY:　A

[865]In the case of an injury causing a person to be incapacitated，the Master or person in charge of a mobile offshore drilling unit must submit a report to the ______.

A.　nearest Marine Safety or Marine Inspection Office

B.　nearest hospital

C.　Department of Energy

D.　American Bureau of Shipping

KEY:　A

[866]In the event of a casualty to a ship，who is responsible to make records available to the Coast Guard official authorized to investigate the casualty?

A.　Officer in Charge，Marine Inspection

B.　The person who caused the casualty

C.　The company man

D.　The owner

KEY:　D

[867]Inspection of a Halon extinguisher involves checking the hose，handle，nozzle，and ______.

A.　sight glass

B.　weighing the extinguisher

C.　service technicians report

D.　last date it was charged

KEY:　B

[868]Official proof of an American vessel's nationality is contained in the ______.

A.　Certificate of Inspection

B.　Official Log

C.　Certificate of Documentation

D.　Shipping Articles

KEY:　C

[869]On vessels over 100 gross tons，whose responsibility is it to ensure that all able seamen in the vessel's crew have the documents and qualifications required by law and regulation?

A.　Master of the vessel

B.　USCG Officer in Charge of Marine Inspection

C.　Owner or operator of the vessel

D.　Company's personnel director

KEY:　A

[870]Port of Registry refers to the port where ______.

A.　the ship has been named

B.　the ship has been built

C.　the ship has been signed

D.　the ship has been registered

KEY:　D

[871]Records of tests and inspections of a cargo vessel's fire extinguishing systems shall be kept on board ______.

A.　for 1 year

B.　for 2 years

C.　until the next Coast Guard inspection

D.　until the vessel's Certificate of Inspection expires

KEY:　D

[872]Spontaneous ignition can result from ______.

A.　an unprotected drop-light bulb

B.　careless disposal or storage of material

C.　smoking in bed

D.　worn electrical wires on power tools

KEY:　B

[873]Temporary Certificates of Inspection for offshore drilling units are effective until the ______.

A.　Operations Manual is approved

B.　Minerals Management Service approval is issued

C.　classification society approval is issued

D.　permanent Certificate of Inspection is issued

KEY:　D

[874]The American Petroleum Institute recommends magnetic particle inspection for ______.

A.　anchor chain

B.　wire rope

C.　connecting links

D.　pendant wires

KEY:　C

[875]The American Petroleum Institute recommends that connecting links and anchor shackles be inspected using ______.

A.　visual examinations

B.　magnetic particle inspection

C.　dye penetrant inspection

D.　x-ray inspection

KEY:　B

[876]The arrangement of charts in the catalogue is ______.

A.　on a regional basis

B.　on a national basis

C.　shown orderly

D.　appeared from A to W

KEY:　A

[877]The authority to grant an alternate procedure for oil transfer operations rests with the ______.

A.　nearest Coast Guard office

B.　Officer-in-Charge，Marine Inspection

C.　Area Commander

D.　Captain of the Port

KEY:　D

[878]The capacity of any liferaft on board a vessel can be determined by ______.

A.　examining the Certificate of Inspection

B.　examining the plate on the outside of the raft container

C.　referring to the Muster List ("Station Bill")

D.　referring to the shipping articles

KEY:　B

[879]The Certificate of Inspection for your tanker authorizes the carriage of grade A and lower products. Which chemical may NOT be carried unless it is specifically endorsed on the Certificate of Inspection?

A.　Amylene

B.　Diisobutyl Phthalate

C.　Oleic acid

D.　Vinyl acetate

KEY:　D

[880]The Certificate of Inspection on a passenger vessel of over 100 gross tons must be posted ______.

A.　in the Master's office

B.　on the bridge

C.　in a conspicuous place where observation by the passengers is likely

D.　in the passengers' dining room or，where there is no dining room，in the lounge area

KEY:　C

[881]The complete details of a crude oil washing system aboard your vessel，including the operating sequences and procedures，design characteristics，a description of the system，and required personnel will be found in the ______.

A.　Oil Transfer Procedures Manual

B.　Crude Oil Washing Operations and Equipment Manual

C.　Code of Federal Regulations

D.　Crude Oil Washing addendum to the Certificate of Inspection

KEY:　B

[882]The consistent winds blowing from the horse latitudes to the doldrums are called the ______.

A.　prevailing westerlies

B.　polar easterlies

C.　trade winds

D.　roaring forties

KEY:　C

[883]The damage to a vessel is over $25，000. Who must notify the nearest Coast Guard Marine Safety or Marine Inspection Office as soon as possible?

A.　The Master of the vessel

B.　The Radio Officer

C.　The Chief Officer

D.　All of the above

KEY:　A

[884]The damage to a vessel is over $25，000. Who must notify the nearest Coast Guard Marine Safety or Marine Inspection Office as soon as possible?

A.　The owner of the vessel

B.　The Master of the vessel

C.　The person in charge of the vessel at the time of casualty

D.　Any one of the above

KEY:　D

[885]The damage to a vessel is over $25，000. Who must notify the nearest Coast Guard Marine Safety or Marine Inspection Office as soon as possible?

A.　The Master of the vessel

B.　The Radio Officer

C.　The Chief Officer

D.　All of the above

KEY:　A

[886]The damage to a vessel is over $25，000. Who must notify the nearest Coast Guard Marine Safety or Marine Inspection Office as soon as possible?

A.　The owner of the vessel

B.　The Master of the vessel

C.　The person in charge of the vessel at the time of casualty

D.　Any one of the above

KEY:　D

[887]The dangerous semicircle of a hurricane in the Northern Hemisphere is that area of the storm ______.

A.　to the right of the storm's track

B.　measured from true north clockwise to 180T

C.　measured from true north counterclockwise to 180T

D.　between the ship's heading and the bearing to the eye

KEY:　A

[888]The dangerous semicircle of a typhoon in the Southern Hemisphere is that area ______.

A.　measured from due south clockwise 180

B.　measured from due south counterclockwise 180

C.　to the left of the storm's track

D.　ahead of the typhoon measured from the storm's track to 90on each side

KEY:　C

[889]The datum from which the predicted heights of tides are reckoned in the tide tables is ______.

A.　mean low water

B.　the same as that used for the charts of the locality

C.　the highest possible level

D.　given in table three of the tide tables

KEY:　B

[890]The depth ______ a shallow patch，lying about 1．3 miles southeastward of Nanshan Tou lighthouse reduces to about 6．1 meters．

A.　above

B.　over

C.　up

D.　upon

KEY:　B

[891]The design specifications for cranes and crane foundations on ship's are set and published by the ______.

A.　American Society of Mechanical Engineers

B.　American Petroleum Institute

C.　Society of Petroleum Engineers

D.　American Society of Civil Engineers

KEY:　B

[892]The dew point is reached when the ______.

A.　temperature of the air equals the temperature of the seawater

B.　atmospheric pressure is 14.7 lbs. per square inch

C.　relative humidity reaches 50%

D.　air becomes saturated with water vapor

KEY:　D

[893]The difference in water spray pattern between the high-velocity tip and low-velocity applicator used with the all-purpose nozzle is due to ______.

A.　a difference in water pressure

B.　the method of breaking up the water stream

C.　the length of the applicator

D.　All of the above

KEY:　B

[894]The direction a vessel is pointed at any given time is the ______.

A.　course

B.　track

C.　heading

D.　course over the ground

KEY:　C

[895]The direction in which a vessel is steered is the course. The path actually followed is the ______.

A.　route

B.　track

C.　heading

D.　course over the ground

KEY:　D

[896]The disadvantage of using a heaving line to pass a towline is that ______.

A.　it increases the likelihood of collision between the towing vessel and the disabled vessel

B.　recreational boaters tend to make the heaving line fast to the towed vessel as if it were the towline

C.　the monkey fist may injure someone

D.　it may reduce the catenary and increase yawing

KEY:　B

[897]The dumping of refuse in a lock is permitted ______.

A.　when approved by the lockmaster

B.　when locking downbound

C.　at no time

D.　during high water only

KEY:　C

[898]The external flotation bladder of an immersion suit should be inflated ______.

A.　only after two hours in the water

B.　only after four hours in the water

C.　before entry into the water

D.　upon entry into the water

KEY:　D

[899]The fog is lifted，so ______ the radar．

A.　close

B.　open

C.　switch off

D.　switch on

KEY:　C

[900]The force exerted by a propeller which tends to throw the stern right or left is called ______.

A.　slip

B.　sidewise force

C.　rotational force

D.　thrust

KEY:　B

04.3 船长业务基本知识

[901]THROWING OVERBOARD OF GOODS IN ORDER TO LIGHTEN THE VESSEL OR IMPROVE ITS STABILITY IN CASE OF AN EMERGENCY defines ______.

A.　Dropping off

B.　Jettison

C.　Throwing away

D.　Taking off

KEY:　B

[902]

A.　The Merchant undertakes that no claim or _______ shall be made against any servant，agent or subcontractor of the Carrier which imposes or attempts to impose upon any of them or any vessel owned by any of them any liability whatsoever in connection with the goods．

B.　determination

C.　allegation

D.　engagement

KEY:　B

[903]______ are included in the Queen's enemies．

A.　Public enemies

B.　Pirates

C.　Robbers

D.　The enemies of the state to which the merchants belong

KEY:　A

[904]______ Charterer and Shipowner agreed on instructions which were ambiguous and were misinterpreted in good faith by the master，it was held that the Charterer could not hold the Shipowner liable．

A.　Whether

B.　Where

C.　That

D.　Which

KEY:　B

[905]______ for loss of contents．

A.　No

B.　N．B．

C.　N．P．

D.　N／R

KEY:　D

[906]______ is the offense against navigational rules and will make the ship unseaworthy with the Shipowner responsible for all the consequences．

A.　Overloading

B.　Overtaking

C.　Overboarding

D.　Overlining

KEY:　A

[907]______ is the price paid to a Shipowner for the transportation of goods or merchandise by sea from one specific port to another．

A.　Freight

B.　Freight rate

C.　Hire

D.　Lighterage

KEY:　A

[908]______ may be defined as a departure without justification and under no necessity from the proper and usual course of an agreed voyage，whereby the character and the incidents of such voyage are altered．

A.　Deviation

B.　Change

C.　Alteration

D.　Amendment

KEY:　A

[909]______ means the sufficiency of a vessel in materials，construction，equipment，crew，and outfit for the trade or service in which it is employed．

A.　Seaworthiness

B.　Cargo-worthiness

C.　Readiness

D.　Proficiency

KEY:　A

[910]A mortgagee does not acquire a right to the freight ______ he has taken actual or constructive possession of the ship．

A.　where

B.　if

C.　and

D.　unless

KEY:　D

[911]A mortgagee only ______ entitled to all the freight which the ship is in the course of earning，when his possession of the ship comes into being．

A.　sustains

B.　maintains

C.　becomes

D.　has

KEY:　C

[912]A port ______ if more than ordinary prudence and skill is needed to avoid exposure to danger there．

A.　safety

B.　is safe

C.　is not safe

D.　is not safety

KEY:　C

[913]A port is ______ if the delay in leaving it would be such as to frustrate the adventure．

A.　not good

B.　unsafe

C.　safe

D.　obtainable

KEY:　B

[914]A refusal to work part of the day only may ______ to be a strike even though such refusal is not in breach of contract．

A.　hold

B.　be held

C.　be maintained

D.　be reported

KEY:　B

[915]A seaman lost his continuous discharge book during the voyage. Upon discharge from Articles，he should be issued a ______.

A.　letter of service on company letterhead signed and sealed by the master

B.　Record of Entry in a Continuous Discharge Book for use in applying for a duplicate book

C.　Certificate of Discharge with the white copy forwarded to the Commandant

D.　Mutual Release (CG-2119)，and the articles should be annotated as to the loss

KEY:　C

[916]A ship crane which has been idle for a period of over six months shall be inspected to the same standards as a ______.

A.　yearly inspection

B.　quarterly inspection

C.　monthly inspection

D.　weekly inspection

KEY:　A

[917]A temporary condition of danger will not make the port ______，provided that such condition will not last an unreasonable time．

A.　unsafe

B.　safe

C.　good

D.　bad

KEY:　A

[918]A trade custom has been proved to the effect that in trade between Sarawak and Singapore mate's receipts ______ universally adopted as documents of title as in same way as bills of lading．

A.　are

B.　takes

C.　makes

D.　has

KEY:　A

[919]A vessel proceeding to London，England，from New York makes an unscheduled call at the port of Hamilton，Bermuda. What term is used to denote the voluntary departure from the usual course?

A.　Alteration

B.　Deviation

C.　Liberty

D.　Unscheduled stop

KEY:　B

[920]Advance freight together with other charges ______ due on shipment．

A.　is

B.　are

C.　has

D.　have

KEY:　A

[921]After an occurrence of oil pollution，what shall be done first by a vessel in Chinese port？ ______.

A.　use oil dispersion agent and wait for investigation

B.　try to collect the oil on deck or in water

C.　report to Harbor Master

D.　B and C

KEY:　D

[922]All accidents and damage to ship，equipment or personnel，must be ______in the vessel deck log.

A.　entered into

B.　entered

C.　entered with

D.　entered in

KEY:　B

[923]An implied warranty of seaworthiness on the part of the vessel's owner lies in the ______.

A.　Cargo Manifest

B.　Certificate of Inspection

C.　Classification Certificate

D.　contract of carriage，i.e. Bill of Lading

KEY:　D

[924]An underwriter is liable for ______.

A.　loss arising from the subject itself because of its inherent qualities

B.　loss caused by the ordinary evaporation of liquids

C.　loss caused by heavy weather

D.　the natural decay of the vessel due to the passage of time

KEY:　C

[925]Any action ______ collision shall，if the circumstances of the case admit，made in ample time and with due regard to the observance of good seamanship．

A.　taking to avoid

B.　taking to avoiding

C.　taken to avoid

D.　taken to avoiding

KEY:　C

[926]Any action taken to avoid collision shall，if the circumstances of the case admit，______.

A.　be positive

B.　be positive and made in ample time

C.　be positive，made in ample time and with due regard to the observance of good seamanship．

D.　be positive，made in ample time，with due regard to the observance of good seamanship and avoiding complying with local regulations．

KEY:　C

[927]Any partial loss or damage shall be ______ pro rata on the basis of such declared value.

A.　adjusted

B.　arranged

C.　determined

D.　fixed

KEY:　A

[928]Cargo damaged due to water being present in a cargo hold，as a result of a leak in the hull plating，is said to be damage by ______. ① Free moisture． ② Transfer of hygroscopic moisture．

A.　① only

B.　② only

C.　Both ① and ②

D.　Neither ① nor ②

KEY:　A

[929]Compliance with the terms of the load line certificate on a vessel is the responsibility of the ______.

A.　Ballast Control Operator

B.　Barge Supervisor

C.　Master or Offshore Installation Manager

D.　operators representative

KEY:　C

[930]Either ______ may be discharged on the ground of impossibility of performance or on the ground of delay，where no breach of contract by either party has taken place．

A.　contract

B.　party

C.　person

D.　company

KEY:　B

[931]Every wrongful act willfully committed by the master or crew against their vessel and her cargo defines ______ ．

A.　Negligence

B.　Barratry

C.　Piracy

D.　Mutiny

KEY:　B

[932]Except as provided for otherwise，an oil tanker might discharge certain amount at certain rate of oil or oily mixtures into sea when ______.

A.　the tanker is within 50 nautical miles from the nearest land

B.　the tanker is no more than 50 nautical miles from the nearest land

C.　the tanker is more than 50 nautical miles from the nearest land

D.　the tanker is not more than 50 nautical miles from the nearest land

KEY:　C

[933]Fueling results in the collection of waste oil in drip pans and containers. Which is an approved method of disposing of the waste oil?

A.　Draining it overboard when the vessel gets underway

B.　Placing it in proper disposal facilities

C.　Adding sinking agents and discharging it into the water

D.　Mixing it with dispersants before draining it overboard

KEY:　B

[934]Functions aboard a tanker or tank barge such as connecting，disconnecting，and topping off must be supervised by ______.

A.　any certificated tankerman

B.　the Master of the vessel

C.　the officer of the watch

D.　the person designated as person in charge

KEY:　D

[935]If a chartered ship is prevented from，or delayed in，getting to the loading port by peril excepted during the voyage，the exception clauses ______.

A.　apply

B.　comply

C.　supply

D.　reply

KEY:　A

[936]If there is a strike or lock-out affecting the cargo on or after vessel's arrival at port of discharge and the same has not been settled within 48 hours，Receivers shall have the option of keeping vessel waiting until such strike or lock-out is at an end against paying half demurrage after expiration of the time prescribed for discharging or ordering the vessel to a substituting port．The “same” refers to ______

A.　the cargo

B.　the port

C.　the arrival

D.　the strike or lock-out

KEY:　D

[937]If you are guilty of failure to properly perform your duties as Operator of Uninspected Towing Vessels，which of the following actions may NOT be taken?

A.　Issuance of a letter of warning

B.　Suspension of your license

C.　Revocation of your license

D.　A fine placed against your license

KEY:　D

[938]If you had started loading earlier，you ______ loading now．

A.　would finish

B.　will have finished

C.　would have finished

D.　will be finished

KEY:　C

[939]In a charter-party，the amount of despatch money payable is often fixed ______.

A.　at one fourth the demurrage rate

B.　at one third the demurrage rate

C.　at half the demurrage rate

D.　at the demurrage rate

KEY:　C

[940]In most cases，the mere reception or dumping down of the cargo on the ship ______ involve completion of loading，because the operation of loading involves all that is required to put the cargo in a condition in which it can be carried．

A.　is not

B.　has not

C.　do not

D.　does not

KEY:　D

[941]In order to accelerate discharging of the main hatches，you are requested to arrange for two gangs of stevedores to work hatches Nos．2 and 4 {round the clock}．

A.　24 hrs a day

B.　16 hrs a day

C.　12 hrs a day

D.　8 hrs a day

KEY:　A

[942]In the case of new crude oil tankers，the additional ballast permitted shall be carried in cargo tanks only if such tanks have been ______ before departure from an oil unloading port or terminal.

A.　inerted

B.　crude oil washed

C.　diluted

D.　cleaned

KEY:　B

[943]In towing，chocks are used to ______.

A.　protect the towline from chafing

B.　secure the end of the towline on the tug

C.　stop off the towline while retrieving it

D.　absorb shock loading on the towline

KEY:　A

[944]In view of ______ bilge water from any vessel，an application shall be made to Harbor Authorities for approval．

A.　discharging

B.　recircling

C.　loading

D.　taking in

KEY:　A

[945]No oil or mixture containing oil ______ into Singapore waters from any vessel．

A.　shall not be discharged

B.　shall be discharged

C.　shall be filled

D.　shall not be filled

KEY:　B

[946]Off hire is when a vessel breaks down，strands，or has any damage that prevents the working of cargo or delays the sailing of the vessel．Under which of the following charter parties is off hire most likely to be mentioned？ ______.

A.　Bareboat

B.　Voyage

C.　Time

D.　Lease

KEY:　C

[947]Oil may NOT be transferred unless ______.

A.　there are two certificated tankermen on each vessel

B.　the vessel is equipped with constant-tension winches

C.　discharge containment equipment (i.e. drip pans) are in place

D.　All of the above

KEY:　C

[948]On a ship's crane，the load chart relates the allowable load to the combination of the boom length and ______.

A.　winch speed

B.　boom strength

C.　load radius

D.　cable strength

KEY:　C

[949]Overcarriage is best prevented by ______.

A.　assuring heavy cargo is stowed low in the vessel

B.　checking the vessel's load line calculations carefully

C.　inspection of the hold at completion of discharge

D.　stowing the cargo which is to be discharged last on the bottom

KEY:　C

[950]Particular average is ______.

A.　the average distance steamed per day over the duration of the voyage

B.　charges against all parties in a marine venture to pay for damages

C.　loss sustained by only one party

D.　claimed after all liens against the vessel are settled

KEY:　C

04.4 船员职务基本知识

[951]The Chief Mate must ascertain that the deck loads are stowed in such a manner as ______.

A.　not to affect the draught trim

B.　not to affect the cargo ventilation

C.　not to affect the free surface

D.　not to affect the vessel's stability

KEY:　D

[952]Which statement concerning the carriage of containers is TRUE? ______.

A.　The Chief Mate and Master of a container ship should have the proposed stowage plan ready for the stevedore upon arrival in port.

B.　When stowed on deck of a break bulk ship，the bottom of the container must be evenly supported throughout.

C.　With tiered containers，a 40-foot container may be stowed on top of two 20-foot containers.

D.　Deck load calculations must take into account the square footage of the entire container bottom.

KEY:　C

[953]Prior to getting underway，the Master or person in charge of a vessel must ______.

A.　conduct a fire drill

B.　conduct a boat drill

C.　log the fore and aft draft marks

D.　test the emergency generator

KEY:　C

[954]Shipping articles are contracts of agreement between the members of the crew and the __________.

A.　Charterers

B.　Coast Guard

C.　Master

D.　vessel's owner

KEY:　C

[955]The Master to prosecute all voyages with the utmost__________ and to render customary assistance with the Vessel's Crew.

A.　fastness

B.　speeding

C.　swiftness

D.　despatch

KEY:　。

[956]The Master to sign or if required by Charterers to authorise Charterers or their Agents to sign__________　for cargo as presented in conformity with Mate's or Tally Clerks' receipts.

A.　Charter Party

B.　Mate’s Receipts

C.　Document relating to freight

D.　Bills of Lading

KEY:　。

[957]______ is a room on or near the bridge provided with the necessary fittings and furniture for the handling and stowage of charts and where the chronometers are placed．

A.　Captain's cabin

B.　Chief Officer's locker

C.　Chartroom

D.　Pilot's cabin

KEY:　C

[958]A copy of the Articles of Agreement，less the names，is required to be posted. This document is called the______.

A.　Shipping Articles

B.　Fo'c'sle Card

C.　Voyage Agreement

D.　Articles Copy

KEY:　B

[959]A refusal to work part of the day only may ______ to be a strike even though such refusal is not in breach of contract．

A.　hold

B.　be held

C.　be maintained

D.　be reported

KEY:　B

[960]A seaman is entitled by law to a release from Foreign Articles when ______.

A.　intoxicated

B.　the vessel is overloaded

C.　there is a change of home port

D.　there is a change of Master

KEY:　B

[961]A seaman is reported missing in the morning and was last seen after coming off the mid-watch. Which type of turn would you use to return to the trackline steamed during the night?

A.　Williamson

B.　Racetrack

C.　180turn

D.　Anderson

KEY:　A

[962]A seaman may not make an allotment to his ______.

A.　minor children

B.　grandparents

C.　brother

D.　mother-in-law

KEY:　D

[963]After a seaman is discharged，at the end of the voyage，the final payment of wages due must be made at the earlier of within 24 hours after the cargo has been discharged or______.

A.　within 4 days after the seaman's discharge

B.　prior to the vessel's departure on the next voyage

C.　prior to loading any out-bound cargo

D.　prior to any change of Master

KEY:　A

[964]All vessel personnel should be familiar with the survival craft's ______.

A.　boarding and operating procedures

B.　maintenance schedule

C.　navigational systems

D.　fuel consumption rates

KEY:　A

[965]As Chief Officer of a vessel underway，it comes to your attention that the vessel is，in some manner，unseaworthy. Under such circumstances the Master is required to take action upon receiving ______.

A.　information of such condition from yourself

B.　notification of such condition from yourself and the Second Officer

C.　notification of such condition from yourself and any other member of the crew

D.　notification of such condition from yourself or the Second Officer

KEY:　B

[966]At common law，the master，as agent of ______，has the right to land or warehouse unclaimed goods．

A.　Charterer

B.　Shipowner

C.　cargo owner

D.　shipper

KEY:　B

[967]Before sailing，mariners on duty shall check the headings of magnetic compass by comparison with ______.

A.　ship's clock

B.　engine movement recorder

C.　repeaters

D.　navigation lights

KEY:　C

[968]By regulation，your cargo vessel must have ______.

A.　sufficient facilities for the crew to wash their clothes

B.　a wash basin in each room

C.　separate toilet facilities for engineers where their number exceeds six

D.　All of the above

KEY:　A

[969]Chief officer，I have ______ things to tell you．

A.　a little

B.　a few

C.　little

D.　much

KEY:　B

[970]Chief，I see that ship ______ to her port．

A.　turns

B.　turning

C.　to turn

D.　be turning

KEY:　B

[971]Compliance with the terms of the load line certificate on a vessel is the responsibility of the ______.

A.　Ballast Control Operator

B.　Barge Supervisor

C.　Master or Offshore Installation Manager

D.　operators representative

KEY:　C

[972]Do you know ______ ？

A.　where is the chief officer

B.　where the chief officer is

C.　is where the chief officer

D.　the chief officer is where

KEY:　B

[973]During an abandonment or drill，the first person to arrive at the survival craft should ______.

A.　pass out food and water to personnel

B.　open the doors and start the sprinkler system

C.　activate the emergency release handle

D.　open the doors and prepare the craft for boarding

KEY:　D

[974]Even in the absence of express instructions，the authority of the master of a ship is very large and ______ to all acts that are usual and necessary for the employment of the ship．

A.　expends

B.　spreads

C.　extends

D.　enlarges

KEY:　C

[975]He is ______ a carpenter as a chief officer．

A.　more

B.　not so much

C.　rather

D.　not such

KEY:　B

[976]In terms of vessel manning，a watch is the ______.

A.　direct performance of deck or engine operations in a scheduled and fixed rotation

B.　performance of maintenance work necessary for the vessel's safe operation，on a daily basis

C.　performance of lookout duties

D.　direct performance of cargo loading and discharge operations only

KEY:　A

[977]In the event that the Company pays the taxes on your behalf，you agree to ______ the Company against all such payments it may make in respect thereof.

A.　claim

B.　indemnify

C.　recover

D.　settle

KEY:　B

[978]It is requested that the ship's personnel are ______ the port regulations．

A.　to conform to

B.　to draw up

C.　to comply with

D.　to be applicable to

KEY:　C

[979]It is the chief mate's job ______ the tank cleaning operation．

A.　to plan

B.　planning

C.　for planning

D.　in planning

KEY:　A

[980]It is the responsibility of the Master or person in charge of a ship to ensure that ______.

A.　the station bill is posted in each compartment

B.　temporary personnel and visitors are advised of emergency stations

C.　names of crew members are listed on the station bill

D.　no changes are made to the station bill

KEY:　B

[981]It is the responsibility of the person in charge to ______.

A.　revise the operating manual when rig personnel change

B.　be fully aware of the provisions in the operating manual

C.　maintain the rig's construction portfolio

D.　All of the above

KEY:　B

[982]It should be observed ______ the master is not bound to show both the number of packages and the weight．

A.　that

B.　which

C.　where

D.　when

KEY:　A

[983]Masters are ______ solely responsible for all damages or accidents of whatever kind resulting from the navigating or handling of their vessels by day or by night．

A.　hold

B.　holded

C.　held

D.　holding

KEY:　C

[984]Masters are ______ to make themselves well acquainted with these regulations．

A.　binding

B.　bound

C.　bounded

D.　bounding．

KEY:　B

[985]Masters are required to notify Port Authorities of anchorage bearings after ______.

A.　departure

B.　leaving

C.　anchored

D.　anchoring

KEY:　D

[986]Maybe the Chief Officer is ______ in his calculations．

A.　up

B.　out

C.　over

D.　down

KEY:　A

[987]No person may serve as the person in charge of both the vessel and the facility during oil transfer operations unless ______.

A.　there is ready access between the two

B.　the vessel and facility are immediately adjacent

C.　the person in charge has a rapid means of transportation between the two

D.　the Captain of the Port authorizes such procedure

KEY:　D

[988]No person whose license has been revoked shall be issued another license except upon ______.

A.　approval of the Commandant

B.　taking a new examination

C.　approval of the Officer-in-Charge，Marine Inspection

D.　approval of an administrative law judge

KEY:　A

[989]On a tanker，sanitary inspections of the crew's quarters are the responsibility of the ______.

A.　Master and Second Mate

B.　Master and Chief Engineer

C.　Master and union delegate

D.　Master and Chief Mate

KEY:　B

[990]Orders of the Master to the officer of the watch which s / he must comply with are ______.

A.　Night orders

B.　Standing orders

C.　Commands by master

D.　Requirements by master

KEY:　B

[991]Please give particulars of every case of {illness or death} occurring on board，the Health officer said to the master．

A.　Symptoms or Case

B.　disease or accident

C.　sickness or mortality

D.　Health or person

KEY:　C

[992]Shipping articles are a contract of agreement between the members of the crew and the ______.

A.　charterers

B.　Coast Guard

C.　Master

D.　vessel's owner

KEY:　C

[993]Thank you for ______.

A.　a information

B.　the informations

C.　your information

D.　your informations

KEY:　C

[994]The Chief Checker reminded the Chief Officer ______ the dried turnips on top of the turpentine．

A.　to put not

B.　not to put

C.　of putting

D.　of not putting

KEY:　C

[995]The Chief Officer ______ told the stevedores to stow the cargo lot by lot．

A.　plainly

B.　playfully

C.　absolutely

D.　completely

KEY:　A

[996]The Chief Officer estimated the broken space ______ 3 500 cubic feet．

A.　at

B.　by

C.　with

D.　in

KEY:　A

[997]The chief officer has the responsibility for ______.

A.　supervising the handling of cargo

B.　the upkeep of the ship and her equipment

C.　supervising the handling of cargo and the upkeep of the ship

D.　supervising the handling of cargo and the upkeep of deck equipment

KEY:　D

[998]The Chief Officer insisted on putting remarks regardless ______ my objection．

A.　at

B.　in

C.　of

D.　with

KEY:　C

[999]The Declaration of Inspection made before oil transfer operations must be signed by the ______.

A.　Master of the vessel

B.　Captain of the Port

C.　person(s) in charge

D.　All of the above

KEY:　C

[1000]The document which acknowledges that the cargo has been received and is in the carrier's custody is called the ______.

A.　Dock Receipt

B.　Hatch Report and Recapitulation

C.　Cargo Manifest

D.　Stowage Plan

KEY:　A

六　　国际公约

06.1 STCW公约的有关内容

[1701]Your ship must {speed up}．

A.　reduce speed

B.　increase speed

C.　change speed

D.　stop engine

KEY:　B

[1702]You'd better ______ your course to port to avoid collision．

A.　to alter

B.　alter

C.　altering

D.　altered．

KEY:　B

[1703]The damage to the hatch cover was caused by your stevedores，and you ought to have known we never ______ lies．

A.　say

B.　talk

C.　tell

D.　speak

KEY:　C

[1704]During a storm，the chance of fatigue failure of a mooring line will increase as ______.

A.　vessel motions increase

B.　mooring tensions decrease

C.　KG increases

D.　KG decreases

KEY:　A

[1705]A passenger vessel is underway. When may passengers visit the pilothouse?

A.　Passengers are excluded from the pilothouse while underway.

B.　Passengers are permitted in the pilothouse during daylight hours only.

C.　Passengers are permitted to visit the pilothouse when authorized by the Master and officer of the watch.

D.　Passengers are permitted in the pilothouse when they are escorted by a ship's officer.

KEY:　A

[1706]At 0900，you take the following radar ranges: Watch Hill Point 5.4 miles; Block Island Grace Point 8.3 miles. Which statement is TRUE?

A.　You are within 3 nautical miles of the coast.

B.　The bottom in the area is sand and gravel.

C.　The fix is indeterminate.

D.　You are governed by the Inland Rules.

KEY:　C

[1707]Functions aboard a tanker or tank barge such as connecting，disconnecting，and topping off must be supervised by ______.

A.　any certificated tankerman

B.　the Master of the vessel

C.　the officer of the watch

D.　the person designated as "person in charge"

KEY:　D

[1708]If your passenger vessel is fitted with a loudspeaker system，it must be tested at least once ______.

A.　every week

B.　a day

C.　every trip

D.　a watch or once a trip，whichever is shorter

KEY:　A

[1709]In writing up the logbook at the end of your watch，you make an error in writing an entry. What is the proper way of correcting this error?

A.　Carefully and neatly erase the entry and rewrite it correctly.

B.　Cross out the error with a single line，write the correct entry，and initial it.

C.　Blot out the error completely and rewrite the entry correctly.

D.　Remove this page of the log book and rewrite all entries on a clean page.

KEY:　B

[1710]Maritime Administration personnel may be allowed in the pilothouse upon the responsibility of the ______.

A.　Chief Officer

B.　Navigator

C.　most senior person present from the Maritime Administration

D.　officer in charge of the watch

KEY:　D

[1711]On which vessels are the officers and deck crew allowed to operate under a two-watch system on voyages of less than 600 miles?

A.　Towing vessels

B.　Offshore supply vessels

C.　Barges，when manned

D.　All of the above

KEY:　D

[1712]Reducing free surfaces has the effect of lowering the ______.

A.　uncorrected KG

B.　virtual height of the center of gravity

C.　metacenter

D.　metacentric height

KEY:　B

[1713]Sideways movement of the mast is resisted by the ______.

A.　shrouds

B.　halyards

C.　sheets

D.　forestay

KEY:　A

[1714]The BEST information on the nature and extent of damage to the vessel is obtained from ______.

A.　alarms and monitoring devices

B.　the engineroom watch

C.　personnel at the scene of the damage

D.　the bridge watch

KEY:　C

[1715]The coastline between Point Judith and Watch Hill is ______.

A.　steep with rocky bluffs

B.　sandy and broken by rocky points

C.　low and marshy

D.　heavily forested

KEY:　B

[1716]The crude oil washing installation and associated equipment and arrangements shall ______ with the requirements established by the Administration.

A.　comply

B.　supply

C.　provide

D.　deny

KEY:　A

[1717]The fitting that allows a boom to move freely both vertically and laterally is called the ______.

A.　swivel

B.　lizard

C.　spider band

D.　gooseneck

KEY:　D

[1718]The fluke angle of an anchor system is the angle between the ______.

A.　flukes and the shank

B.　shank and the sea bottom

C.　mooring line and the sea bottom

D.　flukes and the shackle

KEY:　A

[1719]The grip of a joint represents the ______.

A.　thickness of the connected members

B.　diameter of the head

C.　entire length of the rivet

D.　diameter of the shank

KEY:　A

[1720]The horizontal flat surfaces where the upper stock joins the rudder are the ______.

A.　rudder keys

B.　rudder palms

C.　lifting flanges

D.　shoes of the rudder

KEY:　B

[1721]The irregular black line around a charted light such as Race Rock Light indicates that it is ______.

A.　unwatched

B.　surrounded by riprap

C.　a minor light

D.　constructed on an artificial island

KEY:　B

[1722]The owner or Master of a towing vessel shall ensure that all tests and inspections of gear take place and are logged ______.

A.　when a new Master assumes command

B.　daily，at 0800 local zone time

C.　weekly，before 2400 Saturday

D.　immediately after assuming the watch

KEY:　A

[1723]The owner or Master of a towing vessel shall ensure that all tests and inspections of gear take place and are logged ______.

A.　on each watch，immediately before being relieved

B.　before embarking on a voyage of more than 24 hours

C.　daily，at 1200 local zone time

D.　weekly，before 0000 Sunday

KEY:　B

[1724]The projecting lugs of the rudderpost which furnish support to the rudder are called ______.

A.　bases

B.　gudgeons

C.　pintles

D.　rudder lugs

KEY:　B

[1725]The ratio of the height of a vessel's rudder to its width is referred to as the ______.

A.　aspect ratio

B.　constriction ratio

C.　rudder ratio

D.　steering ratio

KEY:　A

[1726]The sprocket teeth on a wildcat are known as the ______.

A.　pawls

B.　devil's claws

C.　whelps

D.　pockets

KEY:　C

[1727]The sprocket wheel in a windlass，used for heaving in the anchor，is called a ______.

A.　capstan

B.　dog wheel

C.　fairlead

D.　wildcat

KEY:　D

[1728]The term PINTLE AND GUDGEON are associated with the ______.

A.　anchor windlass

B.　jumbo boom

C.　rudder

D.　steering engine

KEY:　C

[1729]The Tonnage Certificate gives ______ of a vessel which is classified into gross tonnage and net tonnage．

A.　the descriptions

B.　the measurements

C.　the specifications

D.　the prescriptions

KEY:　B

[1730]The upward slope of a ships bottom from the keel to the bilge is known as ______.

A.　camber

B.　slope

C.　deadrise

D.　keel height

KEY:　C

[1731]The upward slope of a vessels bottom from the keel to the bilge is called ______.

A.　camber

B.　sheer

C.　rake

D.　rise of bottom

KEY:　D

[1732]The use of liners in riveted construction is eliminated by using ______.

A.　lapped construction

B.　strapped construction

C.　joggled construction

D.　belted construction

KEY:　C

[1733]The vessel's quarter is located ______.

A.　abeam

B.　dead astern

C.　just forward of the beam on either side

D.　on either side of the stern

KEY:　D

[1734]There are ______ rats on board．

A.　not

B.　no

C.　any

D.　none

KEY:　B

[1735]Upon completion of fueling a gasoline driven vessel it is necessary to ______.

A.　keep ports，doors，windows，and hatches closed

B.　start engines immediately

C.　ventilate before starting engine

D.　None of the above

KEY:　C

[1736]What is the BEST conductor of electricity?

A.　Carbon dioxide

B.　Distilled water

C.　Fresh water

D.　Salt water

KEY:　D

[1737]What is the best instrument for establishing a safe working area before welding in a confined space?

A.　An oxygen indicator

B.　A combustible gas indicator

C.　A combination combustible gas and oxygen indicator

D.　A flame safety lamp

KEY:　C

[1738]What is the female section of a tool joint called?

A.　Stem

B.　Pin

C.　Box

D.　Stand

KEY:　C

[1739]What is the function of an air receiver in the compressed air system on a ship?

A.　Condenses moisture

B.　Provides overpressure protection

C.　Purifies the air

D.　Acts as an accumulator

KEY:　D

[1740]What is the most important difference between the bow type anchor shackle and the D-type anchor shackle?

A.　The bow type shackle provides a superior connection.

B.　The D-type shackle is weaker than the bow type.

C.　The bow type shackle is weaker than the D-type.

D.　The D-type shackle provides an inferior connection.

KEY:　C

[1741]What is the purpose of the limit switch on gravity davits?

A.　To cut off the power when the davits hit the track safety stops

B.　To stop the davits from going too fast

C.　To cut off the power when the davits are about 12 inches or more from the track safety stops

D.　None of the above

KEY:　C

[1742]What time shall we {pass} the Suez Canal？

A.　reach

B.　bear

C.　transit

D.　watch

KEY:　C

[1743]When must the Master or person in charge of a ship log the position of load line marks in relation to the surface of the water in the logbook?

A.　Once a day

B.　At the change of every watch

C.　Only when in fresh or brackish water

D.　Prior to getting underway

KEY:　D

[1744]When the pilot is embarked he or she ______.

A.　relieves the officer of the watch

B.　relieves the Master of his duties

C.　is solely responsible for the safe navigation of the vessel

D.　is a specialist hired for his or her local navigational knowledge

KEY:　D

[1745]Which device is designed to automatically hold the load if power should fail to an electric winch?

A.　Pneumatic brake

B.　Electromagnetic brake

C.　Hand brake

D.　Motor controller

KEY:　B

[1746]Which fitting is used to connect the boom to the mast?

A.　Clevis pin

B.　Gunter-lug

C.　Gooseneck

D.　Transom

KEY:　C

[1747]Which statement describes the shore between Watch Hill Point and Point Judith?

A.　Low，rocky cliffs with heavily wooded hills inland

B.　Sandy beaches broken by rocky points

C.　Sand dunes and beaches with a mud and sand bottom

D.　Wooded，barren hills with isolated prominent buildings

KEY:　B

[1748]Which two components pass through the shank of an LWT anchor?

A.　Anchor shackle and stock

B.　Tripping palm and flukes

C.　Crown and chocks

D.　Swivel and stabilizer bar

KEY:　A

[1749]Which word indicates color variation in the characteristics of a light?

A.　Opposing

B.　Changing

C.　Reversing

D.　Alternating

KEY:　D

[1750]While the Pilot is maneuvering the vessel to a dock，what is the PRIMARY responsibility of the watch officer?

A.　Insure that helm and throttle orders given by the Pilot are correctly executed.

B.　Judge the appropriateness of the Pilot's orders and countermand them if necessary.

C.　Supervise the signaling and flag etiquette.

D.　Record the bells and their times in the bell book.

KEY:　A

06.2 SOLAS公约的有关内容

[1751]The Safety Radio Certificate for the Cargo Ship is issued ______.

A.　by the Radiotelegraphy Authority of a country．

B.　in accordance with the Government Regulations．

C.　by complying with the Provisions of the National Conventions．

D.　under the Provisions of the SOLAS Convention

KEY:　D

[1752]Instructions on how to conduct search and rescue are given in the ______ which is primarily designed for use by merchant ship．

A.　MARPOL

B.　IRPCS

C.　SOLAS

D.　MERSAR

KEY:　D

[1753]Due to the nature of a vessel's construction for a particular trade，it does not fully comply with the provisions of SOLAS. Where will this be indicated? ______.

A.　On the Exemption Certificate

B.　On the reverse of the particular SOLAS certificate affected

C.　On the face of the Certificate of Inspection

D.　Nowhere； the vessel must engage in international trade.

KEY:　A

[1754]The Safety Equipment Certificate shows that the vessel conforms to the standards of the ______.

A.　MSA

B.　American Bureau of Shipping

C.　American Salvage Association

D.　SOLAS Convention

KEY:　D

[1755]Temporary Certificates of Inspection are effective until the ______.

A.　SOLAS Certificate is issued

B.　Loadline Certificate is renewed

C.　classification society approval is issued

D.　permanent Certificate of Inspection is issued

KEY:　D

[1756]______ should be consulted to obtain the information concerning port documents required by the port Authority．

A.　Admiralty Notices to Mariners

B.　SOLAS

C.　Admiralty Sailing Directions

D.　Guide to Port Entry

KEY:　D

[1757]The Charterers shall have the privilege to ship dangerous cargo in accordance with __________ Code or any competent authorities' regulations，as applicable.

A.　IMDG

B.　Hague

C.　SOLAS

D.　MARPOL

KEY:　危

[1758]The Vessel is a __________ bulk carrier which is permitted to carry grain in bulk without requiring any fittings under the Rules of the 1974 International Safety of Life at Sea Convention.

A.　self-loading

B.　self-trimming

C.　self-unloading

D.　self-discharging

KEY:　s

[1759]A documented vessel operating over 50 miles offshore must carry an inflatable liferaft with a ______.

A.　SOLAS A pack

B.　SOLAS B pack

C.　coastal pack

D.　small vessel pack

KEY:　A

[1760]A shipboard Oil Pollution Emergency Plan is required of ______.

A.　all vessels，regardless of size and commercial application

B.　any barge or ship constructed or operated in such a manner that no oil in any form can be carried aboard

C.　an oil tanker of 150 gross tons and above，or other ship of 400 gross tons and above

D.　an oil tanker of 400 gross tons and above，or other ships of 150 gross tons and above

KEY:　C

[1761]A SOLAS passenger ship safety certificate is required on all ______.

A.　T-Boats carrying more than 49 passengers for hire

B.　T-Boats that carry more than 12 passengers on an international voyage

C.　T-Boats carrying more than 150 passengers for hire

D.　large excursion vessels on lakes，bays，sounds，and river routes

KEY:　B

[1762]A SOLAS passenger ship safety certificate is required on all ______.

A.　T-Boats carrying more than 49 passengers for hire

B.　T-Boats that carry more than 12 passengers on an international voyage

C.　T-Boats carrying more than 150 passengers for hire

D.　large excursion vessels on lakes，bays，sounds，and river routes

KEY:　B

[1763]A vessel cannot comply with all of the SOLAS requirements due to its construction. Where will this be indicated?

A.　Nowhere; the vessel must comply to engage in international trade.

B.　On the reverse of the SOLAS certificate

C.　On the Exemption Certificate

D.　On the Certificate of Inspection

KEY:　C

[1764]A vessel cannot comply with all of the SOLAS requirements due to its construction. Where will this be indicated?

A.　Nowhere; the vessel must comply to engage in international trade.

B.　On the reverse of the SOLAS certificate

C.　On the Exemption Certificate

D.　On the Certificate of Inspection

KEY:　C

[1765]An inflatable liferaft equipped with a SOLAS B pack must be stowed ______.

A.　so as to float free

B.　with the vessel's emergency equipment

C.　near the wheelhouse

D.　as far forward as possible

KEY:　A

[1766]Due to the nature of a vessel's construction for a particular trade，it does not fully comply with the provisions of SOLAS. Where will this be indicated?

A.　On the Exemption Certificate

B.　On the reverse of the particular SOLAS certificate affected

C.　On the face of the Certificate of Inspection

D.　Nowhere; the vessel must comply to engage in international trade.

KEY:　A

[1767]Due to the nature of a vessel's construction for a particular trade，it does not fully comply with the provisions of SOLAS. Where will this be indicated? ______.

A.　On the Exemption Certificate

B.　On the reverse of the particular SOLAS certificate affected

C.　On the face of the Certificate of Inspection

D.　Nowhere； the vessel must comply to engage in international trade.

KEY:　A

[1768]Goods of an inflammable，explosive or dangerous nature to the shipment whereof the carrier，master or agent of the carrier，has not consented，with knowledge of their nature and character，may at any time before discharge，______ at any place or destroyed or rendered innocuous by the carrier without compensation．

A.　be loaded

B.　be landed

C.　be lended

D.　be loaned

KEY:　B

[1769]Halon gas will decompose and may form very hazardous toxic fumes when discharged ______.

A.　directly on flames

B.　at room temperature

C.　in an extremely cold climate

D.　None of the above

KEY:　A

[1770]How many operators are needed／required on board according to the provisions of the International Convention for the Safety of Life at Sea，1974？ ______.

A.　4 operators

B.　3 operators

C.　2 operators

D.　1 operator

KEY:　D

[1771]In ______ convention，a vessel which carries more than 12 passengers shall be deemed as a passenger ship．

A.　COSCO

B.　STCW

C.　SOLAS

D.　MARPOL

KEY:　C

[1772]In accordance with SOLAS，the batteries that power interior lighting in inflatable liferafts can be made to last longer by ______.

A.　unscrewing the bulb during the daylight

B.　switching the light on only when necessary

C.　taking no action as there is no way on saving power

D.　taking no action as they shut off automatically in daylight

KEY:　B

[1773]On a rigid liferaft (SOLAS B pack) which is equipped with all of the required equipment you may NOT find a ______.

A.　bailer

B.　sponge

C.　whistle

D.　fishing kit

KEY:　D

[1774]Safety of navigation is dealt with in chapter V of the SOLAS Convention which identifies certain navigation safety services which should be provided by Contracting Governments and sets forth provisions of an operational nature applicable in general to all ships on all voyages. This is ______ the Convention as a whole.

A.　in contrast to

B.　in compliance with

C.　the essence of

D.　representing

KEY:　C

[1775]SOLAS is the abbreviation of ______.

A.　International Maritime Dangerous Goods Carried by Sea

B.　International Code for the Construction and Equipment of Ships Carrying Gases in Bulk

C.　International Convention for the Safety of Life at Sea

D.　International Regulations for Preventing Collisions at Sea

KEY:　C

[1776]SOLAS requires a lifesaving training manual be provided in each crew cabin or in the ______.

A.　bridge

B.　engineering control station

C.　recreation and messrooms

D.　fire control room

KEY:　C

[1777]Temporary Certificates of Inspection are effective until the ______.

A.　SOLAS Certificate is issued

B.　Load Line Certificate is renewed

C.　classification society approval is issued

D.　permanent Certificate of Inspection is issued

KEY:　D

[1778]The document that certifies the correctness of the load line marks on a ship is called the ______.

A.　Certificate of Inspection

B.　Load line certificate

C.　Certificate of Documentation

D.　SOLAS Certificate

KEY:　B

[1779]The document that certifies the correctness of the load line marks on a ship is called the ______.

A.　Certificate of Inspection

B.　Load line certificate

C.　Certificate of Documentation

D.　SOLAS Certificate

KEY:　B

[1780]The number of able seamen required on board is stated in the ______.

A.　American Bureau of Shipping code

B.　Solas Certificate

C.　Classification Certificate

D.　Certificate of Inspection

KEY:　D

[1781]The number of certificated able seamen and lifeboatmen required on board is listed in the ______.

A.　Certificate of Inspection

B.　American Bureau of Shipping code

C.　Muster List ("Station Bill")

D.　Safety of Life at Sea Convention

KEY:　A

[1782]The number of certificated able seamen and lifeboatmen required on board is listed in the ______.

A.　Certificate of Inspection

B.　American Bureau of Shipping code

C.　station bill

D.　Safety of Life at Sea Convention

KEY:　A

[1783]The number of certificated able seamen and lifeboatmen required on a ship is stated in the ______.

A.　Certificate of Inspection

B.　American Bureau of Shipping code

C.　Minerals Management Service regulations

D.　Safety of Life at Sea Convention

KEY:　A

[1784]The Safety of Life at Sea Convention was developed by the ______.

A.　IMDG conference

B.　American Bureau of Shipping

C.　International Maritime Organization

D.　American Institute of Maritime Shipping

KEY:　C

[1785]The Safety Radio Certificate for the cargo Ship is issued ______.

A.　by the Radiotelegraphy Authority of a country．

B.　in accordance with the Government Regulations．

C.　by Complying with the Provisions of the international Conventions．

D.　under the Provisions of the SOLAS Convention

KEY:　D

[1786]The use of sinking and dispersing chemical agents for removal of surface oil is ______.

A.　the most common method

B.　too expensive for common use

C.　generally safe to sea life

D.　authorized only with prior approval of the governmental On-Scene Coordinator

KEY:　D

[1787]To determine the number of Able Seamen required on a mobile offshore drilling unit，you should check the ______.

A.　load line certificate

B.　Operations Manual

C.　Safety of Life at Sea Certificate

D.　Certificate of Inspection

KEY:　D

[1788]To determine the number of able seamen required on an inspected vessel，you should check the ______.

A.　Load Line Certificate

B.　operations manual

C.　Safety of Life at Sea Certificate

D.　Certificate of Inspection

KEY:　D

[1789]To determine the number of able seamen required on an inspected vessel，you should check the ______.

A.　Load Line Certificate

B.　operations manual

C.　Safety of Life at Sea Certificate

D.　Certificate of Inspection

KEY:　D

[1790]To determine the number of certificated lifeboatmen required on a vessel，you should check the ______.

A.　Load Line Certificate

B.　Certificate of Inspection

C.　Safety of Life at Sea Certificate

D.　operations manual

KEY:　B

[1791]To determine the number of inflatable liferafts required on a vessel，you should check the ______.

A.　Load Line Certificate

B.　SOLAS Certificate

C.　Stability Letter

D.　Certificate of Inspection

KEY:　D

[1792]To determine the number of portable fire extinguishers required on an inspected vessel，you should check the ______.

A.　hot work permit

B.　Certificate of Inspection

C.　Safety of Life at Sea Certificate

D.　Muster List ("Station Bill")

KEY:　B

[1793]To determine the number of portable fire extinguishers required on an inspected vessel，you should check the ______.

A.　hot work permit

B.　Certificate of Inspection

C.　Safety of Life at Sea Certificate

D.　station bill

KEY:　B

[1794]To determine the number of portable fire extinguishers required on an inspected vessel，you should check the ______.

A.　hot work permit

B.　Certificate of Inspection

C.　Safety of Life at Sea Certificate

D.　station bill

KEY:　B

[1795]To determine what navigation lights and day-shapes must be displayed on mobile offshore drilling units under tow，you should check the ______.

A.　American Bureau of Shipping classification rules

B.　International Regulations for Preventing Collisions at Sea

C.　Safety of Life at Sea Convention

D.　Minerals Management Service rules

KEY:　B

[1796]To determine what navigation lights and dayshapes must be displayed on mobile offshore drilling units under tow，you should check the ______.

A.　American Bureau of Shipping classification rules

B.　International Regulations for Preventing Collisions at Sea

C.　Safety of Life at Sea Convention

D.　Minerals Management Service rules

KEY:　B

[1797]What is the period of validity of the SOLAS required Cargo Ship Safety Construction Certificate?

A.　12 months

B.　24 months

C.　48 months

D.　60 months

KEY:　D

[1798]Where will you find the requirements for the signals that must be sounded by a mobile offshore drilling unit that is being towed through an area of restricted visibility?

A.　COLREGS

B.　MMS Rules

C.　SOLAS Convention

D.　Coast Pilot

KEY:　A

[1799]Which document lists all the lifesaving equipment required for a vessel?

A.　Certificate of Inspection

B.　American Bureau of Shipping Classification Certificate

C.　International Convention for the Safety of Life at Sea Certificate

D.　Certificate of Registry

KEY:　A

[1800]Which document lists all the lifesaving equipment required for a vessel?

A.　Certificate of Inspection

B.　American Bureau of Shipping Classification Certificate

C.　International Convention for the Safety of Life at Sea Certificate

D.　Certificate of Registry

KEY:　A

06.3 MARPOL公约的有关内容

[1801]A Certificate of Financial Responsibility attests that the vessel ______.

A.　has financial backing to meet any liability resulting from the discharge of oil

B.　has the minimum required amount of P & I and hull insurance

C.　will assume the responsibility for any damage or loss to the shipper

D.　has financial reserves to meet reasonable expected crew costs of an intended voyage

KEY:　A

[1802]A ship required to carry an Oil Record Book must maintain the book on board for ______.

A.　one year

B.　two years

C.　three years

D.　four years

KEY:　C

[1803]A ship which is required to carry an Oil Record Book must log in the book ______.

A.　discharge of ballast or cleaning water from fuel tanks

B.　sounding levels of all fuel tanks on a daily basis

C.　the grade and specific gravity of all fuel oil carried

D.　fuel consumption rates on a weekly basis

KEY:　A

[1804]A vessel emitting harmful substances into the air or spilling oil into the sea is a ______.

A.　Polluter

B.　Emitter

C.　Spiller

D.　Oiler

KEY:　A

[1805]A vessel in ocean service that does not have an approved means of processing oily bilge slops or oily ballast must have ______.

A.　a fixed piping system for ballast discharge to a reception facility

B.　a discharge outlet for the ballast system on each side of the weather deck

C.　one portable adapter for a shore connection to the ballast line

D.　All of the above

KEY:　A

[1806]A vessel to which Annex V to MARPOL 73/78 applies is 24 nautical miles from the nearest land. Which type of garbage is prohibited from being discharged?

A.　Glass

B.　Crockery

C.　Metal

D.　Dunnage

KEY:　D

[1807]A vessel to which Annex V to MARPOL 73/78 applies is located in a MARPOL designated special area，14 nautical miles from nearest land. What type of garbage is permitted to be discharged?

A.　Paper products

B.　Glass ground to less than 1

C.　Metal ground to less than 1

D.　Food waste

KEY:　D

[1808]According to Annex V to MARPOL 73/78，garbage containing plastic is permitted to be disposed of by ______.

A.　incinerating offshore

B.　discharging when at least 12 nautical miles from nearest land

C.　grinding to less than 1 and discharging at least 12 nautical miles from nearest land

D.　grinding to less than 1 and discharging at least 25 nautical miles from nearest land

KEY:　A

[1809]All oil spills must be reported to the ______.

A.　MOC

B.　MSA OF CHINA

C.　local police

D.　local fire department

KEY:　B

[1810]An oil tanker with dedicated clean ballast tanks shall have adequate tank capacity dedicated solely to the carriage of ______ as defined.

A.　clean ballast

B.　crude oil

C.　fresh water

D.　fuel oil

KEY:　A

[1811]Annex V to MARPOL 73/78 contains requirements pertaining to the discharge into the marine environment of ______.

A.　oil

B.　garbage

C.　noxious liquid substances

D.　None of the above

KEY:　B

[1812]Any discharge into the sea of oil or oily mixtures from ships to which this Annex of MARPOL 73/78 applies shall be ______ except when special conditions are satisfied.

A.　required

B.　requested

C.　promoted

D.　prohibited

KEY:　D

[1813]Application for a waiver of any requirements of the regulations for oil transfer operations must be submitted to the ______.

A.　District Commander

B.　Commandant

C.　Captain of the Port

D.　nearest MSA office

KEY:　C

[1814]Before operating a non-oceangoing ship greater than 100 gross tons it must have a fixed piping system to discharge oily mixtures ashore. This system must include______.

A.　approved oily-water separating equipment

B.　a fixed or portable containment system at the shore connection

C.　a spare pump in case the main pump is inoperative

D.　at least one outlet accessible from the weather deck

KEY:　D

[1815]Biodegradation of the oil slick in sea water is assisted by all the following marine micro-organisms which are capable of metabolising oil compounds except ______

A.　bacteria

B.　moulds

C.　yeasts

D.　seals

KEY:　D

[1816]How is the Master or operator of a vessel required to keep the crew informed of the regulations concerning the discharging of garbage overboard? ______.

A.　Give each crewmember a copy of ANNEX V of MARPOL.

B.　Call an all hands meeting before sailing.

C.　Keep placards prominently posted.

D.　Have each person read and sign a copy of the regulations.

KEY:　C

[1817]How many PPM of the oily water can be pumped out within 12 miles along the China coast line？ ______.

A.　at least 15 PPM

B.　about 15 PPM

C.　up to 15 PPM

D.　no less than 15 PPM

KEY:　C

[1818]How should cargo tank hatches be protected when the ullage opening is open and the tank NOT gas free? ______.

A.　With gooseneck vents

B.　With warning signs

C.　With flame screens

D.　With pressure-vacuum relief valves

KEY:　C

[1819]If Annex V to MARPOL 73/78 applies to your vessel，you willnot be able to discharge ______ anywhere at sea.

A.　plastic

B.　metal

C.　glass

D.　paper

KEY:　A

[1820]In case of a ship of less than 400 tons gross tonnage other than an oil tanker whist outside the ______，the Administration shall ensure that it is equipped as far as practicable and reasonable with installations to ensure the storage of oil residues with the requirements of MARPOL 73/78．

A.　special area

B.　national waters

C.　the designated zone

D.　the designated channels

KEY:　A

[1821]No discharge into the sea shall contain ______ in quantities or concentrations which are hazardous to the marine environment or chemicals or other substances introduced for the purpose of circumventing the conditions of discharge specified in MARPOL 73/78.

A.　chemicals

B.　chemicals or other substances

C.　substances

D.　other substances

KEY:　B

[1822]No person on board any vessel to which Annex V to MARPOL 73/78 applies may discharge garbage of any type when ______.

A.　less than 12 nautical miles from PRC

B.　less than 12 nautical miles from nearest land

C.　in the navigable waters of the PRC

D.　less than 25 nautical miles from nearest land

KEY:　C

[1823]Oil slick caused by spillage of oil from tanker ship is ______.

A.　a drop of oil

B.　the emulsion of oil at sea surface

C.　the weathered oil

D.　the oil floating on the surface of the sea water

KEY:　D

[1824]Open ullage holes in tanks which are not gas-free must be protected by ______.

A.　PV valves

B.　warning signs

C.　flame screens

D.　stop-check valves

KEY:　C

[1825]Providing you are not sailing in the Red Sea or another special area as listed in ANNEX V of MARPOL，how many miles from land must you be to throw garbage including bottles，rags，and glass that has not been ground up into the sea?

A.　3 nm

B.　6 nm

C.　12 nm

D.　25 nm

KEY:　C

[1826]The international body responsible for drafting the convention prohibiting marine pollution (MARPOL) is the ______.

A.　Maritime Advisory Council

B.　International Maritime Organization

C.　International Association of Shipping

D.　Association of Seafaring Nations

KEY:　B

[1827]The International Oil Pollution Prevention (IOPP) Certificate required by MARPOL is issued to CHINA flag ships by the ______.

A.　International Maritime Organization

B.　MSA OF CHINA

C.　CCS

D.　Environmental Protection Agency

KEY:　B

[1828]The oil residues which cannot be discharged into the sea in compliance with MARPOL 73/78 shall be ______.

A.　discharged to into special area

B.　discharged into national waters

C.　discharged into the designated zone

D.　retained on board or discharged to reception facilities

KEY:　D

[1829]The operator of each vessel subject to the pollution regulations is NOT required to keep written records of ______.

A.　the name of each person designated as a person in charge

B.　the date and results of the most recent equipment inspection

C.　cargoes carried and dates delivered，including destinations

D.　hose information not marked on the hose

KEY:　C

[1830]The requirement to carry an Oil Record Book does not apply to a ship that ______.

A.　is not equipped to discharge overboard any oil or oily mixture

B.　is on an international voyage to a country that is a party to MARPOL

C.　has an International Oil Pollution Prevention Certificate

D.　has an oily water separator of sufficient capacity for the oil wastes generated

KEY:　A

[1831]The tanker has in operation，except as provided for in Regulation 15(5) and (6) of Annex A of MARPOL 73/78，an oil discharge monitoring and control system and ______ as required.

A.　an IGS

B.　a scriber

C.　a P/V

D.　a slop tank arrangement

KEY:　D

[1832]Vessels to which Annex V to MARPOL 73/78 applies may discharge garbage containing plastics ______.

A.　5 nautical miles from nearest land

B.　12 nautical miles from nearest land

C.　25 nautical miles from nearest land

D.　None of the above

KEY:　D

[1833]What is the MINIMUM distance a vessel subject to the requirements of Annex V to MARPOL 73/78 must be located from nearest land to legally discharge paper trash?

A.　5 nautical miles

B.　10 nautical miles

C.　12 nautical miles

D.　25 nautical miles

KEY:　C

[1834]Whenever visible traces of oil are observed on or below the surface of the water in the immediate vicinity of ship or its wake，Governments of Parties to the Convention should，to the extent they are reasonably able to do so，promptly ______ the facts bearing on the issue of whether there has been a violation of the provisions of this Regulation or Regulation 10 of this Annex of MARPOL 73/78.

A.　observe

B.　investigate

C.　see

D.　look

KEY:　B

[1835]Which area is designated a special area by Annex V to MARPOL 73/78?

A.　Gulf of Mexico

B.　Sargasso Sea

C.　Red Sea

D.　Great Lakes

KEY:　C

[1836]Which is an exception to the garbage discharge requirements in Annex V to MARPOL 73/78?

A.　The garbage to be discharged will sink.

B.　Garbage accumulation on board has exceeded storage space.

C.　A person falls overboard，and a plastic ice chest is thrown for flotation.

D.　The destination port or terminal cannot receive garbage.

KEY:　C

[1837]Which statement is TRUE concerning protection of double bottom tanks against excessive pressure?

A.　Each vent for the tank must be equal to the area of the tank filling line.

B.　The tanks must be protected by overflows.

C.　The total area of the vents or the overflow shall be at least equal to the area of the fill line.

D.　There must be twice as much vent area as the area of the fill line.

KEY:　C

[1838]Which statement is TRUE concerning the ventilation of engine and fuel tank compartments on uninspected towing vessels using fuel with a flash point of 100F?

A.　There shall be at least 3 ventilator ducts open to the atmosphere.

B.　At least one exhaust duct shall extend from the open atmosphere to the lower portion of the bilge.

C.　At least one intake duct shall extend from the open atmosphere to the lower portion of the bilge.

D.　Only vessels using fuel with a flash point above 110F need ventilate engine and fuel compartments.

KEY:　B

[1839]Which step is NOT generally taken when gas-freeing a tank? ______.

A.　Washing the tank interior with sea water

B.　Application of degreasing solvents

C.　Removal of corrosion products and sludge

D.　Fresh air ventilation

KEY:　B

[1840]Which substance is NOT considered to be Oil under the pollution prevention regulations?

A.　Petroleum and fuel oil

B.　Sludge

C.　Oil mixed with dredge spoil

D.　Oil refuse and oil mixed with wastes

KEY:　C

[1841]Which vessel in ocean service is not subject to Annex V of MARPOL 73/78?

A.　A 20-foot sailing vessel

B.　A 26-foot tug and tow

C.　An uninspected 35-foot passenger vessel

D.　A Navy Destroyer

KEY:　D

[1842]While testing a cargo tank，your oxygen indicator reads 25% oxygen in the tank. You would then ______.

A.　enter the tank safely

B.　suspect the accuracy of the reading

C.　ventilate the tank

D.　test for nitrogen

KEY:　B

[1843]While underway and towing an unmanned tank barge you are required to ______.

A.　maintain a strict watch on the barge from the towing vessel

B.　fly a red flag from the towing vessel

C.　open the tops of all empty tanks on the barge

D.　take hourly soundings of any loaded tanks on the barge

KEY:　A

[1844]You are cleaning the tanks after carrying a cargo of crude oil. Which statement is TRUE? ______.

A.　Washing water should be recirculated if possible because it has the same electric potential as the cargo tank being cleaned.

B.　The hoses to portable cleaning machines should be disconnected before the machines are removed from the tank.

C.　The principal hazard with steaming cargo tanks is raising the ambient temperature above the flame point of the cargo residue.

D.　Steam cleaning and water washing are both capable of generating electrostatic charges within a tank.

KEY:　D

[1845]You are crossing a narrow channel in a 15-meter vessel when you sight a tanker off your port bow coming up the channel. Which statement is TRUE?

A.　Yours is the give-way vessel because it is less than 30 meters long.

B.　You shall not impede the safe passage of the tanker.

C.　The tanker is the stand-on vessel because it is to port of your vessel.

D.　The tanker is the stand-on vessel because it is the larger of the two vessels.

KEY:　B

[1846]You are keeping the required garbage disposal records. The amount of garbage disposed must be stated in ______.

A.　cubic meters

B.　both cubic meters and cubic feet

C.　both kilos and pounds

D.　barrels of 55 gallon capacity

KEY:　A

[1847]You are on a multiple product tanker and carrying methyl acrylate，diethanolamine，and triethylamine. Which statement is TRUE?

A.　Methyl acrylate may be loaded in a tank adjacent to diethanolamine but not triethylamine.

B.　The heating coils to the cargo tank loaded with methyl acrylate must be blanked off.

C.　Valve components made of aluminum，copper，or zinc may not be in contact with the cargoes.

D.　Each crew member working on deck must have an emergency escape breathing apparatus.

KEY:　B

[1848]You are on a multiple-product chemical tanker and will carry cargoes of allyl alcohol，benzene，and propanolamine. Which of the following is true?

A.　All of these cargoes are mutually compatible.

B.　Benzene may not be carried in a tank adjacent to either of the other two cargoes.

C.　Allyl alcohol is incompatible with propanolamine but both are compatible with benzene.

D.　Propanolamine is compatible with allyl alcohol but must be segregated from benzene.

KEY:　C

[1849]You are on a multiple-product chemical tanker and will carry cargoes of isophorone，ethylenediamine，and creosote. Which of the following is TRUE?

A.　All of these cargoes are compatible.

B.　Isophorone is incompatible with ethylenediamine but may be stowed adjacent to creosote.

C.　All of these cargoes are incompatible.

D.　Ethylenediamine is compatible with isophorone but both are incompatible with creosote.

KEY:　A

[1850]You are on a multiple-product chemical tanker and will carry cargoes of ethanolamine，methyl acrylate，and glycerine. Which statement is true?

A.　All of these cargoes are mutually incompatible.

B.　Glycerine may be stowed adjacent to methyl acrylate but must be segregated from ethanolamine.

C.　Methyl acrylate and ethanolamine are compatible but both must be segregated from glycerine.

D.　Glycerine is compatible with both of the other cargoes.

KEY:　D

007.2 锚泊与靠离泊作业

[2201]Your vessel has anchored in a channel known to have strong tidal currents． To check your position you take a round of bearings，one of which is a range in line． One hour later the bearing on the lights in range opens up． This indicates the ship is ______.

A.　swinging

B.　dredging her anchor

C.　taking bearings of the lights

D.　taking distances of the lights

KEY:　A

[2202]Your 15-meter tug is underway and crossing a deep and narrow channel. A large container vessel is off your port bow on a steady bearing. Which statement is TRUE concerning this situation? ______.

A.　You should maintain course and speed.

B.　The container vessel is the stand-on as it is the larger vessel.

C.　You are not to impede the safe passage of the container vessel in the channel.

D.　None of the above

KEY:　C

[2203]A Danforth lightweight anchor does NOT hold well in which type of bottom?

A.　Mud

B.　Grass

C.　Sand

D.　Clay

KEY:　B

[2204]A mooring line is described as being 6x24，1-3/4 inch wire rope. What do the above numbers refer to?

A.　Strands，yarns，circumference

B.　Strands，wires，diameter

C.　Wires，yarns，diameter

D.　Strands，circumference，wires

KEY:　B

[2205]A mooring system that results in a spread system without anchor buoys is called a ______.

A.　permanent chasing system

B.　wire rope mooring system

C.　shepherd's crook mooring system

D.　spring buoy mooring system

KEY:　A

[2206]A vessel brought alongside should be fended off the towing vessel by ______.

A.　crew members using their arms

B.　crew members using the strong muscles of their legs

C.　fenders

D.　No fending is necessary due to the rugged construction of most towing vessels.

KEY:　C

[2207]A vessel is wind rode when it is ______.

A.　at anchor and heading into the wind

B.　backing into the wind

C.　carrying lee rudder

D.　necessary to apply a leeway correction to the course

KEY:　A

[2208]A vessel moored with two anchors，sometimes，at an exposed roadstead to ______.

A.　aid turning the ship

B.　obtain a fine bearing

C.　increase ship swings to wind or tide

D.　lighten the stress of anchor chains

KEY:　D

[2209]After casting off moorings at a mooring buoy in calm weather，you should ______.

A.　go full ahead on the engine(s)

B.　back away a few lengths to clear the buoy and then go ahead on the engines

C.　go half ahead on the engines and put the rudder hard right

D.　go half ahead on the engines and pass upstream of the buoy

KEY:　B

[2210]An example of an anchor which has a stock is a ______.

A.　Bruce anchor

B.　Dunn anchor

C.　Hook anchor

D.　LWT anchor

KEY:　D

[2211]Anchoring and fishing in this area are ______.

A.　danger

B.　dangerous

C.　dangerously

D.　dangering

KEY:　B

[2212]Before letting the anchor go，you should check that the ______.

A.　chain is clear

B.　anchor is clear of obstructions

C.　wildcat is disengaged

D.　All of the above

KEY:　D

[2213]By paying out more anchor cable，you ______.

A.　decrease the holding power of your anchor

B.　decrease the swing of your vessel while at anchor

C.　increase the holding power of your anchor

D.　increase the possibility that your vessel will drag anchor

KEY:　C

[2214]Chafing gear should be placed ______.

A.　at all wearing points of mooring lines

B.　at the bitter ends of all standing rigging

C.　around running rigging

D.　on wire rope only

KEY:　A

[2215]Consideration should be given in planning for the mooring orientation in a new location so that in adverse weather a crane is available to off-load the supply vessel on what side of the unit? ______.

A.　Weather side

B.　Leeward side

C.　Upwind side

D.　Crosswind side

KEY:　B

[2216]During a storm，the chance of fatigue failure of a mooring line will increase as ______.

A.　vessel motions increase

B.　mooring tensions decrease

C.　KG increases

D.　KG decreases

KEY:　A

[2217]Galvanizing would not be suitable for protecting wire rope which is used for ______.

A.　cargo runners

B.　mooring wires

C.　shrouds

D.　stays

KEY:　A

[2218]In a combination chain and wire rope mooring system，the anchor chain is deployed at the anchor end of the line to ______.

A.　increase fatigue life of the system

B.　reduce the time to retrieve the line

C.　increase the holding power

D.　reduce the catenary

KEY:　C

[2219]In a crossing situation，a vessel fishing must keep out of the way of a vessel which is ______.

A.　under sail

B.　towing

C.　restricted in her ability to maneuver

D.　engaged in pilotage duty

KEY:　C

[2220]Installing tandem anchors on the same mooring line is referred to as ______.

A.　doubling

B.　pretensioning

C.　piggybacking

D.　paralleling

KEY:　C

[2221]It is sometimes necessary to moor bow and stern to two mooring buoys in order to ______.

A.　avoid any swing in a restricted space

B.　shelter the ship from strong winds

C.　prevent from touching with other vessels

D.　make a convenience of cargo discharging

KEY:　A

[2222]Leg penetration to depths which require pullout forces greater than that which can be supplied by the buoyancy of the hull may exist in ______.

A.　soft mud

B.　sand

C.　silt

D.　soft to firm clays

KEY:　D

[2223]Mooring with two bow anchors has which major advantage over anchoring with one bow anchor? ______.

A.　The vessel will not reverse direction in a tidal current.

B.　The radius of the vessel's swing will be shortened.

C.　A mooring approach may be made from any direction.

D.　The vessel will not swing with a change in wind.

KEY:　B

[2224]On a single-screw vessel，when coming port side to a pier and being set off the pier，you should ______.

A.　swing wide and approach the pier so as to land starboard side to

B.　approach the pier on a parallel course at reduced speed

C.　make your approach at a greater angle than in calm weather

D.　point the vessel's head well up into the slip and decrease your speed

KEY:　C

[2225]Owing to the big draught of the ship，she has to go alongside by the time of ______.

A.　spring tide

B.　flood tide

C.　ebb tide

D.　slack water

KEY:　A

[2226]Pilotage in this country is {compulsory}．

A.　free of charge

B.　important

C.　required

D.　unnecessary

KEY:　C

[2227]The anchor chain should be kept moderately taut during a Mediterranean moor to ______.

A.　facilitate speed of recovery during the weighing process

B.　indicate the anchor's location to passing or mooring ships

C.　prevent damage to the stern in the event of a headwind

D.　provide a steady platform for the gangway between the fantail and pier

KEY:　C

[2228]The anchors should be dropped well out from the pier while at a Mediterranean moor to ______.

A.　eliminate navigational hazards by allowing the chain to lie along the harbor bottom

B.　increase the anchor's reliability by providing a large catenary in the chain

C.　permit the ship to maneuver in the stream while weighing anchors

D.　prevent damage to the stern caused by swinging against the pier in the approach

KEY:　C

[2229]The best method of determining if a vessel is dragging anchor is to note ______.

A.　the amount of line paid out

B.　how much the vessel sheers while at anchor

C.　any change in the tautness of the anchor chain

D.　changes in bearings of fixed objects onshore

KEY:　D

[2230]The holding capabilities of an anchor are determined PRIMARILY by the ______.

A.　design of the anchor

B.　weight of the anchor

C.　scope of the anchor chain

D.　size of the vessel

KEY:　C

[2231]The holding power of an anchor at a given scope of cable increases when the ______.

A.　amount of chain lying along the bottom increases

B.　length of the catenary is reduced

C.　mooring line tension is increased

D.　amount of chain lying along the bottom decreases

KEY:　A

[2232]The length of chain between the anchor and the end of the pendant line is called the ______.

A.　pigtail chain

B.　thrash chain

C.　crown chain

D.　wear chain

KEY:　C

[2233]The lockmaster has given you permission to tie off on the lower guide wall to wait your turn to lock through. What should you be most concerned with?

A.　A downbound vessel

B.　An upbound vessel

C.　Current reaction when the lock chamber is being emptied

D.　Current reaction when the lock chamber is being filled

KEY:　C

[2234]The most doubtful and unpredictable factor in a mooring system is the ______.

A.　ability of the anchors to hold in a seabed

B.　anchor chain catenary length

C.　variability of the fairlead

D.　angle of the flukes

KEY:　A

[2235]The Pilotage-quarantine anchorage，______ between Damen Dao and Qingshan Dao，is defined by the lines ______ the given points．

A.　situated／connected

B.　situated／connecting

C.　situating／connected

D.　situating／connecting

KEY:　B

[2236]The purpose of the stripping bar on an anchor windlass is to ______.

A.　clean off any mud that may have accumulated on the chain

B.　engage or disengage the wildcat

C.　fairlead the chain from the hawse pipe to the wildcat

D.　prevent the chain from fouling the wildcat

KEY:　D

[2237]The safest device used to secure the end of the pendant wire when it is initially passed to the anchor handling vessel is a ______.

A.　pelican hook

B.　hydraulic deck stopper

C.　connecting link

D.　shackle

KEY:　B

[2238]The ship ______ on a low rock was broken in two by the waves．

A.　that had been driven

B.　had been driven

C.　have been driven

D.　which to have been driven

KEY:　A

[2239]The ship is moored ______ buoys.

A.　of

B.　on

C.　alongside

D.　to

KEY:　D

[2240]There are two classes of vessels which，to the extent necessary to carry out their work，do not have to comply with the rule regarding traffic separation schemes. One of these is a vessel ______.

A.　engaged in fishing in a traffic lane

B.　servicing a submarine cable

C.　towing another

D.　engaged on pilotage duty

KEY:　B

[2241]To ensure the best results during the Mediterranean moor，the chains should ______.

A.　be crossed around the bow

B.　tend out at right angles to the bow

C.　tend aft 60from each bow

D.　tend forward 30on either bow

KEY:　D

[2242]Vessels lying afloat at buoys for more than 24 hours should use anchor chain for ______ and insurance wire for ______.

A.　bow mooring／stern line

B.　forward spring／after spring

C.　forward breast rope／forward spring

D.　slip rope／stern line

KEY:　A

[2243]Wale shores would be used when drydocking a vessel with ______.

A.　tumble home

B.　excessive deadrise

C.　excessive trim

D.　a list

KEY:　B

[2244]We took usual steps：sounding the depth，reversing and stopping engine，paying out anchor and 60 fathoms of chain cable in the depth of 12 fathoms．The operation above is ______.

A.　berthing

B.　searching

C.　anchoring

D.　picking up pilot

KEY:　C

[2245]What could cause a significant difference between actual chain tension and the tension measured by the tensiometer?

A.　The type of anchor and mooring line being used

B.　The type of bottom in which the ship is anchored

C.　A significant difference between air and water temperature

D.　The chain contacting a chock or fairlead between the tensiometer and the lower swivel fairlead

KEY:　D

[2246]What is the purpose of the intermediate spring?

A.　Serves as a backup for the main tow hawser in case of failure.

B.　Provides weight and flexibility to the total tow makeup.

C.　Lengthens the main tow hawser to keep the tow in step.

D.　Distributes the towing load.

KEY:　B

[2247]What type of stopper would you use on a nylon mooring line?

A.　Chain

B.　Nylon

C.　Manila

D.　Wire

KEY:　B

[2248]When anchored，increasing the scope of the anchor chain normally serves to ______.

A.　prevent fouling of the anchor

B.　decrease swing of the vessel

C.　prevent dragging of the anchor

D.　reduce strain on the windlass

KEY:　C

[2249]When anchoring a vessel under normal conditions，which scope of chain is recommended?

A.　Four times the depth of water

B.　Two and one-half times the depth of water

C.　Five to seven times the depth of water

D.　Fifteen times the depth of water

KEY:　C

[2250]When anchoring，good practice requires 5 to 7 fathoms of chain for each fathom of depth. In deep water you should use ______.

A.　the same ratio

B.　more chain for each fathom of depth

C.　less chain for each fathom of depth

D.　two anchors with the same ratio of chain

KEY:　C

07.3 沿海与大洋航行

[2251]You are to ______ the convoy at 1745 hours．

A.　get

B.　take

C.　have

D.　join

KEY:　D

[2252]Which statement about a tunnel bow thruster is TRUE?

A.　It provides lateral control without affecting headway.

B.　It is fully effective at speeds up to about six knots.

C.　It can be used to slow the ship in addition to backing down.

D.　It will allow you to hold a position when the current is from astern.

KEY:

[2253]______ is prohibited in this area．

A.　Anchor

B.　Anchoring

C.　Anchored

D.　Being anchored

KEY:　B

[2254]A reaching course is one in which the wind ______.

A.　comes directly over the bow

B.　comes directly over the stern

C.　comes over an area extending from broad on the bow to the quarter

D.　has no effect on the vessel

KEY:　C

[2255]A single vessel being towed alongside shall exhibit ______.

A.　one all-round white light

B.　sidelights and a sternlight

C.　only the outboard sidelight and a sternlight

D.　a masthead light，sidelights，and a sternlight

KEY:　B

[2256]A tow astern is veering from side to side on its towline. The best way of controlling the action is to ______.

A.　trim the tow by the bow

B.　trim the tow by the stern

C.　list the tow to windward

D.　adjust the length of the towing bridle

KEY:　B

[2257]A tow that is properly aligned to pass through a narrow opening between two bridge piers is ______.

A.　on course

B.　headed fair

C.　holding on

D.　in shape

KEY:　D

[2258]A towing vessel and her tow are severely restricted in their ability to change course. When making way，the towing vessel will show ONLY ______.

A.　the masthead lights for a towing vessel

B.　the lights for a vessel restricted in her ability to maneuver

C.　sidelights，stern light，and towing light

D.　All of the above.

KEY:　D

[2259]A towing vessel is tripped when ______.

A.　it is overtaken by the tow

B.　it is pulled sideways by the tow

C.　the weight of the towing hawser causes loss of maneuverability

D.　the propeller is fouled by the towing hawser

KEY:　B

[2260]A tug would NOT assist a ship to steer if the tug is made up to the large vessel ______.

A.　by a tow line ahead of the vessel

B.　forward on either bow of the vessel

C.　approximately amidships of the vessel

D.　on the vessel's quarter

KEY:　C

[2261]A vessel is proceeding from a very cold climate to a warm climate．With a cargo of non-hygroscopic nature，which of the following is correct？ ______.

A.　There is danger of heavy cargo sweat if outside air is introduced by ventilation．

B.　Danger of the ship sweat exists．Constant and vigorous ventilation is required．

C.　There is little danger of ship's sweat；slight possibility of cargo sweat．

D.　The possibility of any ship or cargo sweat is remote

KEY:　A

[2262]A vessel must proceed at a safe speed ______.

A.　in restricted visibility

B.　in congested waters

C.　during darkness

D.　at all times

KEY:　D

[2263]A wedge of water building up between the bow and nearer bank which forces the bow out and away describes ______.

A.　bank cushion

B.　bank suction

C.　combined effect

D.　bend effect

KEY:　A

[2264]An ocean tow is sinking in deep water. Attempts to sever the towing hawser are unsuccessful. Which action should now be taken?

A.　Abandon the towing vessel.

B.　Radio for emergency assistance.

C.　Slip the towline and allow it to run off the drum.

D.　Secure all watertight openings on the towing vessel.

KEY:　C

[2265]An overtaking situation would be one in which one vessel is approaching another from more than how many degrees abaft the beam?

A.　0

B.　10

C.　22.5

D.　None of the above

KEY:　C

[2266]As you approach Kate Aubrey Bar Light (mile 788 AHP)，yoursearchlight will show what type of marking at the light?

A.　Green square

B.　Red and green banded square

C.　Green triangle

D.　Green diamond

KEY:　A

[2267]As you approach Shreves cut-off you see Red River LandingGage (302.4 AHP) which reads 4.2 feet. Which of thefollowing statements is TRUE?

A.　This reading is at the same elevation as the 6.2 ft. mark on the gage at Head of Passes.

B.　A vessel drawing 8 ft would be able to pass over the sill at Old River Lock

C.　This reading is 6.4 feet below the Low Water Reference Plane.

D.　A vessel drawing 7 ft. would be able to pass through the locks at Lower Old River.

KEY:　C

[2268]Every vessel shall at all times proceed at a safe speed so that she can ______.

A.　take proper and effective action to avoid collision

B.　be stopped within a distance appropriate to the prevailing circumstances and conditions．

C.　neither A nor B

D.　both A and B．

KEY:　D

[2269]Every vessel shall proceed at a safe speed ______ the prevailing circumstances and conditions．

A.　adapted to

B.　used by

C.　liable to

D.　adaptation to．

KEY:　A

[2270]Every vessel should at all times proceed at a safe speed. Safe speed is defined as that speed where ______.

A.　you can stop within your visibility range

B.　you can take proper and effective action to avoid collision

C.　you are traveling slower than surrounding vessels

D.　no wake comes from your vessel

KEY:　B

[2271]How does an icebreaker normally free a ship which has become beset while navigating independently?

A.　By backing down the track and cutting out ice on either bow

B.　By approaching from the stern and crossing ahead at an angle of 20to 30to the beset ship's course

C.　By overtaking the beset ship，running ahead and then backing down the track to the beset vessel

D.　By approaching the vessel from astern and towing the beset vessel stern first

KEY:　B

[2272]If a ship is proceeding towards the magnetic equator，the uncorrected deviation due to permanent magnetism ______.

A.　increases

B.　remains the same

C.　decreases

D.　is unimportant and may be neglected

KEY:　C

[2273]If the PSCO from general impressions or observations on board has _______for believing that the ship，its equipment or its crew do not substantially meet the requirements，the PSCO should proceed to a more detailed inspection.

A.　clean report

B.　serious deficiencies

C.　clear ground

D.　detention report

KEY:　C

[2274]If you do NOT understand the course or intention of an approaching vessel you should sound ______.

A.　one short blast

B.　one prolonged blast

C.　not less than five short blasts

D.　not less than five prolonged blasts

KEY:　C

[2275]In stopping distances of vessels，head reach can best be described as the ______.

A.　difference between the vessel's speed through the water at any instant and the new speed ordered on the telegraph

B.　distance the vessel has actually run through the water since a change of speed was ordered

C.　distance the vessel will run between taking action to stop her and being stationary in the water

D.　speed at which a vessel should proceed to ensure that she will run a predetermined distance，once her engines have been stopped

KEY:　C

[2276]It is an ______ condition precedent in every voyage charter-party that the ship shall proceed on the voyage without departure from her proper course．

A.　implemented

B.　implied

C.　impossible

D.　improper

KEY:　B

[2277]Large vessel leaving． Keep clear ______ approach channel．

A.　off

B.　reach

C.　from

D.　of

KEY:　D

[2278]Mainly east 3 or 4 partly cloudy to overcast occasional rain ______ slowly south moderate or good．

A.　spreading

B.　proceeding

C.　bounding for

D.　going to

KEY:　A

[2279]Mariners are reminded that they should proceed in the general direction indicated by the arrows or，if ______ a lane，they should do so as nearly as practicable at right angles to it．

A.　cross

B.　crossing

C.　crossed

D.　crosses

KEY:　B

[2280]Mariners proceeding across the main routes are ______ to do so at as wide an angle as practicable．

A.　recommended

B.　reported

C.　applied

D.　complied

KEY:　A

[2281]Mariners proceeding across the main routes are recommended to do so at ______.

A.　as wide an angle as practicable

B.　as wide an angle so practicable

C.　like wide an angle as practicable

D.　like wide as angle so practicable

KEY:　A

[2282]On a gnomonic chart，a great circle track between Los Angeles and Brisbane will appear as a ______.

A.　loxodromic curve

B.　curved line concave to the equator

C.　straight line

D.　spiral approaching the poles as a limit

KEY:　C

[2283]Risk of collision exists when an approaching vessel has a(n) ______.

A.　generally steady bearing and decreasing range

B.　generally steady range and increasing bearing

C.　increasing range and bearing

D.　decreasing bearing only

KEY:　A

[2284]RYC8/TKS PLSPROCEED PEAKIS MEET/PLT SHIFT W/ANCH FOR/QTINE THEN/TO E/ANCH FOR/SUPPLY. This telex indicates that ______.

A.　the vessel will anchor at eastern anchorage and take supply at the western anchorage

B.　the vessel will anchor at western anchorage and take supply at the eastern anchorage

C.　the vessel will anchor at eastern anchorage and take supply there

D.　the vessel will anchor at western anchorage and take supply there

KEY:　B

[2285]Safe speed is defined as that speed where ______.

A.　you can stop within your visibility range

B.　the vessel is not subject to vibrations

C.　you are traveling slower than surrounding vessels

D.　you can take proper and effective action to avoid collision

KEY:　D

[2286]Some of these shoals have been disproved and are not charted． Nevertheless mariners should ______ with particular caution in this area．

A.　go

B.　proceed

C.　move

D.　remove

KEY:　B

[2287]Sometimes a tropical storm moves so slowly that a vessel，if astern of it，can ______ it．

A.　cross

B.　approach

C.　proceed near

D.　run into

KEY:　D

[2288]The approach to the roads and harbor from the ______ entails no difficulty．

A.　eastern

B.　easterly

C.　eastward

D.　eastwards

KEY:　C

[2289]The court has the power to ______ the time for commencing arbitration proceedings if it is of the opinion that in the circumstances of the case undue hardship would otherwise be caused．

A.　pretend

B.　extend

C.　contend

D.　intend

KEY:　B

[2290]The Light List shows a lighted aid to navigation on the left bank. This means that the light can be seen on the starboard side of a vessel ______.

A.　ascending the river

B.　descending the river

C.　crossing the river

D.　proceeding towards sea

KEY:　A

[2291]The rules state that vessels may depart from the requirements of the Rules when ______.

A.　there are no other vessels around

B.　operating in a narrow channel

C.　the Master enters it in the ship's log

D.　necessary to avoid immediate danger

KEY:　D

[2292]The term of Landfall means ______.

A.　Land first sighted when vessel approaching from seaward

B.　Land last sighted when vessel leaving from a port

C.　in sight of one another when vessel underway

D.　in sight of an island during a ship on her voyage

KEY:　A

[2293]Two vessels are approaching each other near head on. What action should be taken to avoid collision? ______.

A.　The first vessel to sight the other should give way.

B.　The vessel making the slower speed should give way.

C.　Both vessels should alter course to starboard.

D.　Both vessels should alter course to port.

KEY:　C

[2294]When approaching Block Island Sound from Long Island Sound，you will enter waters governed by the International Rules of the Road when you ______.

A.　pass through The Race

B.　cross the territorial sea boundary

C.　exit Block Island Sound to the east or south

D.　None of the above，as Long Island Sound is governed by the International Rules of the Road

KEY:　A

[2295]When attempting an upstream landing while pushing empty barges ahead in a hard onshore wind，the approach is best made ______.

A.　with bow out，stern in

B.　with bow in，stern out

C.　parallel to the dock，as close in as possible

D.　parallel to the dock，as far out as possible

KEY:　A

[2296]When steering a tow downstream around the shape of a sand bar，and staying on the proper side of the buoys，an operator should be cautious of ______.

A.　eddies under the bar

B.　swift current under the bar causing loss of control

C.　cross-currents pushing the tow away from the bar

D.　cross-currents pushing the tow into the bar

KEY:　A

[2297]When towing astern，increased catenary will ______.

A.　increase control of the tow

B.　prevent the towing vessel from going in irons

C.　make the towing vessel less maneuverable

D.　reduce shock stress on the towing hawser

KEY:　D

[2298]When towing in an open seaway，it is important to use a towing line ______.

A.　made only of wire rope，due to possible weather conditions

B.　that will have the tow on a crest while your vessel is in a trough

C.　that will have the tow on a crest while your vessel is on a crest

D.　with little dip to gain maximum control of the tow

KEY:　C

[2299]When underway and proceeding ahead，as the speed increases，the pivot point tends to ______.

A.　move aft

B.　move forward

C.　move lower

D.　remain stationary

KEY:　B

[2300]When using the anchor to steady the bow while approaching a dock you must be aware of the fact that ______.

A.　the vessel will tend to take a large sheer towards the side where the anchor is down

B.　steering control is ineffective in trying to turn to the side opposite to that of the anchor being used

C.　the anchor cable must never lead under the hull

D.　using an offshore anchor decreases the chances of the anchor holding

KEY:　D

九　　船舶避碰

09.1 海上避碰规则

[2451]A sailing vessel is overtaking a steam vessel in a fairway on International waters，so as to pass on the steam vessel's port side． The sailing vessel is the ______.

A.　privileged vessel and would sound two short blasts．

B.　burdened vessel and would sound two prolonged blasts followed by two short blasts．

C.　burdened vessel and would sound no whistle signal．

D.　privileged vessel and would sound no whistle signal．

KEY:　B

[2452]A stand-on vessel in a crossing situation is allowed to take action when ______.

A.　on a collision course

B.　the vessels will pass within one mile

C.　it becomes apparent to her that the give-way vessel is not taking appropriate action

D.　the relative speed of the vessels indicates collision in less than six minutes

KEY:　C

[2453]A towing vessel becomes tripped while towing on a hawser astern. What factor is MOST important when assessing the risk of capsizing?

A.　Length of the towline

B.　Height of the towline connection

C.　Longitudinal position of the towline connection

D.　Direction of opposing force

KEY:　B

[2454]A vessel constrained by her draught shall，when risk of collision with another vessel in a crossing or head-on situation exists，______.

A.　apply the relevant Steering and Sailing Rules as a power-driven vessel

B.　apply the relevant Steering and Sailing Rules as a Sailing vessel

C.　apply the relevant Steering and Sailing Rules as a vessel not under command

D.　apply the relevant Steering and Sailing Rules as a vessel engaged in a towing operation．

KEY:　A

[2455]A vessel engaged in fishing underway sounds the same fog signal as a ______.

A.　power-driven vessel stopped and making no way through the water

B.　vessel being towed

C.　vessel restricted in her ability to maneuver at anchor

D.　sailing vessel at anchor

KEY:　C

[2456]A vessel has sustained damage in a collision with another vessel. It is necessary to have a Seaworthy Certificate before the vessel sails. Who will issue this certificate?

A.　American Consul

B.　Classification Society

C.　Captain of the Port

D.　Officer in Charge，Marine Inspection

KEY:　B

[2457]A vessel is approaching from dead ahead． Both of her sidelights are visible and her range lights are in line． Which of the following could you do first？ ______.

A.　Sound one blast of the whistle．

B.　Alter course to starboard．

C.　Construct a radar plot．

D.　Sound the danger signal．

KEY:　B

[2458]A vessel is in sight of another vessel when ______.

A.　she can be observed by radar

B.　she can be observed visually from the other vessel

C.　she can be plotted on radar well enough to determine her heading

D.　her fog signal can be heard

KEY:　B

[2459]A vessel is in sight of another vessel when she ______.

A.　can be observed by radar

B.　has determined that risk of collision exists

C.　is sounding a fog signal which can be heard on the other vessel

D.　can be observed visually from the other vessel

KEY:　D

[2460]A vessel restricted in her ability to maneuver is one which ______.

A.　through some exceptional circumstance is unable to maneuver as required by the Rules

B.　from the nature of her work is unable to maneuver as required by the Rules

C.　due to adverse weather conditions is unable to maneuver as required by the Rules

D.　has lost steering and is unable to maneuver

KEY:　B

[2461]All of the following are practices of good seamanship EXCEPT ______.

A.　When meeting，altering course to the left to increase sea room．

B.　Maintaining an alert radar watch in reduced visibility．

C.　Showing a flare-up light to attract attention when you are not under command．

D.　Maintaining a proper lookout from sunrise to sunset．

KEY:　A

[2462]All of the following constitute special circumstances EXCEPT ______.

A.　Three vessels meeting so as to cross each other's course．

B.　A vessel meeting a vessel that is backing out of a slip．

C.　An overtaking situation that has reached in extremis．

D.　A vessel fishing with nets meeting a sailing vessel．

KEY:　D

[2463]Any alteration of course and／or speed to avoid collision shall，if the circumstances of the case admit，______ to another vessel observing visually or by radar．

A.　be large enough to be readily apparent

B.　be a succession of small alterations

C.　be with due regard to the power and speed of the vessel

D.　leave sufficient room for the other vessel to take action．

KEY:　A

[2464]Any vessel ______ any other shall keep out of the way of the vessel ______.

A.　overtaking／overtaken

B.　being overtake／being overtaken

C.　overtaking／being overtaken

D.　being overtaking／overtaken．

KEY:　C

[2465]If it becomes necessary for a stand-on vessel to take action to avoid collision，she shall NOT，if possible，______.

A.　decrease speed

B.　increase speed

C.　turn to port for a vessel on her own port side

D.　turn to starboard for a vessel on her own port side

KEY:　C

[2466]If there is any doubt as to the proper operation of a radar，which statement is TRUE? ______.

A.　Only a radar expert can determine if the radar is operating.

B.　All radars have indicator lights and alarms to signal improper operation.

C.　A radar range compared to the actual range of a known object can be used to check the operation of the radar.

D.　The radar resolution detector must be energized to check the radar.

KEY:　C

[2467]If there is sufficient sea room，alteration of ______ may be the most effective action to avoid close-quarters situation．

A.　speed and course

B.　speed alone

C.　course alone

D.　speed or course．

KEY:　C

[2468]If your vessel is underway in fog and you hear one prolonged and three short blasts，this is a ______.

A.　vessel not under command

B.　sailing vessel

C.　vessel being towed (manned)

D.　vessel being towed (unmanned)

KEY:　C

[2469]In a crossing situation on open waters，a sailing vessel shall keep out of the way of all the following vessels EXCEPT a vessel ______.

A.　not under command

B.　restricted in her ability to maneuver

C.　engaged in towing

D.　fishing

KEY:　C

[2470]In determining if risk of collision exists the following considerations shall be among ______.

A.　these taken into calculation

B.　those taking into account

C.　that taken into account

D.　those taken into account．

KEY:　D

[2471]In determining safe speed，the Rules list all of the following as factors which must be taken into account EXCEPT the ______.

A.　limitations of radar equipment

B.　presence of background lights at night

C.　maximum horsepower of your vessel

D.　maneuverability of your vessel

KEY:　C

[2472]In fog you observe your radar and determine that risk of collision exists with a vessel which is 2 miles off your port bow. You should ______.

A.　stop your engines

B.　sound the danger signal at two-minute intervals

C.　hold course and speed until the other vessel is sighted

D.　take avoiding action as soon as possible

KEY:　D

[2473]In order for a vessel to be engaged in fishing she must be ______.

A.　underway

B.　using gear which extends more than 50 meters outboard

C.　using a seine of some type

D.　using gear which restricts her maneuverability

KEY:　D

[2474]In order to maintain speed while changing course from a close reach to a broad reach，the sails should be ______.

A.　lowered

B.　reefed

C.　hauled in

D.　eased out

KEY:　D

[2475]In reduced visibility，you hear two prolonged blasts of a whistle. This signal is sounded by a ______.

A.　power-driven vessel dead in the water

B.　sailing vessel on the port tack

C.　vessel not under command

D.　vessel fishing with nets

KEY:　A

[2476]In restricted visibility a towed vessel must sound a fog signal when it is ______.

A.　the last vessel in the tow

B.　the last vessel in the tow and it is carrying a crew

C.　manned，regardless of its position in the tow

D.　None of the above are correct

KEY:　B

[2477]In restricted visibility the speed of a vessel without operational radar may be ______ enable effective avoiding action to be taken on sighting another ship．

A.　too low to

B.　too high to

C.　too high for

D.　too low for．

KEY:　A

[2478]In the daytime，you see a large sailing vessel on the beam. You know that she is also propelled by machinery if she shows ______.

A.　a basket

B.　a black ball

C.　a black cone

D.　two black cones

KEY:　C

[2479]It has often been stressed in the Courts that the look-out man should，preferably，be stationed forward，unless weather conditions make this ______.

A.　possible

B.　practicable

C.　impossible

D.　probably．

KEY:　C

[2480]On open waters，a power-driven vessel shall keep out of the way of a ______.

A.　vessel on her port side that is crossing her course

B.　vessel that is overtaking her

C.　seaplane on the water

D.　sailing vessel

KEY:　D

[2481]On open waters，a vessel fishing is in a crossing situation with a vessel sailing located on the fishing vessel's starboard side. Which vessel is the stand-on vessel? ______.

A.　The fishing vessel because it is to port of the sailing vessel.

B.　The fishing vessel because it is fishing.

C.　The sailing vessel because it is to starboard of the fishing vessel.

D.　The sailing vessel because it is sailing.

KEY:　B

[2482]Power-driven vessels must keep out of the way of sailing vessels except ______.

A.　in a crossing situation

B.　when they are making more speed than the power-driven vessel

C.　when the sailing vessel is overtaking

D.　on the Inland Waters of the United States

KEY:　C

[2483]Risk of collision is considered to exist if ______.

A.　four vessels are nearby

B.　a vessel has a steady bearing at a constant range

C.　there is any doubt that a risk of collision exists

D.　a special circumstance situation is apparent

KEY:　C

[2484]Sailing vessels are stand-on over power-driven vessels except ______.

A.　in a crossing situation

B.　in a meeting situation

C.　when they are the overtaking vessel

D.　on the inland waters of the PR China.

KEY:　C

[2485]The effectiveness of the action shall be carefully checked until the other vessel is finally ______.

A.　past

B.　clear

C.　past or clear

D.　past and clear．

KEY:　D

[2486]The risk of collision shall be deemed to exist if the compass bearing of an approaching vessel ______.

A.　did not appreciably change

B.　does not appreciably change

C.　do not appreciably change

D.　not appreciably change

KEY:　B

[2487]The rules require that a stand-on vessel SHALL take action to avoid collision when she determines that ______.

A.　risk of collision exists

B.　the other vessel will cross ahead of her

C.　the other vessel is not taking appropriate action

D.　collision cannot be avoided by the give-way vessel's maneuver alone

KEY:　D

[2488]The Rules state that certain factors are to be taken into account when determining safe speed. One of the factors is the ______.

A.　radio communications that are available

B.　maximum speed of your vessel

C.　temperature

D.　current

KEY:　D

[2489]The Rules state that certain factors are to be taken into account when determining safe speed. Those factors include ______.

A.　state of wind，sea，and current，and the proximity of navigational hazards

B.　maximum attainable speed of your vessel

C.　temperature

D.　aids to navigation that are available

KEY:　A

[2490]The steering and sailing rules for vessels in restricted visibility apply to vessels ______.

A.　in sight of one another in fog

B.　navigating in or near an area of restricted visibility

C.　only if they are showing special purpose lights

D.　only if they have operational radar

KEY:　B

[2491]The term power-driven vessel means ______ in these Rules．

A.　any sailing vessel with propelling machinery

B.　any vessel propelled by machinery

C.　any sailing vessel with or without machinery for propelling

D.　any sailing vessel with propelling machinery not in use．

KEY:　B

[2492]The term restricted visibility，when used in the Rules，refers to ______.

A.　situations when you can see vessels on radar that you cannot see visually

B.　visibility of less than half a mile

C.　any condition where visibility is restricted

D.　visibility where you cannot see shore

KEY:　C

[2493]The wind is ESE，and a sailing vessel is steering NW. Which fog signal should she sound?

A.　One blast at one-minute intervals

B.　One blast at two-minute intervals

C.　Two blasts at one-minute intervals

D.　One prolonged and two short blasts at two-minute intervals

KEY:　D

[2494]Under International Rules ______.

A.　all vessels must carry an after range light．

B.　vessels less than 50 meters in length need not carry an after range light．

C.　vessels stopped dead in the water should turn off their range lights．

D.　vessels over 20 meters in length must carry both range lights from sunset to sunrise．

KEY:　B

[2495]Underway vessels shall at all time maintain a proper look-out ______.

A.　by sight and hearing only

B.　by all available means besides sight and hearing

C.　by all available means except sight and hearing

D.　by none of above means．

KEY:　B

[2496]Vessel constrained by her draught is ______.

A.　a non-power driven vessel

B.　a sailing vessel

C.　a power driven vessel

D.　a power driven vessel or a non-power driven vessel．

KEY:　C

[2497]Vessel underway means that a vessel is ______.

A.　on the way

B.　at anchor

C.　made fast to a terminal

D.　moving against water．

KEY:　D

[2498]Vessels shall be deemed to be in sight of one another only when one ______ from the other．

A.　can be observed visually

B.　can be observed by radar

C.　can be located on the radar

D.　can be heard．

KEY:　A

[2499]What determines if a vessel is restricted in her ability to maneuver? ______.

A.　Whether or not all of the vessel's control equipment is in working order

B.　The vessel's draft in relation to the available depth of water

C.　Whether the nature of the vessel's work limits maneuverability required by the Rules

D.　Whether or not the vessel is the give-way vessel in a meeting situation

KEY:　C

[2500]When a pushing vessel and a vessel being pushed are rigidly connected in a composite unit，they shall be regarded as ______.

A.　a tugboat

B.　a sailing vessel

C.　a power-driven vessel

D.　a vessel propelled by machinery and under sail

KEY:　C

09.2 航行值班与海上避碰

[2501]You are approaching another vessel and are not sure whether danger of collision exists. You must assume ______.

A.　there is a risk of collision

B.　you are the give-way vessel

C.　the other vessel is the give-way vessel

D.　there is no risk of collision

KEY:　A

[2502]You are in restricted visibility and hear a fog signal forward of the beam. Nothing appears on your radar screen. You must ______.

A.　stop your engines

B.　sound two prolonged blasts of the whistle

C.　sound the danger signal

D.　slow to bare steerageway

KEY:　D

[2503]While underway at night you observe a vessel's range lights in line off your starboard beam． You should ______.

A.　come right．

B.　sound the danger signal．

C.　continue on your course．

D.　come left．

KEY:　C

[2504]Where does the look-out man stand？ ______.

A.　In positions in which the noises in the ship are least likely to interfere with the hearing of a fog signal and the man can see clearly．

B.　In positions in which the noises in the ship are much likely to interfere with the hearing of a fog signal．

C.　In positions in which the hearing of a signal is reduced to a minimum．

D.　In a position in which the man can see clearly．

KEY:　A

[2505]When your vessel enters thick fog，she should sound ______ every two minutes in accordance with these Rules．

A.　one short blast

B.　one prolonged blast

C.　two short blasts

D.　three short blasts．

KEY:　B

[2506]When you doubt the existence of risk of collision，______.

A.　such risk shall be deemed to exist

B.　such risk shall not be deemed to exist

C.　you needn't take any action to avoid collision with any vessel

D.　you should accelerate your speed ahead．

KEY:　A

[2507]When two power-driven vessels are meeting on reciprocal or nearly reciprocal courses so as to involve risk of collision，they shall be deemed to be in ______.

A.　crossing situation

B.　head-on situation

C.　restricted visibility

D.　overtaking situation．

KEY:　B

[2508]When in sight of another vessel，any action taken to avoid collision must ______.

A.　be accompanied by sound signals

B.　not result in another close quarters situation

C.　include a speed change

D.　All of the above

KEY:　B

[2509]When a vessel sees the other ahead or nearly ahead，by night she could see the masthead lights of the other in a line or nearly in a line or both side lights，______ shall be deemed to exist．

A.　head-on situation

B.　end-situation

C.　crossing situation

D.　close-quaters situation．

KEY:　A

[2510]In determining safe speed，all of the following must be taken into account EXCEPT the ______.

A.　maximum horsepower of your vessel

B.　presence of background lights at night

C.　draft of your vessel

D.　maneuverability of your vessel

KEY:　A

[2511]In determining if risk of collision exists，if there is any doubt，such risk shall be deemed to ______.

A.　exist

B.　exists

C.　existing

D.　to be existed．

KEY:　A

[2512]In determining a safe speed ______ shall not be among those taken into account．

A.　the characteristics，efficiency and limitation of the radar equipment

B.　any constraints imposed by the radar range scale in use

C.　the effect on radar detection of the sea state，weather and other sources of interference．

D.　the safe working load of the cranes both on board and ashore．

KEY:　D

[2513]In determining a safe speed ______ shall not be among those taken into account．

A.　the presence of the background light at night

B.　the state of wind，sea and current

C.　the number of crew on board

D.　the draught in relation to the available depth of water．

KEY:　C

[2514]In determining a safe speed ______ shall not be among those taken into account．

A.　the state of visibility

B.　the power of the vessel

C.　the traffic density

D.　the maneuverability of the vessel．

KEY:　B

[2515]In dense fog a vessel without operational radar may not be justified ______ at all but should anchor if it is safe and practicable for her to do so．

A.　to be underway

B.　on under way condition

C.　to stop engine

D.　in being under way．

KEY:　D

[2516]In complying with the Rules，of what must the mariner take due regard? ______.

A.　Limited backing power of his vessel

B.　Radar information about nearby vessels

C.　The occupation of the other vessel，if known

D.　All of the above

KEY:　D

[2517]In a crossing situation，the stand-on vessel should normally ______.

A.　take action to cross ahead of the other vessel

B.　take action to pass astern of the other vessel

C.　maintain course and speed

D.　change course and increase speed

KEY:　C

[2518]In a crossing situation，a stand-on vessel which is forced to take action in order to avoid collision with a vessel on her own port side shall，if possible，avoid ______.

A.　turning to port

B.　turning to starboard

C.　decreasing speed

D.　increasing speed

KEY:　A

[2519]If the circumstances of the case admit，any vessel ______ in a narrow channel．

A.　shall have the liberty to anchor

B.　shall avoid anchoring

C.　shall moor to a buoy when necessary

D.　shall keep anchoring．

KEY:　B

[2520]If the circumstances admit，______ of small alteration of course and speed shall not be taken to avoid collision．

A.　depression

B.　oppression

C.　succession

D.　possession．

KEY:　C

[2521]If a vessel of special construction cannot fully comply with the Rules，her degree of compliance shall be determined by ______.

A.　IMO

B.　The owners of the vessel．

C.　The government．

D.　The builder．

KEY:　C

[2522]If a vessel is engaged in fishing according to the definitions in the Rules，it will have ______.

A.　gear extending from the side or stern

B.　gear that restricts maneuverability

C.　less than 50 percent trolling lines

D.　None of the above

KEY:　B

[2523]If a seaplane is not used as a means of transpiration on water but is capable of being used so，it shall ______.

A.　be regarded as a vessel not under command

B.　be regarded as a vessel underway

C.　be regarded as a vessel

D.　be regarded as a sailing vessel．

KEY:　C

[2524]Which statement about the Navigation Rules is TRUE? ______.

A.　The rules require vessels to comply with Traffic Separation Scheme regulations.

B.　The rules use the term safe speed.

C.　The Rules permit a stand-on vessel to take action prior to being in extremis.

D.　All of the above are correct.

KEY:　D

[2525]The Rules state that vessels may depart from the Rules when ______.

A.　there are other vessels in the vicinity

B.　operating in a traffic separation scheme

C.　engaged in a situation involving more than two vessels

D.　necessary to avoid immediate danger

KEY:　D

[2526]Vessels，______ support vessels，should not enter the safety zone of a deepwater port unless clearance has been obtained from the Vessel Traffic Supervisor．

A.　besides

B.　excepted

C.　otherwise

D.　other than

KEY:　D

[2527]One of the factors which affects the circulation of ocean currents is ______.

A.　humidity

B.　varying densities of water

C.　vessel traffic

D.　the jet stream

KEY:　B

[2528]A head-on situation at night is one in which you see ______.

A.　one sidelight of a vessel ahead of you

B.　one sidelight and a masthead light of a vessel ahead of you

C.　one sidelight，a masthead light，and a range light of a vessel ahead of you

D.　both sidelights of a vessel dead ahead of you

KEY:　D

[2529]A proper look-out must be kept ______.

A.　only in fog

B.　only between the hours of sunset and sunrise

C.　only when entering and leaving port

D.　at all times

KEY:　D

[2530]A proper look-out shall be maintained ______.

A.　only at night

B.　only during restricted visibility

C.　at night and during restricted visibility

D.　at all times

KEY:　D

[2531]A vessel ______ a crossing vessel，shall not normally enter a separation zone or cross a separation line．

A.　including

B.　inclusive of

C.　except that

D.　other than．

KEY:　D

[2532]A vessel ______ when underway shall so far as possible，keep out of the way of a vessel not under command．

A.　having engaged in fishing

B.　engaging in fishing

C.　engaged in fish

D.　engaged in fishing．

KEY:　D

[2533]A vessel aground at night is required to show two red lights in a vertical line as well as ______.

A.　not under command lights

B.　restricted in her ability to maneuver lights

C.　anchor lights

D.　sidelights and a sternlight

KEY:　C

[2534]A vessel approaching a narrow channel shall ______.

A.　avoid crossing the channel if it impedes another vessel navigating in the channel

B.　not overtake any vessels within the channel

C.　keep as close as possible to the edge of the channel on her port side

D.　anchor only in the middle of the channel

KEY:　A

[2535]A vessel approaching your vessel from 235relative is in what type of situation? ______.

A.　Meeting

B.　Overtaking

C.　Crossing

D.　Passing

KEY:　B

[2536]A vessel being pushed ahead is required to carry which of the following lights？ ______.

A.　Sidelights and a stern light．

B.　Sidelights only．

C.　Sidelights and a small white light at each end．

D.　A small white light at each end．

KEY:　B

[2537]A vessel being towed astern shall show at night ______.

A.　the lights required for a power-driven vessel underway

B.　only the required masthead lights

C.　a sternlight only

D.　sidelights and a sternlight

KEY:　D

[2538]A vessel being towed will show ______.

A.　a forward masthead light

B.　sidelights and a sternlight

C.　a towing light

D.　All of the above

KEY:　B

[2539]A vessel conducting mine clearing operations will show ______.

A.　three balls in a vertical line

B.　two balls in a vertical line

C.　one ball near the foremast and one ball at each fore yard

D.　one diamond near the foremast head and one ball at each fore yard

KEY:　C

[2540]A vessel constrained by her draught may，in addition to the lights prescribed for power-driven vessels in Rule 23，exhibit where they can best be seen ______ or a cylinder．

A.　two all-round red lights in a vertical line

B.　three all-round red lights in a vertical line

C.　two all-round yellow lights in a vertical line

D.　three all-round green lights in a vertical line．

KEY:　B

[2541]A vessel continually lists to one side and has a normal rolling period. Which statement is TRUE? ______.

A.　The vessel has negative GM.

B.　The center of gravity is on the centerline.

C.　The list can be corrected by reducing KM.

D.　The vessel has asymmetrical weight distribution.

KEY:　D

[2542]A vessel crossing a river on the Western Rivers has the right of way over ______.

A.　vessels ascending the river

B.　vessels descending the river

C.　all vessels ascending and descending the river

D.　None of the above

KEY:　D

[2543]A vessel enagaged in fishing，and at anchor，shall show ______.

A.　an anchor light

B.　sidelights and a sternlight

C.　three lights in a vertical line，the highest and lowest being red，and the middle being white

D.　None of the above

KEY:　D

[2544]A vessel engaged in fishing during the day would show ______.

A.　one black ball

B.　two cones with bases together

C.　a cone，apex downward

D.　two cones，apexes together

KEY:　D

[2545]A vessel engaged in fishing must display a light in the direction of any gear that extends outward more than 150 meters. The color of this light is ______.

A.　white

B.　green

C.　red

D.　yellow

KEY:　A

[2546]A vessel engaged in fishing shall keep out of the way of a vessel ______.

A.　under sail

B.　restricted in her ability to maneuver

C.　crossing a channel

D.　All of the above

KEY:　B

[2547]A vessel engaged in mine clearance operations is called ______.

A.　a vessel restricted in her ability to maneuver

B.　a vessel not under command

C.　a vessel constrained by her draught

D.　a vessel not under way．

KEY:　A

[2548]A vessel engaged in trawling will show identification lights of ______.

A.　a red light over a white light

B.　a white light over a red light

C.　a green light over a white light

D.　two red lights in a vertical line

KEY:　C

[2549]A vessel may exhibit lights other than those prescribed by the Rules as long as the additional lights ______.

A.　do not interfere with the keeping of a proper look-out

B.　are not the color of either sidelight

C.　have a lesser range than the prescribed lights

D.　All of the above

KEY:　A

[2550]As look-out，you spot an object 45off your port bow. You should report the object as ______.

A.　broad on the port bow

B.　3 points on the port bow

C.　3 points forward of the port beam

D.　on the port beam

KEY:　A

10.2 地文航海

[2701]Leeway is the ______.

A.　difference between the true course and the compass course

B.　momentum of a vessel after her engines have been stopped

C.　lateral movement of a vessel downwind of her intended course

D.　displacement of a vessel multiplied by her speed

KEY:　C

[2702]______ is the process by which the position of the vessel at any moment is found by applying the last well-determined position to the run that has been made since，using for this purpose the ship's course and the distance being those indicated by log．

A.　dead reckoning

B.　dead weight

C.　dead slow ahead

D.　dead slow astern

KEY:　A

[2703]______ means the curve on the earth's surface which cuts all the meridians at the same angle．

A.　Great Circle

B.　Position Line

C.　Rhumb line

D.　True Bearing

KEY:　C

[2704]A bold reef is a reef ______.

A.　with part of it extending above the water

B.　that can be detected by water turbulence

C.　that drops off sharply

D.　perpendicular to the current

KEY:　B

[2705]A buoy having red and green horizontal bands would have a light characteristic of ______.

A.　interrupted quick flashing

B.　composite group flashing

C.　Morse (A)

D.　quick flashing

KEY:　B

[2706]A buoy marking a wreck will show a(n) ______.

A.　white light FL (2) and a topmark of 2 black spheres

B.　occulting green light and may be lettered

C.　yellow light and will be numbered

D.　continuous quick white light and may be numbered

KEY:　A

[2707]A buoy with a composite group-flashing light indicates a(n) ______.

A.　anchorage area

B.　fish net area

C.　bifurcation

D.　dredging area

KEY:　C

[2708]A buoy，being a shape of cylinder and with red group flashing，shows that it is ______.

A.　left-side buoy

B.　right-side buoy

C.　a buoy of safety

D.　a buoy of danger

KEY:　A

[2709]A cardinal mark showing an uninterrupted quick-flashing white light indicates the deepest water in the area is on the ______.

A.　north side of the mark

B.　west side of the mark

C.　east side of the mark

D.　south side of the mark

KEY:　A

[2710]A dead reckoning (DR) plot ______.

A.　ignores the effect of surface currents

B.　is most useful when in sight of land

C.　must be plotted using magnetic courses

D.　may be started at an assumed position

KEY:　A

[2711]A dead reckoning (DR) plot ______.

A.　must utilize magnetic courses

B.　must take set and drift into account

C.　should be replotted hourly

D.　should be started each time the vessel's position is fixed

KEY:　D

[2712]A fairly accurate estimation of a ship's position can be calculated by a technique known as ______.

A.　dead reckoning

B.　Pilotage

C.　great circle sailing

D.　geographic navigation

KEY:　A

[2713]A large navigational buoy (LNB) is painted ______.

A.　red

B.　yellow

C.　with red and white vertical stripes

D.　with a distinct color and pattern unique to each buoy

KEY:　A

[2714]A line connecting all possible positions of your vessel at any given time is a ______.

A.　longitude line

B.　latitude line

C.　line of position

D.　fix

KEY:　C

[2715]A line of position is ______.

A.　a line connecting two charted objects

B.　a line on some point of which the vessel may be presumed to be located

C.　the position of your vessel

D.　not used in a running fix

KEY:　B

[2716]A position obtained by applying only your vessel's course and speed to a known position is a ______.

A.　dead-reckoning position

B.　fix

C.　probable position

D.　running fix

KEY:　A

[2717]A position obtained by crossing lines of position taken at different times and advanced to a common time is a(n) ______.

A.　running fix

B.　dead-reckoning position

C.　fix

D.　estimated position

KEY:　A

[2718]A position that is obtained by applying estimated current and wind to your vessel's course and speed is a(n) ______.

A.　dead reckoning position

B.　estimated position

C.　fix

D.　None of the above

KEY:　B

[2719]A position that is obtained by using two or more intersecting lines of position，taken at nearly the same time，is a(n) ______.

A.　estimated position

B.　fix

C.　running fix

D.　dead-reckoning position

KEY:　B

[2720]A relative bearing is always measured from ______.

A.　true north

B.　magnetic north

C.　the vessel's beam

D.　the vessel's head

KEY:　D

[2721]A single line of position combined with a dead-reckoning position results in a(n) ______.

A.　running fix

B.　fix

C.　estimated position

D.　assumed position

KEY:　C

[2722]A vessel's position should be plotted using bearings of ______.

A.　buoys close at hand

B.　fixed known objects on shore

C.　buoys at a distance

D.　All of the above

KEY:　B

[2723]At the magnetic equator there is no induced magnetism in the vertical soft iron because ______.

A.　the lines of force cross the equator on a 0-180alignment

B.　the quadrantal error is 0

C.　there is no vertical component of the Earth's magnetic field

D.　the intercardinal headings have less than 1error

KEY:　C

[2724]In plotting a running fix，how many fixed objects are needed to take your lines of position from?

A.　Three

B.　Two

C.　One

D.　None

KEY:　C

[2725]Lighted white and orange buoys must show which color light?

A.　Orange

B.　White

C.　Red

D.　Alternating yellow and white

KEY:　B

[2726]Preferred channel buoys indicate the preferred channel to transit by ______.

A.　odd or even numbers

B.　the color of their top band

C.　the location of the buoy in the channel junction

D.　the buoy's light rhythms

KEY:　B

[2727]Restricted areas at locks and dams are indicated by ______.

A.　flashing red lights upstream and fixed red lights downstream

B.　yellow unlighted buoys

C.　signs and/or flashing red lights

D.　red daymarks upstream and green daymarks downstream

KEY:　C

[2728]The luminous range of a light takes into account the ______.

A.　glare from background lighting

B.　existing visibility conditions

C.　elevation of the light

D.　observer's height of eye

KEY:　B

[2729]The maximum distance at which a light may be seen under the existing visibility conditions is called ______.

A.　nominal range

B.　luminous range

C.　charted range

D.　geographic range

KEY:　B

[2730]The nominal range of a light may be accurately defined as the maximum distance at which a light may be seen ______.

A.　under existing visibility conditions

B.　under perfect visibility

C.　with ten miles visibility

D.　with fifteen miles visibility

KEY:　C

[2731]The period of a lighted aid to navigation refers to the ______.

A.　date of construction or establishment

B.　length of time between flashes of the light

C.　time required for the longest flash of each cycle

D.　time required for the light to complete each cycle

KEY:　D

[2732]The positions and characteristics of lights and buoys shown within the port area are ______.

A.　untrue

B.　impossible

C.　suspectful

D.　unimportant

KEY:　A

[2733]The shortest distance between any two points on earth defines a ______.

A.　small circle

B.　great circle

C.　rhumb line

D.　hyperbola

KEY:　B

[2734]There are a number of buoys {in the vicinity}．

A.　nearby

B.　in view

C.　visible

D.　far from

KEY:　A

[2735]There is sufficient anchorage for ______ at all time around the No．1 buoy and for smaller vessels around the No．4 buoy．

A.　deep laded vessels

B.　deeply load vessels

C.　deep loading vessels

D.　deeply loaded vessels

KEY:　D

[2736]This is ______ where we shall anchor．

A.　the light-house

B.　the anchorage

C.　the pilot station

D.　the light-buoy

KEY:　B

[2737]Two navigational hazards are located near to each other，but each is marked by an individual cardinal buoyage system. The buoys of one cardinal system may be identified from the other system by ______.

A.　the differing light colors

B.　one system having odd numbers while the other system has even numbers

C.　one system using horizontal bands while the other system uses vertical stripes

D.　the difference in the periods of the light

KEY:　D

[2738]Uncharted lights，fog signals and radar beacon transmissions may be ______ near the station．

A.　encountered

B.　met

C.　contacted

D.　seen

KEY:　A

[2739]Under the IALA - A Buoyage System，a buoy used as a port hand mark would not show which light characteristic?

A.　Isophase

B.　Quick flashing

C.　Long flashing

D.　Group Flashing (2 + 1)

KEY:　D

[2740]What describes an accurate position that is NOT based on any prior position?

A.　Dead-reckoning position

B.　Estimated position

C.　Fix

D.　Running fix

KEY:　C

[2741]What indicates a dual purpose buoy?

A.　Red buoy with a horizontal yellow band

B.　Red and white vertically-striped buoy with a vertical yellow stripe

C.　Red and white vertically-striped buoy with a red spherical topmark

D.　Green buoy with a yellow square

KEY:　D

[2742]What is the light characteristic of a lighted，preferred-channel buoy?

A.　Group flashing

B.　Composite group flashing

C.　Interrupted quick flashing

D.　Fixed and flashing

KEY:　B

[2743]What is the major advantage of high altitude observations?

A.　Errors due to unusual parallax are eliminated.

B.　The same body can be used for a fix from observations separated by several minutes.

C.　The declination is the only information needed from the almanac.

D.　The semidiameter correction of the sextant altitude is eliminated.

KEY:　B

[2744]When a buoy marks a channel bifurcation，the preferred channel is NOT indicated by ______.

A.　the shape of an unlighted buoy

B.　the light color of a lighted buoy

C.　the color of the topmost band

D.　whether the number is odd or even

KEY:　D

[2745]When approaching a preferred-channel buoy，the best channel is NOT indicated by the ______.

A.　light characteristic

B.　color of the uppermost band

C.　shape of an unlighted buoy

D.　color of the light

KEY:　A

[2746]When entering a channel from seaward，the numbers on buoys ______.

A.　are the same as their Light List number

B.　are marked in 6 inch figures with retroreflective material

C.　increase with the even numbers to starboard

D.　decrease with the odd numbers to starboard

KEY:　C

[2747]When entering from seaward，a buoy displaying a single-flashing red light would indicate ______.

A.　a junction with the preferred channel to the left

B.　a sharp turn in the channel to the right

C.　the starboard side of the channel

D.　a wreck to be left on the vessel's port side

KEY:　C

[2748]When making landfall at night，you can determine if a light is a major light or an offshore buoy by ______.

A.　the intensity of the light

B.　checking the period and characteristics against the Light List

C.　the color，because the buoy will have only a red or a green light

D.　Any of the above can be used to identify the light.

KEY:　B

[2749]When navigating a vessel，you ______.

A.　can always rely on a buoy to be on station

B.　can always rely on a buoy to show proper light characteristics

C.　should assume a wreck buoy is directly over the wreck

D.　should never rely on a floating aid to maintain its exact position

KEY:　D

[2750]When using a buoy as an aid to navigation which of the following should be considered?

A.　The buoy should be considered to always be in the charted location.

B.　If the light is flashing，the buoy should be considered to be in the charted location.

C.　The buoy may not be in the charted position.

D.　The buoy should be considered to be in the charted position if it has been freshly painted.

KEY:　C

1

07.9 潮汐与涌浪

[2801]A stream of water immediately surrounding a moving vessel's hull，flowing in the same direction as the vessel is known as ______.

A.　directional current

B.　forward current

C.　propeller current

D.　wake current

KEY:　D

[2802]A structure，usually made of stone，or cement pilings，which extends from the bank at approximately right angles to the current is called a ______.

A.　dike

B.　revetment

C.　cutoff

D.　crib

KEY:　A

[2803]Current refers to the ______.

A.　vertical movement of the water

B.　horizontal movement of the water

C.　density changes in the water

D.　None of the above

KEY:　B

[2804]Currents are ______ in rate and direction．

A.　vary

B.　variable

C.　variably

D.　variability

KEY:　B

[2805]Generally speaking，a ship steaming across the North Pacific from Japan to Seattle is likely to experience ______.

A.　adverse currents for practically the entire crossing

B.　favorable currents for practically the entire crossing

C.　favorable currents in the summer months and adverse currents in the winter months

D.　variable currents having no significant effect on the total steaming time

KEY:　B

[2806]In mid-ocean，the characteristics of a wave are determined by three factors. What is NOT one of these factors?

A.　Effect of the moon's gravity

B.　Fetch

C.　Wind velocity

D.　Length of time a wind has been blowing

KEY:　A

[2807]In which month will the equatorial counter current be strongest?

A.　January

B.　April

C.　August

D.　October

KEY:　C

[2808]Neap tides occur ______.

A.　at the start of spring，when the Sun is nearly over the equator

B.　only when the Sun and Moon are on the same sides of the Earth and are nearly in line

C.　when the Sun and Moon are at approximately 90to each other，as seen from the Earth

D.　when the Sun，Moon，and Earth are nearly in line，regardless of alignment order

KEY:　C

[2809]Neap tides occur only ______.

A.　at a new or full Moon

B.　when the Sun，Moon，and Earth are in line

C.　at approximately 28-day intervals

D.　when the Moon is at quadrature

KEY:　D

[2810]Neap tides occur when the ______.

A.　Moon is in its first quarter and third quarter phases

B.　Sun and Moon are on opposite sides of the Earth

C.　Moon's declination is maximum and opposite to that of the Sun

D.　Sun and Moon are in conjunction

KEY:　A

[2811]One of the causes of ocean currents is density differences in the water. This is true because ______.

A.　in an area of high density the water's surface is lower than in an area of low density

B.　surface water flows from an area of high density to one of low density

C.　the lesser the density gradient the freer the water is to move

D.　it is the density differences that cause the currents to stay in the troughs

KEY:　B

[2812]Priming of the tides occurs ______.

A.　at times of new and full Moon

B.　when the Earth，Moon，and Sun are lying approximately on the same line

C.　when the Moon is between first quarter and full and between third quarter and new

D.　when the Moon is between new and first quarter and between full and third quarter

KEY:　D

[2813]River currents tend to ______.

A.　pick up speed where the channel widens

B.　run slower in the center of the channel

C.　hug the inside of a bend

D.　cause the greatest depth of water to be along the outside of a bend

KEY:　D

[2814]Scouring is the result of ______.

A.　interruption of the normal current flow by the footing

B.　elevating in mud-slide zones

C.　penetrating rapidly through a hard crust into weaker soils

D.　extracting legs from soils with high cohesion

KEY:　A

[2815]Set of current is ______.

A.　its velocity in knots

B.　direction from which it flows

C.　estimated current

D.　direction toward which it flows

KEY:　D

[2816]Spring tides are tides that ______.

A.　have lows lower than normal and highs higher than normal

B.　have lows higher than normal and highs lower than normal

C.　are unpredictable

D.　occur in the spring of the year

KEY:　A

[2817]Spring tides occur ______.

A.　at the start of spring，when the Sun is nearly over the equator

B.　only when the Sun and Moon are on the same side of the Earth and nearly in line

C.　when the Sun and Moon are at approximately 90to each other as seen from the Earth

D.　when the Sun，Moon，and Earth are nearly in line，in any order

KEY:　D

[2818]Spring tides occur ______.

A.　when the moon is new or full

B.　when the moon and sun have declination of the same name

C.　only when the moon and sun are on the same sides of the earth

D.　at the beginning of spring when the sun is over the equator

KEY:　A

[2819]Stand of the tide is that time when ______.

A.　the vertical rise or fall of the tide has stopped

B.　slack water occurs

C.　tidal current is at a maximum

D.　the actual depth of the water equals the charted depth

KEY:　A

[2820]Swell is the rise and fall of the ocean's surface due to ______.

A.　fetch

B.　distant winds

C.　local storms

D.　the pull of the moon

KEY:　B

[2821]The Benguela Current flows in a ______.

A.　SW'ly direction along the NW coast of Africa

B.　S'ly direction off the East Coast of Australia

C.　NW'ly direction along the SW coast of Africa

D.　SW'ly direction along the SE coast of Greenland

KEY:　C

[2822]The best time to work a boat into a slip is ______.

A.　when the wind is against you

B.　with the current setting against you

C.　at slack water

D.　with a cross current

KEY:　C

[2823]The Brazil Current flows in which general direction?

A.　Northwesterly

B.　Southwesterly

C.　Southerly

D.　Northerly

KEY:　B

[2824]The current rates ______ excess of 2 kn may occur with prolonged strong winds．

A.　by

B.　with

C.　on

D.　in

KEY:　D

[2825]The difference between the heights of low and high tide is the ______.

A.　period

B.　range

C.　distance

D.　depth

KEY:　B

[2826]The easiest way to anchor a vessel in a current is to ______.

A.　stem the current and make very slow headway when the anchor is dropped

B.　stem the current and be falling aft very slowly when the anchor is dropped

C.　stem the current and endeavor to make neither headway nor sternway when the anchor is dropped

D.　stop all headway through the water and keep the current astern when the anchor is dropped

KEY:　B

[2827]The Humboldt Current flows in which direction?

A.　North

B.　South

C.　East

D.　West

KEY:　A

[2828]The interval of the average elapsed time from the meridian transit of the moon until the next high tide is called the ______.

A.　harmonic constant

B.　establishment of the port

C.　half-tide level

D.　tide cycle

KEY:　B

[2829]The movement of water away from the shore or downstream is called a(n) ______.

A.　reversing current

B.　ebb current

C.　flood current

D.　slack current

KEY:　B

[2830]The navigational term SET OF CURRENT means ______.

A.　velocity of current in knots

B.　direction from which the current flows

C.　estimated current

D.　direction toward which the current flows

KEY:　D

[2831]The north equatorial current flows to the ______.

A.　east

B.　northeast

C.　southwest

D.　west

KEY:　D

[2832]The set of the current is the ______.

A.　speed of the current at a particular time

B.　maximum speed of the current

C.　direction from which the current flows

D.　direction in which the current flows

KEY:　D

[2833]The set of the equatorial countercurrent is generally to the ______.

A.　north

B.　east

C.　southwest

D.　northwest

KEY:　B

[2834]The set of the ocean current is caused by ______.

A.　density differences of the water

B.　rotation of the earth

C.　direction of primary circulation of air

D.　all of the above

KEY:　D

[2835]The speed of an ocean current is dependent on ______.

A.　the density of the water

B.　the air temperature

C.　the presence of a high pressure area near it

D.　underwater soil conditions

KEY:　A

[2836]The steady current circling the globe at about 60S is the ______.

A.　Prevailing Westerly

B.　Sub-Polar Flow

C.　West Wind Drift

D.　Humboldt Current

KEY:　C

[2837]The term flood current refers to that time when the water ______.

A.　is flowing towards the land

B.　is moving towards the ocean

C.　level is not changing

D.　level is rising because of heavy rains

KEY:　A

[2838]The tide ______.It's 2 hours before low water，and ______ your present position you will be aground at low water．

A.　falls／in

B.　has been falling／on

C.　has fallen／at

D.　is falling／in

KEY:　D

[2839]The tide is ______.

A.　black

B.　setting

C.　being

D.　having

KEY:　B

[2840]The two most effective generating forces of surface ocean currents are ______.

A.　temperature and salinity differences in the water

B.　wind and density differences in the water

C.　water depth and underwater topography

D.　rotation of the Earth and continental interference

KEY:　B

[2841]The type of current which will have the greatest effect on the course made good for your vessel is ______.

A.　one flowing in the same direction as your course steered

B.　one flowing in the opposite direction as your course steered

C.　one that flows at nearly right angles to your course steered

D.　a rotary current in which the direction of current flow constantly changes

KEY:　C

[2842]The velocity of a rotary tidal current will be decreased when the Moon is ______.

A.　at apogee

B.　new

C.　full

D.　All of the above

KEY:　A

[2843]The velocity of the current in large coastal harbors is ______.

A.　predicted in Tidal Current Tables

B.　unpredictable

C.　generally constant

D.　generally too weak to be of concern

KEY:　A

[2844]The velocity of the wind，its steady direction，and the amount of time it has blown determines a wind driven current's ______.

A.　temperature

B.　density

C.　deflection

D.　speed

KEY:　D

[2845]The West Wind Drift is located ______.

A.　near 60S

B.　on each side of the Equatorial Current

C.　in the North Atlantic between Greenland and Europe

D.　in the South Pacific near 5S

KEY:　A

[2846]The winds with the greatest effect on the set，drift，and depth of the equatorial currents are the ______.

A.　doldrums

B.　horse latitudes

C.　trade winds

D.　prevailing westerlies

KEY:　C

[2847]Tide is ______ in direction ENE．

A.　getting

B.　setting

C.　being

D.　having

KEY:　B

[2848]To make sure of getting the full advantage of a favorable current，you should reach an entrance or strait at what time in relation to the predicted time of the favorable current?

A.　One hour after the predicted time

B.　At the predicted time

C.　30 minutes before flood，one hour after an ebb

D.　30 minutes before the predicted time

KEY:　D

[2849]Tropic tides are caused by the ______.

A.　Moon being at its maximum declination

B.　Moon crossing the equator

C.　Sun and Moon both being near 0declination

D.　Moon being at perigee

KEY:　A

[2850]Water may boil up around the stern of a vessel in a channel due to ______.

A.　slack water when upbound

B.　shallow water

C.　a cross current

D.　a head current

KEY:　B

08.7 货物装卸货作业

[3101]Which action will affect the trim of a vessel?

A.　Moving high weights lower

B.　Adding weight at the tipping center

C.　Moving a weight forward

D.　Moving lower weight high

KEY:　C

[3102]A large basin cut into the shore，closed off by a caisson，and used for drydocking of ships is known as a ______.

A.　slipway

B.　graving dock

C.　ground warp

D.　caisson dock

KEY:　B

[3103]A large GM would give a vessel which of the following characteristics？ ______. ① More safety in case of flooding． ② Increased racking stresses．

A.　① only

B.　② only

C.　Both ① and ②

D.　Neither ① nor ②

KEY:　C

[3104]A sling is a device used in ______.

A.　hoisting cargo aboard a vessel

B.　hoisting personnel aboard a vessel

C.　securing a small boat to a large vessel

D.　hoisting the anchor

KEY:　A

[3105]All the damage and shortage，______ those incurred after discharge and before delivery to receivers，should be noted in the discharging report．

A.　including

B.　besides

C.　except

D.　in addition to

KEY:　A

[3106]All the holds to be loaded with grain must be swept ______ commencement of loading．

A.　cleanly／during

B.　clear／meanwhile

C.　clean／between

D.　clean／prior to

KEY:　D

[3107]Backstays are ______.

A.　running rigging leading aft from the masts

B.　running rigging leading forward from the masts

C.　standing rigging leading aft from the masts

D.　standing rigging from the cross trees to the mast head

KEY:　C

[3108]Bonding cables are used during cargo transfer to ______.

A.　provide safe electrical power connection to barge equipment

B.　keep the vessel from surging excessively

C.　prevent an accidental discharge of static electricity

D.　secure the cargo hose to a hatch when loading overall

KEY:　C

[3109]For a given weight of cargo，the stress on the heel block of a cargo boom ______.

A.　increases as the cargo closes the head while hoisting

B.　increases if the cargo runner is doubled up

C.　decreases as the boom is topped to a greater angle

D.　is determined by the thrust stresses on the boom

KEY:　C

[3110]For any given pedestal crane，when the boom is lengthened，the lifting capacity is ______.

A.　unchanged

B.　increased

C.　eliminated

D.　decreased

KEY:　D

[3111]Oil product samples should be taken from the ______.

A.　dock riser

B.　ship's cargo tanks

C.　shore tank discharge

D.　All of the above

KEY:　D

[3112]On a ship crane，the load chart relates the allowable load to the combination of boom length and ______.

A.　boom angle

B.　boom strength

C.　load radius

D.　cable strength

KEY:　C

[3113]On no account ______ to be exceeded．

A.　is the S．W．L． of the crane

B.　are the S．W．L． of the crane

C.　the S．W．L． of the crane is

D.　the S．W．L．of the crane are

KEY:　A

[3114]Stop ______ the derrick at once，the gooseneck is bent．

A.　to use

B.　to operate

C.　maneuvering

D.　using

KEY:　D

[3115]The boom indicator on a crane will indicate the ______.

A.　length of the boom

B.　angle of the boom

C.　lifting capacity of the boom

D.　direction of the boom

KEY:　B

[3116]The boom indicator tells the operator at what angle the boom is compared to the ______.

A.　horizontal position of the boom

B.　vertical position of the boom

C.　horizontal position of the load being lifted

D.　vertical position of the load being lifted

KEY:　A

[3117]The load chart of a ship crane enables the operator to combine the load radius with boom length to determine the ______.

A.　maximum counter weight required

B.　minimum horsepower required

C.　hoist rope strength

D.　allowable load

KEY:　D

[3118]The port clearance for a ship should be granted ______ the completion of loading or discharging in the port．

A.　before

B.　when

C.　after

D.　during

KEY:　C

[3119]The proper place ______ the sling is made up is the square of the hatch．

A.　which

B.　where

C.　when

D.　how

KEY:　B

[3120]The ship ______ about 1/2 miles southward of the lighthouse．

A.　moors

B.　moor

C.　is moored

D.　mooring

KEY:　C

[3121]The ship is listing too much due to your faulty method in loading operation．You must keep her ______.

A.　flat

B.　up

C.　upright

D.　vertical

KEY:　C

[3122]The term DISCHARGE，as it applies to the pollution regulations，means ______.

A.　spilling

B.　leaking

C.　dumping

D.　All of the above

KEY:　D

[3123]The term Loading and unloading Cargo means ______.

A.　arranging cargo

B.　shipping cargo

C.　cargo handling

D.　weighing cargo

KEY:　C

[3124]The terminal indicates to you that they are going to use a booster pump to assist the discharging operation. You start the discharge，and in a few minutes the pressure drops sharply. This could be a result of the ______.

A.　booster pump coming on the line and discharging properly

B.　booster pump failing to start

C.　booster pump being lined up in the wrong direction

D.　ship's pump speeding up

KEY:　A

[3125]The terminal stuff tallies the cargo mainly before the ship's ______.

A.　loading

B.　leaving

C.　arriving

D.　sailing

KEY:　A

[3126]The total leg load reactions on an independent leg，self-elevating unit include static and ______.

A.　environmental loading

B.　variable loading

C.　fixed loading

D.　basic loading

KEY:　A

[3127]The total quantity of oil discharge into the sea ______ for existing tankers 1/15000 of the total quantity of the particular cargo or which the residue formed a part ______.

A.　does not exceed

B.　should exceed

C.　should be up to

D.　should be no more than

KEY:　A

[3128]The tricing pendants should be released ______.

A.　before the gripes are removed

B.　before loading the passengers

C.　after loading the passengers

D.　after the boat is afloat

KEY:　C

[3129]The valve on the discharge side of a cargo pump on a tanker will usually be a ______.

A.　butterfly valve

B.　spectacle valve

C.　check valve

D.　globe valve

KEY:　C

[3130]These weights are ______ the ship's discharging facilities．

A.　out

B.　beyond

C.　from

D.　off

KEY:　B

[3131]They {commenced} loading at 1400 hours．

A.　started

B.　finished

C.　completed

D.　reported

KEY:　A

[3132]They decided to discharge the {heavy lifts}) soon．

A.　heavy cargo

B.　heavy weather

C.　cranes

D.　derricks

KEY:　A

[3133]To ______ sanitary water from any vessel，an application shall be made to Harbor Authorities for approval．

A.　discharge

B.　recircle

C.　load

D.　take in

KEY:　A

[3134]To prevent the riser system from buckling under its own weight，upward tension is provided by the ______.

A.　riser stabilizers

B.　guideline tensioning system

C.　riser tensioning system

D.　guideline stabilizers

KEY:　C

[3135]We have finished ______ and are ready for sailing．

A.　load

B.　loading

C.　will load

D.　to load

KEY:　B

[3136]We'll put off ______ until Friday．

A.　to discharge

B.　discharge

C.　discharging

D.　to be discharging

KEY:　C

[3137]We've changed our ______ not to start loading this evening．

A.　heart

B.　hearts

C.　mind

D.　minds

KEY:　C

[3138]What is a characteristic of all centrifugal cargo pumps?

A.　They are self-priming.

B.　Decreasing the speed of rotation will decrease the discharge pressure.

C.　Opening the discharge valve wider will increase the discharge pressure.

D.　All of the above

KEY:　B

[3139]What is an example of the term Restraint of Rulers，Princes，or Peoples in a marine insurance policy?

A.　A prohibition from loading a cargo from a country when the cargo may be a carrier of infectious diseases.

B.　Arrest of a vessel by legal authorities to satisfy claims through exercise of a maritime lien.

C.　Damage caused by riot of the population of a port.

D.　Losses caused by fines from polluting the harbor after malfunction of a piping system.

KEY:　A

[3140]What is NOT a precaution to be taken when topping off?

A.　Reduce the loading rate.

B.　Notify the engine room of the procedure.

C.　Maintain communications with the dock man.

D.　Give the operation your undivided attention.

KEY:　B

[3141]What is NOT a responsibility of the tankerman in charge of loading an unmanned tank barge?

A.　Enforcement of smoking restrictions

B.　Restricting vessels from coming alongside the barge

C.　Insuring that a combustible gas indicator is aboard

D.　Protecting cargo hose with chafing gear

KEY:　C

[3142]What is the best indication of the loading of a diesel engine?

A.　Oil temperature

B.　Manifold pressure

C.　Exhaust gas temperature

D.　Fuel consumption

KEY:　C

[3143]What is the function of wearing rings found on some centrifugal pumps?

A.　Absorb erosion of high velocity discharge stream

B.　Seal pump shaft against entry of air

C.　Isolate the outlet side from the inlet side

D.　Dampen the turbulent discharge flow

KEY:　C

[3144]What is the holding power ratio of an anchor?

A.　Maximum mooring line tension divided by the anchor's weight in air

B.　Anchor's weight in air divided by the maximum mooring line tension

C.　Preloading tension divided by the anchor's weight in air

D.　Operating tension divided by the anchor's weight in air

KEY:　A

[3145]What is the principle behind dynamic braking as used on an anchor windlass?

A.　Electrical loading

B.　Mechanical distribution

C.　Compressibility of air

D.　Mechanical friction

KEY:　A

[3146]What is the purpose of pressure-vacuum relief valves?

A.　Regulation of discharge pressure from cargo pumps

B.　Maintaining constant velocity in cargo lines

C.　Regulation of suction head on cargo pumps

D.　To maintain a tank or void at atmospheric pressure

KEY:　D

[3147]What would be prima facie evidence of unseaworthiness?

A.　Overloading

B.　Overstowage

C.　Overbooking

D.　Overcarriage

KEY:　A

[3148]When ______ clean ballast water from any vessel，an application shall be made to Harbor Authorities for approval．

A.　discharging

B.　recircling

C.　loading

D.　taking in

KEY:　A

[3149]When a cargo boom or crane is rated at varying capacities，there will be a table at the controls which relates safe working load to ______.

A.　winch speed

B.　boom strength

C.　load radius

D.　cable strength

KEY:　C

[3150]When checking the oxygen content of the cargo tanks prior to loading cargoes requiring vapor recovery，the atmosphere must be sampled ______.

A.　one meter from the tank bottom and one meter below the tank top

B.　one half the ullage of the tank and one meter below the tank top

C.　one half the ullage of the tank and one meter above the tank bottom

D.　at three meter intervals from the tank top

KEY:　B

08.8 理货与计量作业

[3151]______ this electric fan to the tally room．

A.　Fetch

B.　Take

C.　Bring

D.　Let

KEY:　B

[3152]______ will be paid by shipowners after tallyman doing the tally work．

A.　Cargo-handling expenses

B.　Tally money

C.　Cargo-tallying dues

D.　Tally fees

KEY:　D

[3153]A ship is responsible for the cargo loaded or unloaded ______.

A.　only when the cargo is within its rail

B.　after the cargo is checked by tallymen

C.　as soon as the cargo loading or unloading begins

D.　during the time when loading or unloading is going on

KEY:　A

[3154]A shore is a piece of securing dunnage that ______.

A.　runs from a low supporting level up to the cargo at an angle

B.　is also known as a distance piece

C.　is placed on the deck under the cargo to distribute its weight evenly

D.　is run horizontally from a support to the cargo

KEY:　A

[3155]A vessel fishing at night，with gear extending more than 150 meters horizontally outwards，will show in the direction of the gear ______.

A.　one white light

B.　two vertical white lights

C.　one yellow light

D.　two vertical yellow lights

KEY:　A

[3156]After being launched from a vessel，totally enclosed survival craft which have been afloat over a long period require ______.

A.　frequent opening of hatches to permit entry of fresh air

B.　regular checks of bilge levels

C.　use of ear plugs to dampen engine noise

D.　frequent flushing of the water spray system with fresh water

KEY:　B

[3157]As a general rule，tally clerks should make their tallying ______.

A.　in warehouses

B.　on deck

C.　ashore

D.　in the tallyroom

KEY:　B

[3158]By day，the horizontal motion of a white flag，or arms extended horizontally，by a person on the beach indicates ______.

A.　Haul away

B.　Tail block is made fast

C.　Negative

D.　Affirmative

KEY:　C

[3159]Can you tell me something about the tallying methods at your port？ ______.

A.　Certain

B.　Be certain

C.　Sure

D.　Be sure

KEY:　C

[3160]During loading or discharging，the tallymen must make contact with ______ on duty so as to solve problems in time．

A.　ship owner

B.　ship’s officers

C.　shippers

D.　consignors

KEY:　B

[3161]Green lights may appear on ______.

A.　horizontally banded buoys

B.　vertically striped buoys

C.　yellow buoys

D.　spherical buoys

KEY:　A

[3162]How do you dispose ______ the general cargo in the fore part of Hatch No．5？

A.　at

B.　to

C.　of

D.　with

KEY:　C

[3163]Hygroscopic cargoes should be ventilated when ______.

A.　going from a warm to a cold climate

B.　the dew point of the outside air is greater than the dew point of the air in the hold

C.　the dew point of the air in the hold is very low

D.　the outside dew point is 16℃ and the cargo temperature is 12℃

KEY:　A

[3164]If a hydraulic pump on a winch accidentally stops while hoisting，the load will stay suspended because ______.

A.　a check valve will close that prevents reverse circulation

B.　a centrifugal counterweight counteracts the force of gravity.

C.　the electric pump motor will cut out

D.　the control lever will move to the stop position

KEY:　A

[3165]If a void occurs in the cargo hold，it is better to ______ to control the broken stowage．

A.　brace it with dunnage

B.　cover it with large pieces

C.　fill it with small pieces

D.　leave it as it is

KEY:　C

[3166]If you came into contact with nitrobenzene while disconnecting the cargo hose，you should be aware that it is ______.

A.　highly toxic when absorbed through the skin

B.　a blood poison

C.　a nerve poison

D.　All of the above

KEY:　D

[3167]In the absence of exceptions clauses，there is ______ obligation to supply a cargo of the contractual description and quantity．

A.　a conditional

B.　an absolute

C.　a considerable

D.　a substantial

KEY:　B

[3168]In the case of any actual or apprehended loss or damage the carrier and receiver must ______ all reasonable facilities to each other for inspecting and tallying the goods．

A.　make

B.　take

C.　have

D.　give

KEY:　D

[3169]In the Uniform Cardinal System of Buoyage，a buoy in the eastern quadrant from a danger could ______.

A.　be black and white horizontally striped

B.　be black and white vertically striped

C.　have a red top mark

D.　be any of the above

KEY:　C

[3170]Inflammable cargo ______ in the holds adjacent to the engine room．

A.　shall never be stowed

B.　may be stowed

C.　can be stowed

D.　should be stowed

KEY:　A

[3171]It ______ us three days to load the cargo，I think．

A.　will take

B.　takes

C.　taking

D.　to take

KEY:　A

[3172]Limitation of liability to a specified sum，or a clause totally exempting the Shipowner from liability if the claim is not made within a given time，cannot ______ the Shipowner where the loss is due to unseaworthiness unless clear and express words are used to indicate it．

A.　avoid

B.　avail

C.　amend

D.　awake

KEY:　B

[3173]Making cargo plan for appropriate distribution of the cargo on board is ______ duty．

A.　a ship's officer's

B.　a Tallyman's

C.　a port planner's

D.　a foreman's

KEY:　A

[3174]Masts and derricks of cargo ships are made of ______.

A.　steel

B.　copper

C.　bross

D.　zinc

KEY:　A

[3175]May I have a copy of the cargo plan？Certainly，here ______.

A.　is it

B.　are you

C.　they are

D.　you are

KEY:　D

[3176]Most totally enclosed lifeboats are equipped with which of the following?

A.　Tanks for the storage of drinking water

B.　Ballast tanks to prevent the boat from capsizing

C.　A hydraulic start diesel engine

D.　Auxiliary mechanical propulsion (Fleming gear)

KEY:　C

[3177]Navigational marks used for informational or regulatory purposes are ______.

A.　solid yellow

B.　white with orange geometric shapes

C.　red and white vertically-striped

D.　green and red horizontally-banded

KEY:　B

[3178]Planning to stow ______ in the end lower holds will result in much broken stowage．

A.　large crates or cases

B.　small curved items

C.　drums

D.　filler cargo

KEY:　A

[3179]Please ______ a cleaning gang to sweep out the hold．

A.　arrange for

B.　to arrange

C.　to arrange

D.　arrange to

KEY:　A

[3180]Please ______ that my ship is ready in every respect to load her cargo．

A.　advised

B.　to be advised

C.　be advised

D.　are advised

KEY:　C

[3181]Please arrange for the workers to do the ______ work during loading the trucks and heavy cargo on the ship，and also supply the necessary materials for the work．

A.　lashing

B.　shifting

C.　lifting

D.　tallying

KEY:　A

[3182]Please don't change the cargo plan ______ will．

A.　at

B.　by

C.　with

D.　on

KEY:　A

[3183]Please shift some light cargo to fill it ______.

A.　in

B.　up

C.　over

D.　with

KEY:　B

[3184]Please tell your tallymen to ______ out the marks in the holds during discharging．

A.　pick

B.　choose

C.　sort

D.　decide

KEY:　C

[3185]Preventer bars are fitted on lifeboat releasing hooks to prevent ______.

A.　the falls from unhooking if the releasing gear is operated accidentally while the boat is being lowered

B.　operation of the release lever until the boat is waterborne

C.　the falls from rehooking after they have been released

D.　accidental unhooking when the falls become slack

KEY:　D

[3186]Red lights may appear on ______.

A.　horizontally banded buoys

B.　vertically striped buoys

C.　yellow buoys

D.　spherical buoys

KEY:　A

[3187]The Captain of the Port has closed to navigation，and buoyed，a section of a harbor. These buoys would be painted ______.

A.　red or green to conform with the other lateral aids

B.　red and green horizontally-striped

C.　solid yellow

D.　white with orange marks

KEY:　D

[3188]The cargo fall over the dock is called the ______.

A.　burton fall

B.　down fall

C.　hatch fall

D.　stay fall

KEY:　A

[3189]The cargo is liable to burn．Please keep it ______ the engine room．

A.　out of

B.　away from

C.　from away

D.　out from

KEY:　B

[3190]The cargo，when ______，should be completely covered with plastic．

A.　stow

B.　stowing

C.　stowed

D.　be stowed

KEY:　C

[3191]The Chief Tally would like to inquire ______ the rotation of the discharging ports．

A.　at

B.　on

C.　about

D.　after

KEY:　C

[3192]The dew point of the air in the ship's hold is to ______. ① Be kept below the temperature of the cargo to prevent cargo sweat． ② Be kept above the temperature of the hull and bulkheads to prevent ship's sweat．

A.　① only

B.　② only

C.　Both ① and ②

D.　Neither ① nor ②

KEY:　A

[3193]The force acting on a single cargo runner which is vertically lifting or lowering a load is greatest when ______.

A.　decelerating when lowering the load

B.　decelerating when raising the load

C.　lowering the load at constant speed

D.　raising the load at constant speed

KEY:　A

[3194]The foreman says that the stevedores stopped working owing to the ______ of the winch．

A.　breakdown

B.　breaked down

C.　broken down

D.　broke down

KEY:　A

[3195]The full utilization of berths can be achieved if ______.

A.　ships arrive at random

B.　cargo-handling time varies greatly

C.　ships have to queue to get onto berths

D.　ships do not have to wait to get onto berths

KEY:　C

[3196]The inability to supply a cargo of the contractual quantity in the country of shipment ______ the doctrine of frustration into operation．

A.　is capable of bringing

B.　is sufficient to bring

C.　is not capable of bringing

D.　is probally to bring

KEY:　C

[3197]The majority of cargo gear breakdowns are a result of ______.

A.　Guy failures

B.　Topping lift failures

C.　Bending of the boom from compression

D.　Gooseneck failure

KEY:　A

[3198]The payment proportionate to the part of the voyage accomplished or to the part of the cargo delivered is referred as ______.

A.　advance freight

B.　back freight

C.　dead freight

D.　pro rata freight

KEY:　D

[3199]The ship will be responsible for the cargo loaded or unloaded ______.

A.　only when the cargo is within its rail

B.　during the time when loading or unloading is going on

C.　after the cargo is checked by tallymen

D.　as soon as loading or discharging begins

KEY:　A

[3200]The Tally Fees will be settled ______ my Shipowner ______ the agents．

A.　to／for

B.　for／to

C.　by／through

D.　through／for

KEY:　C

09.2 海上气象报告与气象导航

[3301]When winds of force reach 10-11 in Beaufort Scale，usually we call such winds as ______.

A.　Hurricane

B.　Breeze

C.　Storm

D.　Gale

KEY:　C

[3302]The Routing System which aimed at reducing the risk of casualties is called as ______.

A.　Traffic Separation Schemes

B.　Recommended tracks

C.　Precautionary areas

D.　Inshore traffic zones

KEY:　A

[3303]______ LOW ELY SLWLY WITH COLD FRONT FM CEN TO 12N 179W AND WARM FROM FM CEN TO 11N 171W．

A.　SHLW

B.　SCTD

C.　OBS

D.　SW

KEY:　A

[3304]______ moving rather quickly east expected 150 miles south of Cape Farewell 972 by 160000 GMT．

A.　Developed low

B.　Developing low

C.　Being developed low

D.　Low to be developed

KEY:　B

[3305]______ rain means it rains in some parts of the area．

A.　Squally

B.　Isolated

C.　Scattered

D.　Occasional

KEY:　C

[3306]{On account of} the bad weather，my ETA is amended to read 1830 his on July 15th．

A.　For the account of

B.　Because

C.　Owing to

D.　For fear of

KEY:　C

[3307]A cold front moving in from the northwest can produce ______.

A.　thunderstorms，hail，and then rapid clearing

B.　increasing cloud cover lasting for several days

C.　lengthy wet weather

D.　low ceilings with thick cirrus clouds

KEY:　A

[3308]A feature of an inflatable raft which helps keep people stationary in rough weather is ______.

A.　lashings on the floor of the raft for the passenger's feet

B.　straps from the overhead

C.　lifelines on the inside of the raft

D.　ridges in the floor of the raft

KEY:　C

[3309]A line on a weather chart connecting places which have the same barometric pressure is called an ______.

A.　isotherm

B.　isallobar

C.　isobar

D.　isotope

KEY:　C

[3310]A local wind which occurs during the daytime and is caused by the different rates of warming of land and water is a ______.

A.　foehn

B.　chinook

C.　land breeze

D.　sea breeze

KEY:　D

[3311]A local wind which occurs during the daytime and is caused by the different rates of warming of land and water is a ______.

A.　Foehn

B.　Chinook

C.　land breeze

D.　sea breeze

KEY:　D

[3312]A low，uniform layer of cloud resembling fog，but not resting on the ground，is called ______.

A.　cumulus

B.　nimbus

C.　stratus

D.　cirrus

KEY:　C

[3313]A mercurial barometer at sea is subject to rapid variations in height (pumping) due to the pitch and roll of the vessel. To avoid this error，measurements of atmospheric pressure at sea are usually measured with a(n) ______.

A.　syphon barometer

B.　cistern barometer

C.　aneroid barometer

D.　fortin barometer

KEY:　C

[3314]A microbarograph is a precision instrument that provides a ______.

A.　charted record of atmospheric temperature over time

B.　charted record of atmospheric pressure over time

C.　graphic record of combustible gases measured in an atmosphere

D.　graphic record of vapor pressure from a flammable/combustible liquid

KEY:　B

[3315]A millibar is a unit of ______.

A.　humidity

B.　precipitation

C.　pressure

D.　temperature

KEY:　C

[3316]A sea breeze is a wind ______.

A.　that blows towards the sea at night

B.　that blows towards an island during the day

C.　caused by cold air descending a coastal incline

D.　caused by the distant approach of a hurricane

KEY:　B

[3317]A sylphon cell is a part of a ______.

A.　maximum thermometer

B.　barograph

C.　thermograph

D.　hygrometer

KEY:　B

[3318]A tropical cyclone has recurved and entered temperate latitudes. In the Northern Hemisphere when a large high pressure system lies north of the storm，what situation may occur?

A.　The low may suddenly deepen，and the cyclone intensify and pick up speed.

B.　The left semicircle may become the dangerous semicircle.

C.　The low and the high may merge and cancel out the weather characteristics of each.

D.　The high may force the cyclone to reverse its track.

KEY:　B

[3319]A tropical storm is building strength some distance from your ship. Waves are coming from the east，with periods increasing from 5 seconds to 15 seconds. The swell is from the east. Where was the storm when these new swells were generated?

A.　To the north of you

B.　To the south of you

C.　To the east of you

D.　To the west of you

KEY:　C

[3320]A tropical wave is located 200 miles due west of your position，which is north of the equator. Where will the wave be in 24 hours?

A.　Farther away to the west

B.　Farther away to the east

C.　In the same place

D.　Closer and to the west

KEY:　A

[3321]A tropical wave is usually preceded by ______.

A.　tropical storms

B.　good weather

C.　heavy rain and cloudiness

D.　heavy seas

KEY:　B

[3322]A weather forecast states that the wind will commence backing. In the Northern Hemisphere，this would indicate that it will ______.

A.　shift in a clockwise manner

B.　shift in a counterclockwise manner

C.　continue blowing from the same direction

D.　decrease in velocity

KEY:　B

[3323]A weather map is a synoptic data because it ______.

A.　summarizes a great deal of information

B.　can be interpreted accurately

C.　appears daily

D.　is prepared by the Weather Bureau

KEY:　A

[3324]A WIND BLOWS ROUND ANTICLOCKWISE defines ______.

A.　Backing (of wind)

B.　Beach (to)

C.　Veering (of wind)

D.　Maintaining direction of the wind

KEY:　A

[3325]After a cold front passes the barometric pressure usually ______.

A.　fluctuates

B.　remains the same

C.　remains the same，with clouds forming rapidly

D.　rises，often quite rapidly，with clearing skies

KEY:　D

[3326]After a cold front passes，the barometric pressure ______.

A.　drops，and the temperature drops

B.　drops，and the temperature rises

C.　rises，and the temperature drops

D.　rises，and the temperature rises

KEY:　C

[3327]All of the following are associated with cumulonimbus clouds EXCEPT ______.

A.　steady rainfall

B.　hail storms

C.　thunderstorms

D.　tornadoes or waterspouts

KEY:　A

[3328]An air mass is termed warm if ______.

A.　it is above 70F

B.　the ground over which it moves is cooler than the air

C.　it originated in a high pressure area

D.　it originated in a low pressure area

KEY:　B

[3329]An aneroid barometer is an instrument ______.

A.　used to measure the speed of wind

B.　in which the pressure of the air is measured

C.　that tells which direction a storm is coming from

D.　used to measure the height of waves

KEY:　B

[3330]An occluded front on a weather map is colored ______.

A.　blue line

B.　purple line

C.　dashed blue line

D.　alternate red and blue line

KEY:　B

[3331]Anticyclones are usually characterized by ______.

A.　dry，fair weather

B.　high winds and cloudiness

C.　gustiness and continuous precipitation

D.　overcast skies

KEY:　A

[3332]As the temperature for a given mass of air increases，the ______.

A.　dew point increases

B.　dew point decreases

C.　relative humidity increases

D.　relative humidity decreases

KEY:　D

[3333]BASHI：E TO SE 9-10．500M IN HVY SQUALLY SHWRS AND TS．SEA 8-9M．SWELL SE 6-7M．From the above forecast，the wind in Beaufort Scale will be ______.

A.　STRONG GALE TO STORM

B.　GALE TO STRONG GALE

C.　NEAR GALE TO GALE

D.　STRONG BREEZE TO NEAR GALE

KEY:　A

[3334]CAPE WEST WIND WEST TO SOUTHWEST 15 TO 20 KNOTS WEATHER SHOWERS．This description is likely to be under the heading of ______.

A.　FORECAST

B.　STORM WARNING

C.　GENERAL SITUATION

D.　MAP ANALYSIS

KEY:　A

[3335]During the voyage he encountered boisterous winds and heavy weather during which time the vessel ______ heavily and to such an extent that at times it was necessary to change course．

A.　moved

B.　labored

C.　drove

D.　went

KEY:　B

[3336]During the voyage my ship ______ bad weather in the Pacific Ocean．

A.　meets with

B.　has met with

C.　met with

D.　meeted with

KEY:　C

[3337]Especially in adverse weather，risk of collision with an offshore supply vessel increases when the vessel is moored to what side of the unit? ______.

A.　Upwind

B.　Downwind

C.　Crosswind

D.　Downcurrent

KEY:　A

[3338]Fleet weather forecast messages are the expected conditions in ______.

A.　a certain area

B.　all ocean areas

C.　some areas

D.　important areas

KEY:　A

[3339]For an accurate barometer check，you would ______.

A.　check it with a barometer on another vessel

B.　take readings from several barometers and average them

C.　check it with the barometer at the ship chandlery

D.　check it against radio or National Weather Service reports of the immediate vicinity

KEY:　D

[3340]Gale warnings are usually issued when winds of ______ 8-9 are expected．

A.　wind Speed

B.　Air Force

C.　wind Velocity

D.　Beaufort Force

KEY:　D

[3341]HIGH WAVES；DENSE STREAKS OF FOAM ALONG THE DIRECTION OF THE WIND；CRESTS OF WAVES BEGIN TO TOPPLE，TUMBLE AND ROLL OVER；SPRAY MAY AFFECT VISIBILITY．This condition is likely to be termed ______.

A.　strong breeze

B.　near gale

C.　gale

D.　strong gale

KEY:　D

[3342]If a weather bulletin shows the center of a low pressure system to be 100 miles due east of you，what winds can you expect in the Southern Hemisphere? ______.

A.　South-southwesterly

B.　North-northwesterly

C.　South-southeasterly

D.　North-northeasterly

KEY:　A

[3343]If the sky was clear，with the exception of a few cumulus clouds，it would indicate ______.

A.　rain

B.　hurricane weather

C.　fair weather

D.　fog setting in

KEY:　C

[3344]If your weather bulletin shows the center of a low pressure area to be 100 miles due east of your position，what winds can you expect in the Northern Hemisphere? ______.

A.　East to northeast

B.　East to southeast

C.　North to northwest

D.　South to southeast

KEY:　C

[3345]In Beaufort Scale，the wind force 7 with speed of 28-33 knots is defined as ______.

A.　Moderate gale

B.　Near gale

C.　Strong gale

D.　Whole gale

KEY:　B

[3346]INTERTROPICAL CONVERGENCE ZONE OVER SOUTH PHILIPPINE PASSING OVER MINDANAO NORTHERN SULU AND PALAWAN．This paragraph is likely to be under the heading of ______.

A.　GALE WARNING

B.　24 HOURS FORECAST

C.　SYNOPSIS

D.　STORM WARNING

KEY:　C

[3347]It is predicted that heavy rains are ______ to flood the harbor and its vicinity in a few days．

A.　frightening

B.　threatening

C.　scattering

D.　warning

KEY:　B

[3348]Mechanical lifting of air by the upslope slant of the terrain is called ______.

A.　vertical lifting

B.　convective lifting

C.　advective lifting

D.　topographic lifting

KEY:　D

[3349]Of the following，______ is not a wind．

A.　bora

B.　gust

C.　gregale

D.　floe

KEY:　D

[3350]OWING TROPICAL STORM9706 CROSSING OUR/COURSE PLSPERMIT SHELTERING KAGOSHIMA KAIWAN. This cable indicates that ______.

A.　she requested shelter permission

B.　she intended to berth at port of KAGOSHIMA

C.　she intended to change her course

D.　she was reporting to the port that a tropical storm is coming

KEY:　A

09.3 海上天气现象

[3351]When a wind blows round clockwise，it is ______.

A.　Variable

B.　Changing

C.　Backing

D.　Veering

KEY:　D

[3352]Isobars on a synoptic chart are useful in predicting ______.

A.　temperature

B.　dew point

C.　wind velocity

D.　relative humidity

KEY:　C

[3353]______ causing strong winds and rough sea north west Bay of Biscay．

A.　Strong low pressure

B.　Strong high pressure

C.　Steep pressure gradient

D.　Steep gradient pressure

KEY:　C

[3354]A hurricane moving northeast out of the Gulf passes west of your position. You could expect all of the following EXCEPT ______.

A.　higher than normal swells

B.　high winds

C.　winds veering from south，through west，to northwest

D.　light showers

KEY:　D

[3355]A katabatic wind blows ______.

A.　up an incline due to surface heating

B.　in a circular pattern

C.　down an incline due to cooling of the air

D.　horizontally between a high and a low pressure area

KEY:　C

[3356]A steep barometric gradient indicates ______.

A.　calms

B.　light winds

C.　strong winds

D.　precipitation

KEY:　C

[3357]A storm is forecast for the area where your vessel is moored. For its safety you should put ______.

A.　more slack in the mooring lines

B.　a strain on the mooring lines

C.　chafing gear on the mooring lines

D.　grease on the mooring lines

KEY:　C

[3358]A strong，often violent，northerly wind occurring on the Pacific coast of Mexico，particularly during the colder months，is called ______.

A.　Tehuan tepecer

B.　Papagayo

C.　Norther

D.　Pampero

KEY:　B

[3359]A strong，often violent，northerly wind occurring on the Pacific coast of Mexico，particularly during the colder months，is a ______.

A.　Papagayo

B.　fall wind

C.　foehn

D.　williwaw

KEY:　B

[3360]A thin，whitish，high cloud popularly known as mares tails is ______.

A.　altostratus

B.　stratus

C.　cumulus

D.　cirrus

KEY:　D

[3361]A very light breeze that causes ripples on a small area of still water is a ______.

A.　cat's paw

B.　hog's breath

C.　williwaw

D.　chinook

KEY:　A

[3362]Above-normal tides near the center of a hurricane maybe caused by the ______.

A.　high barometric pressure

B.　jet stream

C.　storm surge

D.　torrential rains

KEY:　C

[3363]Altocumulus clouds are defined as ______.

A.　high clouds

B.　middle clouds

C.　low clouds

D.　vertical development clouds

KEY:　B

[3364]An anemometer on a moving vessel measures ______.

A.　apparent wind speed only

B.　true wind speed and true wind direction

C.　true wind speed only

D.　apparent wind speed and true wind direction

KEY:　A

[3365]An aneroid barometer reading should be corrected for differences in ______.

A.　elevation

B.　temperature

C.　wind speed

D.　latitude

KEY:　A

[3366]Cirrus clouds are composed primarily of ______.

A.　ice crystals

B.　water droplets

C.　snow crystals

D.　nitrogen

KEY:　A

[3367]Early indications of the approach of a hurricane may be all of the following EXCEPT ______.

A.　short confused swells

B.　gradually increasing white clouds (mare's tails)

C.　pumping barometer

D.　continuous fine mist-like rain

KEY:　A

[3368]Especially in adverse weather，risk of collision with an offshore supply vessel increases when the vessel is moored to what side of the unit? ______.

A.　Upwind

B.　Downwind

C.　Crosswind

D.　Downcurrent

KEY:　A

[3369]Fetch is the ______.

A.　distance a wave travels between formation and decay

B.　stretch of water over which a wave-forming wind blows

C.　time in seconds required for two crests to pass a given point

D.　measurement of a wave's steepness

KEY:　B

[3370]Fleet weather forecast messages are the expected conditions in ______.

A.　a certain area

B.　all ocean areas

C.　some areas

D.　important areas

KEY:　A

[3371]Fog is likely to occur when there is little difference between the dew point and the ______.

A.　relative humidity

B.　air temperature

C.　barometric pressure

D.　absolute humidity

KEY:　B

[3372]For an accurate barometer check，you would ______.

A.　check it with a barometer on another vessel

B.　take readings from several barometers and average them

C.　check it with the barometer at the ship chandlery

D.　check it against radio or National Weather Service reports of the immediate vicinity

KEY:　D

[3373]Gasoline tank vent lines on board small passenger vesselsmust be fitted with removable flame screens ______.

A.　and 30 square inches of louvers

B.　three inches in diameter with a check valve to prevent water from entering in heavy weather

C.　three inches in circumference inside the fill pipe

D.　consisting of a single screen of at least 30 X 30 mesh，corrosion resistant wire

KEY:　D

[3374]Generally speaking，with ______ the weather becomes cloudy or overcast with strong winds．

A.　high pressures

B.　low pressures

C.　weak pressures

D.　strong pressures

KEY:　B

[3375]Good weather is usually associated with a region of ______.

A.　low barometric pressure

B.　high barometric pressure

C.　falling barometric pressure

D.　pumping barometric pressure

KEY:　B

[3376]High clouds，composed of small white flakes or scaly globular masses，and often banded together to form a mackerel sky，would be classified as ______.

A.　cirrus

B.　cirrocumulus

C.　altostratus

D.　cumulonimbus

KEY:　B

[3377]High pressure ridge ______ from Sevastopol to eastern Libya．

A.　reaching

B.　extending

C.　increasing

D.　upgrading

KEY:　B

[3378]Hurricane warnings are issued ______ when winds of force 12 or above are expected．

A.　in all parts of the world

B.　in some parts of the world

C.　from all corners of the world

D.　from every corner of the world

KEY:　B

[3379]If you are caught in the left semicircle of a tropical storm，in the Southern Hemisphere，you should bring the wind ______.

A.　on the starboard quarter，hold course and make as much way as possible

B.　2 points on the port quarter，and make as much way as possible

C.　on the port bow，and make as much way as possible

D.　dead ahead and heave to

KEY:　C

[3380]In a storm，the leeward lines of a ship's mooring system will ______.

A.　pull the unit in the same direction that the weather is pushing it

B.　pull the unit in the opposite direction that the weather is pushing it

C.　tend to keep the unit on its original location over the well head

D.　affect the unit's draft and inclination as maximum mooring tensions are reached

KEY:　A

[3381]In bad weather，what length of chain should be used with a single anchor?

A.　3 times the depth of water

B.　6 times the depth of water

C.　10 times the depth of water

D.　15 times the depth of water

KEY:　C

[3382]In North America the majority of the weather systems move from ______.

A.　north to south

B.　south to north

C.　east to west

D.　west to east

KEY:　D

[3383]In reading a weather map，closely spaced pressure gradient lines would indicate ______.

A.　high winds

B.　high overcast clouds

C.　calm or light winds

D.　fog or steady rain

KEY:　A

[3384]In the doldrums you can expect ______.

A.　steady，constant winds

B.　frequent rain showers and thunderstorms

C.　steep pressure gradients

D.　low relative humidity

KEY:　B

[3385]In the doldrums you will NOT have ______.

A.　high relative humidity

B.　frequent showers and thunderstorms

C.　steep pressure gradients

D.　frequent calms

KEY:　C

[3386]In the relatively calm area near the hurricane center，the seas are ______.

A.　moderate but easily navigated

B.　calm

C.　mountainous and confused

D.　mountainous but fairly regular as far as direction is concerned

KEY:　C

[3387]Little or no change in the barometric reading over a twelve hour period indicates ______.

A.　stormy weather is imminent

B.　that present weather conditions will continue

C.　a defect in the barometer

D.　increasing wind strength

KEY:　B

[3388]MAINLY GOOD BECOMING MODERATE．This forecast probably refers to ______ in a certain area．

A.　visibility

B.　winds

C.　sea

D.　fog

KEY:　A

[3389]MAINLY VARIABLE 3 to 4 VEERING NELY 5 TOMORROW MORNING．This forecast refers to ______ in the designated area．

A.　visibility

B.　winds

C.　sea

D.　fog

KEY:　B

[3390]Most high pressure areas are accompanied by ______.

A.　precipitation

B.　clear，cool weather

C.　humid，sticky weather

D.　cool fogs

KEY:　B

[3391]NASHA VERELLA ST JACQUES ZENMU ANSHA REEF CLOUDY TO OVERCAST WITH SHOWERS WIND SW 17 TO 21 SEA MOD VIS GOOD TO MOD．The abovementioned forecast is probably from ______.

A.　Radio Dalian

B.　Radio Guangzhou

C.　Radio Singapore

D.　Radio Bangkok

KEY:　B

[3392]Navigational warnings and weather bulletins for shipping ______ from Singapore Radio．

A.　broadcast

B.　broadcasted

C.　are broadcast

D.　are broadcasted

KEY:　C

[3393]On a working copy of a weather map，a cold front is represented by what color line?

A.　Red

B.　Blue

C.　Alternating red and blue

D.　Purple

KEY:　B

[3394]On the pole side of the high pressure belt in each hemisphere，the pressure diminishes. The winds along these gradients are diverted by the Earth's rotation toward the east and are known as the ______.

A.　geostrophic winds

B.　doldrums

C.　horse latitudes

D.　prevailing westerlies

KEY:　D

[3395]Plain language is usually used on marine weather ______.

A.　forecasts

B.　observations

C.　analyses

D.　reports

KEY:　A

[3396]Prevailing winds between 30N and 60N latitude are from the ______.

A.　north

B.　south

C.　east

D.　west

KEY:　D

[3397]Recurvature of a hurricane's track usually results in the forward speed ______.

A.　increasing

B.　decreasing

C.　remaining the same

D.　varying during the day

KEY:　A

[3398]Relative humidity is the percentage of water vapor that is in the air as compared to the maximum amount it can hold at ______.

A.　a specific barometric pressure

B.　a specific temperature

C.　a specific wind speed

D.　any time

KEY:　B

[3399]Snow has obliterated surface features and the sky is covered with uniform，altostratus clouds. There are no shadows and the horizon has disappeared. What is this condition called?

A.　Ice blink

B.　Whiteout

C.　Water sky

D.　Aurora reflection

KEY:　B

[3400]Steady precipitation is typical of ______.

A.　coming cold weather conditions

B.　a warm front weather condition

C.　high pressure conditions

D.　scattered cumulus clouds

KEY:　B

10.1 船舶结构基本知识

[3451]______ is the main center-line structural member，running fore and aft along the bottom of a ship，sometimes referred to as the backbone．

A.　Frame

B.　Deckbeam

C.　Stringer

D.　Keel

KEY:　D

[3452]A band or collar on the top end of a boom to which the topping lift，midships guy，and outboard guys are secured，is called the ______.

A.　collar band

B.　guy band

C.　pad eye collar

D.　spider band

KEY:　D

[3453]A continuous watertight bulkhead is normally also a(n) ______.

A.　structural bulkhead

B.　exterior bulkhead

C.　centerline bulkhead

D.　joiner bulkhead

KEY:　A

[3454]A heated bulkhead has the effect on a hygroscopic commodity of ______.

A.　causing moisture to accumulate against the bulkhead

B.　lowering the vapor pressure of the commodity

C.　lowering the dew point of the air

D.　raising the vapor pressure of the commodity

KEY:　D

[3455]A large vessel is equipped with a controllable pitch propeller. Which statement is TRUE? ______.

A.　When dead in the water，it is often difficult to find the neutral position and slight headway or sternway may result.

B.　When going directly from full ahead to full astern，there is complete steering control.

C.　When the vessel has headway and the propeller is in neutral，there is no effect on rudder control.

D.　When maneuvering in port，full ahead or astern power can usually be obtained without changing shaft RPM.

KEY:　A

[3456]A liner in riveted construction of a vessel is a(n) ______.

A.　small plate which fills the aperture between riveted strakes and the vessel framing

B.　backing plate which is used to level the strakes while riveting，and then removed

C.　internal frame to which the side shell is riveted

D.　seam that is welded after riveting is completed

KEY:　A

[3457]A record of the types and strengths of steels used on a ship must be included in the ______.

A.　general plans

B.　builder's documentation

C.　Certificate of Inspection

D.　construction portfolio

KEY:　D

[3458]A single-screw vessel going ahead tends to turn more rapidly to port because of propeller ______.

A.　discharge current

B.　suction current

C.　sidewise force

D.　thrust

KEY:　C

[3459]A twin-screw vessel with a single rudder is making headway. The engines are full speed ahead. There is no wind or current. Which statement is FALSE? ______.

A.　If one screw is stopped，the ship will turn toward the side of the stopped screw.

B.　The principal force which turns the ship is set up by the wake against the forward side of the rudder.

C.　Turning response by use of the rudder only is greater than on a single-screw vessel.

D.　With the rudder amidships，the ship will steer a fairly steady course.

KEY:　C

[3460]A vessel is equipped with twin propellers，both turning outboard with the engines half ahead. If there is no wind or current and the rudders are amidships，what will happen? ______.

A.　The bow will swing to starboard.

B.　The bow will swing to port.

C.　The vessel will steer a zigzag course.

D.　The vessel will steer a fairly straight course.

KEY:　D

[3461]A virtual rise in the center of gravity may be caused by ______.

A.　filling a partially filled tank

B.　using an on board crane to lift a freely swinging heavy object

C.　emptying a partially filled tank

D.　transferring ballast from the forepeak to the after peak

KEY:　B

[3462]Another name for the garboard strake is the ______.

A.　A strake

B.　Z strake

C.　side keel plate

D.　stringer plate

KEY:　A

[3463]As to the stained plates，the fault lies ______ you．

A.　on

B.　to

C.　up

D.　with

KEY:　A

[3464]Flanking rudders effect a vessel's heading because of the ______.

A.　effect of the propeller flow on the rudders

B.　water flow due to the vessel's movement through the water

C.　tunnel affect of the water flow past opposing rudders

D.　discharge current being channeled to impinge on the vessel's deadwood

KEY:　A

[3465]In a twin screw ship going half-ahead，both screws turning outboard and the rudder amidships，no current or wind，the vessel will ______ ．

A.　Move bodily to port．

B.　Move bodily to starboard．

C.　Move in a zig-zag motion．

D.　Steer a fairly straight course

KEY:　D

[3466]In ship construction，bulkheads in the quarters are generally ______.

A.　structural

B.　watertight

C.　non-structural

D.　continuous

KEY:　C

[3467]Many vessels are provided with flume tanks，which also have a dump tank located under the flume tanks. In the event the ship is damaged，you could dump the flume tanks into the dump tank which would ______.

A.　reduce the free surface effect and raise the KG

B.　not have any effect on free surface and raise the KG

C.　reduce the free surface effect and lower the KG

D.　not have any effect on free surface and lower the KG

KEY:　C

[3468]Metal plates that cover the top of the hawsepipe are called ______.

A.　footings

B.　plugs

C.　buckler plates

D.　stop waters

KEY:　C

[3469]No deduction is allowed for the ______ mess-room in ships having passenger accommodation，which are not also provided with a passengers’ mess-room．

A.　officer

B.　officers

C.　officer's

D.　officers’

KEY:　D

[3470]Panting frames are located in the ______.

A.　after double bottoms

B.　centerline tanks on tankers

C.　fore and after peaks

D.　forward double bottoms

KEY:　C

[3471]Shell plating that has curvature in two directions and must be heated and hammered to shape over specially prepared forms is called ______.

A.　compound plate

B.　furnaced plate

C.　flat plate

D.　rolled plate

KEY:　B

[3472]Sidewise force of the propeller tends to throw a vessel's stern to the right or left，depending on rotation. This force is caused by ______.

A.　back current from the rudder

B.　greater pressure on the right or left side of the propeller，depending on rotation

C.　lower pressure on the right or left side of the propeller，depending on rotation

D.　torque from the velocity and angle at which the surrounding water impinges upon the propeller blades

KEY:　D

[3473]Signs of racking stresses generally appear at the ______.

A.　bow and stern shell frames and plating

B.　junction of the frames with the beams and floors

C.　garboard strake，at each side of the keel

D.　thrust bearing of the main shaft

KEY:　B

[3474]Structural bulkheads on a ship are usually ______.

A.　continuous

B.　watertight

C.　transverse

D.　non-watertight

KEY:　B

[3475]The bow thruster generally is ineffective at ______.

A.　over 3 knots headway

B.　at any speed astern

C.　at any speed ahead

D.　over 1 knot sternway

KEY:　A

[3476]The circular steel structure installed around the propeller of a towboat is the ______.

A.　nozzle

B.　shroud

C.　strut

D.　hood

KEY:　A

[3477]The construction portfolio may be included as part of the ship ______.

A.　general plans

B.　operating manual

C.　builders documentation

D.　CCS file

KEY:　B

[3478]The flood stream here is at an average rate of 0．8 ______.

A.　cables

B.　miles

C.　kilometers

D.　knot

KEY:　D

[3479]The inner bottom is the ______.

A.　tank top

B.　compartment between the tank top and shell of the vessel

C.　inner side of the vessel's shell

D.　space between two transverse bottom frames

KEY:　A

[3480]The joint formed when two steel plates are placed end-to-end is called a ______.

A.　bevel

B.　seam

C.　butt

D.　bond

KEY:　C

[3481]The main function of the drawworks on a ship is to ______.

A.　transport tubulars from the pipe racks to the derrick floor

B.　maintain constant tension on the marine riser

C.　store and secure the excess drill line

D.　lower and hoist the drill string into and out of the drilled hole

KEY:　D

[3482]The objective of shoring a damaged bulkhead is to ______.

A.　force the warped，bulged，or deformed sections back into place

B.　support and hold the area in the damaged position

C.　withstand subsequent additional damage

D.　make a watertight seal at the damaged area

KEY:　B

[3483]The pivoting point of a fully loaded vessel with normal trim proceeding ahead at sea speed is ______.

A.　right at the bow

B.　one-third the length of the vessel from the bow

C.　one-half the length of the vessel from the bow

D.　two-thirds the length of the vessel from the bow

KEY:　B

[3484]The safety stopper that prevents the anchor cable from running free if the cable jumps the wildcat is the ______.

A.　riding pawl

B.　devil's claw

C.　buckler plate

D.　spill pipe

KEY:　A

[3485]The shearing stresses on a ship's structure are usually greatest at ______.

A.　the bow

B.　the stern

C.　midships

D.　the ship's quarter-length points

KEY:　D

[3486]The strictest load line regulations apply to ______.

A.　gas carriers

B.　freighters (break-bulk)

C.　passenger ships

D.　tankers

KEY:　C

[3487]The system of valves and cargo lines in the bottom piping network of a tanker that connects one section of cargo tanks to another section is called a ______.

A.　come-along

B.　crossover

C.　manifold

D.　runaround

KEY:　B

[3488]The type of joint formed when a third small plate is riveted over two plates butted together is called a ______.

A.　butted joint

B.　lap joint

C.　strap joint

D.　stringer joint

KEY:　C

[3489]The type of joint formed when an edge of one plate is laid over the edge of the plate to which it is riveted is a ______.

A.　grip joint

B.　strap joint

C.　thread joint

D.　lap joint

KEY:　D

[3490]The welds used to attach stiffeners to a plate are known as ______.

A.　butt welds

B.　seam welds

C.　fillet welds

D.　plate welds

KEY:　C

[3491]The welds used to join shell plates in flush construction are known as ______.

A.　butt welds

B.　seam welds

C.　fillet welds

D.　continuous welds

KEY:　A

[3492]The words PIPE，BUTT，TUN AND PUNCHEON all refer to ______.

A.　the sizes of casks

B.　various lengths of timber

C.　structural shapes used in construction

D.　viscosities of liquid cargoes

KEY:　A

[3493]Those ship's tanks that are particularly important for trimming the ship are the ______.

A.　domestics

B.　settlers

C.　deeps

D.　peaks

KEY:　D

[3494]To reduce the number of strakes at the bow，two strakes are tapered and joined at their ends by a single plate. This plate is known as a ______.

A.　cover plate

B.　joiner

C.　lap strake

D.　stealer plate

KEY:　D

[3495]Upon underwater ______ by divers，there are no apparent ______ on the bottom plates of the said vessel．

A.　detection，defects

B.　defection，defects

C.　defects，defection

D.　defects，detection

KEY:　A

[3496]What best describes the structure from which Stratford Point Light is shown?

A.　Brown conical tower with white horizontal band in center of light on black pier

B.　Red conical tower on brown cylindrical pier

C.　White octagonal house on brown cylindrical pier

D.　White conical tower，with brown band midway of height

KEY:　D

[3497]What is NOT used as a stern tube bearing，on a large vessel?

A.　Wooden (lignum vitae) strips

B.　Hard rubber

C.　Resin bonded composition

D.　Bronze bushings

KEY:　D

[3498]What is the purpose of a striker plate?

A.　Provides surface for applying force on machinery

B.　Provides landing surface for the sounding bob

C.　Absorbs machinery vibration

D.　Prevents valve stem over-travel

KEY:　B

[3499]What is the purpose of cant frames in steel vessels?

A.　To support the overhang of the stern

B.　To provide strength to shell plating at the stern

C.　To add strength to the deck beams which support the weather decks

D.　To support the plating of a cylindrical tank

KEY:　B

[3500]What is used to prevent twisting of a towing bridle?

A.　A bitt

B.　A bulkhead

C.　A V-spring

D.　A fishplate

KEY:　D

11.2 雷达及ARPA的使用

[3851]It may be found that，in certain circumstances，radar beacon emissions can cause ______ with the normal radar display，particularly at close range．

A.　unwanted interference

B.　unexpected figures

C.　abnormal data

D.　unwanted information

KEY:　A

[3852]My radar is not working．I require shore based radar assistance．Is shore based radar assistance ______ ？

A.　used

B.　available

C.　in use

D.　can be used

KEY:　B

[3853]No assumptions ______ on the basis of scanty information，especially scanty radar information．

A.　should not be made

B.　are not to be taken

C.　should be taken

D.　shall be made．

KEY:　D

[3854]Proper use shall be made ______ radar equipment．

A.　with

B.　to

C.　for

D.　of

KEY:　D

[3855]Radar beacons are transmitters designed to produce a ______ on the screens of ship's radar sets．

A.　incorrect image

B.　distinctive image

C.　pictorial image

D.　pictographic image

KEY:　B

[3856]Radar makes it possible and much safer for us to sail ______.

A.　in dense fog

B.　in boisterous weather

C.　in the open sea

D.　in rivers

KEY:　A

[3857]Radar reflectors are required for ______.

A.　all fishing vessels over 39 feet in length

B.　sail-propelled fishing vessels

C.　all fishing vessels of less than 200 GT

D.　wooden hull fishing vessels with a poor radar echo

KEY:　D

[3858]SWEEP INT on radar panel is the abbreviation of ______.

A.　sweep introduction

B.　sweep interest

C.　sweep intention

D.　sweep intensity

KEY:　D

[3859]The 10-cm radar as compared to a 3-cm radar of similar specifications will ______.

A.　be more suitable for river and harbor navigation

B.　provide better range performance on low lying targets during good weather and calm seas

C.　have a wider horizontal beam width

D.　have more sea return during rough sea conditions

KEY:　C

[3860]The 3-cm radar as compared to a 10-cm radar with similar specifications will ______.

A.　give better range performance in rain，hail，etc.

B.　display small targets in a mass of dense sea clutter at a greater range

C.　have less sea return in choppy rough seas

D.　display a more map like presentation for inshore navigation

KEY:　D

[3861]The ARPA may swap targets when automatically tracking if two targets ______.

A.　are tracked on reciprocal bearings

B.　are tracked at the same range

C.　are tracked on the same bearing

D.　pass close together

KEY:　D

[3862]The articulated light is superior to other types of buoys because ______.

A.　the radar reflectors reflect better signals

B.　fog horn signals travel farther to sea

C.　it is equipped with strobe lights

D.　it has a reduced watch circle

KEY:　D

[3863]The closest point of approach (CPA) of a contact on a relative motion radar may be determined ______.

A.　immediately when the contact is noted on radar

B.　only if the radar scope is watched constantly

C.　after the contact has been marked at least twice

D.　by an occasional glance at the radar

KEY:　C

[3864]The common way to obtain your ship's position is ______.

A.　keeping a close watch and lookout

B.　taking a radar range and bearing

C.　observing a radar target and listening to signals

D.　keeping a well clear caution

KEY:　B

[3865]The correct method of switching off a marine radar is to turn power switch to ______ position first，then to ______ position．

A.　off／standby

B.　standby／off

C.　standby／close

D.　close／standby

KEY:　B

[3866]The description Racon beside an illustration on a chart would mean a ______.

A.　radar transponder beacon

B.　radar conspicuous beacon

C.　radar calibration beacon

D.　circular radiobeacon

KEY:　A

[3867]The fitting of an efficient radar reflector is likely to considerably increase the ship's probability of ______.

A.　detection

B.　being detected

C.　the detective

D.　the detected

KEY:　B

[3868]The officer of the watch should use radar when appropriate and at all times in ______ waters．

A.　congesting

B.　congested

C.　congestion

D.　congest．

KEY:　B

[3869]The radar control that reduces weak echoes out to a limited distance from the ship is the ______.

A.　sensitivity time control (sea-clutter control)

B.　receiver gain control

C.　brilliance control

D.　fast time constant (differentiator)

KEY:　A

[3870]The radar control that shortens all echoes on the display and reduces clutter caused by rain or snow is the ______.

A.　sensitivity time control (sea clutter control)

B.　receiver gain control

C.　brilliance control

D.　fast time constant (differentiator)

KEY:　D

[3871]The radar control used to reduce sea return at close ranges is the ______.

A.　gain control

B.　sensitivity time control

C.　fast time constant

D.　pulse length control

KEY:　B

[3872]The radio waves used for radar are very short，only ______ long．

A.　a few centimeters

B.　a few fathoms

C.　a few meters

D.　a few feet

KEY:　A

[3873]The shoreline along Rocky Point should give a good radar return because ______.

A.　the shore is bluff and rocky

B.　of offshore exposed rocks

C.　submerged reefs cause prominent breakers

D.　the lookout tower is marked with radar reflectors

KEY:　A

[3874]To determine if risk of collision exists，a vessel which is fitted with radar must use ______.

A.　radar scanning

B.　radar plotting

C.　compass bearings

D.　All of the above

KEY:　D

[3875]Vessel required to have an Automatic Radar Plotting Aid must have a device to indicate the ______.

A.　speed of the vessel over the ground or through the water

B.　distance to the next port

C.　time of the next navigational satellite pass

D.　None of the above

KEY:　A

[3876]What is TRUE of the history display of a target's past positions on an ARPA?

A.　It provides a graphic display to emphasize which vessel is on a collision course.

B.　In the true presentation，it provides a quick visual check to determine if a vessel has changed course.

C.　The display is one of the primary inputs and must be in use when using the trial maneuver capability.

D.　It provides a graphic display of a target vessel's relative course，speed，and CPA.

KEY:　B

[3877]What is TRUE when operating in fog and other vessels are detected by radar? ______.

A.　You should make an ample change to port for a vessel crossing on the starboard bow.

B.　You should maneuver in ample time if a close-quarters situation is developing.

C.　You should determine the course and speed of all radar contacts at six minute intervals.

D.　Long-range scanning will provide early warning of ALL other vessels within the radar's range.

KEY:　B

[3878]What provides little or no indication that a vessel is dragging anchor?

A.　Increasing radar range to a fixed object ahead

B.　Drift lead with the line leading perpendicular to the centerline

C.　Vibrations felt by placing a hand on the cable

D.　Changing bearings to distant fixed objects abeam

KEY:　B

[3879]What will cause the ARPA to emit a visual alarm，audible alarm，or both?

A.　An acquired target entering into a guard zone

B.　A tracked target lost for one radar scan

C.　A tracked target entering your preset CPA-TCPA limits

D.　A target being initially detected within a guard zone

KEY:　C

[3880]When hitting a solid object such as a ship or an airplane，the radar waves are reflected back ______ they came．

A.　in the way

B.　the way

C.　by the way

D.　through the path

KEY:　D

[3881]When navigating in thick fog with the radar on，you should ______.

A.　station the look-out in the wheelhouse to keep a continuous watch on the radar

B.　secure the sounding of fog signals until a vessel closes within five miles

C.　station a look-out as low down and far forward as possible

D.　keep the radar on the shortest available range for early detection of approaching vessels

KEY:　C

[3882]When using a radar in a unstabilized mode，fixes are determined most easily from ______.

A.　center bearings

B.　tangent bearings

C.　ranges

D.　objects that are close aboard

KEY:　C

[3883]When using an ARPA，what should you consider in order to evaluate the information displayed?

A.　The target vessel's generated course and speed are based solely on radar inputs.

B.　Navigational constraints may require a target vessel to change course.

C.　The trial maneuver feature will automatically determine a course that will clear all targets.

D.　You cannot determine if a small target has been lost due to sea return.

KEY:　B

[3884]When using the ARPA in heavy rain，which action should you take?

A.　Increase the radar gain to pick up weak echoes through the rain.

B.　Increase the STC setting to reduce close-in spurious signals.

C.　Navigate as though the effective range of the radar has been reduced.

D.　Increase the range of the inner and outer guard rings.

KEY:　C

[3885]When using the radar for navigating ______.

A.　the best fix is obtained by using a tangent bearing and a range

B.　and using two radar ranges for a fix，the objects of the ranges should be close to reciprocal bearings

C.　and using ranges，the most rapidly changing range should be measured last

D.　and crossing a radar range of one object with the visual bearing of a second object，the two objects should be 80to 110apart

KEY:　C

[3886]When will the float-free emergency position indicating radiobeacon be activated after abandoning ship?

A.　Immediately after floating free

B.　After about one hour when the salt water activates the battery

C.　Only when keyed by the radar of another vessel

D.　Only when daylight activates the photovoltaic cell

KEY:　A

[3887]When you turn on the fast time constant (differentiator) control of a radar it will ______.

A.　enhance weak target echoes and brighten them on the PPI

B.　reduce clutter over the entire PPI by shortening the echoes

C.　only suppress weak targets to a limited distance from the ship (sea clutter)

D.　reduce the beam width to provide a map-like presentation for navigation

KEY:　B

[3888]Which ARPA data should you use in order to determine if a close quarters situation will develop with a target vessel?

A.　Set and drift of the current

B.　Relative track information

C.　Predicted time of CPA

D.　Initial range of acquisition

KEY:　B

[3889]Which condition indicates that your radar needs maintenance?

A.　Serrated range rings

B.　Indirect echoes

C.　Multiple echoes

D.　Blind sector

KEY:　A

[3890]Which general statement concerning radar is FALSE? __________.

A.　Raising the antenna height increases the radar range.

B.　The ability of radar to detect objects is unaffected by weather conditions.

C.　Radar bearings are less accurate than radar ranges.

D.　Radar should be checked regularly during clear weather to ensure that it is operating properly.

KEY:　B

[3891]Which procedure(s) shall be used to determine risk of collision?

A.　Watching the compass bearing of an approaching vessel

B.　Systematic observation of objects detected by radar

C.　Long-range radar scanning

D.　All of the above

KEY:　D

[3892]Which statement about an estimated position is TRUE?

A.　It is more reliable than a fix based on radar bearings.

B.　It may be based on a single LOP or questionable data.

C.　When a 3-LOP fix plots in a triangle，the center of the triangle is the estimated position.

D.　It is usually based on soundings.

KEY:　B

[3893]Which statement concerning the operation of radar in fog is TRUE?

A.　Radar ranges are less accurate in fog.

B.　Navigation buoys will always show up on radar.

C.　A sandy beach will show up clearer on radar than a rocky cliff.

D.　Small wooden boats may not show up on radar.

KEY:　D

[3894]Which statement is TRUE concerning a vessel equipped with operational radar? ______.

A.　She must use this equipment to obtain early warning of risk of collision.

B.　The use of a radar excuses a vessel from the need of a look-out.

C.　The radar equipment is only required to be used in restricted visibility.

D.　The safe speed of such a vessel will likely be greater than that of vessels without radar.

KEY:　A

[3895]Which statement is true concerning a vessel equipped with operational radar? ______.

A.　The Master of the vessel must be on the bridge when the radar is in use.

B.　The radar equipment is only required to be used in restricted visibility.

C.　The use of a radar excuses a vessel from the need of a look-out.

D.　This equipment must be used to obtain early warning of risk of collision.

KEY:　D

[3896]You are scanning the radar screen for a buoy fitted with racon. How should this signal appear on the PPI display?

A.　Starting with a dash and extending radially outward from the target

B.　As a broken line from center of PPI to the target

C.　Starting with a dot and extending radially inward from the target

D.　Starting with a dash and extending to the right of the target

KEY:　A

[3897]You are underway at 5 knots and see on your radar a contact 10 miles directly astern of you. 12 minutes later，the contact is 8 miles directly astern of you. What is the estimated speed of the contact?

A.　Dead in the water

B.　1 knot

C.　10 knots

D.　15 knots

KEY:　D

[3898]You are underway in restricted visibility and hear a fog signal forward of the beam. Nothing appears on your radar screen. You must ______.

A.　stop your engines

B.　sound two prolonged blasts of the whistle

C.　sound the danger signal

D.　slow to bare steerageway

KEY:　D

[3899]You are underway in thick fog. You have not determined if risk of collision exists. Which statement is TRUE? ______.

A.　Your speed must be reduced to bare steerageway.

B.　A look-out is not required if the radar is on.

C.　Fog signals are only required when a vessel is detected by radar.

D.　The radar should always be kept on a short-range scale.

KEY:　A

[3900]You are using a radar in which your own ship is shown at the center，and the heading flash always points to 0. If bearings are measured in relation to the flash，what type of bearings are produced?

A.　Relative

B.　TRUE

C.　Compass

D.　Magnetic

KEY:　A

11.3 磁罗经与陀螺罗经的使用

[3901]If a magnetic compass is not affected by any magnetic field other than the Earth's，which statement is TRUE?

A.　Compass error and variation are equal.

B.　Compass north will be true north.

C.　Variation will equal deviation.

D.　There will be no compass error.

KEY:　A

[3902]If the compass heading and the magnetic heading are the same then ______.

A.　the deviation has been offset by the variation

B.　there is something wrong with the compass

C.　the compass is being influenced by nearby metals

D.　there is no deviation on that heading

KEY:　D

[3903]If the gyrocompass error is east，what describes the error and the correction to be made to gyrocompass headings to obtain true headings?

A.　The readings are too low (small numerically) and the amount of the error must be added to the compass to obtain true

B.　The readings are too low and the amount of the error must be subtracted from the compass to obtain true

C.　The readings are too high (large numerically) and the amount of the error must be added to the compass to obtain true

D.　The readings are too high and the amount of the error must be subtracted from the compass to obtain true

KEY:　A

[3904]If the magnetic heading is greater than the compass heading，the deviation is ______.

A.　east

B.　west

C.　north

D.　south

KEY:　A

[3905]Indications of the master gyrocompass are sent to remote repeaters by the ______.

A.　follow-up system

B.　transmitter

C.　phantom element

D.　azimuth motor

KEY:　B

[3906]Knowing the compass heading that is 90to the side of a ship will enable the operator of a survival craft to initially steer ______.

A.　into the wind

B.　away in fire and smoke

C.　directly to the standby boat

D.　directly to the nearest land

KEY:　B

[3907]Magnetic compass deviation ______.

A.　varies depending upon the bearing used

B.　is the angular difference between magnetic north and compass north

C.　is published on the compass rose on most nautical charts

D.　is the angular difference between geographic and magnetic meridians

KEY:　B

[3908]Magnetic dip is a measurement of the angle between the ______.

A.　geographic pole and the magnetic pole

B.　lubber's line and true north

C.　horizontal and the magnetic line of force

D.　compass heading and the magnetic heading

KEY:　C

[3909]Magnetic heading differs from compass heading by ______.

A.　compass error

B.　true heading

C.　variation

D.　deviation

KEY:　D

[3910]Magnetic variation changes with a change in ______.

A.　the vessel's heading

B.　sea conditions

C.　seasons

D.　the vessel's position

KEY:　D

[3911]Magnets are placed in horizontal trays in the compass binnacle to compensate for the ______.

A.　induced magnetism in the vessel's horizontal soft iron

B.　change in the magnetic field when the vessel inclines from vertical

C.　permanent magnetism of the vessel

D.　magnetic fields caused by electrical currents in the vicinity

KEY:　C

[3912]Quadrantal error in a gyrocompass has its GREATEST effect ______.

A.　in high latitudes

B.　near the equator

C.　on north or south headings

D.　on intercardinal headings

KEY:　D

[3913]The compass deviation changes as the vessel changes ______.

A.　geographical position

B.　speed

C.　heading

D.　longitude

KEY:　C

[3914]The compass error of a magnetic compass that has no deviation is ______.

A.　zero

B.　equal to variation

C.　eliminated by adjusting the compass

D.　constant at any geographical location

KEY:　B

[3915]The compass heading of a vessel differs from the true heading by ______.

A.　compass error

B.　variation

C.　magnetic dip

D.　deviation

KEY:　A

[3916]The difference between magnetic heading and compass heading is called ______.

A.　variation

B.　deviation

C.　compass error

D.　drift

KEY:　B

[3917]The directive force of a gyrocompass ______.

A.　increases with latitude，being maximum at the geographic poles

B.　decreases with latitude，being maximum at the geographic equator

C.　is greatest when a vessel is near the Earth's magnetic equator

D.　remains the same at all latitudes

KEY:　B

[3918]The error in a magnetic compass caused by the vessel's magnetism is called ______.

A.　variation

B.　deviation

C.　compass error

D.　bearing error

KEY:　B

[3919]The Flinders bar on a magnetic compass compensates for the ______.

A.　induced magnetism in vertical soft iron

B.　induced magnetism in horizontal soft iron

C.　permanent magnetism in ship's steel

D.　vessel's inclination from the vertical

KEY:　A

[3920]The greatest directive force is exerted on the magnetic compass when the ______.

A.　needles are nearly in line with the meridian

B.　vessel is near the magnetic poles

C.　variation is near zero

D.　vessel is near the magnetic equator

KEY:　D

[3921]The gyrocompass error resulting from your vessel's movement in OTHER than an east-west direction is called ______.

A.　damping error

B.　ballistic deflection

C.　quadrantal error

D.　speed error

KEY:　D

[3922]The horizontal angle between the magnetic meridian and the north-south line of the magnetic compass is ______.

A.　deviation

B.　variation

C.　compass error

D.　dip

KEY:　A

[3923]The lubber's line of a magnetic compass ______.

A.　always shows true north direction

B.　indicates the vessel's heading

C.　is always parallel to the vessel's transom

D.　is located on the compass card

KEY:　B

[3924]The lubber's line on a magnetic compass indicates ______.

A.　compass north

B.　the direction of the vessel's head

C.　magnetic north

D.　a relative bearing taken with an azimuth circle

KEY:　B

[3925]The magnetic compass magnets are acted on by the horizontal component of the Earth's total magnetic force. This magnetic force is GREATEST at the ______.

A.　north magnetic pole

B.　south magnetic pole

C.　magnetic prime vertical meridian

D.　magnetic equator

KEY:　D

[3926]The magnetic compass operates on the principle that ______.

A.　like magnetic poles attract

B.　unlike magnetic poles repel

C.　unlike poles attract

D.　the poles of the compass line up with the geographic poles of the earth

KEY:　C

[3927]The MOST important feature of the material used for making the binnacle of a standard magnetic compass is that it is ______.

A.　nonmagnetic

B.　weatherproof

C.　corrosion resistant

D.　capable of being permanently affixed to the vessel

KEY:　A

[3928]The permanent magnetism of a vessel may change in polarity due to ______.

A.　being moored for a long time on one heading

B.　being struck by lighting

C.　steaming from the north magnetic hemisphere to the south magnetic hemisphere

D.　loading a homogenous magnetic cargo such as steel plate，iron bars，etc.

KEY:　B

[3929]The permanent magnetism of a vessel may change in strength due to ______.

A.　a collision with another vessel

B.　being moored on a constant heading for a long period of time

C.　being struck by lightning

D.　All of the above

KEY:　D

[3930]The principal purpose of magnetic compass adjustment is to ______.

A.　reduce the variation as much as possible

B.　reduce the deviation as much as possible

C.　reduce the magnetic dip as much as possible

D.　allow the compass bowl to swing freely on its gimbals

KEY:　B

[3931]The quadrantal spheres are used to ______.

A.　remove deviation on the intercardinal headings

B.　remove deviation on the cardinal compass headings

C.　remove heeling error

D.　compensate for induced magnetism in vertical soft iron

KEY:　A

[3932]The reaction of a gyrocompass to an applied force is known as ______.

A.　precession

B.　earth rate

C.　gyroscopic inertia

D.　gravity effect

KEY:　A

[3933]The standard magnetic compass heading differs from the true heading by ______.

A.　compass error

B.　latitude

C.　variation

D.　deviation

KEY:　A

[3934]The total magnetic effects which cause deviation of a vessel's compass can be broken down into a series of components which are referred to as ______.

A.　divisional parts

B.　coefficients

C.　fractional parts

D.　equations

KEY:　B

[3935]The vertical angle between the horizontal and the magnetic line of force is the ______.

A.　elevation

B.　magnetic angle

C.　vertical angle

D.　dip

KEY:　D

[3936]The vertical component of the Earth's magnetic field causes induced magnetism in vertical soft iron. This changes with latitude. What corrects for this coefficient of the deviation?

A.　The Flinders bar

B.　The heeling magnet

C.　Quadrantal soft iron spheres

D.　Bar magnets in the binnacle

KEY:　A

[3937]To center a compass bowl in its binnacle，you should have the ship on an even keel，heading north or south，and adjust the screws until ______.

A.　the compass heading is in line with the lubber's line

B.　there is no lost motion in the gimbal rings

C.　no change of heading by compass is observed if you raise and lower the heeling magnet

D.　the gimbal rings do not strike the compass frame when they are tilted

KEY:　C

[3938]True heading differs from magnetic heading by ______.

A.　deviation

B.　variation

C.　compass error

D.　northerly error

KEY:　B

[3939]Variation in a compass is caused by ______.

A.　worn gears in the compass housing

B.　magnetism from the earth's magnetic field

C.　magnetism within the vessel

D.　lack of oil in the compass bearings

KEY:　B

[3940]Variation is a compass error that you ______.

A.　can correct by adjusting the compass card

B.　can correct by adjusting the compensating magnets

C.　can correct by changing the vessel's heading

D.　cannot correct

KEY:　D

[3941]Variation is the angular measurement between ______.

A.　compass north and magnetic north

B.　compass north and true north

C.　magnetic meridian and the geographic meridian

D.　your vessel's heading and the magnetic meridian

KEY:　C

[3942]What are the only magnetic compass correctors that correct for both permanent and induced effects of magnetism?

A.　Quadrantal spheres

B.　Heeling magnets

C.　Athwartships magnets

D.　Fore-and-aft magnets

KEY:　B

[3943]What is an advantage of the magnetic compass aboard vessels?

A.　Compass error is negligible at or near the earth's magnetic poles.

B.　It does not have to be checked as often.

C.　It is reliable due to it's essential simplicity.

D.　All points on the compass rose are readily visible.

KEY:　C

[3944]What is the basic principle of the magnetic compass?

A.　Magnetic materials of the same polarity repel each other and those of opposite polarity attract.

B.　The Earth's magnetic lines of force are parallel to the surface of the Earth.

C.　Magnetic meridians connect points of equal magnetic variation.

D.　The compass needle(s) will，when properly compensated，lie parallel to the isogonic lines of the Earth.

KEY:　A

[3945]What is the name of the movable，radial guide line used to measure direction on a radar?

A.　Compass rose

B.　Cursor

C.　Plan position indicator

D.　Variable range marker

KEY:　B

[3946]When a magnetic compass is not in use for a prolonged period of time it should ______.

A.　be shielded from direct sunlight

B.　be locked into a constant heading

C.　have any air bubbles replaced with nitrogen

D.　have the compensating magnets removed

KEY:　A

[3947]When a vessel changes course from one cardinal heading to another cardinal heading while adjusting the compass，which action should be taken?

A.　The course change should be made rapidly to prevent transient induced magnetism while passing the intercardinal headings.

B.　After the new heading is reached，the vessel should steam on that course for at least two minutes before the adjustment.

C.　During the course change，you should gently tap the compass to remove any error caused by friction on the pivot bearing.

D.　After steadying on the new heading，the compass card should be slewed by a magnet and allowed to oscillate freely to remove any gaussin error.

KEY:　B

[3948]When adjusting a magnetic compass for error，a deviation table should be made ______.

A.　before correcting for any deviation

B.　after correcting for variation

C.　after adjusting the fore-and-aft and athwartships permanent magnets

D.　before the quadrantal correctors are placed on the compass

KEY:　C

[3949]When adjusting a magnetic compass using the fore-and-aft permanent magnets，you should ______.

A.　use the magnets one at a time，putting one in one side and then one on the opposite side，one step higher.

B.　use the magnets in pairs，starting at the top，with trays at the highest point of travel

C.　use the magnets in pairs，from the bottom up，with the trays at the lowest point of travel

D.　fill all the trays with magnets，then remove them one-by-one until the deviation is removed

KEY:　C

[3950]When changing from a compass course to a true course you should apply ______.

A.　variation

B.　deviation

C.　variation and deviation

D.　a correction for the direction of current set

KEY:　C

11.4 GPS、DF及LC的使用

[3951]Which statement about satellite navigation is TRUE?

A.　While a fix can be generated by signals received from two satellites，three satellites are necessary for reliable accuracy.

B.　The satellites are in equatorial orbits around the earth about 60of longitude apart.

C.　The satellite navigation system determines a fix by measuring the doppler shift of the radio signals from the satellite.

D.　The ship's receiver cannot begin processing data until the antenna locks onto the satellite and starts continuous tracking.

KEY:　C

[3952]The ship is equipped with {DF}．

A.　discharging facilities

B.　direction finder

C.　log indicator

D.　a kind of radar

KEY:　B

[3953]More exact assessment of visibility when ______ is used to determine the range of vessel or other objects in the vicinity．

A.　radar

B.　omega

C.　satellite navigator

D.　DF．

KEY:　A

[3954]We'll see ______ the compasses are in good condition．

A.　if

B.　how

C.　when

D.　where

KEY:　A

[3955]Vessels in port may use ______ for receiving typhoon warnings during the typhoon season．

A.　their transmitters

B.　their receivers

C.　their radars

D.　their lorans

KEY:　B

[3956]To obtain accuracy in fixing by DF，______ and three stations should be used．

A.　parallel bearings

B.　long-range bearings

C.　short-range bearings

D.　cross bearings

KEY:　D

[3957]To obtain accuracy in fixing by DF cross bearings，______ should be used．

A.　two stations，including one on the nearest coast

B.　two stations，including one on the nearer coast

C.　three stations，including one on the farthest coast

D.　three stations，including one on the nearest coast

KEY:　C

[3958]Calibration Stations give special transmissions for the calibration of ship's ______.

A.　gyro-compass

B.　marine radar

C.　DF

D.　navigational satellite

KEY:　C

[3959]Before switching on gyro-compass，you should make sure that the power supply on board is ______.

A.　high or low

B.　AC or DC

C.　strong or weak

D.　on or off

KEY:　B

[3960]At the master gyrocompass，the compass card is attached to the ______.

A.　spider element

B.　sensitive element

C.　link arm

D.　pickup transformer

KEY:　B

[3961]A compass card without north-seeking capability that is used for relative bearings is a(n) ______.

A.　bearing circle

B.　pelorus

C.　bearing bar

D.　alidade

KEY:　B

[3962]______ is on the panel of DF．

A.　ZERO CLEARING

B.　ANTI-CLUTTER-RAIN

C.　DIMMER

D.　CURSOR

KEY:　A

[3963]______ is on the panel of DF．

A.　RINGS INT

B.　SWEEP INT

C.　REPEATER

D.　GONIOMETER

KEY:　D

[3964]Magnetic compass deviation ______.

A.　varies depending upon the bearing used

B.　is the angular difference between magnetic north and compass north

C.　is published on the compass rose on most nautical charts

D.　is the angular difference between geographic and magnetic meridians

KEY:　B

[3965]You have calibrated your RDF. When compiling the calibration table，the correction to be applied to any future RDF bearings is listed against the ______.

A.　true bearing of the transmitter

B.　relative bearing of the transmitter

C.　heading of the vessel

D.　time of reception

KEY:　B

[3966]As a vessel changes course to starboard，the compass card in a magnetic compass ______.

A.　remains aligned with compass north

B.　also turns to starboard

C.　first turns to starboard then counterclockwise to port

D.　turns counterclockwise to port

KEY:　A

[3967]Quadrantal error in a gyrocompass has its GREATEST effect ______.

A.　in high latitudes

B.　near the equator

C.　on north or south headings

D.　on intercardinal headings

KEY:　D

[3968]______ is on the panel of DF．

A.　RINGS INT

B.　SWEEP INT

C.　REPEATER

D.　GONIOMETER

KEY:　D

[3969]A compass card without north-seeking capability that is used for relative bearings is a(n) ______.

A.　bearing circle

B.　pelorus

C.　bearing bar

D.　alidade

KEY:　B

[3970]A single vertical magnet placed underneath the compass in the binnacle is used to compensate for ______.

A.　the horizontal component of the permanent magnetism

B.　deviation caused by the vessel's inclination from the vertical

C.　induced magnetism in the horizontal soft iron

D.　induced magnetism in the vertical soft iron

KEY:　B

[3971]A system of reservoirs and connecting tubes in a gyro compass is called a ______.

A.　spider element

B.　mercury ballistic

C.　gyrotron

D.　rotor

KEY:　B

[3972]As a vessel changes course to starboard，the compass card in a magnetic compass ______.

A.　remains aligned with compass north

B.　also turns to starboard

C.　first turns to starboard then counterclockwise to port

D.　turns counterclockwise to port

KEY:　A

[3973]Assumption shall ______ on the basis of scanty information especially scanty radar information．

A.　make

B.　be made

C.　not make

D.　not be made．

KEY:　D

[3974]At sea，we can use ______ to fix the ship's position．

A.　VHF

B.　Steering Gear

C.　Rudder Indicator

D.　Loran

KEY:　D

[3975]At the master gyrocompass，the compass card is attached to the ______.

A.　spider element

B.　sensitive element

C.　link arm

D.　pickup transformer

KEY:　B

[3976]Based on a DR，at approximately 1817 you would expect to______.

A.　enter a traffic separation zone

B.　cross a submerged pipeline

C.　depart a regulated area

D.　depart a restricted area

KEY:　C

[3977]Based on your 2209 fix，which would be a warning that you are being set down on Block Island Sound South Entrance Obstruction Lighted BIS Buoy?

A.　Decreasing loran readings on 9960-W

B.　Visual bearings of Montauk Point Lt. changing to the left

C.　Increasing bearings of Southeast Point Light

D.　Decreasing soundings

KEY:　A

[3978]Before a magnetic compass is adjusted certain correctors must be checked to ensure that they are free of permanent magnetism. These correctors are the ______.

A.　fore-and-aft and athwartships magnets

B.　dip needle and heeling magnet

C.　heeling magnet and Flinders bar

D.　Flinders bar and quadrantal spheres

KEY:　D

[3979]Compass deviation is caused by ______.

A.　magnetism from the earth's magnetic field

B.　misalignment of the compass

C.　magnetism within the vessel

D.　a dirty compass housing

KEY:　C

[3980]Compass error is equal to the ______.

A.　deviation minus variation

B.　variation plus compass course

C.　combined variation and deviation

D.　difference between true and magnetic heading

KEY:　C

[3981]Deviation in a compass is caused by the ______.

A.　vessel's geographic position

B.　vessel's heading

C.　earth's magnetic field

D.　influence of the magnetic materials of the vessel

KEY:　D

[3982]Deviation is the angle between the ______.

A.　true meridian and the axis of the compass card

B.　true meridian and the magnetic meridian

C.　magnetic meridian and the axis of the compass card

D.　axis of the compass card and the degaussing meridian

KEY:　C

[3983]Deviation which is maximum on intercardinal compass headings may be removed by the ______.

A.　Flinders bar

B.　transverse magnets

C.　fore-and-aft magnets

D.　soft iron spheres on the sides of the compass

KEY:　D

[3984]Due to GPS roll over of the clock cycle，GPS receivers may give the wrong time and position or may lock up permanently on ______.

A.　21-Aug-99

B.　21-Sep-99

C.　Octoer 31，1999

D.　31-Dec-99

KEY:　A

[3985]Enquiries regarding the Maritime Mobile Service may be made to the Supervisor at either Singapore Radio ______ the International Telephone Exchange as appropriate．

A.　and

B.　nor

C.　with

D.　or

KEY:

[3986]Error may be introduced into a magnetic compass by______.

A.　making a structural change to the vessel

B.　a short circuit near the compass

C.　belt buckles

D.　All of the above

KEY:　D

[3987]Gyrocompass repeaters reproduce the indications of the master gyrocompass. They are ______.

A.　accurate only in the Polar regions

B.　accurate electronic servomechanisms

C.　hand operated

D.　accurate only if the vessel is underway

KEY:　B

[3988]Heeling error is defined as the change of deviation for a heel of ______.

A.　2while the vessel is on an intercardinal heading

B.　1while the vessel is on a compass heading of 000

C.　2and is constant on all headings

D.　1while the vessel is on a compass heading of 180

KEY:　B

[3989]How is variation indicated on a small-scale nautical chart?

A.　Magnetic compass table

B.　Magnetic meridians

C.　Isogonic lines

D.　Variation is not indicated on small-scale nautical charts.

KEY:　C

[3990]If several fixed navigational lights are visible at the same time，each one may be positively identified by checking all of the following EXCEPT what against the Light List?

A.　Rhythm

B.　Period

C.　Intensity

D.　Color

KEY:　C

[3991]Induced magnetism is found in ______.

A.　hard iron

B.　soft iron

C.　vertical iron only

D.　horizontal iron only

KEY:　B

[3992]It is dangerous for vessels without the use of radar ______ the estuary．

A.　to get

B.　to approach

C.　to proceed

D.　to close

KEY:　B

[3993]It may be found that，in certain circumstances，Radar Beacon can cause unwanted interference particularly ______.

A.　at close range

B.　at end on situation

C.　at head on situation

D.　at crossing situation

KEY:　A

[3994]Magnets in the binnacles of magnetic compasses are used to reduce the effect of ______.

A.　deviation

B.　variation

C.　local attraction

D.　All of the above

KEY:　A

[3995]Most GPS receivers use the doppler shift of the carrier phase to compute ______.

A.　Latitude

B.　Longitude

C.　Speed

D.　Time

KEY:　C

[3996]Radar is not only to obtain ______，but also to measure their distance away from you．

A.　brightness of objects

B.　color of targets

C.　shape of targets

D.　bearing of objects

KEY:　D

[3997]Radar makes the most accurate determination of the ______.

A.　direction of a target

B.　distance to a target

C.　size of a target

D.　shape of a target

KEY:　B

[3998]The basic collision avoidance display presented by CASII is designed for easy interpretation and ______ of possible collision threat situations．

A.　full confirmation

B.　precise appreciation

C.　immediate assessment

D.　timely recognition

KEY:　C

[3999]The correction to be applied to a Loran-C reading when matching a sky wave to a ground wave may be found ______.

A.　on loran charts covering areas where sky waves are used

B.　in the Radio Aids to Navigation，PUB 117

C.　in the Loran-C Correction Tables

D.　Sky waves cannot be matched to ground waves in LORAN-C to produce a usable reading.

KEY:　A

[4000]The heading of a vessel is indicated by what part of the compass?

A.　Card

B.　Needle

C.　Lubber's line

D.　Gimbals

KEY:　C

12.2 GMDSS

[4051]______ is the chain？

A.　How

B.　What

C.　Why

D.　How about

KEY:　A

[4052]______ is the course to reach you？

A.　What

B.　How

C.　Which

D.　When

KEY:　A

[4053]A check line is ______.

A.　a safety line attached to a man working over the side

B.　used to measure water depth

C.　used to slow the headway of a barge

D.　used to measure the overhead height of a bridge

KEY:　C

[4054]A Class A EPIRB is NOT ______.

A.　approved by the CCS

B.　designed for float free，automatic activation

C.　operated on 121.5/243 MHz

D.　operated manually

KEY:　D

[4055]A Class C EPIRB ______.

A.　is primarily designed for vessels operating up to 50 miles offshore or on foreign waters

B.　transmits a signal that can be detected by satellite

C.　must be manually activated

D.　operates in the medium and low frequency bands

KEY:　C

[4056]A Class C EPIRB ______.

A.　should be used on board vessels operating not more than 200 miles offshore

B.　transmits an alerting signal on channel 16 followed by a homing signal on channel 15

C.　is automatically activated when it floats free of a sinking vessel

D.　transmits a swept tone signal on the aviation distress frequency

KEY:　B

[4057]A Class S EPIRB is a ______.

A.　submarine EPIRB

B.　safety EPIRB

C.　ship EPIRB

D.　survival craft EPIRB

KEY:　D

[4058]A major advantage of the NAVTEX system when compared to other systems is that ______.

A.　the information can be received on an ordinary FM radio

B.　warnings are printed out for reading when convenient

C.　broadcasts are at scheduled times

D.　a low frequency band is used for long distance transmission

KEY:　B

[4059]A NAVAREA warning carries the following number; 1986/87 (11). Which statement is TRUE?

A.　The warning was issued in 1986，the 87th sequentially numbered warning and broadcast 11 times.

B.　This is warning number 1986 issued in 1987，and it affects sub-region 11.

C.　This warning is valid in 1986 and 1987 and is the eleventh two-year warning.

D.　The subject of this warning first appeared in 1986; this warning is in 1987 and is the eleventh on this topic.

KEY:　B

[4060]A vessel operating outside of coastal waters must carry an automatically activated Emergency Position Indicating Radio Beacon (EPIRB) if she ______.

A.　does not have berthing facilities

B.　has berthing and galley facilities

C.　is a workboat and her mother ship carries an EPIRB

D.　None of the above are correct.

KEY:　B

[4061]All self-propelled vessels on an international voyage must be equipped with how many Emergency Position Indicating Radiobeacons (EPIRB)?

A.　One approved Class A EPIRB

B.　Two approved Class A EPIRBs

C.　One approved Class B EPIRBs

D.　Two approved Class B EPIRBs

KEY:　A

[4062]At sea，all required GMDSS equipment (other than survival craft equipment) must be proven operational by ______.

A.　daily testing

B.　either A or C

C.　operational use of the equipment

D.　testing at least every 48 hours

KEY:　B

[4063]CATEGORY A EPIRB's transmit on frequencies that are monitored by ______.

A.　orbiting satellites in space

B.　commercial radio stations

C.　private，commercial，and military aircraft

D.　ordinary radio stations

KEY:　A

[4064]Class A EPIRBs are required on ______.

A.　self-propelled，inspected vessels

B.　towing vessels

C.　recreational vessels

D.　uninspected passenger vessels

KEY:　A

[4065]Class C EPIRBs are ______.

A.　manually activated and operate on maritime (VHF) channels

B.　automatically activated and operate on maritime (VHF) channels

C.　manually operated and detected by satellite

D.　automatically operated and detected by satellite

KEY:　B

[4066]During an annual FCC inspection ______.

A.　all required documents and publications may have to be produced

B.　licensed GMDSS radio operators may be required to demonstrate equipment competencies

C.　all required equipment must be fully operational

D.　All of the above

KEY:　D

[4067]Each Class A EPIRB shall be stowed in a manner which will permit ______.

A.　easy access to its storage compartment

B.　replacement of the battery without exposure to the weather

C.　it to float free if the vessel sinks

D.　it to remain attached to the vessel

KEY:　C

[4068]Each EPIRB required on a ship shall be tested using the integrated test circuit and output indicator every ______.

A.　week

B.　two weeks

C.　month

D.　two months

KEY:　C

[4069]For GMDSS，when may a compulsory vessel not be allowed to leave port?

A.　When the vessel is in an overloaded condition

B.　When the vessel has arranged for both duplication of equipment AND shore-based maintenance

C.　When the vessel has replaced a required piece of GMDSS-related equipment but its performance has not been verified or logged

D.　When the vessel is carrying only two licensed GMDSS Radio Operators and is capable of performing all required functions

KEY:　C

[4070]For GMDSS，which statement concerning a compulsory vessel is FALSE?

A.　A conditional or partial exemption may be granted，in exceptional circumstances，for a single voyage outside the sea area for which the vessel is equipped.

B.　Once a compulsory vessel's GMDSS station has been fitted and inspected，the station must be inspected only once every five years.

C.　All passenger vessels and all cargo vessels that are 300 Gross Tons or larger must comply.

D.　Compulsory vessels must carry at least two licensed GMDSS Radio Operators.

KEY:　B

[4071]How long must GMDSS Radio Logs be retained by the licensee when they relate to a distress situation or disaster?

A.　6 months

B.　1 year

C.　2 years

D.　3 years

KEY:　D

[4072]How long must the GMDSS radio log be retained on board?

A.　At least two years after the last entry

B.　At least one year after the last entry

C.　At least 90 days after the last entry

D.　At least 30 days after the last entry

KEY:　D

[4073]I have ______ you on my radar．

A.　seen

B.　sighted

C.　watched

D.　located

KEY:　D

[4074]I have altered course to 245 degrees for ______.

A.　identify

B.　identifier

C.　identity

D.　identification

KEY:　D

[4075]I have no choice ______ to put such remarks．

A.　or

B.　and

C.　but

D.　unless

KEY:　C

[4076]If a GMDSS radio operator initiates a DSC distress transmission but does not insert a message，what happens?

A.　The transmission is aborted and an alarm sounds to indicate this data must be provided by the operator.

B.　The transmission is not initiated and "ERROR" is indicated on the display readout.

C.　The transmission will be made with "default" information provided automatically.

D.　The receiving station will poll the DSC unit of the vessel in distress to download the necessary information

KEY:　C

[4077]If your vessel is equipped with a radiotelephone，what must also be aboard?

A.　Certificate of Inspection

B.　List of ship stations

C.　Copy of ship to shore channels

D.　Radio station license

KEY:　D

[4078]On which vessels is GMDSS required?

A.　All vessels capable of international voyages

B.　Vessels operating outside of the range of VHF coastal radio stations

C.　SOLAS Convention ships of 300 gross tonnage or more.

D.　Coastal vessels of less than 300 gross tons

KEY:　C

[4079]One method of reducing the length of radio transmissions without distorting the meaning of your words is by using ______.

A.　slang

B.　secret codes

C.　procedure words

D.　analogies

KEY:　C

[4080]Our ship ______ with many modern navigational instruments．

A.　equipped

B.　is equipped

C.　equips

D.　has equipped

KEY:　B

[4081]Proper GMDSS watchkeeping includes ______.

A.　monitoring all required frequencies in the proper mode

B.　reading all displays and/or printouts after silencing an alarm

C.　notifying the Master of any distress alerts

D.　All of the above

KEY:　D

[4082]Proper GMDSS watchkeeping includes ______.

A.　understanding the GMDSS console's normal operational indicators

B.　maintaining a proper GMDSS radio station log

C.　responding to and comprehending alarms

D.　All of the above

KEY:　D

[4083]Survival craft EPIRBs (Class S) are ______.

A.　permanently mounted in the survival craft

B.　portable and stowed for carriage to the survival craft

C.　permanently mounted on the bridge wings

D.　portable and stowed in the emergency gear locker

KEY:　B

[4084]The Class A EPIRB transmits a signal ______.

A.　that follows the curvature of the earth

B.　that can be picked up by SARSAT satellite

C.　that activates an alarm on board nearby aircraft

D.　to alert shore stations and then transmits a homing signal

KEY:　B

[4085]The Consol navigation system，used in Russian and Northern European waters，can be used ______.

A.　for precise navigation in coastal waters

B.　by measuring the phase difference of the dots and dashes

C.　as an aid to ocean navigation

D.　if the vessel is fitted with a special Consol receiver

KEY:　C

[4086]The difference between a class A and a class B EPIRB is that the class A EPIRB ______.

A.　operates on both military and civilian aircraft distress frequencies

B.　transmits both an alerting signal and a homing signal

C.　must be the float free type and automatically activated

D.　operates with a greater output of signal power

KEY:　C

[4087]The Emergency Position Indicating Radiobeacon on a cargo vessel must be stowed ______.

A.　in an inside passageway

B.　in an approved bracket

C.　so that it is accessible from the bridge of the vessel

D.　so that it will float free if the vessel sinks

KEY:　D

[4088]The maritime radio system consisting of a series of coast stations transmitting coastal warnings is called ______.

A.　NAVTEX

B.　HYDROLANT/HYDROPAC

C.　NAVAREA

D.　SAFESEA

KEY:　A

[4089]The Master shall insure that the Emergency Position Indicating Radiobeacon (EPIRB) is ______.

A.　secured inside the wheelhouse

B.　tested annually

C.　tested monthly

D.　secured in the emergency locker

KEY:　C

[4090]The principal advantage of NAVTEX radio warnings is that ______.

A.　they can be used by mariners who do not know Morse code

B.　only an ordinary FM radio is necessary to receive these warnings

C.　information on a given topic is only broadcast at specified times

D.　they cover a broad spectrum of the radio band allowing reception on almost any type of receiver

KEY:　A

[4091]Under the International Code of Signals how are geographical locations such as New York City transmitted?

A.　A commonly used abbreviation such as NYC is used.

B.　The name is spelled out.

C.　The geographical coordinates are used.

D.　The radio station call sign for the nearest marine radio station is used.

KEY:　B

[4092]VHF Radio is one of the very important navigational instruments on board，______ messages can be sent．

A.　by which

B.　through which

C.　by that

D.　through that

KEY:　B

[4093]What are the conditions，under GMDSS，whereby a ship is NOT allowed to depart from any port?

A.　The vessel is not capable of performing all required distress and safety functions.

B.　The vessel is carrying more than the required number of qualified GMDSS radio operators.

C.　The vessel has a temporary waiver of its radio license and Safety Certificate.

D.　The vessel is not carrying a GMDSS radio maintainer，but has provided for shoreside maintenance plus duplication of equipment if required.

KEY:　A

[4094]What are the vessel equipment and personnel requirements for GMDSS?

A.　Two licensed GMDSS radio operators

B.　Equipment carriage requirements

C.　Distress alerting and response

D.　All of the above

KEY:　D

[4095]What does EPIRB stand for?

A.　Emergency Position Indicating Radar Buoy

B.　Electronic Pulse Indicating Radiobeacon

C.　Emergency Position Indicating Radiobeacon

D.　None of the above

KEY:　C

[4096]What does the DSC control unit do if the GMDSS radio operator fails to insert updated information when initiating a DSC distress alert?

A.　It will abort the transmission and set off an audible alarm that must be manually reset.

B.　It will initiate the DSC distress alert but，as no information will be transmitted，rescue personnel will not be able to identify the vessel，its position or its situation.

C.　It will initiate the DSC distress alert and default information will automatically be transmitted.

D.　It will initiate the DSC distress alert，but any station receiving it will have to establish contact with the distressed vessel to determine its identity，position and situation.

KEY:　C

[4097]What is a functional or carriage requirement for compulsory vessels?

A.　A compulsory vessel must carry at least two licensed GMDSS Radio Operators.

B.　A compulsory vessel must satisfy certain equipment carriage requirements that are determined by where the vessel sails.

C.　A compulsory vessel must be able to transmit and respond to distress alerts.

D.　All of the above

KEY:　D

[4098]What is a GMDSS functional or carriage requirement for compulsory vessels?

A.　A compulsory vessel must carry at least two (2) licensed GMDSS operators

B.　A compulsory vessel must satisfy certain equipment carriage requirements that are determined by where the vessel sails.

C.　A compulsory vessel must be able to transmit and respond to distress alerts.

D.　All of the above

KEY:　D

[4099]What is the action that a GMDSS Radio Operator should take when a DSC distress alert is received?

A.　No action is necessary，as the DSC control unit will automatically switch to the NBDP follow-on communications frequency.

B.　The operator should immediately set continuous watch on the radiotelephone frequency that is associated with the frequency band on which the distress alert was received.

C.　The Operator should immediately set continuous watch on VHF channel 70.

D.　The Operator should immediately set continuous watch on the NBDP frequency that is associated with the frequency band on which the distress alert was received.

KEY:　B

[4100]What is the basic concept of GMDSS?

A.　Search and rescue authorities ashore can be alerted to a distress situation.

B.　Shipping in the immediate vicinity of a ship in distress will be rapidly alerted.

C.　Shoreside authorities and vessels can assist in a coordinated SAR operation with minimum delay.

D.　All of the above

KEY:　D

12.3 VHF通信

[4151]Our company is a professional one，and you may rely ______ it．

A.　on

B.　in

C.　at

D.　up

KEY:　A

[4152]When the changing of a VHF channel / frequency is accepted，say ______.

A.　I am going to change to VHF channel.

B.　I am certainly going to change to VHF channel.

C.　Changing to VHF channel.

D.　VHF channel. has been changed

KEY:　C

[4153]When it is accepted to remain on the frequency indicated，you should say ______.

A.　Standing by on VHF frequency.

B.　Coming to VHF frequency.

C.　VHF frequency. is the best place for you to stay by

D.　remaining in frequency. and do not change

KEY:　A

[4154]There are salvage operations ______ position 120．0N 12．5E．

A.　on

B.　at

C.　in

D.　above

KEY:　C

[4155]International distress signal in VHF calling for help is ______.

A.　Help，Help，Help

B.　Mayday，Mayday，Mayday

C.　Save，Save，Save

D.　Rescue，Rescue，Rescue

KEY:　B

[4156]______ tankers over 100 meters long must not proceed beyond Dongbei Buoy．

A.　Loading

B.　Load

C.　Lade

D.　Laden

KEY:　D

[4157]______ the captain said is reasonable．

A.　That

B.　It

C.　Which

D.　What

KEY:　D

[4158]______ the jumbo everything is ready for loading．

A.　Except

B.　Excepting

C.　Except for

D.　Except that

KEY:　A

[4159]______ time will the pilot be available？

A.　What

B.　When

C.　At what

D.　At when

KEY:　A

[4160]______ Two degree on starboard.

A.　What rudder？

B.　How heading？

C.　Are you on your course？

D.　How answer？

KEY:　A

[4161]______ will be broadcast every one hour on VHF Channel 6．

A.　VHF News

B.　Channel Rules

C.　Visibility Time

D.　Navigational Warnings

KEY:　D

[4162]______ will take three days to load the cargo．

A.　That

B.　This

C.　It

D.　What

KEY:　C

[4163]______ Yokohama，We'll also call at some other Japanese ports．

A.　Beside

B.　Besides

C.　Except

D.　Excepting

KEY:　B

[4164]______ your latest tropical storm warning information？

A.　What is

B.　How is

C.　What are

D.　How are

KEY:　A

[4165]______ your present speed？

A.　How is

B.　How are

C.　What is

D.　What are

KEY:　C

[4166]______，the cargo plan is not well done．

A.　Frank speaked

B.　Frankly to speak

C.　Frankly spoken

D.　Frankly speaking

KEY:　D

[4167]______，you can't finish discharging tomorrow！

A.　What pity

B.　What a pity

C.　What the pity

D.　What pities

KEY:　B

[4168]{Heave up} both anchors．

A.　drag

B.　dredge

C.　weigh

D.　lie

KEY:　C

[4169]A call between any two ship stations on an intership working frequency shall have a maximum duration of ______.

A.　2 minutes

B.　3 minutes

C.　4 minutes

D.　5 minutes

KEY:　B

[4170]A mark or place at which a vessel comes under obligatory entry，transit，or escort procedure is know as ______.

A.　Way point

B.　Receiving point

C.　Reporting point

D.　Closest Point

KEY:　B

[4171]All VHF marine band radios operate in the simplex mode，which means that ______.

A.　only one person may talk at a time

B.　only two persons may talk at the same time

C.　the radio only transmits

D.　the radio only receives

KEY:　A

[4172]Channel 13 (156.65 MHz)，the designated bridge-to-bridge channel，may NOT be used to ______.

A.　exchange navigational information between vessels

B.　exchange navigational information between a vessel and a shore station

C.　conduct necessary tests

D.　exchange operating schedules with company dispatcher

KEY:　D

[4173]How ______ the cargo at loading port？

A.　about

B.　in

C.　with

D.　from

KEY:　A

[4174]How should the letter O be pronounced when spoken on the radiotelephone?

A.　OCK TOW BER

B.　O RI AN

C.　OSS CAH

D.　OA KAM

KEY:　C

[4175]I am ready ______ tow line．

A.　cast

B.　to cast of

C.　cast off

D.　to cast off

KEY:　D

[4176]I have been instructed that on my voyage ______，I shall call at Maderra to pick up some cargo．

A.　home

B.　family

C.　China

D.　returning

KEY:　A

[4177]I’d like to know something about the cargo condition at the loading port ______ detail．

A.　at

B.　by

C.　in

D.　on

KEY:　C

[4178]If it ______ fine tomorrow，we shall start unloading．

A.　to be

B.　is

C.　will be

D.　has been

KEY:　B

[4179]If you know that the vessel you are about to call on the VHF radio maintains a radio watch on both the working and the calling frequencies，which frequency should you call on?

A.　Calling frequency

B.　Distress frequency

C.　Urgency frequency

D.　Working frequency

KEY:　D

[4180]My ship ______ that port next month．

A.　calls at

B.　has called at

C.　call at

D.　will call at

KEY:　D

[4181]One hour before arrival at Elbe 1 light vessel，masters are advised to contact the pilot boat by ______.

A.　telephone

B.　telegraph

C.　telex

D.　VHF

KEY:　D

[4182]Pilot Station，this is Chinese ship Jinsha ______.Over.

A.　calls

B.　called

C.　calling

D.　call

KEY:　C

[4183]Please ______ all the lines fore and aft，the vessel is just in her position of the berth now．

A.　pay out

B.　make fast

C.　shorten

D.　hold on

KEY:　B

[4184]PUBLIC ADDRESS SYSTEM: LOUDSPEAKERS IN THE VESSEL'S CABINS，MESS ROOMS，ETC.，AND ON DECK VIA WHICH IMPORTANT INFORMATION CAN BE BROADCAST FROM A CENTRAL POINT，MOSTLY FROM THE NAVIGATION BRIDGE defines ______.

A.　PP-system

B.　PA-system

C.　AP-system

D.　AA-system

KEY:　B

[4185]Should that kind of weather ______，the two ships would touch each other．

A.　persist

B.　persists

C.　to persist

D.　persisted

KEY:　A

[4186]Some goods are missing ______ terrible weather．

A.　due

B.　because of

C.　because

D.　since

KEY:　B

[4187]The expression STAND-BY ENGINE means： ______.

A.　to get the engine ready

B.　to respond to the engine

C.　to maneuver the engine

D.　to change the engine

KEY:　A

[4188]The generators on your rig have shut down，leaving you without navigation lights. Which emergency signal would you transmit over the VHF radio to alert vessels in the area of your predicament?

A.　Mayday，Mayday，Mayday

B.　Pan，Pan，Pan

C.　Security，Security，Security

D.　Lights out，Lights out，Lights out

KEY:　C

[4189]The height of a VHF radio antenna is more important than the power output wattage of the radio because ______.

A.　VHF communications are basically line of sight

B.　the air is more dense the higher you go

C.　salt water is a poor conductor of sound

D.　sea water absorbs the radiated energy

KEY:　A

[4190]The present voyage of my ship began at the port of Shanghai after loading 8 000 tons of Cargo on board and ended after discharging the Cargo at the port of Hong Kong，which was ______.

A.　the port of destination．

B.　the next port of call．

C.　the port of arrival

D.　the last port of call

KEY:　A

[4191]The space in Hatch No．4 tweendeck is too ______.

A.　near

B.　short

C.　close

D.　small

KEY:　D

[4192]The weather is often ______ in a high-pressure area．

A.　fine

B.　cloudy

C.　rainy

D.　windy

KEY:　A

[4193]There is a ______ mine in position 50N，30W．

A.　drifting

B.　drifted

C.　drift

D.　draft

KEY:　A

[4194]There is a drifting mine ______ in position 21 degrees 31 minutes North 124 degrees 20 minutes East．

A.　report

B.　reports

C.　reported

D.　reporting

KEY:　C

[4195]There is a VHF set ______ the front of the wheelhouse．

A.　at

B.　on

C.　of

D.　in

KEY:　D

[4196]These vessels are all {sailing} for Shanghai．

A.　arriving

B.　bound

C.　loading

D.　discharging

KEY:　B

[4197]Vessel on fire is ______ position of 123.32′N，10.13′E．

A.　on

B.　in

C.　at

D.　of

KEY:　B

[4198]Vessels wishing to communicate with the Port Health Officer should do ______ on VHF Channel 14．

A.　so

B.　that

C.　this

D.　same

KEY:　A

[4199]What does the abbreviation VHF stand for？ ______.

A.　Vessel's Hoisting Flag．

B.　Very High Safety．

C.　Vessel's Homing Frequency．

D.　Very High Frequency

KEY:　D

[4200]What is meant by veering the anchor chain?

A.　Bringing the anchor to short stay

B.　Heaving in all the chain

C.　Locking the windlass to prevent more chain from running out

D.　Paying out more chain

KEY:　D

13.2 甲板与船体保养

[4401]______ about 20 sq．m to be sand-blasted．

A.　The rusting area

B.　The rusted area

C.　The corroded area

D.　The corrosive area

KEY:　B

[4402]A "stopper" is ______.

A.　a short length of line used for temporarily holding another line

B.　a snatch block for handling a topping lift

C.　an engine order telegraph

D.　the brake on a cargo winch

KEY:　A

[4403]A bollard is found on the ______.

A.　beach

B.　deck

C.　pier

D.　towed vessel

KEY:　C

[4404]A bridle for an ocean tow consists of ______.

A.　two chains of equal length

B.　a single nylon pendant rove through a heavy ring free to move on the pendant

C.　two long legs of wire rope shackled to a fishplate

D.　a single length of heavy chain with both ends secured on deck to welded pad eyes

KEY:　A

[4405]A device used to tighten up remaining slack in wire rope when you are making up to a tow in inland waters is a ______.

A.　tripping line

B.　tripping bracket

C.　norman pin

D.　steamboat ratchet

KEY:　D

[4406]A partial deck in a hold is called a(n) ______.

A.　weather deck

B.　orlop deck

C.　shelter deck

D.　main deck

KEY:　B

[4407]A pumproom is suspected of accumulating gases after a ventilation machinery breakdown. Where should the combustible gas indicator case be placed when testing the pumproom atmosphere for combustible gases? ______

A.　In the lower level of the pumproom

B.　In the middle level of the pumproom

C.　In the upper level of the pumproom

D.　On the deck outside the pumproom

KEY:　D

[4408]A shroud is ______.

A.　any wire running fore-and-aft on a ship

B.　any wire with a fiber core

C.　a heavy wire extending from the trunk of the mast to the head of the boom

D.　a heavy wire extending from the mast，atwartships to the deck，to support the mast

KEY:　D

[4409]An inland oil barge must have equipment available to remove an on-deck oil spill of at least ______.

A.　one barrel

B.　two barrels

C.　five barrels

D.　ten barrels

KEY:　A

[4410]An upright vessel has negative GM. GM becomes positive at the angle of loll because the ______.

A.　free surface effects are reduced due to pocketing

B.　KG is reduced as the vessel seeks the angle of loll

C.　effective beam is increased causing BM to increase

D.　underwater volume of the hull is increased

KEY:　C

[4411]Between the side frames on a vessel，support for the deck beams is provided by ______.

A.　stanchions

B.　brackets

C.　web frames

D.　deck stringers

KEY:　A

[4412]Bulkheads which form part of the tanks on a vessel are stiffened to withstand ______.

A.　deck loads from above

B.　dynamic forces while afloat

C.　hydrostatic pressure

D.　over-pressurization

KEY:　C

[4413]By definition，a spar deck is the ______.

A.　lower most continuous deck not broken by water tight bulkheads

B.　after most weather deck above the main strength deck

C.　upper or weather deck above the main strength deck

D.　deck of light construction below the main or strength deck

KEY:　C

[4414]In plugging submerged holes on a vessel，rags，wedges，and other materials should be used in conjunction with plugs to ______.

A.　reduce the water pressure on the hull

B.　reduce the water leaking around the plugs

C.　prevent progressive flooding

D.　reduce the possibility of stress fractures

KEY:　B

[4415]Laying out a line in successive circles flat on deck with the bitter end in the center is known as ______.

A.　coiling

B.　faking

C.　flemishing

D.　lining

KEY:　C

[4416]Lines or gear NOT in use should be ______.

A.　conspicuously marked

B.　stowed anywhere

C.　left on deck

D.　secured or stowed out of the way

KEY:　D

[4417]Mechanical gearing of deck machinery such as the windlass or boat hoists should ______.

A.　be open to view so，if a foreign object gets in the gearing，the operator can immediately stop the machinery

B.　have a guard over the gearing

C.　be painted a contrasting color from the base color in order to call attention to the gearing

D.　not be operated if there is any crew within 10 feet of the machinery

KEY:　B

[4418]On cargo booms，preventers are ______.

A.　auxiliary guys

B.　extra fair leads

C.　steel bands

D.　stops

KEY:　A

[4419]On navigational aids，what does the light characteristic Fl(2+1) mean?

A.　A flashing light combined with a fixed light of greater brightness

B.　Light flashes combined in groups，with a different number of flashes in each group

C.　A light showing groups of two or more flashes at regular intervals

D.　A fixed light varied at regular intervals by groups of two or more flashes of greater brightness

KEY:　B

[4420]On what type of pump would you find an impeller?

A.　Centrifugal

B.　Gear

C.　Piston

D.　Vane

KEY:　A

[4421]On which type davit does the davit head stay at the same height?

A.　Radial

B.　Sheath-screw

C.　Quadrantal

D.　Gravity

KEY:　A

[4422]Our jumbo boom can ______ 20 tons．

A.　lift

B.　rise

C.　elevate

D.　hoist

KEY:　A

[4423]Our ship is more than 150 meters in ______.

A.　height

B.　breadth

C.　depth

D.　length

KEY:　D

[4424]Our starboard derrick boom of hatch No．4 is badly cracked ______ that it has become entirely useless．

A.　at its goose neck to such a extent

B.　at it's goose neck to such an extent

C.　at it's goose neck for such a extent

D.　at its goose neck to such an extent

KEY:　D

[4425]Pinching of the cargo hose between the vessel and the dock should be prevented by ______.

A.　adjusting the hose supports

B.　laying out an excess length of hose on deck

C.　tying off the topping lifts and runners to winch heads

D.　All of the above

KEY:　A

[4426]PLACE ON DECK，IN MESS ROOMS，ETC.，ASSIGNED TO CREW AND PASSENGERS WHERE THEY HAVE TO MEET ACCORDING TO THE MUSTER LIST WHEN THE CORRESPONDING ALARM IS RELEASED OR ANNOUNCEMENT MADE means ______.

A.　Deck area

B.　Dining room on board

C.　Assembly station

D.　Hold

KEY:　C

[4427]Tell the stevedores not to touch the goods ______ the upper tweendeck of Hatch No．2．

A.　on

B.　in

C.　at

D.　into

KEY:　B

[4428]The angle of maximum righting arm corresponds approximatelyto the angle of ______.

A.　deck edge immersion

B.　the load line

C.　downflooding

D.　loll

KEY:　A

[4429]The carrier is entitled to carry the goods on deck only if such carriage is in accordance ______ an agreement with the shipper or with the usage of the particular trade or is required by statutory rules or regulations．

A.　to

B.　in

C.　on

D.　with

KEY:　D

[4430]The crane manufacturer's operating tables are posted near the ______.

A.　crane pedestal

B.　wire-rope locker

C.　main deck

D.　crane controls

KEY:　D

[4431]The holding power of an anchor increases when the ______.

A.　amount of chain lying along the bottom increases

B.　length of the catenary is reduced

C.　mooring line tension is increased

D.　amount of chain lying along the bottom decreases

KEY:　A

[4432]The internal volume of a cargo hold measured from the inside faces of the cargo battens，the lower side of the deck beams，and the top of the tank top ceiling is known as the ______.

A.　gross tonnage

B.　deadweight space

C.　bale cubic

D.　stowage area

KEY:　C

[4433]The International Load Line Certificate is a document indicating the ______ of the various load lines of the vessel．

A.　decks

B.　cabins

C.　holds

D.　positions

KEY:　D

[4434]The lifeboats on your vessel are stowed on cradles on deck and are handled by sheath-screw boom davits. Which of the following statements about launching a boat is TRUE?

A.　The boat should be hoisted a few inches clear of the cradle before cranking out the davits.

B.　The inboard gripes should be cast off before the outboard gripes.

C.　The outboard section of the cradle must be released.

D.　The tricing pendants will automatically bring the boat alongside at the embarkation deck.

KEY:　C

[4435]The main advantage and chief characteristic of a Steulchen boom is that it can be ______.

A.　operated by one winchman

B.　cradled on deck

C.　swung from one hatch to the adjacent hatch

D.　collared to the mast

KEY:　C

[4436]The margin plate is the ______.

A.　outboard strake of plating on each side of an innerbottom

B.　outer strake of plating on each side of the main deck of a vessel

C.　plate which sits atop the center vertical keel

D.　uppermost continuous strake of plating on the shell of a vessel

KEY:　A

[4437]The minimum length of a boat painter for a lifeboat in ocean service is ______.

A.　60 fathoms

B.　the distance from the main deck to the light waterline

C.　twice the distance from the main deck to the light waterline or 50 feet whichever is greater

D.　two times the distance from the boat deck to the light waterline or 50 feet whichever is greater

KEY:　D

[4438]The process of lowering a boom to a horizontal position and onto its deck support is called ______.

A.　spotting a boom

B.　collaring a boom

C.　cradling a boom

D.　toppling a boom

KEY:　C

[4439]The ship is rolling and pitching violently，and shipping a large quantity of sea water on deck {at times}．

A.　from time to time

B.　at this time

C.　at one time

D.　infrequently

KEY:　A

[4440]The stevedores have already stowed the porcelain ware ______ up to the deckhead．

A.　loosely

B.　closely

C.　heavily

D.　strongly

KEY:　B

[4441]The terms ceiling and margin plate are associated with the ______.

A.　crew's quarters

B.　engine room

C.　main deck

D.　tank top

KEY:　D

[4442]The tweendecker has ______ below the main deck．

A.　another deck

B.　two other decks

C.　no deck

D.　additional deck

KEY:　A

[4443]The usual method of arranging a line on deck so that it will run out easily without kinking or fouling is ______.

A.　coiling the line

B.　faking down the line

C.　flemishing the line

D.　racking the line

KEY:　B

[4444]To warp a vessel means to ______.

A.　anchor the vessel

B.　bring the head into the wind

C.　clean the decks

D.　move the vessel by hauling on lines

KEY:　D

[4445]What ______ to him is whether the job of a deck officer allows him to pursue his studies．

A.　matters

B.　happens

C.　refers

D.　applies

KEY:　A

[4446]What danger is presented if a waterspout passes over a ship?

A.　Heavy rains could cause downflooding.

B.　Visibility could be restricted by low clouds.

C.　Large waves could cause structural damage.

D.　Personnel may be injured by loose deck gear blown by the wind.

KEY:　D

[4447]What is meant by the term TOPPING THE BOOM?

A.　Lowering the boom

B.　Raising the boom

C.　Spotting the boom over the deck

D.　Swinging the boom athwartships

KEY:　B

[4448]What is TRUE about hoisting operations?

A.　Personnel may work beneath suspended loads，as long as they are alert and wear hard hats.

B.　If a suspended load with no tag begins to spin，personnel should attempt to stop the spinning if the load is within reach.

C.　If tag lines are used to control a suspended load，they should be secured to the deck.

D.　Personnel not involved in the hoisting operation should be kept clear of the transfer area.

KEY:　D

[4449]When a line is spirally coiled about its end and lying flat on deck，it is said to be ______.

A.　coiled

B.　faked

C.　flemished

D.　seized

KEY:　C

[4450]When hoisting a boat on gravity type davits using an electric motor driven winch，the davit arms should be brought up ______.

A.　to their final position with the winch operating at slow speed

B.　to the bar stop，and then hand cranked to their final position

C.　until just before they make contact with the limit switch，and then hand cranked to their final position

D.　to the embarkation deck，and then hand cranked to their final position

KEY:　C

10.3 船舶设备

[3501]Limber is a term associated with ______.

A.　emergency gear

B.　drainage

C.　deck cargo storage

D.　securing gear

KEY:　B

[3502]A bilge suction line，in a fishing vessel with more than 16 individuals aboard，must have a strainer with an open area not less than how many times the open area of the suction line?

A.　one

B.　two

C.　three

D.　four

KEY:　C

[3503]A block and tackle is rove to advantage. This means that the ______.

A.　blocks have been overhauled

B.　hauling parts of two tackles are attached

C.　hauling part leads through the movable block

D.　hauling part leads through the standing block

KEY:　C

[3504]A bluff bar is a bar ______.

A.　extending out from a bluff alongside the river

B.　that tends to give a false indication of its position

C.　that has a sharp drop off into deep water

D.　that is perpendicular to the current

KEY:　C

[3505]A boom vang ______.

A.　holds the boom down and flattens the main sail

B.　draws the head of the sail to windward

C.　tautens the standing rigging

D.　douses the gaff topsail

KEY:　A

[3506]A carling is used aboard ship ______.

A.　as a connecting strap between the butted ends of plating

B.　to stiffen areas under points of great stress between beams

C.　to prevent the anchor from fouling when the brake is released

D.　to provide an extra heavy fitting in a heavy lift cargo rig

KEY:　B

[3507]A chain stripper is used to ______.

A.　prevent chain from clinging to the wildcat

B.　clean the marine debris from the chain

C.　flake chain from a boat's chain locker

D.　clean chain prior to an x-ray inspection

KEY:　A

[3508]A common means of connecting shots of anchor chain in the field is to use a ______.

A.　sprocket

B.　Kenter link

C.　swivel

D.　end shackle

KEY:　B

[3509]A contra-guide is a type of ______.

A.　bow thruster

B.　cargo gear

C.　steering engine

D.　rudder

KEY:　D

[3510]A deck fitting，used to secure line or wire rope，consisting of a single body with two protruding horns is called a ______.

A.　bitt

B.　bollard

C.　capstan

D.　cleat

KEY:　D

[3511]A design modification of an anchor chain which prevents kinking is the ______.

A.　detachable link

B.　stud link

C.　Kenter link

D.　connecting link

KEY:　B

[3512]A measurement device for inspecting anchor chain is the ______.

A.　slide rule

B.　go-no-go gauge

C.　derrick tape

D.　amp probe

KEY:　B

[3513]A set of interior steps on a ship leading up to a deck from below is known as ______.

A.　a companion way

B.　tween-decks

C.　stairs

D.　Any of the above are acceptable

KEY:　A

[3514]A single fitting installed in a pipeline that either blanks off the pipe or allows a full flow passage of a liquid through the pipe is referred to as a ______.

A.　blind flange

B.　pivot coupling

C.　quick-release coupling

D.　spectacle flange

KEY:　D

[3515]A wildcat is a ______.

A.　deeply-grooved drum on the windlass with sprockets which engage the links of the anchor chain

B.　winch that is running out of control due to a failure of the overspeed trips

C.　line that has jumped off the gypsyhead while under strain

D.　nylon line that parts under strain and whips back in a hazardous manner

KEY:　A

[3516]Bilge keels are fitted on ships to ______.

A.　assist in drydock alignment

B.　improve the vessel's stability

C.　protect the vessel from slamming against piers

D.　reduce the rolling of the vessel

KEY:　D

[3517]Buckler plates are ______.

A.　triangular-shaped plates connecting the bull chain to the topping lift

B.　metal plates secured over the tops of the hawsepipes

C.　faired shell plates with curvature in two directions

D.　sheets of dunnage used to prevent heavy cargo from buckling the deck plates

KEY:　B

[3518]Damaged bulkheads often take a permanent set which is independent of the panting or bulge caused by water pressure. To control this，you should ______.

A.　install shoring so the shoring supports the damaged bulkheads without pushing on them

B.　install shoring so that it pushes on the damaged bulkhead while supporting it

C.　use jacks or chain falls to remove the set before installing shores

D.　place sandbags by the bulkhead without installing shores

KEY:　A

[3519]Electric cargo winches have an overload safety device which normally cuts the current to the winch motor ______.

A.　after torque causes line pull to exceed the rated capacity of the winch

B.　before the line pull reaches the rated capacity of the winch

C.　when the line pull reaches the breaking strength of the fall

D.　before the safe working load of the fall is reached

KEY:　A

[3520]Forecastle deck is located in the ship's ______.

A.　bow stem

B.　stern

C.　portside

D.　starboard side

KEY:　A

[3521]Freeboard is measured from the upper edge of the ______.

A.　bulkwark

B.　deck line

C.　gunwale bar

D.　sheer strake

KEY:　B

[3522]Fuel oil tank vents are fitted with a screen which will stop ______.

A.　oil from flowing out of the tank vent

B.　air from entering the tank vent

C.　vapors from leaving the tank vent

D.　flames on deck from entering the tank vent

KEY:　D

[3523]Grinding to eliminate shallow surface defects should be done ______.

A.　parallel to the longitudinal direction of the chain

B.　perpendicular to the direction of the anchor chain

C.　diagonally across the link of the anchor chain

D.　around the circumference of the chain link

KEY:　A

[3524]Gross tonnage indicates the vessel's ______.

A.　displacement in metric tons

B.　total weight including cargo

C.　volume in cubic feet

D.　the volume of the enclosed space

KEY:　D

[3525]Holes in the bulwark，which allow deck water to drain into the sea，are ______.

A.　doggers

B.　fidleys

C.　freeing ports

D.　swash ports

KEY:　C

[3526]In vessel construction，a greater number of watertight bulkheads results in ______.

A.　increased capacity to set flooding boundaries

B.　decreased capacity to set flooding boundaries

C.　reduced compartmentation

D.　greater deck load capacity

KEY:　C

[3527]In vessel construction，the garboard strake is ______.

A.　located next to and parallel to the keel

B.　located next to and parallel to the gunwale

C.　another term for the bilge keel

D.　another term for the rub rail

KEY:　A

[3528]On a ship，the deck stringer is the outboardmost deck ______.

A.　plating

B.　beam

C.　stiffener

D.　stanchion

KEY:　A

[3529]One function of a bulwark is to ______.

A.　help keep the deck dry

B.　prevent stress concentrations on the stringer plate

C.　protect against twisting forces exerted on the frame of the vessel

D.　reinforce the side stringers

KEY:　A

[3530]Owing to the greater girth of a ship amidships than at the ends，certain strakes are dropped as they approach the bow and stern to reduce the amount of plating at the ends. These strakes are called ______.

A.　drop strakes

B.　stealers

C.　throughs

D.　voids

KEY:　A

[3531]Pollution regulations require that each scupper in an enclosed deck area have a ______.

A.　wooden plug

B.　soft rubber plug

C.　two-piece soft patch

D.　mechanical means of closing

KEY:　D

[3532]Prior to magnetic particle inspection of anchor chain，the chain should be ______.

A.　degaussed

B.　demagnetized

C.　soaked

D.　sandblasted

KEY:　D

[3533]Regulations define the bulkhead deck as ______. (subdivision and stability regulations)

A.　any deck extending from stem to stern

B.　the uppermost deck to which transverse watertight bulkheads extend

C.　the lowermost deck to which transverse watertight bulkheads extend

D.　the uppermost complete deck

KEY:　B

[3534]Regulations require that access to a cargo pumproom in a tanker carrying grade D liquid cargo，shall be ______.

A.　away from galleys，living quarters，or navigation spaces

B.　only from areas equipped with power ventilation systems

C.　from the open deck

D.　isolated from sources of vapor ignition

KEY:　C

[3535]Shell plating is ______.

A.　the galvanizing on steel

B.　a hatch cover

C.　the outer plating of a vessel

D.　synonymous with decking

KEY:　C

[3536]Ship's steering gear refers to ______.

A.　cargo handling machines

B.　deck winches and derricks

C.　engine-room tools

D.　course controlling system

KEY:　D

[3537]The antennas of the trucks on deck are not fitted ______.

A.　in

B.　on

C.　out

D.　with

KEY:　B

[3538]The athwartship hull structural members of a ship are ______.

A.　stringers

B.　girders

C.　breasthooks

D.　deck beams

KEY:　D

[3539]The joint of the riser in a marine riser system that compensates for heave，the up-and-down motion of a floater，is the ______.

A.　flex joint

B.　slip joint

C.　heave joint

D.　riser joint

KEY:　B

[3540]The machinery associated with heaving in and running out anchor chain is the ______.

A.　winch

B.　windlass

C.　draw works

D.　dynamic pay out system

KEY:　B

[3541]The opening in the deck beneath the anchor windlass that leads to the chain locker is the ______.

A.　hawse pipe

B.　fall pipe

C.　drop-pipe

D.　spill pipe

KEY:　D

[3542]The opening in the deck that leads the anchor cable outside the hull is the ______.

A.　hawsepipe

B.　fall pipe

C.　drop-pipe

D.　spill pipe

KEY:　A

[3543]The perforated，elevated bottom of the chain locker，which prevents the chains from touching the main locker bottom and allows seepage water to flow to the drains，is called a ______.

A.　cradle

B.　draft

C.　harping

D.　manger

KEY:　D

[3544]The platform that supports the derrickman when tripping pipe is called the ______.

A.　Texas deck

B.　water table

C.　stabbing board

D.　monkey board

KEY:　D

[3545]The purpose of cant frames in steel vessels is ______.

A.　To support the overhang of the stern

B.　To strengthen the plating against the pounding of the sea

C.　To add strength to the deck beams which support the weather deck

D.　To support the plating of a cylindrical tank

KEY:　A

[3546]The riding pawl is ______.

A.　a safety interlock in a cargo winch that prevents the runner from overspeeding

B.　a stopper that prevents the anchor cable from running free if the cable jumps the wildcat

C.　the device that locks the deck lashings of the Peck and Hale system

D.　the lug that rides on the eccentric rib and engages the locking ring on the windlass

KEY:　B

[3547]The strake on each side of the keel is called a ______.

A.　sheer strake

B.　gatewood strake

C.　insulation strake

D.　garboard strake

KEY:　D

[3548]The term strake is used in reference to ______.

A.　rudder mountings

B.　anchor gear

C.　hull plating

D.　vessel framing

KEY:　C

[3549]The Tonnage Certificate indicates ______.

A.　deadweight tons

B.　displacement tons

C.　net tons

D.　gross tons

KEY:　C

[3550]To determine the weight capacity of a deck in a cargo hold，you would refer to the ______.

A.　deadweight scale

B.　deck capacity plan

C.　cubic capacity tables

D.　general arrangement plan

KEY:　B

13.3 船舶索具及保养

[4451]Nylon rope is often used in the makeup of a towline because it ______.

A.　floats

B.　stretches

C.　handles easily

D.　resists rot

KEY:　B

[4452]On a light tow，what could you substitute for a fishplate?

A.　heart-shaped shackle

B.　pelican hook

C.　swivel

D.　ring

KEY:　A

[4453]On a long ocean tow，the bridle should be made up of two equal lengths of ______.

A.　chain

B.　wire

C.　nylon

D.　manila

KEY:　A

[4454]On an anchor windlass，the wheel over which the anchor chain passes is called a ______.

A.　brake compressor wheel

B.　devil's claw

C.　wildcat

D.　winchhead

KEY:　C

[4455]On your vessel，a wire rope for the cargo gear shows signs of excessive wear and must be replaced. In ordering a new wire for this 10-ton boom，what safety factor should you use?

A.　Three

B.　Five

C.　Six

D.　Seven

KEY:　B

[4456]Ropes or wires attached to derricks to prevent them from swinging during cargo handling operations are __________.

A.　Preventers

B.　Side ropes

C.　Stays

D.　Guide lines

KEY:　A

[4457]Separating both blocks of a tackle to prepare it for reuse is called ______.

A.　chockablocking

B.　fleeting

C.　overhauling

D.　two blocking

KEY:　C

[4458]Serving is ______.

A.　marline or ratline wound along the grooves of a rope

B.　narrow strips of light canvas or cotton cloth spiral-wrapped along the rope

C.　marline tightly wound on the rope by means of a board or mallet

D.　a splice made by laying the strand of one rope into the vacated grooves of another rope

KEY:　C

[4459]Stuffer-braid rope has ______.

A.　a yarn core

B.　no core

C.　three strands

D.　12 threads

KEY:　A

[4460]Sufficient cargo tanks shall be ______ prior to each ballast voyage in order that，taking into account the tanker's trading pattern and expected weather conditions，ballast water is put only into cargo tanks which have been crude oil washed.

A.　inerted

B.　crude oil washed

C.　gas freed

D.　batterned down

KEY:　B

[4461]Temporary seizings on wire rope are made with ______.

A.　marline

B.　sail twine

C.　tape

D.　wire

KEY:　D

[4462]The best method for tying two lines of the same size together is by using a ______.

A.　becket bend

B.　two bowlines

C.　single carrick bend

D.　square knot

KEY:　D

[4463]The best method of protecting that portion of a fiber anchor line nearest the anchor from chafing on the bottom is by ______.

A.　using a small scope ratio

B.　replacing that portion with a short length of chain

C.　using a hockle to keep that portion of the anchor line off the bottom

D.　using a synthetic line

KEY:　B

[4464]The best way to lift many small articles aboard your vessel is with a ______.

A.　pallet

B.　barrel hook

C.　spreader

D.　snotter

KEY:　A

[4465]The biggest problem encountered when towing bridle legs are too short is ______.

A.　retrieval

B.　adjusting tension

C.　excessive strain

D.　hookup to main towline

KEY:　C

[4466]The carrick bend is used to ______.

A.　add strength to a weak spot in a line

B.　join two hawsers

C.　be a stopper to transfer a line under strain

D.　join lines of different sizes

KEY:　B

[4467]The catenary in a towline is ______.

A.　a short bridle

B.　the downward curvature of the hawser

C.　another name for a pelican hook

D.　used to hold it amidships

KEY:　B

[4468]The correct way to make an eye in a wire rope with clips is to place the clips with the ______.

A.　first and third U-bolts on the bitter end and the second U-bolt on the standing part

B.　first and third U-bolts on the standing part and the second U-bolt on the bitter end

C.　U-bolts of all clips on the bitter end

D.　U-bolts of all clips on the standing part

KEY:　C

[4469]The disadvantage of using three strand nylon line for towing is its ______.

A.　inherent weakness

B.　tendency to rot if left damp

C.　danger to crew if it parts

D.　strength and shock absorbing abilities

KEY:　C

[4470]The heel block is located ______.

A.　at the base of the boom

B.　at the cargo hook

C.　near the midships guy

D.　near the spider band

KEY:　A

[4471]The knot at the end of the heaving line used to pass the towing hawser is called a ______.

A.　monkey's fist

B.　ball or baseball knot

C.　heaving knot

D.　three strand Turk's head

KEY:　A

[4472]The knot used to join two lines of different diameter is a ______.

A.　square knot

B.　carrick bend

C.　becket bend

D.　sheepshank

KEY:　C

[4473]The knot used to join two lines or two large hawsers for towing is called a ______.

A.　square knot

B.　carrick bend

C.　sheet bend

D.　bowline

KEY:　B

[4474]The latch of a safety hook ______.

A.　increases the strength of the hook

B.　prevents the sling ring from coming out of the hook if the strain is abruptly eased

C.　prevents the sling ring from coming out of the hook if there is a strain on the sling ring

D.　All of the above

KEY:　B

[4475]The line with the most stretch is ______.

A.　manila

B.　nylon

C.　polypropylene

D.　dacron

KEY:　B

[4476]The main advantage of a Chinese stopper over the one line stopper is that it ______.

A.　will not jam on the mooring line

B.　is stronger

C.　is easier to use when under heavy tension

D.　is safer to use when under heavy tension

KEY:　A

[4477]The metal，teardrop-shaped object sometimes used within an eyesplice is a ______.

A.　grommet

B.　reinforcement

C.　splice form

D.　thimble

KEY:　D

[4478]The recessed areas on a wildcat are called ______.

A.　pawls

B.　sockets

C.　pockets

D.　devil's claws

KEY:　C

[4479]The splice designed to pass easily through a block is called a(n) ______.

A.　eye splice

B.　short splice

C.　long splice

D.　block splice

KEY:　C

[4480]The spray of water in low-velocity fog will have ______.

A.　greater range than high-velocity fog

B.　lesser range than high-velocity fog

C.　about the same range as high-velocity fog

D.　greater range than a solid stream

KEY:　B

[4481]The strongest of the natural fibers is ______.

A.　cotton

B.　hemp

C.　manila

D.　sisal

KEY:　C

[4482]Towlines should be inspected for chafing where the towline ______.

A.　passes over the stern of the towing vessel

B.　passes through chocks

C.　is attached to the disabled vessel

D.　All of the above

KEY:　D

[4483]Tugs sometimes shackle a length of chain in the towline in order to ______.

A.　take the wear should the towline drag bottom

B.　assure that if the towline is overstressed it will part close to the bridle

C.　prevent the towline from whipping should it part

D.　put spring in the towline

KEY:　D

[4484]Two mooring lines may be placed on the same bollard and either one cast off first if ______.

A.　the eye of the second line is dipped

B.　the mooring lines are doubled

C.　the bollard has two horns

D.　one of the lines is a breast line

KEY:　A

[4485]Uncoiling manila line improperly can result in a(n) ______.

A.　number of fishhooks

B.　kink in the line

C.　50% loss of efficiency of the line

D.　increase in deterioration of the line

KEY:　B

[4486]Under identical load conditions，nylon，when compared with natural fiber line，will stretch ______.

A.　less and have less strength

B.　more and have less strength

C.　more and have greater strength

D.　less and have greater strength

KEY:　C

[4487]What action should you take to compensate for the abovecurrent?

A.　Continue on the same course and speed.

B.　Alter your course to the right.

C.　Slow to 8.5 knots.

D.　Alter your course to the left.

KEY:　D

[4488]What are the two main types of stud link chain?

A.　Oil Rig chain and Oil Field Stud Link chain

B.　Flash-butt welded chain and Di-Lok chain

C.　Flash-butt welded chain and Oil Rig chain

D.　Oil Field Stud Link chain and Flash-butt welded chain

KEY:　B

[4489]What best describes an anchor buoy?

A.　A black ball that is hoisted when the ship anchors

B.　A buoy attached to the anchor

C.　A buoy attached to the scope of an anchor chain

D.　A mark of the number of fathoms in an anchor chain

KEY:　B

[4490]What can be used to measure the percentage of oxygen inside a chain locker?

A.　Flame safety lamp

B.　Combustible gas indicator

C.　Oxygen indicator

D.　H2S meter

KEY:　C

[4491]What does the term end-for-end refer to in regard to a wire towing hawser?

A.　Cutting off the bitter and towing ends of the wire rope

B.　Splicing two wire ropes together

C.　Removing the wire rope from the drum and reversing it so that the towing end becomes the bitter end

D.　Removing the wire rope from the drum and turning it over so that the wire bends in the opposite direction when rolled on a drum

KEY:　C

[4492]What fitting on the mast works in conjunction with the shrouds to control side bend of the mast?

A.　Chainplate

B.　Hound

C.　Crowfoot

D.　Spreader

KEY:　D

[4493]What happens to the pulling power of a winch when retrieving wire rope?

A.　It increases

B.　It decreases

C.　It remains the same

D.　It fluctuates，depending on the gearing system

KEY:　B

[4494]What is a spill pipe?

A.　A drainage pipe that carries rain or spray from an upper deck to a lower deck

B.　A pipe under the anchor windlass leading to the chain locker

C.　A chute，usually over the stern，to lead dumped garbage clear of the hull

D.　An opening in the deck leading outside the hull

KEY:　B

[4495]What is a step in attaching a poured metal socket to a wire rope?

A.　Etch the wire with acid.

B.　Install a wire seizing on the wire that will be inside the socket.

C.　Ensure the fiber core is well lubricated.

D.　Pour molten babbitt metal into the socket.

KEY:　A

[4496]What is an advantage of having wire rope with a fiber core over that of a wire rope of the same size with a wire core?

A.　Fiber core rope offers greater strength.

B.　Fiber core rope offers greater flexibility.

C.　Fiber core rope can be used at higher operating temperatures.

D.　Fiber core rope is the only type authorized for cargo runners.

KEY:　B

[4497]What is NOT suitable for use in making up the towing rig for a heavy，long ocean tow?

A.　Chain

B.　Ring

C.　Solid thimble

D.　A fishplate

KEY:　B

[4498]What is the best guide for determining the proper scope of anchor chain to use for anchoring in normal conditions?

A.　One shot of chain for every ten feet of water

B.　One shot of chain for every fifteen feet of water

C.　One shot of chain for every thirty feet of water

D.　One shot of chain for every ninety feet of water

KEY:　B

[4499]What is the main reason to slush a wire rope?

A.　Keep the wire soft and manageable

B.　Lubricate the inner wires and prevent wear

C.　Prevent kinking

D.　Prevent rotting

KEY:　B

[4500]What is the primary disadvantage of a permanent chain chaser system?

A.　It requires anchor handling boats with more horsepower.

B.　Chain chasers work in shallow water only.

C.　Chain chasers will not work with piggyback anchors.

D.　It takes longer to deploy anchors using chain chasers.

KEY:　C

13.4 船舶物料及管理

[4501]You are ordering ship's stores. Which statement is TRUE? ______.

A.　Up to five gallons of a flammable liquid may be stowed in the engine room.

B.　All stores of line，rags，linens and other similar type stores must be certified by UL as being fire retardant.

C.　Cylinders containing compressed gasses must be constructed and tested in accordance with the Bureau of Standards.

D.　All distress flares when received must be stored in the portable magazine chest.

KEY:　A

[4502]You are ordering ship's stores. Which statement is TRUE? ______.

A.　Stores such as line，rags，mattresses，linens，etc. must be treated to be fire retardant.

B.　Stores certified for use on uninspected vessels may be identified by the certification number used by Underwriter's Laboratories.

C.　A portable container of a flammable liquid used as fuel for portable auxiliary equipment must be stowed in a paint locker or at a designated open location.

D.　Acetylene may be in a ship's storeroom in quantities not exceeding 6000 cubic feet.

KEY:　C

[4503]What is LEAST likely to cause ignition of fuel vapors? ______.

A.　Static electricity

B.　An open running electric motor

C.　Loose wiring

D.　Explosion proof lights

KEY:　D

[4504]A double male coupling is one that ______.

A.　has left hand twist

B.　has inside threads on both ends

C.　has outside threads on both ends

D.　takes two men to operate

KEY:　C

[4505]A fire starting by spontaneous combustion can be expected in which condition?

A.　Paints，varnish，or other liquid flammables are stowed in a dry stores locker.

B.　Inert cargoes such as pig iron are loaded in a wet condition.

C.　Oily rags are stowed in a metal pail.

D.　Clean mattresses are stored in contact with an electric light bulb.

KEY:　C

[4506]A fishing vessel that is required to have a fireman's outfit，must have all of the following in the outfit except a ______.

A.　flashlight

B.　combustible gas indicator

C.　self-contained breathing apparatus

D.　fire axe

KEY:　B

[4507]A hydraulic accumulator is designed to ______.

A.　store fluid under pressure

B.　act as a fluid reservoir

C.　provide overpressure relief

D.　replenish fluid to a system

KEY:　A

[4508]A vessel engaged in replenishment or transferring persons，provisions or cargo while under way can be regarded as ______.

A.　a vessel restricted in her ability to maneuver

B.　a vessel engaged in laying，serving or picking up a navigation mark，submarine cable or pipeline．

C.　a vessel not under command

D.　a power-driven vessel．

KEY:　A

[4509]A vessel transferring provisions or cargo at sea shall display during the day ______.

A.　two black balls in a vertical line

B.　three black balls in a vertical line

C.　three shapes in a vertical line； the highest and lowest shall be red balls and the middle a white diamond

D.　three black shapes in a vertical line； the highest and lowest shall be balls and the middle one a diamond

KEY:　D

[4510]A vessel transferring provisions or cargo at sea shall display during the day ______.

A.　two black balls in a vertical line

B.　three black balls in a vertical line

C.　three shapes in a vertical line; the highest and lowest shall be red balls and the middle a white diamond

D.　three black shapes in a vertical line; the highest and lowest shall be balls and the middle one a diamond

KEY:　D

[4511]All the copper bars are ______ different sizes．

A.　at

B.　in

C.　by

D.　from

KEY:　B

[4512]All the expenses are ______ the ship's account．

A.　of

B.　for

C.　to

D.　at

KEY:　B

[4513]An advantage of nylon rope over manila rope is that nylon rope ______.

A.　can hold a load even when a considerable number of the yarns have been abraded

B.　can be stored on deck，exposed to sunlight

C.　can be used in conjunction with wire or spring-lay rope

D.　gives audible warning of overstress whereas manila does not

KEY:　A

[4514]An instrument useful in predicting fog is the ______.

A.　sling psychrometer

B.　microbarograph

C.　anemometer

D.　aneroid barometer

KEY:　A

[4515]Before operating a non-oceangoing ship greater than 100 gross tons it must have a fixed piping system to discharge oily mixtures ashore. This system must include ______.

A.　approved oil-water separating equipment

B.　a fixed or portable containment system at the shore connection

C.　a spare pump in case the main pump is inoperative

D.　at least one outlet accessible from the weather deck

KEY:　D

[4516]Before starting to hoist provisions，which should be checked?

A.　Hoist rope is not kinked

B.　Multiple part lines are not twisted around each other

C.　The hook is centrally located over the load

D.　All of the above

KEY:　D

[4517]BHP stands for the ______ of the main engine．

A.　breadth dimension

B.　builder's name

C.　brake horsepower

D.　revolutions per minute

KEY:　C

[4518]Blocks and falls used as lifeboat gear must be designed with a minimum safety factor of ______.

A.　4，based on the breaking strength

B.　5，based on the maximum allowable stress

C.　6，based on the maximum working load

D.　8，based on the normal working load

KEY:　C

[4519]Customs officers，upon boarding a vessel desiring entry into one of their port，would inspect which document?

A.　Cargo Manifest

B.　Certified Crew List

C.　Stores List

D.　All of the above

KEY:　D

[4520]Each fireman's outfit and its spare equipment on a ship must be stowed ______.

A.　in a locked cabinet in the machinery space

B.　in an unlocked cabinet in the machinery space

C.　in a separate and accessible location

D.　at a fire hydrant location

KEY:　C

[4521]Fire axes required on ship's must be stored in the enclosure for fire hoses with the location marked " ______".

A.　Fire Axe Location

B.　Hose Station No. ______

C.　Fire Station No. ______

D.　Firefighting Equipment

KEY:　C

[4522]Fire protection regulations for towing vessels require training in all of the following，EXCEPT ______.

A.　putting on a fireman's outfit，if the vessel is so equipped

B.　donning a self-contained breathing apparatus，if the vessel is so equipped

C.　activating the general alarm and reporting inoperative alarm systems and fire-detection systems

D.　refilling and servicing all expended fire extinguishing equipment

KEY:　D

[4523]MTIME PLS B NOTED MY VSL NEEDS TO TAKE PROVISIONS AND BUNKERS SOON AFTER HER BERTHING ALONGSIDE. This fax indicates that ______.

A.　she needs provisions and bunkers immediately after berthing

B.　even she will berth here the vessel does not need any provisions and bunkers

C.　she will take provisions and bunkers at other berth

D.　she needs no provisions and bunkers even after berthing

KEY:　A

[4524]On a class "B" fire，which portable fire extinguisher would be the LEAST desirable?

A.　Carbon dioxide

B.　Water (stored pressure)

C.　Dry chemical

D.　Foam

KEY:　B

[4525]Paints and solvents on a vessel should be ______.

A.　stored safely in a cool dark non-ventilated area until work is completed

B.　resealed and returned to a well ventilated area after each use

C.　covered at all times to protect from ignition sources

D.　stored in a suitable gear locker

KEY:　B

[4526]The jackknife stored on an inflatable liferaft will always be located ______.

A.　in one of the equipment bags

B.　in a special pocket near the forward entrance

C.　on a cord hanging from the canopy

D.　in a pocket on the first aid kit

KEY:　B

[4527]The order of importance in addressing damage control on a ship is ______.

A.　control flooding，control fire，repair structural damage

B.　restore vital services，control fire，control flooding

C.　control fire，restore vital services，control flooding

D.　control fire，control flooding，repair structural damage

KEY:　D

[4528]To clean a dirty fiber towing hawser used in ocean towing，you should ______.

A.　use lye or other mild detergent

B.　wash it with fresh water，dry and store it

C.　wash it with salt water

D.　use cleaning fluid on nylon，dacron and other synthetic lines

KEY:　B

[4529]Vessel's provisions refer to ______ of the vessel．

A.　fuel oil

B.　vegetables and food

C.　spare parts on board

D.　ballast water

KEY:　B

[4530]What should be used to steer an open lifeboat if the rudder becomes lost or damaged?

A.　Sea anchor

B.　Steering oar

C.　Spare rudder

D.　Daggerboard

KEY:　B

[4531]When choosing extinguishers to fight a Class "B" fire do NOT use ______.

A.　carbon dioxide

B.　dry chemical

C.　foam (stored-pressure type)

D.　water (cartridge-operated)

KEY:　D

[4532]When collecting condensation for drinking water， ______.

A.　a sponge used to mop up and store condensation must be kept salt free

B.　only condensation on the bottom of the canopy should be collected

C.　it should be strained through a finely woven cloth

D.　chlorine tablets should be used to make it drinkable

KEY:　A

[4533]When loading or discharging dry mud or cement，crew members should use goggles and ______.

A.　facial respirator mask

B.　ear plugs

C.　rubberized boots

D.　fireman's outfit

KEY:　A

[4534]Which extinguishing agent will cool down a heated bulkhead in the least amount of time?

A.　Water stream

B.　Water fog or apray

C.　Steam

D.　Dry chemical

KEY:　B

[4535]Which item may be substituted for，in the fireman's outfit，on a cargo vessel?

A.　fire ax

B.　flashlight

C.　rigid helmet

D.　flame safety lamp

KEY:　D

[4536]Which of the following would be of immediate concern after discovering a large fire in the ship's galley?

A.　An adjacent storeroom，containing spare parts

B.　A storeroom directly above，containing combustible fluids

C.　An adjacent storeroom，containing mattresses and linen

D.　An adjacent storeroom，marked "Stewards Stores"

KEY:　B

[4537]Which statement about immersion suits is TRUE?

A.　The primary color of the suit's exterior may be red，orange or yellow.

B.　The suit must，without assistance，turn an unconscious person's mouth clear of the water within 5 seconds.

C.　The suit is flameproof and provides protection to a wearer swimming in burning oil.

D.　The suit may be stored in a machinery space where the ambient temperature is 160F.

KEY:　B

[4538]Which types of portable fire extinguishers are designed for use on electrical fires?

A.　Dry chemical and carbon dioxide

B.　Foam (stored pressure) and soda-acid

C.　Carbon dioxide and foam (stored pressure)

D.　Dry chemical and soda-acid

KEY:　A

[4539]Which vessel is NOT to be regarded as restricted in her ability to maneuver? ______.

A.　A vessel transferring provisions while underway

B.　A pushing vessel and a vessel being pushed when connected in a composite unit

C.　A vessel servicing a navigation mark

D.　A vessel launching aircraft

KEY:　B

[4540]You are ordering ships' stores that are NOT consumer commodities. Which statement is TRUE?

A.　All flammable liquids must be stowed in the paint locker or specially constructed integral tanks.

B.　The label of a hazardous ships' store must include instructions for safe stowage.

C.　Replacement CO2 cylinders for the fixed fire fighting system must have been tested within 8 years of receipt.

D.　Cartridges for the line throwing appliance must be stored in the portable magazine chest after receipt.

KEY:　B

[4541]You are ordering ship's stores. Which statement is TRUE?

A.　Up to five gallons of a flammable liquid may be stowed in the engine room.

B.　All stores of line，rags，linens and other similar type stores must be certified by UL as being fire retardant.

C.　Cylinders containing compressed gasses must be constructed and tested in accordance with the Bureau of Standards.

D.　All distress flares when received must be stored in the portable magazine chest.

KEY:　A

[4542]You are ordering ship's stores. Which statement is TRUE?

A.　Stores such as line，rags，mattresses，linens，etc. must be treated to be fire retardant.

B.　Stores certified for use on uninspected vessels may be identified by the certification number used by Underwriter's Laboratories.

C.　A portable container of a flammable liquid used as fuel for portable auxiliary equipment must be stowed in a paint locker or at a designated open location.

D.　Acetylene may be in a ship's storeroom in quantities not exceeding 6000 cubic feet.

KEY:　C

[4543]You are passing the Bayou Sara gage which reads 3.9 feet. Which of the following statements is TRUE?

A.　The river level is above the Low Water Reference Plane.

B.　Red Store Landing Revetment is ahead on your starboard side

C.　This gage reading is at a lower elevation than the same reading on the gage at Head of Passes.

D.　None of the above.

KEY:　D

[4544]You are requisitioning stores for your tank vessel. What type of matches are permitted aboard?

A.　Phosphorous

B.　Safety

C.　Self-extinguishing

D.　Wooden

KEY:　B

[4545]You are requisitioning stores for your tanker. What type of matches are permitted aboard?

A.　Phosphorous

B.　Safety

C.　Self-extinguishing

D.　Wooden

KEY:　B

[4546]Your fireman's outfit includes a(n) ______.

A.　chemical protection face shield

B.　approved work vest

C.　self-contained breathing apparatus

D.　marlinspike

KEY:　C

[4547]Your vessel has just finished bunkering and has a small list due to improper distribution of the fuel oil. This list will cause ______.

A.　a decrease in reserve buoyancy

B.　a decrease in the maximum draft

C.　the vessel to flop to port or starboard

D.　None of the above

KEY:　D

[4548]Your vessel is at a dock taking bunkers through a pipe laid down on the dock. If oil begins to flow out of a tank vent，what should you do FIRST?

A.　Open the intake valve to an adjacent tank.

B.　Set out drip pans and sawdust and begin to mop up the spill.

C.　Signal the shore control point to shut down.

D.　Close the valve on the tank vent line.

KEY:　C

[4549]Your vessel is listing 4 to port and has a short rolling period. There is loose firefighting water in the hull. The ship is trimmed down by the head with one foot of freeboard at the bow. Which action should you take FIRST?

A.　Press up the slack NO.1 starboard double bottom tank.

B.　Pump out the forepeak tank.

C.　Eliminate the water in the 'tween decks aft.

D.　Jettison stores out of the paint locker in the fo'c'sle.

KEY:　B

[4550]Your vessel is equipped with a fixed CO2 system and a fire main system. In the event of an electrical fire in the engine room what is the correct procedure for fighting the fire?

A.　Use the CO2 system and evacuate the engine room.

B.　Use the fire main system and evacuate the engine room.

C.　Evacuate the engine room and use the CO2 system.

D.　Evacuate the engine room and use the fire main system.

KEY:　C

14.1 海上搜索

[4601]______ the survivors known more of how to cope with cold water in the sinking of MV Titanic in 1912 countless lives ______.

A.　Have／could have been saved

B.　Have／could have saved

C.　Had／could has saved

D.　Had／could have been saved

KEY:　D

[4602]A marker pole，with a horseshoe buoy and a sea anchor attached，should be used to ______.

A.　mark the position of a lost mooring

B.　determine your vessel's sideslip underway

C.　determine your speed through the water

D.　indicate location of a man overboard

KEY:　D

[4603]A rescuer can most easily determine whether or not an adult victim has a pulse by checking the pulse at the ______.

A.　carotid artery in the neck

B.　femoral artery in the groin

C.　brachial artery in the arm

D.　radial artery in the wrist

KEY:　A

[4604]A seaman has a small，gaping laceration of the arm that is not bleeding excessively. What can be done as an alternative to suturing to close the wound?

A.　Wrap a tight bandage around the wound.

B.　Apply a compression bandage.

C.　Use temporary stitches of sail twine.

D.　Apply butterfly strips，then a sterile dressing.

KEY:　D

[4605]A seaman is reported missing in the morning and was last seen after coming off the mid-watch. Which type of turn would you use to return to the trackline steamed during the night?

A.　Williamson

B.　Racetrack

C.　180turn

D.　Anderson

KEY:　A

[4606]A ship must have on board a first-aid kit that is approved by the ______.

A.　Minerals Management Service

B.　Occupational Safety and Health Administration

C.　Mine Safety and Health Administration

D.　American Bureau of Shipping

KEY:　C

[4607]A shipmate chokes suddenly，cannot speak，and starts to turn blue. You should ______.

A.　perform the Heimlich maneuver

B.　make the victim lie down with the feet elevated to get blood to the brain

C.　immediately administer CPR

D.　do nothing until the victim becomes unconscious

KEY:　A

[4608]A shipmate suffers a heart attack and stops breathing. You must ______.

A.　administer oxygen

B.　immediately check his pulse and start CPR

C.　make the victim comfortable in a bunk

D.　immediately give a stimulant，by force if necesssary

KEY:　B

[4609]A snag or other underwater obstruction may form a ______.

A.　V-shaped ripple with the point of the V pointing upstream

B.　V-shaped ripple with the point of the V pointing downstream

C.　small patch of smooth water on a windy day

D.　smoothing out of the vessel's wake

KEY:　A

[4610]A survival craft being used to pick up a person who has fallen overboard from a vessel should approach the person ______.

A.　at a high rate of speed

B.　under oars

C.　against the wind

D.　with the wind

KEY:　C

[4611]A tourniquet should be used to control bleeding ONLY ______.

A.　with puncture wounds

B.　when all other means have failed

C.　when the victim is unconscious

D.　to prevent bleeding from minor wounds

KEY:　B

[4612]A tug is approaching a broken down steamer in moderately heavy weather preparing to take it in tow. In most cases the ______.

A.　steamer will drift stern downwind

B.　tug will drift faster than the steamer

C.　tug should approach stern to

D.　tug should approach from downwind

KEY:　D

[4613]After you activate your emergency position indicating radiobeacon，you should ______.

A.　turn it off for five minutes every half-hour

B.　turn it off and on at five-minute intervals

C.　turn it off during daylight hours

D.　leave it on continuously

KEY:　D

[4614]All casualties such as personnel injuries or illness，oil spills，accidental fire，collision，stranding，and unusually severe weather damage，whether at sea，in port，or in shipyard，must be promptly ______to the company.

A.　spoken

B.　said

C.　told

D.　communicated

KEY:　D

[4615]AN APPOINTMENT BETWEEN VESSELS NORMALLY MADE ON RADIO TO MEET IN A CERTAIN AREA OR POSITION defines ______.

A.　Bona fide

B.　Rendez-vous

C.　Prima facie

D.　Action in personam

KEY:　B

[4616]An emergency sea anchor may be constructed by using ______.

A.　a boat bucket

B.　an air tank filled with water

C.　an oar and canvas weighted down

D.　All of the above

KEY:　D

[4617]An immersion suit must be equipped with a(n) ______.

A.　air bottle for breathing

B.　whistle and light

C.　whistle，light，and reflective tape

D.　whistle，light，and sea dye marker

KEY:　C

[4618]If you see anybody fall overboard，act as follows except __________.

A.　tell an officer/crew the person's position in the water，or telephone the bridge immediately

B.　throw lifebuoys overboard

C.　call out "Man overboard" and keep your eyes on the person in the water

D.　try to find medicine for the person to use when he is rescued

KEY:　D

[4619]If，for any reason，it is necessary to abandon ship while far out at sea，it is important that the crew members should ______.

A.　separate from each other as this will increase the chances of being rescued

B.　get away from the area because sharks will be attracted to the vessel

C.　immediately head for the nearest land

D.　remain together in the area because rescuers will start searching at the vessel's last known position

KEY:　D

[4620]Immediately after abandoning a vessel，lookouts should be posted aboard liferafts to look for ______.

A.　survivors in the water

B.　food and water

C.　land

D.　bad weather

KEY:　A

[4621]In a racetrack turn，to recover a man overboard，the vessel is steadied for the SECOND time after a turn of how many degrees from the original heading?

A.　60

B.　135

C.　180

D.　360

KEY:　D

[4622]In relation to the turning circle of a ship，the term advance means the distance ______.

A.　gained at right angles to the original course

B.　gained in the direction of the original course

C.　moved sidewise from the original course when the rudder is first put over

D.　around the circumference of the turning circle

KEY:　A

[4623]Information on search and rescue procedures will be found in the ______.

A.　World Port Index

B.　International Code of Signals

C.　Sailing Directions

D.　Merchant Ship Search and Rescue Manual (MERSAR)

KEY:　D

[4624]MOVING OF AN ANCHOR OVER THE SEA BOTTOM INVOLUNTARILY BECAUSE IT IS NO LONGER PREVENTING THE MOVEMENT OF THE VESSEL defines ______.

A.　Dredging (of anchor)

B.　Dragging (of anchor)

C.　Weighing anchor

D.　Walking out anchor

KEY:　B

[4625]One of your crew members falls overboard from the starboard side. You should IMMEDIATELY ______.

A.　apply left rudder

B.　throw the crew member a life preserver

C.　begin backing your engines

D.　position your vessel to windward and begin recovery

KEY:　B

[4626]The best method to secure a towline to bitts is to ______.

A.　take a round turn on the bitt farthest from the pull and use figure-eights

B.　take a round turn on the bitt closest to the pull and use figure-eights

C.　use figure-eights and take a round turn at the top of the bitts

D.　use only figure-eights

KEY:　B

[4627]The distance a vessel moves at right angles to the original course，when a turn of 180has been completed，is called the ______.

A.　advance

B.　pivoting point

C.　tactical diameter

D.　kick

KEY:　C

[4628]The distance a vessel moves parallel to the original course from the point where the rudder is put over to any point on the turning circle is called the ______.

A.　advance

B.　drift angle

C.　pivoting point

D.　transfer

KEY:　A

[4629]The distance gained in the direction of the original course when you are making a turn is known as ______.

A.　advance

B.　drift

C.　tactical diameter

D.　transfer

KEY:　A

[4630]The major cause of anchor buoy pendant wire failures is ______.

A.　corrosion

B.　rough weather

C.　defective sockets

D.　mishandling

KEY:　B

[4631]The most likely location for a liquid cargo fire to occur on a tanker would be ______.

A.　in the midships house

B.　at the main deck manifold

C.　at the vent header

D.　in the pumproom

KEY:　D

[4632]The only type of helicopter that may be refueled with the engine running and the blades turning is ______.

A.　a helicopter carrying cargo only

B.　a turbine-equipped helicopter

C.　a Sikorsky

D.　a helicopter carrying injured personnel in an emergency situation

KEY:　B

[4633]The patrolman，while on duty on a passenger vessel，must have in his possession a(n) ______.

A.　nightstick

B.　flashlight

C.　passenger list showing assigned berths

D.　A-I fire extinguisher

KEY:　B

[4634]The signal for fire alarm on a ship must be indicated ______.

A.　at each alarm bell

B.　at each alarm actuator

C.　near all exits

D.　on the station bill

KEY:　D

[4635]The survival craft is manufactured with fire retardant ______.

A.　foam

B.　marine plywood

C.　steel

D.　fiberglass

KEY:　D

[4636]Upon receipt of a distress message，a merchant vessel is bound to proceed to the scene of the distress. Under which of the following cases would this NOT be true?

A.　The vessel would arrive at the distress scene more than 36 hours after the receipt of the initial distress message.

B.　There are vessels closer to the distress scene that are proceeding to assist.

C.　The Master of the vessel in distress has requisitioned another vessel，and that vessel has accepted the requisition.

D.　You are on a tanker and the distress involves a major fire on board the other vessel.

KEY:　C

[4637]What is one of the FIRST actions you should take after abandoning and clearing away from a vessel?

A.　Identify the person in charge.

B.　Gather up useful floating objects.

C.　Prepare for arrival of rescue units.

D.　Arrange watches and duties.

KEY:　A

[4638]What should you do if you have transmitted a distress call a number of times on channel 16 and have received no reply?

A.　Repeat the message using any other channel on which you might attract attention.

B.　Key the microphone several times before transmitting again.

C.　Turn up the volume on the receiver before transmitting again.

D.　Report the problem to the head electrician.

KEY:　A

[4639]When evacuating a seaman by helicopter lift，the vessel should be ______.

A.　stopped with the wind dead ahead

B.　stopped with the wind on the beam

C.　underway with the wind 30on the bow

D.　underway on a course to provide no apparent wind

KEY:　C

[4640]When evacuating a seaman by helicopter lift，which course should the ship take?

A.　Downwind so that the apparent wind is close to nil.

B.　A course that will keep a free flow of air，clear of smoke，over the hoist area.

C.　A course that will have the hoist area in the lee of the superstructure.

D.　With the wind dead ahead because the helicopter is more maneuverable when going into the wind.

KEY:　B

[4641]When evacuating a seaman by helicopter lift，which statement is TRUE?

A.　The vessel should be stopped with the wind dead ahead during the hoisting operation.

B.　Flags should be flown to provide a visual reference as to the direction of the apparent wind.

C.　The drop line should be grounded first then secured as close to the hoist point as possible.

D.　The hoist area should be located as far aft as possible so the pilot will have a visual reference while approaching.

KEY:　B

[4642]When joining a vessel，do not forget personal documents and your spectacles，and ______ if necessary.

A.　drug

B.　remedy

C.　medicine

D.　healer

KEY:　C

[4643]When jumping into water upon which there is an oil-fire，you should ______.

A.　break the water surface with your hands when diving head-first

B.　use your hands to hold your knees to your chest

C.　cover your eyes with one hand while pinching your nose shut and covering your mouth with the other

D.　enter the water at the bow or stern on the windward side of the vessel

KEY:　D

[4644]When retrieving the survival craft，the winch operator should stop the winch and check ______.

A.　that all personnel are seated in the craft

B.　that the cable has not jumped any grooves on the drum

C.　which way the wind is blowing

D.　the hydraulic fluid level before lifting

KEY:　B

[4645]Which condition is necessary for a substance to burn?

A.　The temperature of the substance must be equal to or above its fire point.

B.　The air must contain oxygen in sufficient quantity.

C.　The mixture of vapors with air must be within the explosive range.

D.　All of the above

KEY:　D

[4646]Which is the proper method of determining whether a portable CO2 fire extinguisher needs recharging?

A.　Check the tag to see when the extinguisher was last charged.

B.　Release a small amount of CO2; if the CO2 discharges，the extinguisher is acceptable.

C.　Weigh the extinguisher and compare the weight against that stamped on the valve.

D.　Recharge the extinguisher at least once each year.

KEY:　C

[4647]Which of the following is not a maritime perils？ ______.

A.　Stranding or grounding．

B.　Striking upon rocks or shoals

C.　Collision between ships

D.　Fire

KEY:　D

[4648]Which of the following statements regarding low expansion foam and its application is INCORRECT？ ______.

A.　Foam should not be used on electrical fires

B.　One kilo of low expansion foam solution produces much more foam．

C.　Foam is only efficient when it covers the top of burning combustibles．

D.　A stream of foam should be deflected off the deck in order to best agitate the fire

KEY:　D

[4649]Which procedure should be followed when individuals are rescued in cold climates and suffer from hypothermia?

A.　Give them brandy.

B.　Keep them in motion.

C.　Immerse them in a warm bath (105F，40C).

D.　Cover them with an electric blanket set for maximum temperature.

KEY:　C

[4650]Which statement about firefighting foam is TRUE?

A.　Foam conducts electricity.

B.　To be most effective，foam should be directed at the base of the fire.

C.　Foam is most effective on burning liquids which are flowing.

D.　Foam can ONLY be used to extinguish class A fires.

KEY:　A

14.2 海上救助

[4651]As a vessel sinks to a depth of 15 feet，the hydrostatic trip releases the liferaft container from its cradle by ______.

A.　breaking the weak link

B.　releasing the tie-down strap

C.　pulling the operating cord

D.　releasing the CO2 canister

KEY:　B

[4652]An inflatable liferaft should be manually released from its cradle by ______.

A.　cutting the straps that enclose the container

B.　removing the rubber sealing strip from the container

C.　loosening the turnbuckle on the securing strap

D.　pushing the button on the hydrostatic release

KEY:　D

[4653]An inflatable liferaft is hand-launched by ______.

A.　pulling a cord

B.　cutting the wire restraining bands

C.　removing the rubber packing strip

D.　throwing the entire container overboard

KEY:　D

[4654]An inflatable liferaft can be launched by ______.

A.　the float-free method ONLY

B.　breaking the weak link on the painter

C.　throwing the entire container overboard and then pulling on the operating cord to inflate the raft

D.　removing the securing straps

KEY:　C

[4655]Progressive flooding may be indicated by ______.

A.　ballast control alarms

B.　excessive draft

C.　excessive list or trim

D.　a continual worsening of list or trim

KEY:　D

[4656]You are proceeding to a distress site and expect large numbers of people in the water. Which statement is TRUE?

A.　You should stop to windward of the survivors in the water and only use the ship's boats to recover the survivors.

B.　If the survivors are in inflatable rafts you should approach from windward to create a lee for the survivors.

C.　An inflatable liferaft secured alongside can be an effective boarding station for transfer of survivors from the boats.

D.　Survivors in the water should never be permitted alongside due to the possibility of injury from the vessel.

KEY:　C

[4657]A breeches buoy is being rigged from the shore to a stranded vessel. The initial shot line passed to the vessel is normally made fast to a ______.

A.　hawser which is used to pass a tail-block and whip to the vessel

B.　hawser with breeches buoy and harness attached

C.　hawser which should be made fast to the vessel below the intended location of the tail-block

D.　tail-block and whip which may be used to pass a hawser to the vessel

KEY:　D

[4658]A cable used by helicopters for lifting or lowering persons in a pick-up operation is a ______.

A.　Hoist

B.　Rope

C.　Cable

D.　Line

KEY:　A

[4659]A capsized small sail vessel is best righted when what part of the vessel is downwind?

A.　Stern

B.　Bow

C.　Centerboard

D.　Mast

KEY:　D

[4660]A floating ship with an initial negative metacentric height ______.

A.　will capsize

B.　will incline further

C.　may lie at an angle of loll

D.　may be initially level

KEY:　C

[4661]A helicopter making a round trip from a helideck with refueling capabilities to an unmanned platform will take 45 minutes each way. The helicopter should be carrying enough fuel to last ______.

A.　45 minutes

B.　1 hour and 15 minutes

C.　1 hour and 30 minutes

D.　2 hours

KEY:　D

[4662]A mechanical davit is designed to automatically ______.

A.　position the boat at the embarkation station

B.　lift the boat off the inboard chocks

C.　energize the winch for the falls

D.　set the brake on the winch

KEY:　B

[4663]A minor heat burn of the eye should be treated by ______.

A.　gently flooding with water

B.　warming the eye with moist warm packs

C.　laying the person flat on his back

D.　mineral oil drops directly on the eye

KEY:　A

[4664]A negative metacentric height ______.

A.　will always cause a vessel to capsize

B.　should always be immediately corrected

C.　always results from off-center weights

D.　All of the above are correct

KEY:　B

[4665]A person has fallen overboard and is being picked up with a lifeboat. If the person appears in danger of drowning，the lifeboat should make ______.

A.　an approach from leeward

B.　an approach from windward

C.　the most direct approach

D.　an approach across the wind

KEY:　C

[4666]A person has suffered a laceration of the arm. Severe bleeding has been controlled by using a sterile dressing and direct pressure. What should you do next?

A.　Apply a tourniquet to prevent the bleeding from restarting.

B.　Apply a pressure bandage over the dressing.

C.　Remove any small foreign matter and apply antiseptic.

D.　Administer fluids to assist the body in replacing the lost blood.

KEY:　B

[4667]A person may operate an air compressor in which of the following areas on board a tank barge?

A.　Pumproom

B.　Generator room

C.　A space adjacent to a cargo tank

D.　A space two meters from a cargo valve

KEY:　B

[4668]A person who gets battery acid in an eye should IMMEDIATELY wash the eye with ______.

A.　boric acid solution

B.　water

C.　baking soda solution

D.　ammonia

KEY:　B

[4669]A person with diabetes has received a minor leg injured. The symptoms of the onset of a diabetic coma include ______.

A.　reduced appetite and thirst

B.　sneezing and coughing

C.　only a low grade fever

D.　slurred speech and loss of coordination

KEY:　D

[4670]A racetrack turn would be better than a Williamson turn in recovering a man overboard if ______.

A.　the man has been missing for a period of time

B.　the sea water is very cold and the man is visible

C.　there is thick fog

D.　the wind was from astern on the original course

KEY:　B

[4671]A rescuer can most easily determine whether or not an adult victim has a pulse by checking the pulse at the ______.

A.　carotid artery in the neck

B.　femoral artery in the groin

C.　brachial artery in the arm

D.　radial artery in the wrist

KEY:　A

[4672]A right-handed propeller will cause the survival craft to ______.

A.　walk the stern to starboard in reverse

B.　walk the stern to port in reverse

C.　run faster than a left-handed propeller

D.　right itself if capsized

KEY:　B

[4673]A shipmate chokes suddenly，cannot speak，and starts to turn blue. You should ______.

A.　perform the Heimlich maneuver

B.　make the victim lie down with the feet elevated to get blood to the brain

C.　immediately administer CPR

D.　do nothing until the victim becomes unconscious

KEY:　A

[4674]A towline should be fastened to ______.

A.　the chocks at the bow of a towed vessel

B.　the most forward，centermost point of a towed vessel such asa sturdy bow rail

C.　the mast of a towed sailboat

D.　a secure fitting near the bow of the towed vessel

KEY:　D

[4675]Aboard a survival craft，ether can be used to ______.

A.　start the engine in cold weather

B.　aid in helping personnel breathe

C.　prime the sprinkler system

D.　prime the air supply

KEY:　A

[4676]AN ILLNESS PREFERABLY OF AN INFECTIOUS NATURE SEIZING MORE THAN TWO PERSONS ON BOARD AT THE SAME TIME defines ______.

A.　Disease of Crew

B.　Crew illness

C.　Group illness

D.　Mass disease

KEY:　D

[4677]An obstruction on a helideck is any object that might present a hazard to the ______.

A.　rotor blades and landing gear

B.　unloading of passengers

C.　loading of cargo

D.　pilot's visibility

KEY:　A

[4678]Any vessel in need of carrying out deck washing must be ______ by the department concerned beforehand．

A.　requested

B.　allowed

C.　approved

D.　inquired

KEY:　C

[4679]For a ship not on an international voyage，an approved substitute for an impulse projected rocket-type line throwing appliance is a ______.

A.　spring loaded line thrower

B.　hand thrown buoyant line

C.　shoulder-type line throwing gun

D.　heaving line

KEY:　C

[4680]If the coxswain of your lifeboat gives the command HOLD WATER you should ______.

A.　complete the stroke，raise your oar slightly，swinging the oar slightly forward，and place it in the boat

B.　lift the oar in a vertical position

C.　complete the stroke and hold the oar out of the water

D.　dip the blade of your oar into the water vertically and hold it perpendicular to the keel line

KEY:　D

[4681]If you have to jump in the water when abandoning a vessel，your legs should be ______.

A.　spread apart as far as possible

B.　held as tightly against your chest as possible

C.　in a kneeling position

D.　extended straight down and crossed at the ankles

KEY:　D

[4682]If you must land on a beach with an oar-propelled lifeboat through a heavy surf，the recommended method is to ______.

A.　keep the bow directly in toward the beach，and tow the sea anchor off the stern

B.　ride in on the back of a large breaker

C.　keep the bow into the seas with the sea anchor out over the bow，and row to meet the breaking waves

D.　head directly into the beach by staying between the crests of the waves

KEY:　C

[4683]In an open lifeboat，the lifeboat compass is usually ______.

A.　placed in a fixed bracket when being used

B.　clamped to any position convenient for the coxswain to see it

C.　permanently mounted on the lifeboat's centerline

D.　mounted in the center of the boat to eliminate deviation

KEY:　A

[4684]In heavy seas the helmsman should steer the motor lifeboat ______.

A.　into the seas

B.　broadside to the seas

C.　in the same direction as the seas

D.　in a series of figure-eights

KEY:　A

[4685]In heavy seas you decide to heave to. The lifeboat should be ______.

A.　brought to a position with the stern into the seas

B.　allowed to take its own head

C.　brought to lay in the trough

D.　brought bow into the seas

KEY:　D

[4686]In which situation could a vessel most easily capsize?

A.　Running into head seas

B.　Running in the trough

C.　Running with following seas

D.　Anchored with your bow into the seas

KEY:　B

[4687]Inflatable liferafts are less maneuverable than lifeboats due to their ______.

A.　shape

B.　shallow draft

C.　large sail area

D.　All of the above

KEY:　D

[4688]Large volumes of carbon dioxide are safe and effective for fighting fires in enclosed spaces，such as in a pumproom，provided that the ______.

A.　persons in the space wear gas masks

B.　persons in the space wear damp cloths over their mouths and nostrils

C.　ventilation system is secured and all persons leave the space

D.　ventilation system is kept operating

KEY:　C

[4689]Multiple fire pumps may be used for other purposes provided that one pump is ______.

A.　on line to the fire main

B.　kept available for use on the fire main at all times

C.　capable of being connected to the fire main

D.　rated at or above 125 psi

KEY:　B

[4690]Oil fires are best extinguished by ______.

A.　cutting off the supply of oxygen

B.　removing the fuel

C.　cooling below the ignition temperature

D.　spraying with water

KEY:　A

[4691]On a rigid liferaft which is equipped with all of the required equipment you may NOT find a ______.

A.　boathook

B.　fishing kit

C.　lifeline or grab rail

D.　sea painter

KEY:　B

[4692]On a ship，a fire pump may be used for other purposes if ______.

A.　the other services are run off a reducing station with a pressure gage

B.　a separate fire pump is available for use on the fire main

C.　all other services are operated by a manifold near the pump

D.　All of the above

KEY:　B

[4693]On a vessel，if someone fell overboard and you did not know over which side the person fell，you should ______.

A.　immediately reverse the engines

B.　stop the propellers from turning and throw a ring buoy over the side

C.　increase speed to full to get the vessel away from the person

D.　first put the rudder hard over in either direction

KEY:　B

[4694]Releasing oil from the sea anchor of a lifeboat may ______.

A.　keep the propeller from being fouled

B.　increase propeller speed

C.　help calm the waves in the vicinity of the craft

D.　increase the holding power of the sea anchor

KEY:　C

[4695]Seasickness is caused by rolling or rocking motions which affect fluids in the ______.

A.　stomach

B.　lower intestines

C.　inner ear

D.　bladder

KEY:　C

[4696]Separation cloths may be used to ______.

A.　absorb moisture from hygroscopic cargoes

B.　fill gaps between layers of cargo

C.　wrap cargo that leaks from packaging

D.　keep bagged cargo leakage from contacting the deck

KEY:　D

[4697]Severe exposure to chlorine gas can be fatal. Chlorine gas is primarily a ______.

A.　respiratory irritant

B.　skin burning agent

C.　blood poisoning agent

D.　nerve paralyzing irritant

KEY:　A

[4698]Steering a motor lifeboat broadside to the sea could cause it to ______.

A.　capsize

B.　run smoother

C.　run faster

D.　sink

KEY:　A

[4699]Steering a survival craft broadside to the sea could cause it to ______.

A.　capsize

B.　run smoother

C.　run faster

D.　sink

KEY:　A

[4700]Survival practice in the mooring system is to slack off the tensions on the leeward side and ______.

A.　deballast the unit to transit draft

B.　adjust as evenly as practical the windward tensions

C.　release the anchors on the windward side

D.　tighten the anchor buoys on the leeward side anchors

KEY:　B

18.1 燃烧与爆炸

[4701]The emergency signal for fires is sounded on the ship's whistle and general alarm as ______ ．

A.　a continuous ringing for 10 seconds

B.　one short ring followed by one long ring

C.　two long rings of at least 20 seconds

D.　a continuous ringing until the fire is extinguished

KEY:　A

[4702]A carbon dioxide fire extinguisher should be recharged ______.

A.　at least annually

B.　whenever it is below its required weight

C.　only if the extinguisher has been used

D.　before every safety inspection

KEY:　B

[4703]A carburetor is required to have a safety device called a(n) ______.

A.　pressure release

B.　backfire flame arrestor

C.　automatic shut off

D.　flow valve

KEY:　B

[4704]A Certificate of Inspection issued to a small passenger vessel describes ______.

A.　the mimimum fire extinguishing equipment，lifejackets，survival and rescue craft she must carry

B.　the name of the managing operator

C.　any special conditions or restrictions on her operation

D.　All the above

KEY:　D

[4705]A class B fire is most successfully fought by ______.

A.　preventing oxygen from reaching the burning material

B.　cooling the burning material below its ignition temperature

C.　using the extinguishing agent to make the burning material fire-resistant

D.　using the extinguishing agent to absorb the heat

KEY:　A

[4706]A class C fire would be burning ______.

A.　fuel oil

B.　wood

C.　celluloid

D.　electrical insulation

KEY:　D

[4707]A combustible gas indicator meter is calibrated to read the percentage of ______.

A.　vapor to oxygen

B.　the flammable limit concentration

C.　the autoignition concentration

D.　the lower explosive limit concentration

KEY:　D

[4708]A combustible gas indicator will NOT operate correctly when the ______.

A.　hydrocarbon content of the atmosphere exceeds the U.E.L

B.　atmosphere is deficient in oxygen

C.　distance between the operator and the compartment to be tested is greater than 50 feet

D.　All of the above

KEY:　B

[4709]A cutoff valve in the fire-main system of a ship may be closed to protect the portion of the system on an exposed deck from ______.

A.　damage from crane operations

B.　being used for wash down purposes

C.　accidental diversion of flow to wrong location

D.　freezing

KEY:　D

[4710]A definite advantage of using water as a fire extinguishing agent is its characteristic of ______.

A.　alternate expansion and contraction as water in a liquid state becomes a vapor

B.　absorption of smoke and gases as water is converted from a liquid to a vapor

C.　rapid contraction as water is converted from a liquid to a vapor

D.　rapid expansion as water absorbs heat and changes to steam

KEY:　D

[4711]A fire has broken out on the stern of your vessel. You should maneuver your vessel so the wind ______.

A.　blows the fire back toward the vessel

B.　comes over the bow

C.　comes over the stern

D.　comes over either beam

KEY:　B

[4712]A fire hose has a ______.

A.　male coupling at both ends

B.　female coupling at both ends

C.　female coupling at the nozzle end and a male coupling at the hydrant end

D.　male coupling at the nozzle end and a female coupling at the hydrant end

KEY:　D

[4713]A fire in a ballast pumproom can be brought under control with minimal impact on stability by ______.

A.　cooling the outside bulkheads with water

B.　shutting all sources of air into the compartment

C.　closing the sea chest

D.　flooding the compartment with salt water

KEY:　B

[4714]A fire in electrical equipment should be extinguished by using ______.

A.　salt water

B.　foam

C.　low-velocity fog

D.　CO2

KEY:　D

[4715]A fire is discovered in the forepeak of a vessel at sea. The wind is from ahead at 35 knots. You should ______.

A.　remain on course and hold speed

B.　change course and put the stern to the wind

C.　change course to put the wind on either beam and increase speed

D.　remain on course but slack the speed

KEY:　B

[4716]A fire main system must have enough fire hydrants so that each accessible space may be sprayed with ______.

A.　a low velocity spray applicator

B.　a water spray or solid stream

C.　at least two spray patterns of water

D.　at least 25 psi delivered pressure

KEY:　C

[4717]A fire must be ventilated ______.

A.　when using an indirect attack on the fire such as flooding with water

B.　to prevent the gases of combustion from surrounding the firefighters

C.　to minimize heat buildup in adjacent compartments

D.　if compressed gas cylinders are stowed in the compartment on fire

KEY:　B

[4718]A fire pump may be used for other purposes if ______.

A.　the other services are run off a reducing station with a pressure gage

B.　one of the required pumps is kept availabe for use on the fire main system at all times

C.　no relief valves are installed

D.　all of the above conditions are met

KEY:　B

[4719]A fire starting by spontaneous combustion can be expected in which condition? ______.

A.　Paints，varnish，or other liquid flammables are stowed in a dry stores locker.

B.　Inert cargoes such as pig iron are loaded in a wet condition.

C.　Oily rags are stowed in a metal pail.

D.　Clean mattresses are stored in contact with an electric light bulb.

KEY:　C

[4720]According to the T-Boat regulations the reason for providing adequate ventilation for a battery storage area is to prevent ______.

A.　accumulation of carbon dioxide gas that chokes the battery

B.　accumulation of explosive and toxic gases the battery can generate

C.　mildew or dry rot in the battery box

D.　battery failure including battery case meltdown caused by excessive heat

KEY:　B

[4721]Automatic fire dampers in ventilation systems are operated by a ______.

A.　remote operated valve

B.　fusible link

C.　CO2 system pressure switch

D.　heat or smoke detector

KEY:　B

[4722]Automatic fire dampers in ventilation systems are operated by use of ______.

A.　remotely operated valves

B.　fusible links

C.　C02 system pressure switches

D.　heat or smoke detectors

KEY:　B

[4723]Backfire flame arrestors are installed on ______.

A.　fuel tanks

B.　spark plugs

C.　carburetors

D.　distributors

KEY:　C

[4724]Before a tank is to be crude oil washed，the oxygen content in the tank must be measured at a position ______.

A.　immediately above the level of the oil

B.　at the top of the tank

C.　in the vent riser

D.　one meter from the deck

KEY:　D

[4725]Before using a fixed CO2 system to fight an engine room fire，you must ______.

A.　secure the engine room ventilation

B.　secure the machinery in the engine room

C.　evacuate all engine room personnel

D.　All of the above

KEY:　D

[4726]Blocking open or removing fire dampers can cause ______.

A.　fixed foam systems to be ineffective

B.　faster cooling of the fire

C.　the accumulation of explosive gases

D.　the fire to spread through the ventilation system

KEY:　D

[4727]Cargo transfer operations on a tanker need NOT be stopped when ______.

A.　a tug comes alongside while the tanker is loading grade D and E cargoes

B.　a large，fresh oil spill is discovered immediately adjacent to the side of the tanker

C.　there is an electrical storm in the vicinity

D.　there is a fire on the dock or on a nearby vessel

KEY:　A

[4728]CO2 extinguishes a fire by ______.

A.　cooling

B.　smothering

C.　chemical action

D.　All of the above

KEY:　B

[4729]Combustible gas indicators measure the presence of combustible gas as a percentage of the ______.

A.　flash point

B.　upper explosive limit

C.　lower explosive limit

D.　fire point

KEY:　C

[4730]Control of fire on a ship should be addressed ______.

A.　immediately after restoring vital services

B.　immediately

C.　following control of flooding

D.　following establishment of fire boundaries

KEY:　B

[4731]Control of flooding should be addressed ______.

A.　first

B.　following control of fire

C.　following restoration of vital services

D.　only if a threat exists

KEY:　B

[4732]Controls for a fixed carbon dioxide system shall be mounted ______.

A.　directly outside the space protected by the system

B.　as near the gas cylinders as possible

C.　in the pilothouse

D.　on the main deck near the bow

KEY:　A

[4733]Convection spreads a fire by ______.

A.　heated gases flowing through ventilation systems

B.　the transfer of heat across an unobstructed space

C.　burning liquids flowing into another space

D.　transmitting the heat of a fire through the ship's metal

KEY:　A

[4734]Dry chemical extinguishers extinguish class B fires to the greatest extent by ______.

A.　cooling

B.　smothering

C.　oxygen dilution

D.　breaking the chain reaction

KEY:　D

[4735]Dry chemical extinguishers may be used on what class of fires?

A.　A only

B.　B only

C.　B and C only

D.　A，B or C as marked on the extinguisher

KEY:　D

[4736]Dry chemical fire extinguishers are effective on which type(s) of fire?

A.　Burning oil

B.　Electrical

C.　Paint

D.　All of the above

KEY:　D

[4737]Due to the hazards involved with Halon extinguishers on a ship，the size II extinguisher may only be used ______.

A.　outside

B.　on class C fires

C.　in an emergency

D.　on class B fires

KEY:　A

[4738]During a fire drill on a ship，what action is required?

A.　Start each fire pump

B.　Launch and run a lifeboat

C.　Inventory rescue and fire equipment

D.　Inspect fire hoses

KEY:　A

[4739]During a training exercise a submarine indicating that a torpedo has been fired will send up smoke from a float. The smoke's color will be ______.

A.　black

B.　red

C.　orange

D.　yellow

KEY:　A

[4740]During the required periodic abandon ship drill aboard a ship，each person not assigned duties in the muster list is ______.

A.　instructed in the use of portable fire extinguishers

B.　shown a video demonstrating lifeboat launching

C.　instructed in the use of life jackets

D.　not required to attend the boat drill

KEY:　C

[4741]Each fire hose coupling on a ship must have threads that meet the specifications of the ______.

A.　American Petroleum Institute

B.　National Standard Firehose Coupling

C.　American Society of Mechanical Engineers

D.　Underwriter's Laboratories，Inc.

KEY:　B

[4742]Each fire hydrant must have at least one spanner and at least one ______.

A.　hose rack or reel

B.　all purpose nozzle

C.　foam applicator

D.　pick axe

KEY:　A

[4743]Each fire hydrant serving machinery spaces containing oil fired boilers，internal combustion machinery，or oil fuel units must be equipped with a ______.

A.　fireman's outfit

B.　low-velocity spray applicator

C.　marine strainer

D.　pick axe

KEY:　B

[4744]Each fire pump on a ship must have a pressure gauge located at ______.

A.　the pump discharge

B.　the manifold connection

C.　each fire station discharge

D.　the pump station

KEY:　A

[4745]Each fireman's outfit and its spare equipment on a vessel must be stowed ______.

A.　in a locked cabinet in the machinery space

B.　in an unlocked cabinet in the machinery space

C.　in a separate and accessible location

D.　at a fire hydrant location

KEY:　C

[4746]Each hand portable fire extinguisher must be marked with ______.

A.　the name of the vessel on which it is located

B.　the date that it was installed

C.　the names of the individuals qualified to use it

D.　an identification number

KEY:　D

[4747]Each hose in the fuel transfer system for helicopter refueling must meet the standards of the ______.

A.　Federal Aviation Administration

B.　Corps of Engineers

C.　National Fire Protection Association

D.　National Transportation Safety Board

KEY:　A

[4748]Each part of the fire-main system located on an exposed deck must be ______.

A.　protected against freezing

B.　locked to prevent theft

C.　numbered sequentially

D.　pressurized at all times

KEY:　A

[4749]If，during helicopter refueling operations，fuel is spilled on clothing，the person should first ______.

A.　see the medic immediately

B.　spray himself with foam or CO2

C.　complete his task and then see the medic

D.　remove the clothing and wash

KEY:　D

[4750]On a hydrocarbon flammability chart the line which extends from 0% to 21.8% oxygen，lying tangent to the flammability range，is called the ______.

A.　minimum oxygen content line

B.　critical displacement line

C.　critical dilution line

D.　upper threshold limit

KEY:　C

18.2 探火与消防设备

[4751]A cabinet or space containing the controls or valves for the fixed firefighting system must be ______.

A.　posted with instructions on the operation of the system

B.　ventilated and equipped with explosion-proof switches

C.　painted with red and black diagonal stripes

D.　equipped with a battery powered source of emergency lighting

KEY:　A

[4752]Which statement is TRUE concerning spare charges for portable fire extinguishers on unmanned tank barges? ______.

A.　Spare charges shall be carried for at least 50 percent of each size and variety required on board.

B.　If the unit cannot be charged by the vessel's personnel，one spare unit shall be carried in lieu of spare charges.

C.　Spare charges shall be stowed in watertight containers on deck.

D.　Regulations concerning spare charges do not apply to unmanned barges.

KEY:　D

[4753]No outlet on a fire hydrant may point above the horizontal in order to ______.

A.　avoid kinking the hose

B.　avoid personal injury during connection

C.　make connecting easier

D.　prevent spray on electrical equipment

KEY:　A

[4754]The danger of a charged hose left unattended on deck with the nozzle open is ______.

A.　the hose could burst

B.　the nozzle end will whip about causing damage or injury

C.　water damage to vessel's cargo or structure

D.　personnel might trip over the hose

KEY:　B

[4755]______，as the chemical extinguisher agent，should be used for an electric fire．

A.　dry chemical or foam

B.　foam or soda acid

C.　carbon dioxide or foam

D.　carbon dioxide or dry chemical

KEY:　D

[4756]A branch line valve of a fire extinguishing system on a ship must be marked with the ______.

A.　maximum pressure allowed at that branch

B.　name of the space or spaces which it serves

C.　date of the last maintenance inspection

D.　pressure needed to maintain an effective stream at that point

KEY:　B

[4757]A CO2 extinguisher on a ship which has lost 10% of its charge must be ______.

A.　used at the earliest opportunity

B.　hydrotested

C.　recharged

D.　weighed again in one month

KEY:　C

[4758]A CO2 portable extinguisher is annually checked by ______.

A.　reading the gage pressure

B.　weighing the extinguisher

C.　discharging a small amount of CO2

D.　seeing if the seal has been broken

KEY:　B

[4759]A combination or all-purpose nozzle produces ______.

A.　low-velocity fog only

B.　a solid stream only

C.　a solid stream and foam

D.　a solid stream and fog

KEY:　D

[4760]A fire starts on your vessel while refueling. You should FIRST ______.

A.　stop the ventilation

B.　sound the general alarm

C.　determine the source of the fire

D.　attempt to extinguish the fire

KEY:　B

[4761]A fixed CO2 system on a ship with a capacity of over 300 lbs (136 kilograms) CO2 which protects spaces other than tanks must have ______.

A.　two or more releasing stations

B.　automatic release in event of a fire

C.　an audible alarm and time delay

D.　an audible and visible alarm

KEY:　C

[4762]A foam-type portable fire extinguisher is most useful in fighting a fire in ______.

A.　generators

B.　oil drums

C.　the bridge controls

D.　combustible metals

KEY:　B

[4763]A foam-type portable fire extinguisher would be most useful in combating a fire in ______.

A.　solid materials such as wood or bales of fiber

B.　flammable liquids

C.　a piece of electrical equipment

D.　combustible metallic solids

KEY:　B

[4764]A fuel line breaks，sprays fuel on the hot exhaust manifold，and catches fire. Your FIRST action should be to ______.

A.　batten down the engine room

B.　start the fire pump

C.　apply carbon dioxide to the fire

D.　shut off the fuel supply

KEY:　D

[4765]A galley grease fire on the stove may be extinguished using ______.

A.　water

B.　foam

C.　the range hood extinguishing system

D.　fire dampers

KEY:　C

[4766]A galley grease fire would be classified as which class of fire?

A.　A

B.　B

C.　C

D.　D

KEY:　B

[4767]A GROUP OF CREW MEMBERS TRAINED FOR FIGHTING FLOODING IN THE VESSEL are known as ______.

A.　Fire party

B.　Deck cadets

C.　Research team

D.　Damage control team

KEY:　D

[4768]A large oil fire on the deck of a ship can be fought most effectively with ______.

A.　dry chemical

B.　foam

C.　high-velocity fog

D.　Water (cartridge-operated)

KEY:　B

[4769]A marine chemist issues gas free certificates and is certified by the ______.

A.　Mine Safety Appliance Association

B.　American Chemical Society

C.　Marine Chemists Association

D.　National Fire Protection Association

KEY:　D

[4770]A passenger vessel is required to have a supervised patrol when ______.

A.　there are passengers berthed on board

B.　navigating in excess of eight hours

C.　the vessel has substantial wood in its construction

D.　there is no automatic fire-detection system installed

KEY:　A

[4771]A portable dry chemical fire extinguisher discharges by ______.

A.　gravity when the extinguisher is turned upside down

B.　pressure from a small CO2 cartridge on the extinguisher

C.　air pressure from the hand pump attached to the extinguisher

D.　pressure from the reaction when water is mixed with the chemical

KEY:　B

[4772]A portable foam (stored-pressure type) fire extinguisher would be most useful in combating a fire in ______.

A.　generators

B.　oil drums

C.　the bridge controls

D.　combustible metals

KEY:　B

[4773]A ship helicopter landing deck on which fueling operations are conducted must have a fire protection system that is capable of discharging at 100 psi pressure a foam spray of at least ______.

A.　30 gallons per minute

B.　40 gallons per minute

C.　50 gallons per minute

D.　60 gallons per minute

KEY:　C

[4774]A ship must have a self-contained breathing apparatus to be used as protection from gas leaking from a refrigeration unit. To meet this requirement，you may use ______.

A.　a gas mask certified by the Mine Safety and Health Administration

B.　the same self-contained breathing apparatus required with the fireman's outfit

C.　an oxygen breathing apparatus，provided that the device has been inspected within three years

D.　a portable ventilation system that will provide a complete change of air every three minutes

KEY:　B

[4775]A small fuel spillage has occurred during helicopter refueling. After the leak has been stopped and fire-control personnel have been notified and are standing by，the next step is to ______.

A.　proceed with fueling operation

B.　have helicopter move to a safer location

C.　wash spilled fuel away with a flood of water

D.　clean spilled fuel with rags

KEY:　C

[4776]A spanner is a ______.

A.　cross connection line between two main fire lines

B.　special wrench for tightening couplings in a fire hose line

C.　tackle rigged to support a fire hose

D.　None of the above

KEY:　B

[4777]A spring line is ______.

A.　any wire rope used for mooring

B.　a fire-warp

C.　a mooring line running diagonally to the keel

D.　a mooring line perpendicular to the keel

KEY:　C

[4778]A squeeze-grip type carbon dioxide portable fire extinguisher has been partially discharged. It should be ______.

A.　labeled empty and recharged as soon as possible

B.　replaced in its proper location if weight loss is no more than 25%

C.　replaced in its proper location regardless of weight

D.　replaced in its proper location if weight loss is no more than 15%

KEY:　A

[4779]A vessel is constructed with a steel hull and an aluminum superstructure. Which statement is TRUE?

A.　The aluminum will provide greater resistance to the spread of fire by conduction.

B.　The aluminum structure is usually attached to a steel coaming by a method that insulates the two metals.

C.　If the superstructure is stressed，an aluminum structure requires additional expansion joints to prevent fracture.

D.　The steel at the area of the aluminum-to-steel connection must be closely checked for galvanic corrosion.

KEY:　B

[4780]According to the regulations，what fire safety control feature is required in quick-closing shut off valves?

A.　Electrical cut off switch

B.　A fusible link

C.　Manual cut off switch

D.　A water spray actuator

KEY:　B

[4781]After extinguishing a fire with CO2，it is advisable to ______.

A.　use all CO2 available to cool the surrounding area

B.　stand by with water or other agents

C.　thoroughly ventilate the space of CO2

D.　jettison all burning materials

KEY:　B

[4782]After extinguishing a paint locker fire using the fixed CO2 system，the next action is to have the space ______.

A.　opened and burned material removed

B.　left closed with vents off until all boundaries are cool

C.　checked for oxygen content

D.　doused with water to prevent reflash

KEY:　B

[4783]All of the following are part of the fire triangle EXCEPT ______.

A.　electricity

B.　fuel

C.　oxygen

D.　heat

KEY:　A

[4784]All portable fire extinguishers must be capable of being ______.

A.　carried by hand to a fire

B.　carried or rolled to a fire

C.　recharged in the field

D.　used on class 'B' fires

KEY:　A

[4785]An ABC dry chemical fire extinguisher would be LEAST effective against a fire in ______.

A.　a mattress

B.　spilled liquids such as oil or paint

C.　high voltage electrical gear

D.　a trash can

KEY:　A

[4786]An advantage of a dry chemical over a carbon dioxide fire extinguisher is its ______.

A.　greater range

B.　effectiveness on all types of fires

C.　cleanliness

D.　All of the above

KEY:　A

[4787]An aircraft has indicated that he wants you to change course and follow him. You cannot comply because of an emergency on board. Which signal should you make?

A.　Fire a red flare at night or a red smoke signal by day

B.　Send the Morse signal N by flashing light

C.　Make a round turn (360) and resume course

D.　Make an S turn (hard right then hard left) and resume course

KEY:　B

[4788]An effective method for moving patients with spinal injuries onto a spine board is known as the ______.

A.　pack-strap carry

B.　two man extremities carry

C.　fireman's drag

D.　four man log roll

KEY:　D

[4789]An engine compartment gasoline fire requires which type of extinguisher? ______.

A.　Carbon dioxide

B.　Dry chemical

C.　Foam

D.　All of the above

KEY:　D

[4790]An extinguishing agent which effectively cools，dilutes combustible vapors，removes oxygen，and provides a heat and smoke screen is ______.

A.　carbon dioxide

B.　Halon 1301

C.　dry chemical

D.　water fog

KEY:　D

[4791]An important step in fighting any electrical fire is to ______.

A.　stop ventilation

B.　stop the vessel

C.　de-energize the circuit

D.　apply water to extinguish the fire

KEY:　C

[4792]An inert gas system is designed to reduce the possibility of tank explosions by ______.

A.　eliminating sparks and fire in the vicinity of cargo tanks

B.　removing all hydrocarbon gases from the cargo tanks

C.　blanketing cargo tanks with inert foam

D.　reducing the oxygen concentration below levels necessary for combustion

KEY:　D

[4793]An inert gas system on a tanker should be used to ______.

A.　prevent the generation of flammable or combustible gas in tanks

B.　blow out cargo lines to prevent gas concentrations

C.　dilute tank atmospheres to keep gas concentrations below the lower explosive limit

D.　prevent fires in the pumproom by continually displacing flammable vapors

KEY:　C

[4794]Any extinguishing agent used on a Class C fire must have which important property?

A.　Cooling ability

B.　Leaves no residue

C.　Penetrating power

D.　Nonconductivity

KEY:　D

[4795]Any firefighting equipment that is carried in addition to the minimum required must ______.

A.　meet the applicable standards

B.　be marked as additional equipment

C.　be stowed in a separate area

D.　All of the above

KEY:　A

[4796]Any ship damaging a pipe could ______ an immediate danger of fire．

A.　meet

B.　face

C.　be in font of

D.　follow

KEY:　B

[4797]As an extinguishing agent，foam ______.

A.　conducts electricity

B.　should be directed at the base of the fire

C.　is most effective on burning gases which are flowing

D.　extinguishes by cooling oil fires below ignition temperature

KEY:　A

[4798]As compared to carbon dioxide，dry chemical has which advantage? ______.

A.　Cleaner

B.　Effective on metal fires

C.　Greater range

D.　More cooling effect

KEY:　C

[4799]At night while the ship is tied up to the dock，a fire breaks out in a cargo hatch. After sounding the alarm，what should the person on watch do?

A.　Begin fighting the fire immediately.

B.　Await further orders from the Master.

C.　Let go the lines to let the vessel drift to an anchorage.

D.　Send a person to summon the shore authorities.

KEY:　D

[4800]At the required fire drill conducted aboard a ship，all persons must report to their stations and demonstrate their ability to perform the duties assigned to them ______.

A.　by the toolpusher

B.　in the station bill

C.　by the person conducting the drill

D.　at the previous safety meeting

KEY:　B

18.3 消防演习与作业

[4801]Accumulations of oily rags should be ______.

A.　kept in nonmetal containers

B.　discarded as soon as possible

C.　cleaned thoroughly for reuse

D.　kept in the paint locker

KEY:　B

[4802]Records of tests and inspections of a cargo vessel's fire extinguishing systems shall be ______ on board until the vessel's Certificate of Inspection expires.

A.　consealed

B.　detained

C.　contained

D.　kept

KEY:　D

[4803]A fifteen-pound CO2 extinguisher ______.

A.　contains 15 pounds of CO2

B.　weighs 15 pounds when full of CO2

C.　has 15 pounds of pressure at the nozzle

D.　weighs 15 pounds when empty

KEY:　A

[4804]A fire in a ballast pumproom can be brought under control with minimal impact on stability by ______.

A.　cooling the outside bulkheads with water

B.　shutting all sources of air into the compartment

C.　closing the sea chest

D.　flooding the compartment with salt water

KEY:　B

[4805]A fire in the galley ALWAYS poses the additional threat of ______.

A.　contaminating food with extinguishing agent

B.　spreading through the engineering space

C.　causing loss of stability

D.　a grease fire in the ventilation system

KEY:　D

[4806]A safety outlet is provided on the CO2 discharge piping to prevent ______.

A.　over pressurization of the space being flooded

B.　rupture of cylinder due to temperature increase

C.　over pressurization of the CO2 discharge piping

D.　flooding of a space where personnel are present

KEY:　C

[4807]Actuating the CO2 fixed system causes the shutdown of the ______.

A.　fuel supply

B.　exhaust ventilation

C.　supply and exhaust ventilation

D.　mechanical and natural ventilation

KEY:　C

[4808]After each reading of an oxygen indicator，the instrument should be purged with ______.

A.　CO2

B.　fresh air

C.　the tested compartment's air

D.　water

KEY:　B

[4809]After using a CO2 extinguisher on a ship，it should be ______.

A.　put back in service if some CO2 remains

B.　hydrostatically tested

C.　retagged

D.　recharged

KEY:　D

[4810]An inflatable liferaft should be lifted back aboard the ship by using ______.

A.　the single hook at the top of the raft

B.　two lines passed under the raft

C.　the towing bridle

D.　All of the above

KEY:　C

[4811]Automatic mechanical ventilation shutdown is required for CO2 systems protecting the ______.

A.　machinery spaces

B.　cargo compartments

C.　living quarters

D.　galley

KEY:　A

[4812]CO2 cylinders equipped with pressure actuated discharge heads will discharge automatically when ______.

A.　the discharge valve is open

B.　the control box glass is broken

C.　pressure from the control cylinders is detected

D.　the control cylinders have been completely discharged

KEY:　C

[4813]CO2 cylinders，which protect the small space in which they are stored must ______.

A.　NOT contain more than 200 pounds of CO2

B.　be automatically operated by a heat actuator

C.　have an audible alarm

D.　All of the above

KEY:　B

[4814]Control valves of a CO2 system may be located within the protected space when ______.

A.　it is impractical to locate them outside

B.　there is also a control valve outside

C.　the CO2 cylinders are also in the space

D.　an automatic heat-sensing trip is installed

KEY:　C

[4815]Except in rare cases，it is impossible to extinguish a shipboard fire by ______.

A.　removing the heat

B.　removing the oxygen

C.　removing the fuel

D.　interrupting the chain reaction

KEY:　C

[4816]Fire alarm system thermostats are actuated by ______.

A.　smoke sensors

B.　the difference in thermal expansion of two dissimilar metals

C.　pressure loss due to air being heated

D.　an electric eye which actuates when smoke interferes with the beam

KEY:　B

[4817]Fire and abandon ship stations and duties may be found on the ______.

A.　crewman's duty list

B.　Certificate of Inspection

C.　shipping articles

D.　muster list

KEY:　D

[4818]Fire axes required on MV ship's must be stored in the enclosure for fire hoses with the location marked ______.

A.　Fire Axe Location

B.　Hose Station No.…

C.　Fire Station No.…

D.　Firefighting Equipment

KEY:　C

[4819]Fire dampers prevent the spread of fire by ______.

A.　conduction

B.　convection

C.　radiation

D.　direct contact

KEY:

[4820]Fire extinguishers of sizes III，IV，and V are designated as ______.

A.　portable

B.　semi-portable

C.　fixed

D.　disposable

KEY:　B

[4821]Fire extinguishers on inspected vessels are numbered by size I through V，with I being ______.

A.　used for electrical fires only

B.　the smallest

C.　the most accessible

D.　the most effective

KEY:　B

[4822]Fire extinguishing agents used on Class C fires must be ______.

A.　able to absorb heat

B.　water based

C.　nonconducting

D.　nontoxic

KEY:　C

[4823]Fire fighting equipment requirements for a particular vessel may be found on the ______.

A.　4Certificate of Inspection

B.　Certificate of Seaworthiness

C.　Classification Certificate

D.　Certificate of Registry

KEY:　A

[4824]Fire hose couplings ______.

A.　are made of bronze，brass，or soft alloy metals

B.　should be painted red in order to identify hose lengths

C.　are specially hardened to prevent crushing

D.　should be greased frequently

KEY:　A

[4825]Fire hose should be washed with ______.

A.　salt water and a wire brush

B.　caustic soap

C.　mild soap and fresh water

D.　a holystone

KEY:　C

[4826]Fire in an engine compartment is best extinguished with carbon dioxide gas (CO2) and by ______.

A.　closing the compartment except for the ventilators

B.　completely closing the compartment

C.　leaving the compartment open to the air

D.　increasing the air flow to the compartment by blowers

KEY:　B

[4827]Fire may be spread by which means? ______.

A.　Conduction of heat to adjacent surfaces

B.　Direct radiation

C.　Convection

D.　All of the above

KEY:　D

[4828]Firefighting foam is only effective when the foam ______.

A.　penetrates to the bottom of the fire

B.　is kept saturated with low-velocity water fog

C.　mixes with the burning fuel oil

D.　completely covers the top of the burning liquid

KEY:　D

[4829]Fires are grouped into what categories?

A.　Class A，B，C，and D

B.　Type 1，2，3，and 4

C.　Combustible solids，liquids，and gases

D.　Flammable solids，liquids，and gases

KEY:　A

[4830]Fixed CO2 systems would not be used on crew's quarters or ______.

A.　the paint locker

B.　spaces open to the atmosphere

C.　cargo holds

D.　the engine room

KEY:　B

[4831]Flame screens are used to ______.

A.　contain flammable fumes

B.　protect firefighters from flames

C.　prevent flames from entering tanks

D.　keep flames and sparks from getting out of an engine's exhaust system

KEY:　C

[4832]Flames from small leaks of LFG may be extinguished by ______.

A.　utilizing carbon dioxide or dry chemical fire extinguishers

B.　utilizing soda and acid fire extinguishers

C.　blowing the flames out

D.　letting it burn itself out

KEY:　D

[4833]Foam extinguishes a fire by ______.

A.　shutting off the air supply

B.　cooling the fuel to below ignition temperature

C.　dispersing the fuel

D.　removing the source of ignition

KEY:　A

[4834]Foam extinguishes a fire by ______.

A.　smothering the burning material

B.　chemical combination with burning material

C.　absorbing the burning material

D.　organic destruction of the burning material

KEY:　A

[4835]Foam extinguishes a fire mainly by ______.

A.　cooling

B.　chemical action

C.　smothering

D.　inerting the air

KEY:　C

[4836]Foam is a very effective smothering agent and ______.

A.　it provides cooling as a secondary effect

B.　works well on extinguishing electrical fires

C.　can be used to combat combustible metal fires

D.　All of the above

KEY:　A

[4837]Foam-type portable fire extinguishers are most useful in combating fires involving ______.

A.　solid materials such as wood or bales of fiber

B.　inflammable liquids

C.　electrical equipment

D.　metallic solids

KEY:　B

[4838]Fusible-link fire dampers are operated by ______.

A.　8the heat of a fire melting the link

B.　a break-glass and pull-cable system

C.　electrical controls on the bridge

D.　a mechanical arm outside the vent duct

KEY:　A

[4839]Gas masks ______.

A.　should be worn while fighting a fire

B.　can be used in atmospheres deficient in oxygen

C.　filter contaminants from air that is to be breathed

D.　may be substituted for a self-contained breathing apparatus

KEY:　C

[4840]Generally，the first action in extinguishing an LFG fire caused by escaping gas is to ______.

A.　sweep flames away with water spray

B.　shut off the leak

C.　use a chemical foam fire extinguisher

D.　call the local fire department

KEY:　B

[4841]Good housekeeping on a vessel prevents fires by ______.

A.　allowing better access in an emergency

B.　eliminating potential fuel sources

C.　eliminating trip hazards

D.　improving personnel qualifications

KEY:　B

[4842]Halon extinguishers used on a class C fire should be directed at the ______.

A.　base of the equipment

B.　top of the equipment

C.　power source

D.　source of the fire

KEY:　D

[4843]Halon extinguishes a fire by ______.

A.　breaking the chain reaction

B.　smothering the fire

C.　cooling the fire

D.　coating the fuel with a nonflammable surface

KEY:　A

[4844]Halon extinguishes fire primarily by ______.

A.　cooling

B.　smothering

C.　shielding of radiant heat

D.　breaking the chain reaction

KEY:　D

[4845]Halon fire extinguishers are NOT effective when used on which types of fires?

A.　Fires in electrical equipment

B.　Flammable oils and greases

C.　Class A fires in ordinary combustibles

D.　Materials containing their own oxygen

KEY:　D

[4846]Halon from extinguishers used on a class B fire，should be directed ______.

A.　at the top of the flames

B.　at the base of the fire near the edge

C.　in short quick bursts

D.　toward the upwind side of the fire

KEY:　B

[4847]High-velocity fog ______.

A.　is a finer，more diffuse water spray than low-velocity fog

B.　requires that the water pressure be no greater than 60 psi

C.　produces an effective fog pattern no more than 6 feet beyond the nozzle

D.　extinguishes a fire by absorbing heat and reducing the supply of oxygen

KEY:　D

[4848]How do you operate a portable CO2 fire extinguisher? ______.

A.　Point the horn down.

B.　Turn cylinder upside-down.

C.　Break the rupture disc.

D.　Pull pin，squeeze grip.

KEY:　D

[4849]How does foam extinguish an oil fire? ______.

A.　By cooling the oil below the ignition temperature

B.　By removing the fuel source from the fire

C.　By excluding the oxygen from the fire

D.　By increasing the weight of the oil

KEY:　C

[4850]How is the external flotation bladder of an immersion suit inflated?

A.　It is inflated by a small CO2 bottle that is automatically tripped when the front zipper is at the top of the zipper track.

B.　It is inflated by a small CO2 bottle that is manually tripped.

C.　It is inflated by blowing through an inflation tube.

D.　It inflates by sea water bleeding into the flotation bladder and reacting with a chemical therein.

KEY:　C

18.1 燃烧与爆炸

[4701]The emergency signal for fires is sounded on the ship's whistle and general alarm as ______ ．

A.　a continuous ringing for 10 seconds

B.　one short ring followed by one long ring

C.　two long rings of at least 20 seconds

D.　a continuous ringing until the fire is extinguished

KEY:　A

[4702]A carbon dioxide fire extinguisher should be recharged ______.

A.　at least annually

B.　whenever it is below its required weight

C.　only if the extinguisher has been used

D.　before every safety inspection

KEY:　B

[4703]A carburetor is required to have a safety device called a(n) ______.

A.　pressure release

B.　backfire flame arrestor

C.　automatic shut off

D.　flow valve

KEY:　B

[4704]A Certificate of Inspection issued to a small passenger vessel describes ______.

A.　the mimimum fire extinguishing equipment，lifejackets，survival and rescue craft she must carry

B.　the name of the managing operator

C.　any special conditions or restrictions on her operation

D.　All the above

KEY:　D

[4705]A class B fire is most successfully fought by ______.

A.　preventing oxygen from reaching the burning material

B.　cooling the burning material below its ignition temperature

C.　using the extinguishing agent to make the burning material fire-resistant

D.　using the extinguishing agent to absorb the heat

KEY:　A

[4706]A class C fire would be burning ______.

A.　fuel oil

B.　wood

C.　celluloid

D.　electrical insulation

KEY:　D

[4707]A combustible gas indicator meter is calibrated to read the percentage of ______.

A.　vapor to oxygen

B.　the flammable limit concentration

C.　the autoignition concentration

D.　the lower explosive limit concentration

KEY:　D

[4708]A combustible gas indicator will NOT operate correctly when the ______.

A.　hydrocarbon content of the atmosphere exceeds the U.E.L

B.　atmosphere is deficient in oxygen

C.　distance between the operator and the compartment to be tested is greater than 50 feet

D.　All of the above

KEY:　B

[4709]A cutoff valve in the fire-main system of a ship may be closed to protect the portion of the system on an exposed deck from ______.

A.　damage from crane operations

B.　being used for wash down purposes

C.　accidental diversion of flow to wrong location

D.　freezing

KEY:　D

[4710]A definite advantage of using water as a fire extinguishing agent is its characteristic of ______.

A.　alternate expansion and contraction as water in a liquid state becomes a vapor

B.　absorption of smoke and gases as water is converted from a liquid to a vapor

C.　rapid contraction as water is converted from a liquid to a vapor

D.　rapid expansion as water absorbs heat and changes to steam

KEY:　D

[4711]A fire has broken out on the stern of your vessel. You should maneuver your vessel so the wind ______.

A.　blows the fire back toward the vessel

B.　comes over the bow

C.　comes over the stern

D.　comes over either beam

KEY:　B

[4712]A fire hose has a ______.

A.　male coupling at both ends

B.　female coupling at both ends

C.　female coupling at the nozzle end and a male coupling at the hydrant end

D.　male coupling at the nozzle end and a female coupling at the hydrant end

KEY:　D

[4713]A fire in a ballast pumproom can be brought under control with minimal impact on stability by ______.

A.　cooling the outside bulkheads with water

B.　shutting all sources of air into the compartment

C.　closing the sea chest

D.　flooding the compartment with salt water

KEY:　B

[4714]A fire in electrical equipment should be extinguished by using ______.

A.　salt water

B.　foam

C.　low-velocity fog

D.　CO2

KEY:　D

[4715]A fire is discovered in the forepeak of a vessel at sea. The wind is from ahead at 35 knots. You should ______.

A.　remain on course and hold speed

B.　change course and put the stern to the wind

C.　change course to put the wind on either beam and increase speed

D.　remain on course but slack the speed

KEY:　B

[4716]A fire main system must have enough fire hydrants so that each accessible space may be sprayed with ______.

A.　a low velocity spray applicator

B.　a water spray or solid stream

C.　at least two spray patterns of water

D.　at least 25 psi delivered pressure

KEY:　C

[4717]A fire must be ventilated ______.

A.　when using an indirect attack on the fire such as flooding with water

B.　to prevent the gases of combustion from surrounding the firefighters

C.　to minimize heat buildup in adjacent compartments

D.　if compressed gas cylinders are stowed in the compartment on fire

KEY:　B

[4718]A fire pump may be used for other purposes if ______.

A.　the other services are run off a reducing station with a pressure gage

B.　one of the required pumps is kept availabe for use on the fire main system at all times

C.　no relief valves are installed

D.　all of the above conditions are met

KEY:　B

[4719]A fire starting by spontaneous combustion can be expected in which condition? ______.

A.　Paints，varnish，or other liquid flammables are stowed in a dry stores locker.

B.　Inert cargoes such as pig iron are loaded in a wet condition.

C.　Oily rags are stowed in a metal pail.

D.　Clean mattresses are stored in contact with an electric light bulb.

KEY:　C

[4720]According to the T-Boat regulations the reason for providing adequate ventilation for a battery storage area is to prevent ______.

A.　accumulation of carbon dioxide gas that chokes the battery

B.　accumulation of explosive and toxic gases the battery can generate

C.　mildew or dry rot in the battery box

D.　battery failure including battery case meltdown caused by excessive heat

KEY:　B

[4721]Automatic fire dampers in ventilation systems are operated by a ______.

A.　remote operated valve

B.　fusible link

C.　CO2 system pressure switch

D.　heat or smoke detector

KEY:　B

[4722]Automatic fire dampers in ventilation systems are operated by use of ______.

A.　remotely operated valves

B.　fusible links

C.　C02 system pressure switches

D.　heat or smoke detectors

KEY:　B

[4723]Backfire flame arrestors are installed on ______.

A.　fuel tanks

B.　spark plugs

C.　carburetors

D.　distributors

KEY:　C

[4724]Before a tank is to be crude oil washed，the oxygen content in the tank must be measured at a position ______.

A.　immediately above the level of the oil

B.　at the top of the tank

C.　in the vent riser

D.　one meter from the deck

KEY:　D

[4725]Before using a fixed CO2 system to fight an engine room fire，you must ______.

A.　secure the engine room ventilation

B.　secure the machinery in the engine room

C.　evacuate all engine room personnel

D.　All of the above

KEY:　D

[4726]Blocking open or removing fire dampers can cause ______.

A.　fixed foam systems to be ineffective

B.　faster cooling of the fire

C.　the accumulation of explosive gases

D.　the fire to spread through the ventilation system

KEY:　D

[4727]Cargo transfer operations on a tanker need NOT be stopped when ______.

A.　a tug comes alongside while the tanker is loading grade D and E cargoes

B.　a large，fresh oil spill is discovered immediately adjacent to the side of the tanker

C.　there is an electrical storm in the vicinity

D.　there is a fire on the dock or on a nearby vessel

KEY:　A

[4728]CO2 extinguishes a fire by ______.

A.　cooling

B.　smothering

C.　chemical action

D.　All of the above

KEY:　B

[4729]Combustible gas indicators measure the presence of combustible gas as a percentage of the ______.

A.　flash point

B.　upper explosive limit

C.　lower explosive limit

D.　fire point

KEY:　C

[4730]Control of fire on a ship should be addressed ______.

A.　immediately after restoring vital services

B.　immediately

C.　following control of flooding

D.　following establishment of fire boundaries

KEY:　B

[4731]Control of flooding should be addressed ______.

A.　first

B.　following control of fire

C.　following restoration of vital services

D.　only if a threat exists

KEY:　B

[4732]Controls for a fixed carbon dioxide system shall be mounted ______.

A.　directly outside the space protected by the system

B.　as near the gas cylinders as possible

C.　in the pilothouse

D.　on the main deck near the bow

KEY:　A

[4733]Convection spreads a fire by ______.

A.　heated gases flowing through ventilation systems

B.　the transfer of heat across an unobstructed space

C.　burning liquids flowing into another space

D.　transmitting the heat of a fire through the ship's metal

KEY:　A

[4734]Dry chemical extinguishers extinguish class B fires to the greatest extent by ______.

A.　cooling

B.　smothering

C.　oxygen dilution

D.　breaking the chain reaction

KEY:　D

[4735]Dry chemical extinguishers may be used on what class of fires?

A.　A only

B.　B only

C.　B and C only

D.　A，B or C as marked on the extinguisher

KEY:　D

[4736]Dry chemical fire extinguishers are effective on which type(s) of fire?

A.　Burning oil

B.　Electrical

C.　Paint

D.　All of the above

KEY:　D

[4737]Due to the hazards involved with Halon extinguishers on a ship，the size II extinguisher may only be used ______.

A.　outside

B.　on class C fires

C.　in an emergency

D.　on class B fires

KEY:　A

[4738]During a fire drill on a ship，what action is required?

A.　Start each fire pump

B.　Launch and run a lifeboat

C.　Inventory rescue and fire equipment

D.　Inspect fire hoses

KEY:　A

[4739]During a training exercise a submarine indicating that a torpedo has been fired will send up smoke from a float. The smoke's color will be ______.

A.　black

B.　red

C.　orange

D.　yellow

KEY:　A

[4740]During the required periodic abandon ship drill aboard a ship，each person not assigned duties in the muster list is ______.

A.　instructed in the use of portable fire extinguishers

B.　shown a video demonstrating lifeboat launching

C.　instructed in the use of life jackets

D.　not required to attend the boat drill

KEY:　C

[4741]Each fire hose coupling on a ship must have threads that meet the specifications of the ______.

A.　American Petroleum Institute

B.　National Standard Firehose Coupling

C.　American Society of Mechanical Engineers

D.　Underwriter's Laboratories，Inc.

KEY:　B

[4742]Each fire hydrant must have at least one spanner and at least one ______.

A.　hose rack or reel

B.　all purpose nozzle

C.　foam applicator

D.　pick axe

KEY:　A

[4743]Each fire hydrant serving machinery spaces containing oil fired boilers，internal combustion machinery，or oil fuel units must be equipped with a ______.

A.　fireman's outfit

B.　low-velocity spray applicator

C.　marine strainer

D.　pick axe

KEY:　B

[4744]Each fire pump on a ship must have a pressure gauge located at ______.

A.　the pump discharge

B.　the manifold connection

C.　each fire station discharge

D.　the pump station

KEY:　A

[4745]Each fireman's outfit and its spare equipment on a vessel must be stowed ______.

A.　in a locked cabinet in the machinery space

B.　in an unlocked cabinet in the machinery space

C.　in a separate and accessible location

D.　at a fire hydrant location

KEY:　C

[4746]Each hand portable fire extinguisher must be marked with ______.

A.　the name of the vessel on which it is located

B.　the date that it was installed

C.　the names of the individuals qualified to use it

D.　an identification number

KEY:　D

[4747]Each hose in the fuel transfer system for helicopter refueling must meet the standards of the ______.

A.　Federal Aviation Administration

B.　Corps of Engineers

C.　National Fire Protection Association

D.　National Transportation Safety Board

KEY:　A

[4748]Each part of the fire-main system located on an exposed deck must be ______.

A.　protected against freezing

B.　locked to prevent theft

C.　numbered sequentially

D.　pressurized at all times

KEY:　A

[4749]If，during helicopter refueling operations，fuel is spilled on clothing，the person should first ______.

A.　see the medic immediately

B.　spray himself with foam or CO2

C.　complete his task and then see the medic

D.　remove the clothing and wash

KEY:　D

[4750]On a hydrocarbon flammability chart the line which extends from 0% to 21.8% oxygen，lying tangent to the flammability range，is called the ______.

A.　minimum oxygen content line

B.　critical displacement line

C.　critical dilution line

D.　upper threshold limit

KEY:　C

18.2 探火与消防设备

[4751]A cabinet or space containing the controls or valves for the fixed firefighting system must be ______.

A.　posted with instructions on the operation of the system

B.　ventilated and equipped with explosion-proof switches

C.　painted with red and black diagonal stripes

D.　equipped with a battery powered source of emergency lighting

KEY:　A

[4752]Which statement is TRUE concerning spare charges for portable fire extinguishers on unmanned tank barges? ______.

A.　Spare charges shall be carried for at least 50 percent of each size and variety required on board.

B.　If the unit cannot be charged by the vessel's personnel，one spare unit shall be carried in lieu of spare charges.

C.　Spare charges shall be stowed in watertight containers on deck.

D.　Regulations concerning spare charges do not apply to unmanned barges.

KEY:　D

[4753]No outlet on a fire hydrant may point above the horizontal in order to ______.

A.　avoid kinking the hose

B.　avoid personal injury during connection

C.　make connecting easier

D.　prevent spray on electrical equipment

KEY:　A

[4754]The danger of a charged hose left unattended on deck with the nozzle open is ______.

A.　the hose could burst

B.　the nozzle end will whip about causing damage or injury

C.　water damage to vessel's cargo or structure

D.　personnel might trip over the hose

KEY:　B

[4755]______，as the chemical extinguisher agent，should be used for an electric fire．

A.　dry chemical or foam

B.　foam or soda acid

C.　carbon dioxide or foam

D.　carbon dioxide or dry chemical

KEY:　D

[4756]A branch line valve of a fire extinguishing system on a ship must be marked with the ______.

A.　maximum pressure allowed at that branch

B.　name of the space or spaces which it serves

C.　date of the last maintenance inspection

D.　pressure needed to maintain an effective stream at that point

KEY:　B

[4757]A CO2 extinguisher on a ship which has lost 10% of its charge must be ______.

A.　used at the earliest opportunity

B.　hydrotested

C.　recharged

D.　weighed again in one month

KEY:　C

[4758]A CO2 portable extinguisher is annually checked by ______.

A.　reading the gage pressure

B.　weighing the extinguisher

C.　discharging a small amount of CO2

D.　seeing if the seal has been broken

KEY:　B

[4759]A combination or all-purpose nozzle produces ______.

A.　low-velocity fog only

B.　a solid stream only

C.　a solid stream and foam

D.　a solid stream and fog

KEY:　D

[4760]A fire starts on your vessel while refueling. You should FIRST ______.

A.　stop the ventilation

B.　sound the general alarm

C.　determine the source of the fire

D.　attempt to extinguish the fire

KEY:　B

[4761]A fixed CO2 system on a ship with a capacity of over 300 lbs (136 kilograms) CO2 which protects spaces other than tanks must have ______.

A.　two or more releasing stations

B.　automatic release in event of a fire

C.　an audible alarm and time delay

D.　an audible and visible alarm

KEY:　C

[4762]A foam-type portable fire extinguisher is most useful in fighting a fire in ______.

A.　generators

B.　oil drums

C.　the bridge controls

D.　combustible metals

KEY:　B

[4763]A foam-type portable fire extinguisher would be most useful in combating a fire in ______.

A.　solid materials such as wood or bales of fiber

B.　flammable liquids

C.　a piece of electrical equipment

D.　combustible metallic solids

KEY:　B

[4764]A fuel line breaks，sprays fuel on the hot exhaust manifold，and catches fire. Your FIRST action should be to ______.

A.　batten down the engine room

B.　start the fire pump

C.　apply carbon dioxide to the fire

D.　shut off the fuel supply

KEY:　D

[4765]A galley grease fire on the stove may be extinguished using ______.

A.　water

B.　foam

C.　the range hood extinguishing system

D.　fire dampers

KEY:　C

[4766]A galley grease fire would be classified as which class of fire?

A.　A

B.　B

C.　C

D.　D

KEY:　B

[4767]A GROUP OF CREW MEMBERS TRAINED FOR FIGHTING FLOODING IN THE VESSEL are known as ______.

A.　Fire party

B.　Deck cadets

C.　Research team

D.　Damage control team

KEY:　D

[4768]A large oil fire on the deck of a ship can be fought most effectively with ______.

A.　dry chemical

B.　foam

C.　high-velocity fog

D.　Water (cartridge-operated)

KEY:　B

[4769]A marine chemist issues gas free certificates and is certified by the ______.

A.　Mine Safety Appliance Association

B.　American Chemical Society

C.　Marine Chemists Association

D.　National Fire Protection Association

KEY:　D

[4770]A passenger vessel is required to have a supervised patrol when ______.

A.　there are passengers berthed on board

B.　navigating in excess of eight hours

C.　the vessel has substantial wood in its construction

D.　there is no automatic fire-detection system installed

KEY:　A

[4771]A portable dry chemical fire extinguisher discharges by ______.

A.　gravity when the extinguisher is turned upside down

B.　pressure from a small CO2 cartridge on the extinguisher

C.　air pressure from the hand pump attached to the extinguisher

D.　pressure from the reaction when water is mixed with the chemical

KEY:　B

[4772]A portable foam (stored-pressure type) fire extinguisher would be most useful in combating a fire in ______.

A.　generators

B.　oil drums

C.　the bridge controls

D.　combustible metals

KEY:　B

[4773]A ship helicopter landing deck on which fueling operations are conducted must have a fire protection system that is capable of discharging at 100 psi pressure a foam spray of at least ______.

A.　30 gallons per minute

B.　40 gallons per minute

C.　50 gallons per minute

D.　60 gallons per minute

KEY:　C

[4774]A ship must have a self-contained breathing apparatus to be used as protection from gas leaking from a refrigeration unit. To meet this requirement，you may use ______.

A.　a gas mask certified by the Mine Safety and Health Administration

B.　the same self-contained breathing apparatus required with the fireman's outfit

C.　an oxygen breathing apparatus，provided that the device has been inspected within three years

D.　a portable ventilation system that will provide a complete change of air every three minutes

KEY:　B

[4775]A small fuel spillage has occurred during helicopter refueling. After the leak has been stopped and fire-control personnel have been notified and are standing by，the next step is to ______.

A.　proceed with fueling operation

B.　have helicopter move to a safer location

C.　wash spilled fuel away with a flood of water

D.　clean spilled fuel with rags

KEY:　C

[4776]A spanner is a ______.

A.　cross connection line between two main fire lines

B.　special wrench for tightening couplings in a fire hose line

C.　tackle rigged to support a fire hose

D.　None of the above

KEY:　B

[4777]A spring line is ______.

A.　any wire rope used for mooring

B.　a fire-warp

C.　a mooring line running diagonally to the keel

D.　a mooring line perpendicular to the keel

KEY:　C

[4778]A squeeze-grip type carbon dioxide portable fire extinguisher has been partially discharged. It should be ______.

A.　labeled empty and recharged as soon as possible

B.　replaced in its proper location if weight loss is no more than 25%

C.　replaced in its proper location regardless of weight

D.　replaced in its proper location if weight loss is no more than 15%

KEY:　A

[4779]A vessel is constructed with a steel hull and an aluminum superstructure. Which statement is TRUE?

A.　The aluminum will provide greater resistance to the spread of fire by conduction.

B.　The aluminum structure is usually attached to a steel coaming by a method that insulates the two metals.

C.　If the superstructure is stressed，an aluminum structure requires additional expansion joints to prevent fracture.

D.　The steel at the area of the aluminum-to-steel connection must be closely checked for galvanic corrosion.

KEY:　B

[4780]According to the regulations，what fire safety control feature is required in quick-closing shut off valves?

A.　Electrical cut off switch

B.　A fusible link

C.　Manual cut off switch

D.　A water spray actuator

KEY:　B

[4781]After extinguishing a fire with CO2，it is advisable to ______.

A.　use all CO2 available to cool the surrounding area

B.　stand by with water or other agents

C.　thoroughly ventilate the space of CO2

D.　jettison all burning materials

KEY:　B

[4782]After extinguishing a paint locker fire using the fixed CO2 system，the next action is to have the space ______.

A.　opened and burned material removed

B.　left closed with vents off until all boundaries are cool

C.　checked for oxygen content

D.　doused with water to prevent reflash

KEY:　B

[4783]All of the following are part of the fire triangle EXCEPT ______.

A.　electricity

B.　fuel

C.　oxygen

D.　heat

KEY:　A

[4784]All portable fire extinguishers must be capable of being ______.

A.　carried by hand to a fire

B.　carried or rolled to a fire

C.　recharged in the field

D.　used on class 'B' fires

KEY:　A

[4785]An ABC dry chemical fire extinguisher would be LEAST effective against a fire in ______.

A.　a mattress

B.　spilled liquids such as oil or paint

C.　high voltage electrical gear

D.　a trash can

KEY:　A

[4786]An advantage of a dry chemical over a carbon dioxide fire extinguisher is its ______.

A.　greater range

B.　effectiveness on all types of fires

C.　cleanliness

D.　All of the above

KEY:　A

[4787]An aircraft has indicated that he wants you to change course and follow him. You cannot comply because of an emergency on board. Which signal should you make?

A.　Fire a red flare at night or a red smoke signal by day

B.　Send the Morse signal N by flashing light

C.　Make a round turn (360) and resume course

D.　Make an S turn (hard right then hard left) and resume course

KEY:　B

[4788]An effective method for moving patients with spinal injuries onto a spine board is known as the ______.

A.　pack-strap carry

B.　two man extremities carry

C.　fireman's drag

D.　four man log roll

KEY:　D

[4789]An engine compartment gasoline fire requires which type of extinguisher? ______.

A.　Carbon dioxide

B.　Dry chemical

C.　Foam

D.　All of the above

KEY:　D

[4790]An extinguishing agent which effectively cools，dilutes combustible vapors，removes oxygen，and provides a heat and smoke screen is ______.

A.　carbon dioxide

B.　Halon 1301

C.　dry chemical

D.　water fog

KEY:　D

[4791]An important step in fighting any electrical fire is to ______.

A.　stop ventilation

B.　stop the vessel

C.　de-energize the circuit

D.　apply water to extinguish the fire

KEY:　C

[4792]An inert gas system is designed to reduce the possibility of tank explosions by ______.

A.　eliminating sparks and fire in the vicinity of cargo tanks

B.　removing all hydrocarbon gases from the cargo tanks

C.　blanketing cargo tanks with inert foam

D.　reducing the oxygen concentration below levels necessary for combustion

KEY:　D

[4793]An inert gas system on a tanker should be used to ______.

A.　prevent the generation of flammable or combustible gas in tanks

B.　blow out cargo lines to prevent gas concentrations

C.　dilute tank atmospheres to keep gas concentrations below the lower explosive limit

D.　prevent fires in the pumproom by continually displacing flammable vapors

KEY:　C

[4794]Any extinguishing agent used on a Class C fire must have which important property?

A.　Cooling ability

B.　Leaves no residue

C.　Penetrating power

D.　Nonconductivity

KEY:　D

[4795]Any firefighting equipment that is carried in addition to the minimum required must ______.

A.　meet the applicable standards

B.　be marked as additional equipment

C.　be stowed in a separate area

D.　All of the above

KEY:　A

[4796]Any ship damaging a pipe could ______ an immediate danger of fire．

A.　meet

B.　face

C.　be in font of

D.　follow

KEY:　B

[4797]As an extinguishing agent，foam ______.

A.　conducts electricity

B.　should be directed at the base of the fire

C.　is most effective on burning gases which are flowing

D.　extinguishes by cooling oil fires below ignition temperature

KEY:　A

[4798]As compared to carbon dioxide，dry chemical has which advantage? ______.

A.　Cleaner

B.　Effective on metal fires

C.　Greater range

D.　More cooling effect

KEY:　C

[4799]At night while the ship is tied up to the dock，a fire breaks out in a cargo hatch. After sounding the alarm，what should the person on watch do?

A.　Begin fighting the fire immediately.

B.　Await further orders from the Master.

C.　Let go the lines to let the vessel drift to an anchorage.

D.　Send a person to summon the shore authorities.

KEY:　D

[4800]At the required fire drill conducted aboard a ship，all persons must report to their stations and demonstrate their ability to perform the duties assigned to them ______.

A.　by the toolpusher

B.　in the station bill

C.　by the person conducting the drill

D.　at the previous safety meeting

KEY:　B

18.3 消防演习与作业

[4801]Accumulations of oily rags should be ______.

A.　kept in nonmetal containers

B.　discarded as soon as possible

C.　cleaned thoroughly for reuse

D.　kept in the paint locker

KEY:　B

[4802]Records of tests and inspections of a cargo vessel's fire extinguishing systems shall be ______ on board until the vessel's Certificate of Inspection expires.

A.　consealed

B.　detained

C.　contained

D.　kept

KEY:　D

[4803]A fifteen-pound CO2 extinguisher ______.

A.　contains 15 pounds of CO2

B.　weighs 15 pounds when full of CO2

C.　has 15 pounds of pressure at the nozzle

D.　weighs 15 pounds when empty

KEY:　A

[4804]A fire in a ballast pumproom can be brought under control with minimal impact on stability by ______.

A.　cooling the outside bulkheads with water

B.　shutting all sources of air into the compartment

C.　closing the sea chest

D.　flooding the compartment with salt water

KEY:　B

[4805]A fire in the galley ALWAYS poses the additional threat of ______.

A.　contaminating food with extinguishing agent

B.　spreading through the engineering space

C.　causing loss of stability

D.　a grease fire in the ventilation system

KEY:　D

[4806]A safety outlet is provided on the CO2 discharge piping to prevent ______.

A.　over pressurization of the space being flooded

B.　rupture of cylinder due to temperature increase

C.　over pressurization of the CO2 discharge piping

D.　flooding of a space where personnel are present

KEY:　C

[4807]Actuating the CO2 fixed system causes the shutdown of the ______.

A.　fuel supply

B.　exhaust ventilation

C.　supply and exhaust ventilation

D.　mechanical and natural ventilation

KEY:　C

[4808]After each reading of an oxygen indicator，the instrument should be purged with ______.

A.　CO2

B.　fresh air

C.　the tested compartment's air

D.　water

KEY:　B

[4809]After using a CO2 extinguisher on a ship，it should be ______.

A.　put back in service if some CO2 remains

B.　hydrostatically tested

C.　retagged

D.　recharged

KEY:　D

[4810]An inflatable liferaft should be lifted back aboard the ship by using ______.

A.　the single hook at the top of the raft

B.　two lines passed under the raft

C.　the towing bridle

D.　All of the above

KEY:　C

[4811]Automatic mechanical ventilation shutdown is required for CO2 systems protecting the ______.

A.　machinery spaces

B.　cargo compartments

C.　living quarters

D.　galley

KEY:　A

[4812]CO2 cylinders equipped with pressure actuated discharge heads will discharge automatically when ______.

A.　the discharge valve is open

B.　the control box glass is broken

C.　pressure from the control cylinders is detected

D.　the control cylinders have been completely discharged

KEY:　C

[4813]CO2 cylinders，which protect the small space in which they are stored must ______.

A.　NOT contain more than 200 pounds of CO2

B.　be automatically operated by a heat actuator

C.　have an audible alarm

D.　All of the above

KEY:　B

[4814]Control valves of a CO2 system may be located within the protected space when ______.

A.　it is impractical to locate them outside

B.　there is also a control valve outside

C.　the CO2 cylinders are also in the space

D.　an automatic heat-sensing trip is installed

KEY:　C

[4815]Except in rare cases，it is impossible to extinguish a shipboard fire by ______.

A.　removing the heat

B.　removing the oxygen

C.　removing the fuel

D.　interrupting the chain reaction

KEY:　C

[4816]Fire alarm system thermostats are actuated by ______.

A.　smoke sensors

B.　the difference in thermal expansion of two dissimilar metals

C.　pressure loss due to air being heated

D.　an electric eye which actuates when smoke interferes with the beam

KEY:　B

[4817]Fire and abandon ship stations and duties may be found on the ______.

A.　crewman's duty list

B.　Certificate of Inspection

C.　shipping articles

D.　muster list

KEY:　D

[4818]Fire axes required on MV ship's must be stored in the enclosure for fire hoses with the location marked ______.

A.　Fire Axe Location

B.　Hose Station No.…

C.　Fire Station No.…

D.　Firefighting Equipment

KEY:　C

[4819]Fire dampers prevent the spread of fire by ______.

A.　conduction

B.　convection

C.　radiation

D.　direct contact

KEY:

[4820]Fire extinguishers of sizes III，IV，and V are designated as ______.

A.　portable

B.　semi-portable

C.　fixed

D.　disposable

KEY:　B

[4821]Fire extinguishers on inspected vessels are numbered by size I through V，with I being ______.

A.　used for electrical fires only

B.　the smallest

C.　the most accessible

D.　the most effective

KEY:　B

[4822]Fire extinguishing agents used on Class C fires must be ______.

A.　able to absorb heat

B.　water based

C.　nonconducting

D.　nontoxic

KEY:　C

[4823]Fire fighting equipment requirements for a particular vessel may be found on the ______.

A.　4Certificate of Inspection

B.　Certificate of Seaworthiness

C.　Classification Certificate

D.　Certificate of Registry

KEY:　A

[4824]Fire hose couplings ______.

A.　are made of bronze，brass，or soft alloy metals

B.　should be painted red in order to identify hose lengths

C.　are specially hardened to prevent crushing

D.　should be greased frequently

KEY:　A

[4825]Fire hose should be washed with ______.

A.　salt water and a wire brush

B.　caustic soap

C.　mild soap and fresh water

D.　a holystone

KEY:　C

[4826]Fire in an engine compartment is best extinguished with carbon dioxide gas (CO2) and by ______.

A.　closing the compartment except for the ventilators

B.　completely closing the compartment

C.　leaving the compartment open to the air

D.　increasing the air flow to the compartment by blowers

KEY:　B

[4827]Fire may be spread by which means? ______.

A.　Conduction of heat to adjacent surfaces

B.　Direct radiation

C.　Convection

D.　All of the above

KEY:　D

[4828]Firefighting foam is only effective when the foam ______.

A.　penetrates to the bottom of the fire

B.　is kept saturated with low-velocity water fog

C.　mixes with the burning fuel oil

D.　completely covers the top of the burning liquid

KEY:　D

[4829]Fires are grouped into what categories?

A.　Class A，B，C，and D

B.　Type 1，2，3，and 4

C.　Combustible solids，liquids，and gases

D.　Flammable solids，liquids，and gases

KEY:　A

[4830]Fixed CO2 systems would not be used on crew's quarters or ______.

A.　the paint locker

B.　spaces open to the atmosphere

C.　cargo holds

D.　the engine room

KEY:　B

[4831]Flame screens are used to ______.

A.　contain flammable fumes

B.　protect firefighters from flames

C.　prevent flames from entering tanks

D.　keep flames and sparks from getting out of an engine's exhaust system

KEY:　C

[4832]Flames from small leaks of LFG may be extinguished by ______.

A.　utilizing carbon dioxide or dry chemical fire extinguishers

B.　utilizing soda and acid fire extinguishers

C.　blowing the flames out

D.　letting it burn itself out

KEY:　D

[4833]Foam extinguishes a fire by ______.

A.　shutting off the air supply

B.　cooling the fuel to below ignition temperature

C.　dispersing the fuel

D.　removing the source of ignition

KEY:　A

[4834]Foam extinguishes a fire by ______.

A.　smothering the burning material

B.　chemical combination with burning material

C.　absorbing the burning material

D.　organic destruction of the burning material

KEY:　A

[4835]Foam extinguishes a fire mainly by ______.

A.　cooling

B.　chemical action

C.　smothering

D.　inerting the air

KEY:　C

[4836]Foam is a very effective smothering agent and ______.

A.　it provides cooling as a secondary effect

B.　works well on extinguishing electrical fires

C.　can be used to combat combustible metal fires

D.　All of the above

KEY:　A

[4837]Foam-type portable fire extinguishers are most useful in combating fires involving ______.

A.　solid materials such as wood or bales of fiber

B.　inflammable liquids

C.　electrical equipment

D.　metallic solids

KEY:　B

[4838]Fusible-link fire dampers are operated by ______.

A.　8the heat of a fire melting the link

B.　a break-glass and pull-cable system

C.　electrical controls on the bridge

D.　a mechanical arm outside the vent duct

KEY:　A

[4839]Gas masks ______.

A.　should be worn while fighting a fire

B.　can be used in atmospheres deficient in oxygen

C.　filter contaminants from air that is to be breathed

D.　may be substituted for a self-contained breathing apparatus

KEY:　C

[4840]Generally，the first action in extinguishing an LFG fire caused by escaping gas is to ______.

A.　sweep flames away with water spray

B.　shut off the leak

C.　use a chemical foam fire extinguisher

D.　call the local fire department

KEY:　B

[4841]Good housekeeping on a vessel prevents fires by ______.

A.　allowing better access in an emergency

B.　eliminating potential fuel sources

C.　eliminating trip hazards

D.　improving personnel qualifications

KEY:　B

[4842]Halon extinguishers used on a class C fire should be directed at the ______.

A.　base of the equipment

B.　top of the equipment

C.　power source

D.　source of the fire

KEY:　D

[4843]Halon extinguishes a fire by ______.

A.　breaking the chain reaction

B.　smothering the fire

C.　cooling the fire

D.　coating the fuel with a nonflammable surface

KEY:　A

[4844]Halon extinguishes fire primarily by ______.

A.　cooling

B.　smothering

C.　shielding of radiant heat

D.　breaking the chain reaction

KEY:　D

[4845]Halon fire extinguishers are NOT effective when used on which types of fires?

A.　Fires in electrical equipment

B.　Flammable oils and greases

C.　Class A fires in ordinary combustibles

D.　Materials containing their own oxygen

KEY:　D

[4846]Halon from extinguishers used on a class B fire，should be directed ______.

A.　at the top of the flames

B.　at the base of the fire near the edge

C.　in short quick bursts

D.　toward the upwind side of the fire

KEY:　B

[4847]High-velocity fog ______.

A.　is a finer，more diffuse water spray than low-velocity fog

B.　requires that the water pressure be no greater than 60 psi

C.　produces an effective fog pattern no more than 6 feet beyond the nozzle

D.　extinguishes a fire by absorbing heat and reducing the supply of oxygen

KEY:　D

[4848]How do you operate a portable CO2 fire extinguisher? ______.

A.　Point the horn down.

B.　Turn cylinder upside-down.

C.　Break the rupture disc.

D.　Pull pin，squeeze grip.

KEY:　D

[4849]How does foam extinguish an oil fire? ______.

A.　By cooling the oil below the ignition temperature

B.　By removing the fuel source from the fire

C.　By excluding the oxygen from the fire

D.　By increasing the weight of the oil

KEY:　C

[4850]How is the external flotation bladder of an immersion suit inflated?

A.　It is inflated by a small CO2 bottle that is automatically tripped when the front zipper is at the top of the zipper track.

B.　It is inflated by a small CO2 bottle that is manually tripped.

C.　It is inflated by blowing through an inflation tube.

D.　It inflates by sea water bleeding into the flotation bladder and reacting with a chemical therein.

KEY:　C

19.3 应急预案与EMS

[4951]A group of crew members standing by in case of an emergency in order to assist other teams in action if necessary is a ______.

A.　Fire team

B.　Rescue team

C.　Emergency team

D.　Control team

KEY:　B

[4952]Flooding of any compartment in a ship that results in a serious loss of reserve buoyancy will always ______.

A.　increase the trim

B.　increase ship stability

C.　cause a serious list

D.　decrease the heeling moment

KEY:　D

[4953]A shipboard Oil Pollution Emergency Plan is required of ______.

A.　all vessels，regardless of size and commercial application

B.　any barge or ship constructed or operated in such a manner that no oil in any form can be carried aboard

C.　an oil tanker of 150 gross tons and above，or other ship of 400 gross tons and above

D.　an oil tanker of 400 gross tons and above，or other ships of 150 gross tons and above

KEY:　C

[4954]When dry chemical extinguishers are used to put out class B fires，there is a danger of reflash because dry chemical______.

A.　is not an effective agent on Class B fires

B.　does little or no cooling

C.　dissipates quickly

D.　is rapidly absorbed by the liquid

KEY:　B

[4955]When electrical equipment is involved in a fire，the stream of dry chemicals should be ______.

A.　aimed at the source of the flames

B.　fogged above the equipment

C.　shot off a flat surface onto the flames

D.　used to shield against electrical shock

KEY:　A

[4956]When fighting a fire in an enclosed space，the hose team should crouch as low as possible to ______.

A.　protect themselves from smoke

B.　obtain the best available air

C.　allow the heat and steam to pass overhead

D.　All of the above

KEY:　D

[4957]When fighting a fire on a bulkhead using a portable carbon dioxide extinguisher，the stream should be directed at the ______.

A.　base of the flames，moving the horn from side to side，following the flames upward as they diminish

B.　top of the flaming area，moving the horn from side to side，following the flames downward as they diminish

C.　center of the flaming area，moving the horn vertically from top to bottom

D.　bottom of the flaming area，moving the horn vertically to the top following the flames upward as they diminish

KEY:　A

[4958]When fighting a fire with a portable dry chemical fire extinguisher，the stream should be directed ______.

A.　over the top of the flames

B.　off a bulkhead into the fire

C.　in front of the fire

D.　at the base of the fire

KEY:　D

[4959]When fighting an oil or gasoline fire in the bilge，which of the following should NOT be used?

A.　Foam

B.　Solid stream water nozzle

C.　All-purpose nozzle

D.　Carbon dioxide

KEY:　B

[4960]When fire pumps are used for other than firefighting service，each pipe connecting the other service (except for branch lines used for deck washing) must have a ______.

A.　check valve installed in the line

B.　shut off valve at a manifold near the pump

C.　quick disconnect union within ten feet of the pump

D.　regulator in the line set at 125 psi

KEY:　B

[4961]When inspecting a tank barge to see that it has all the required fire extinguishers and other safety items aboard，which of the following is the best source for determining what is required?

A.　The supervisor in charge of the dock to which the barge is tied up

B.　Part 38 of the Rules and Regulations for Tankers

C.　The Certificate of Inspection of the barge

D.　The local fire marshal

KEY:　C

[4962]When joining the female coupling of the fire hose to the male outlet of the hydrant，you should make sure that the ______.

A.　threads are lubricated

B.　nozzle is attached to the hose

C.　female coupling has a gasket

D.　hose is led out

KEY:　C

[4963]When loading or discharging dry mud or cement，crew members should use facial respirator masks and ______.

A.　goggles

B.　ear plugs

C.　soft-soled boots

D.　fireman's outfit

KEY:　A

[4964]When must a dry chemical fire extinguisher be recharged?

A.　After each use

B.　When the air temperature exceeds 90F

C.　Every 6 months

D.　Every 12 months

KEY:　A

[4965]When possible，what is the FIRST step in fighting an engine fuel-pump fire which results from a broken fuel line?

A.　Secure all engine room doors，hatches，and vents.

B.　Close the fuel line valve.

C.　Check the spread of the fire with foam.

D.　Cast the barge off the wharf.

KEY:　B

[4966]When possible，what should be the FIRST step in combating a fire on deck resulting from a cargo overflow or a leaking cargo line?

A.　Blanket the cargo spill with foam.

B.　Prevent the spread of fire with a foam dam.

C.　Apply CO2 on burning fuel at its source.

D.　Shut off the transfer of cargo.

KEY:　D

[4967]When should a fire be ventilated?

A.　When attacking the fire directly

B.　When using a steam smothering system

C.　When using the fixed CO2 system

D.　All of the above

KEY:　A

[4968]When towing astern what equipment should be stowed ready for use near the towline?

A.　First aid kit

B.　Axe or cutting torch

C.　Fire extinguisher

D.　Chafing gear

KEY:　B

[4969]When two fire hose teams are attacking a fire they should ______.

A.　use different fire hose pressures

B.　use fire hoses of different sizes

C.　not attack the fire from opposite sides

D.　not wear protective clothing

KEY:　C

[4970]When used to fight fire，carbon dioxide ______.

A.　is effective if used promptly on an oil fire

B.　has a greater cooling effect than water

C.　is lighter than air

D.　is harmless to cargo and crew

KEY:　A

[4971]When using a high-velocity fog stream in a passageway，the possibility of a blow back must be guarded against. Blow back is most likely to occur when ______.

A.　pressure builds up in the nozzle which causes a surge of water

B.　the only opening in a passageway is the one from which the nozzle is being advanced

C.　pressure in the fire hose drops below 100 psi

D.　a bulkhead collapses due to heat and pressure

KEY:　B

[4972]When using carbon dioxide to fight a fire on a bulkhead，the CO2 should be applied ______.

A.　first to the bottom of the flaming area，sweeping from side to side，and following the flames upward

B.　in a circular motion from the middle of the bulkhead outward

C.　to the top of the flaming area，sweeping from side to side，and working toward the bottom

D.　in an up-and-down motion from one side of the bulkhead to the other

KEY:　A

[4973]When water is used to fight a fire on board a ship，the effect of the weight of the water must be taken into account. How much sea water will increase the weight displacement by one ton?

A.　64 cubic feet

B.　35 cubic feet

C.　100 gallons

D.　500 liters

KEY:　B

[4974]Where foam extinguishing systems are provided on a ship，each machinery flat in the protected space must have a(n) ______.

A.　coaming

B.　alarm

C.　drain

D.　fire sensor

KEY:　A

[4975]Where should life jackets be stowed? ______.

A.　In the forepeak

B.　In the wheelhouse

C.　Throughout the accommodation spaces

D.　In locked watertight and fireproof containers on or above the main deck

KEY:　C

[4976]Where would you find the FCC authorization for transmitting on your ship's EPIRB?

A.　On the Ship Station License

B.　On the side of the EPIRB transmitter

C.　In the radio log

D.　On the Certificate of Inspection

KEY:　A

[4977]Which action is routinely performed at the annual servicing and inspection of a dry-chemical cartridge-operated portable fire extinguisher?

A.　Insure the chemical is powdery.

B.　Replace the cartridge.

C.　Pressure test the discharge hose.

D.　Test the pressure gauge for proper operation.

KEY:　A

[4978]Which advantage does dry chemical have over carbon dioxide (CO2) in firefighting?

A.　Compatible with all foam agents

B.　Cleaner

C.　More protective against re-flash

D.　All of the above

KEY:　C

[4979]Which characteristic is an advantage of Halon as a fire extinguishing medium?

A.　Electrically non-conductive

B.　Relatively inexpensive

C.　Effective against chemicals containing an oxidizer

D.　All of the above

KEY:　A

[4980]Which characteristic of Halon is a disadvantage when it is used as a fire extinguishing medium?

A.　Leaves a residue

B.　Cost，relative to other agents

C.　Breaks down while under prolonged storage

D.　Conducts electricity

KEY:　B

[4981]Which extinguishing agent is best for use on a magnesium fire?

A.　Water

B.　Sand

C.　CO2

D.　Dry chemical

KEY:　B

[4982]Which extinguishing agent is effective in combating an isoprene fire?

A.　Dry chemicals

B.　CO2

C.　Foam

D.　All the above

KEY:　D

[4983]Which extinguishing agent is most effective on a mattress fire?

A.　CO2

B.　Foam

C.　Dry Chemical

D.　Water

KEY:　D

[4984]Which extinguishing agent is NOT recommended in the Chemical Data Guide for use on a cyclohexane fire?

A.　Foam

B.　Carbon dioxide

C.　Dry chemical

D.　Water fog

KEY:　D

[4985]Which extinguishing agent is recommended in the Chemical Data Guide for use on a carbon disulfide fire?

A.　Alcohol foam

B.　Dry chemical

C.　Chemical foam

D.　Water fog

KEY:　B

[4986]Which extinguishing agent is the best for use on electrical fires?

A.　Foam

B.　CO2

C.　Dry chemical

D.　Water fog

KEY:　B

[4987]Which fire detection system is actuated by sensing a heat rise in a compartment?

A.　Manual fire detection system

B.　Automatic fire detection system

C.　Smoke detection system

D.　Watchman's supervisory system

KEY:　B

[4988]Which fire extinguishing agent can NOT be used on an ethylenediamine?

A.　Water foam

B.　Dry chemical powder

C.　Water fog

D.　Alcohol foam

KEY:　A

[4989]Which fire extinguishing agent has the greatest capacity for absorbing heat?

A.　Water

B.　Foam

C.　Dry chemical

D.　Carbon Dioxide

KEY:　A

[4990]Which firefighting method is an example of an indirect attack on a fire? __________.

A.　Bouncing a straight stream of water off the overhead to create spray effect

B.　Spraying foam on a bulkhead and letting it flow down and over a pool of burning oil

C.　Flooding a paint locker with CO2 and sealing the compartment

D.　Cooling adjacent bulkheads with water to prevent the spread of the fire by conduction

KEY:　C

[4991]Which is a disadvantage of Halon as a fire extinguishing medium?

A.　Conducts electricity

B.　Difficult to store

C.　Large volume necessary to be effective

D.　Ineffective in powdered metal fires

KEY:　D

[4992]Which item is NOT included in the fireman's outfit on a cargo vessel?

A.　Fire ax

B.　Flashlight

C.　rigid helmet

D.　Fresh air breathing apparatus

KEY:　D

[4993]Which portable fire extinguisher is normally recharged in a shore facility?

A.　Dry chemical (cartridge-operated)

B.　Water (cartridge-operated)

C.　Water (pump tank)

D.　Carbon dioxide

KEY:　D

[4994]Which portable fire extinguisher is required just outside the exit of the propulsion machinery space of a 75-ton passenger vessel?

A.　2-1/2 gallon foam extinguisher

B.　15 lb. CO2 extinguisher

C.　2 lb. dry chemical extinguisher

D.　None of the above

KEY:　B

[4995]Which portable fire extinguisher should be used on a class C fire on board a vessel?

A.　Carbon dioxide

B.　Water (stored pressure)

C.　Foam

D.　Carbon tetrachloride

KEY:　A

[4996]Which precaution should be taken when testing a line throwing gun?

A.　Never remove the line from the rocket.

B.　Fire it at an angle of approximately 90 degrees to the horizon.

C.　Wear asbestos gloves.

D.　All of the above

KEY:　A

[4997]Which sizes of fire extinguishers are considered to be semi-portable?

A.　I，II，III，IV，and V

B.　I，II，and III only

C.　II，III，and IV only

D.　III，IV，and V only

KEY:　D

[4998]Which space(s) on your cargo vessel must have a fire detection system?

A.　Any compartment containing explosives

B.　Any compartment adjacent to one containing explosives

C.　Enclosed spaces which are specially suitable for vehicles

D.　All of the above

KEY:　D

[4999]Which statement concerning the application of dry chemical powder is FALSE?

A.　At temperatures of less than 32? F，the extinguisher must be recharged more often.

B.　When possible，the fire should be attacked from windward.

C.　The stream should be directed at the base of the fire.

D.　Directing the stream into burning flammable liquid may cause splashing.

KEY:　A

[5000]Which statement is FALSE concerning precautions during fueling operations?

A.　All engines，motors，fans，etc. should be shut down when fueling.

B.　All windows，doors，hatches，etc. should be closed.

C.　A fire extinguisher should be kept nearby.

D.　Fuel tanks should be topped off with no room for expansion.

KEY:　D

19.4 海上医务与MFAG

[5001]First aid means ______.

A.　medical treatment of accident

B.　setting of broken bones

C.　emergency treatment at the scene of the injury

D.　dosage of medications

KEY:　C

[5002]During the course of a voyage，a seaman falls on the main deck and injures his ankle. The Master should submit a Report of Marine Accident，Injury or Death if the ______.

A.　injured needs first aid

B.　injured is incapacitated

C.　injury results in loss of life only

D.　injury is the result of misconduct

KEY:　B

[5003]A man has suffered a burn on the arm. There is extensive damage to the skin with charring present. How is this injury classified using standard medical terminology?

A.　Dermal burn

B.　Third-degree burn

C.　Major burn

D.　Lethal burn

KEY:　B

[5004]In any major injury to a person，first aid includes the treatment for the injury and ______.

A.　application of CPR

B.　removal of any foreign objects

C.　administration of oxygen

D.　for traumatic shock

KEY:　D

[5005]The symptoms of a fractured back are ______.

A.　leg cramps in the muscles in one or both legs

B.　pain and uncontrolled jerking of the legs and arms

C.　vomiting and involuntary urination or bowel movement

D.　pain at the site of the fracture and possible numbness or paralysis below the injury

KEY:　D

[5006]Which should NOT be a treatment for a person who has received a head injury and is groggy or unconscious?

A.　Give a stimulant.

B.　Elevate his head.

C.　Stop severe bleeding.

D.　Treat for shock.

KEY:　A

[5007]A compound fracture is a fracture in which ______.

A.　more than one bone is broken

B.　the same bone is broken in more than one place

C.　there is never any internal bleeding

D.　the bone may be visible

KEY:　D

[5008]A conscious victim who has suffered a blow to the head has symptoms that indicate the possibility of concussion. If the patient feels no indication of neck or spine injury，recommended treatment would include ______.

A.　turning the victims's head to the side to keep his airway open

B.　positioning the victim so the head is lower than the body

C.　giving the victim water if he is thirsty，but no food

D.　elevating the head and shoulders slightly with a pillow

KEY:　D

[5009]A crew member has suffered frostbite to the toes of both feet. You should ______.

A.　immerse the feet in warm water

B.　warm the feet with a heat lamp

C.　warm the feet at room temperature

D.　rub the feet

KEY:　A

[5010]A crew member has suffered frostbite to the toes of the right foot. Which is NOT an acceptable first aid measure? ______.

A.　Rub the toes briskly.

B.　Elevate the foot slightly.

C.　Rewarm rapidly.

D.　Give aspirin or other medication for pain if necessary.

KEY:　A

[5011]A crew member is having an epileptic convulsion. You should ______.

A.　give the victim artificial respiration

B.　completely restrain the victim

C.　give the victim one 30 mg. tablet of phenobarbital

D.　keep the victim from injuring him or herself

KEY:　D

[5012]A crew member is unconscious and the face is flushed. You should ______.

A.　lay the crew member down with the head and shoulders slightly raised

B.　administer a liquid stimulant

C.　lay the crew member down with the head lower than the feet

D.　attempt to stand the crew member upright to restore consciousness

KEY:　A

[5013]According to the T-Boat regulations the reason for providing adequate ventilation for a battery storage area is to prevent ______.

A.　accumulation of carbon dioxide gas that chokes the battery

B.　accumulation of explosive and toxic gases the battery can generate

C.　mildew or dry rot in the battery box

D.　battery failure including battery case meltdown caused by excessive heat

KEY:　B

[5014]After an accident the victim may go into shock and die. What should be done to help prevent shock?

A.　Slightly elevate the head and feet.

B.　Keep the person awake.

C.　Keep the person lying down and at a comfortable temperature.

D.　Give the person a stimulant to increase blood flow.

KEY:　C

[5015]All inspected vessels on unrestricted ocean routes must have equipment on board for testing an individual's ______.

A.　blood

B.　breath

C.　urine

D.　All of the above

KEY:　B

[5016]An advantage of an ABC dry chemical over a carbon dioxide extinguisher is ______.

A.　lack of toxicity

B.　the multipurpose extinguishing ability

C.　burn-back protection

D.　cooling ability

KEY:　B

[5017]An unconscious person should NOT be ______.

A.　placed in a position with the head lower than the body

B.　given an inhalation stimulant

C.　given something to drink

D.　treated for injuries until conscious

KEY:　C

[5018]Antiseptics are used principally to ______.

A.　speed healing

B.　prevent infection

C.　reduce inflammation

D.　increase blood circulation

KEY:　B

[5019]Atmospheres laden with coal dust or grain dust caused by loading these cargoes ______.

A.　require loading operations to be shut down until the atmosphere clears

B.　are toxic to human life

C.　are subject to spontaneous combustion

D.　may be explosive in some concentrations

KEY:　D

[5020]Basic emergency care for third degree electrical burn is to ______.

A.　flood the burned area with warm water for two minutes

B.　brush away the charred skin and wrap the burned area

C.　cover the burned area with a clean cloth and transport the patient to a medical facility

D.　apply ointment or spray to the burned area and wrap with a clean cloth

KEY:　C

[5021]Bleeding from a vein is ______.

A.　dark red and has a steady flow

B.　bright red and slow

C.　bright red and spurting

D.　dark red and spurting

KEY:　A

[5022]Bleeding from a vein may be ordinarily controlled by ______.

A.　applying direct pressure to the wound

B.　heavy application of a disinfectant

C.　pouring ice water directly onto the wound

D.　pinching the wound closed

KEY:　A

[5023]Blood flowing from a cut artery appears ______.

A.　dark red with a steady flow

B.　bright red with a steady flow

C.　bright red and in spurts

D.　dark red and in spurts

KEY:　C

[5024]Cargo that gives off fumes that may contaminate other cargo is known as a(n) ______.

A.　delicate cargo

B.　dirty cargo

C.　toxic cargo

D.　odorous cargo

KEY:　D

[5025]First aid treatment for small cuts and open wounds is to ______.

A.　lay the patient down and cover the wound when the bleeding stops

B.　stop the bleeding，clean，medicate，and cover the wound

C.　apply an ice pack to the wound and cover it when the bleeding stops

D.　apply a hot towel to purge the wound，then medicate and cover it

KEY:　B

[5026]Halon gas will decompose and may form very hazardous toxic fumes when discharged ______.

A.　directly on flames

B.　at room temperature

C.　in an extremely cold climate

D.　None of the above

KEY:　A

[5027]If a crew member is exposed to phenol by way of skin or eye contact，you should IMMEDIATELY ______.

A.　administer oxygen

B.　treat victim for shock

C.　flush skin and eyes with water

D.　give victim stimulant

KEY:　C

[5028]If a crewman suffers a second-degree burn on the arm，you should ______.

A.　drain any blisters

B.　apply antiseptic ointment

C.　scrub the arm thoroughly to prevent infection

D.　immerse the arm in cold water

KEY:　D

[5029]If the patient vomits during mouth-to mouth resuscitation，the rescuer should FIRST ______.

A.　ignore it and continue mouth-to-mouth ventilation

B.　pause for a moment until the patient appears quiet again，then resume ventilation mouth-to-mouth

C.　switch to mouth-to-nose ventilation

D.　turn the patient's body to the side，sweep out the mouth and resume mouth-to-mouth ventilation

KEY:　D

[5030]If you came into contact with nitrobenzene while disconnecting the cargo hose，you should be aware that it is ______.

A.　highly toxic when absorbed through the skin

B.　a blood poison

C.　a nerve poison

D.　All of the above

KEY:　D

[5031]In order to determine the health hazard data for carbolic acid，you would use that information contained in the Chemical Data Guide for which chemical?

A.　Acetic oxide

B.　Carbinol

C.　Carbon bisulfide

D.　Phenol

KEY:　D

[5032]In reviving a person who has been overcome by gas fumes，what would you AVOID doing? ______.

A.　Giving stimulants

B.　Prompt removal of the patient from the suffocating atmosphere

C.　Applying artificial respiration and massage

D.　Keeping the patient warm and comfortable

KEY:　A

[5033]Medical treatment aboard a ship should not go beyond examination and emergency care without first consulting ______.

A.　the designated medic aboard

B.　the shore based superintendent

C.　a medical doctor

D.　the approved company medical manual

KEY:　C

[5034]Persons who have swallowed a non-petroleum based poison are given large quantities of warm soapy water or warm salt water to ______.

A.　induce vomiting

B.　absorb the poison from the blood

C.　neutralize the poison in the blood

D.　increase the digestive process and eliminate the poison

KEY:　A

[5035]Radiotelegrams requesting medical advice should state briefly and clearly the symptoms of the illness of the person ______.

A.　affected

B.　effected

C.　affecting

D.　effecting

KEY:　A

[5036]Segregation of cargoes refers to ______.

A.　separating cargoes so that the inherent characteristics of one cannot damage the other

B.　separating cargoes by destination

C.　classifying cargoes according to their toxicity

D.　listing the cargoes in order of their flammability

KEY:　A

[5037]Severe exposure to chlorine gas can be fatal. Chlorine gas is primarily a ______.

A.　respiratory irritant

B.　skin burning agent

C.　blood poisoning agent

D.　nerve paralyzing irritant

KEY:　A

[5038]Since accumulations of H2S gas on a ship can be dangerous to personnel，it is important to know that H2S gas is ______.

A.　lighter than air

B.　heavier than air

C.　a yellow gas that is easily recognized

D.　mildly toxic

KEY:　B

[5039]Since electrical burn victims may be in shock，the FIRST medical response is to check for ______.

A.　indication of broken bones

B.　breathing and heartbeat

C.　symptoms of concussion

D.　bleeding injuries

KEY:　B

[5040]Storage batteries should be charged in a well ventilated area because ______.

A.　they generate heat

B.　they emit hydrogen

C.　of the toxic fumes they emit

D.　they recharge faster in a well ventilated space

KEY:　B

[5041]Symptoms of sugar diabetes include ______.

A.　increased appetite and thirst

B.　decreased appetite and thirst

C.　gain in weight

D.　elevated temperature

KEY:　A

[5042]The Agreement sha1l commence ______ a satisfactory Medical Report by a doctor nominated by the Company.

A.　on the part of

B.　in lieu of

C.　subject to

D.　in accordance with

KEY:　C

[5043]The danger associated with using carbon dioxide in an enclosed space is ______.

A.　frostbite

B.　skin burns

C.　asphyxiation

D.　an explosive reaction

KEY:　C

[5044]The most effective warming treatment for a crew member suffering from hypothermia is ______.

A.　running or jumping to create heat

B.　lying in the sun

C.　a warm water bath

D.　mouth-to-mouth resuscitation

KEY:　C

[5045]The principal personnel hazard unique to Halon extinguishers is ______.

A.　displacement of oxygen

B.　skin irritation

C.　inhaling toxic vapors

D.　eye irritation

KEY:　C

[5046]The rescuer can best provide an airtight seal during mouth-to-mouth resuscitation by pinching the victim's nostrils and ______.

A.　cupping a hand around the patient's mouth

B.　keeping the head elevated

C.　applying his mouth tightly over the victim's mouth

D.　holding the jaw down firmly

KEY:　C

[5047]Treatment of frostbite includes ______.

A.　rubbing affected area with ice or snow

B.　rubbing affected area briskly to restore circulation

C.　wrapping area tightly in warm cloths

D.　warming exposed parts rapidly

KEY:　D

[5048]Treatments of heat exhaustion consist of ______.

A.　moving to a shaded area and laying down

B.　bathing with rubbing alcohol

C.　placing the patient in a tub of cold water

D.　All of the above

KEY:　A

[5049]What are the symptoms of sun stroke?

A.　Temperature falls below normal，pulse is rapid and feeble，skin is cold and clammy.

B.　Temperature is high，pulse is strong and rapid，skin is hot and dry.

C.　Temperature is high，pulse is slow and feeble，skin is clammy.

D.　Temperature falls below normal，pulse is rapid，skin is clammy.

KEY:　B

[5050]What is a treatment for traumatic shock?

A.　Administer CPR.

B.　Administer fluids.

C.　Open clothing to allow cooling of the body.

D.　Keep the victim in a sitting position.

KEY:　B

19.5 救生作业

[5051]A hydrostatic release mechanism for a liferaft ______.

A.　must be wet before it will release

B.　should be kept in a watertight cover except in an emergency

C.　will inflate the raft in its cradle if operated manually

D.　must be submerged to a certain depth to release automatically

KEY:　D

[5052]A chock ______.

A.　is a deck fitting used to shackle gear to the deck

B.　permits easy jettisoning of deck cargo in an emergency

C.　prevents stress concentration in the bulwark

D.　provides openings through the bulwark for mooring lines

KEY:　D

[5053]A life line must be connected to the liferaft ______.

A.　at the bow

B.　at the stern

C.　in the middle

D.　all around

KEY:　D

[5054]A liferaft which has inflated bottom-up on the water ______.

A.　should be righted by standing on the carbon dioxide cylinder，holding the righting straps，and leaning backwards

B.　should be righted by standing on the life line，holding the righting straps，and leaning backwards

C.　will right itself when the canopy tubes inflate

D.　must be cleared of the buoyant equipment before it will right itself

KEY:　A

[5055]A rigid lifesaving device designed to support survivors in the water is a ______.

A.　rigid liferaft

B.　life float

C.　inflatable liferaft

D.　survival capsule

KEY:　B

[5056]A rigid lifesaving device only designed for survivors to hold on to while in the water is a ______.

A.　liferaft

B.　life float

C.　life preserver

D.　buoyant apparatus

KEY:　D

[5057]After abandoning ship which action should be taken IMMEDIATELY upon entering a liferaft?

A.　Open equipment pack.

B.　Issue anti-seasickness medicine.

C.　Cut painter and clear the ship.

D.　Dry the liferaft floor and inflate.

KEY:　C

[5058]After abandoning ship，you should deploy the sea anchor from a liferaft to ______.

A.　keep the liferaft from capsizing

B.　navigate against the current

C.　keep personnel from getting seasick

D.　stay in the general location

KEY:　D

[5059]After you have thrown the liferaft and stowage container into the water，you inflate the liferaft by ______.

A.　pulling on the painter line

B.　forcing open the container which operates the CO2

C.　hitting the hydrostatic release

D.　using the hand pump provided

KEY:　A

[5060]All lifeboats，rescue boats，and rigid-type liferafts shall be stripped，cleaned，and thoroughly overhauled at least once every ______.

A.　6 months

B.　year

C.　18 months

D.　two years

KEY:　B

[5061]An inflatable liferaft can be launched by ______.

A.　the float-free method ONLY

B.　breaking the weak link on the painter

C.　throwing the entire container overboard and then pulling on the operating cord to inflate the raft

D.　removing the securing straps

KEY:　C

[5062]An inflatable liferaft is hand-launched by ______.

A.　pulling a cord

B.　cutting the wire restraining bands

C.　removing the rubber packing strip

D.　throwing the entire container overboard

KEY:　D

[5063]An inflatable liferaft is thrown into the water from a sinking vessel. What occurs automatically after the painter trips the CO2 bottles to inflate the raft? ______.

A.　The sea anchor deploys.

B.　The floor inflates.

C.　If upside down，the craft rights itself.

D.　The painter detaches from the raft.

KEY:　A

[5064]An inflatable liferaft should be manually released from its cradle by ______.

A.　cutting the straps that enclose the container

B.　removing the rubber sealing strip from the container

C.　loosening the turnbuckle on the securing strap

D.　pushing the button on the hydrostatic release

KEY:　D

[5065]As a vessel sinks to a depth of 15 feet，the hydrostatic trip releases the liferaft container from its cradle by ______.

A.　breaking the weak link

B.　releasing the tie-down strap

C.　pulling the operating cord

D.　releasing the CO2 canister

KEY:　B

[5066]Each lifebuoy shall be marked in block capitals of the Roman alphabet with ______ of the ship．

A.　the call sign and name

B.　the official number and name

C.　the port of registry and name

D.　the port of registry and official number

KEY:　C

[5067]Each liferaft，which does not have an indicated maximum stowage height indicated on the liferaft，must be ______.

A.　limited to carry no more than 10 persons

B.　stowed not more than 59 feet above the lightest waterline

C.　stowed in quick release racks

D.　inspected every six months

KEY:　B

[5068]Generally，what is used to inflate liferafts? ______.

A.　non-toxic gas

B.　Oxygen

C.　Hydrogen

D.　Helium

KEY:　A

[5069]How should signal flares be used after you have abandoned ship and are adrift in a liferaft? ______.

A.　Immediately use all the signals at once.

B.　Use all the signals during the first night.

C.　Employ a signal every hour after abandoning ship until they are gone.

D.　Use them only when you are aware of a ship or plane in the area.

KEY:　D

[5070]If an inflatable liferaft inflates upside down，you can right it by ______.

A.　pushing up on one side

B.　standing on the CO2 bottle，holding the bottom straps，and throwing your weight backwards

C.　getting at least three or four men to push down on the side containing the CO2 cylinder

D.　doing nothing; it will right itself after the canopy supports inflate

KEY:　B

[5071]If an inflatable liferaft is overturned，it may be righted by ______.

A.　filling the stabilizers on one side with water

B.　releasing the CO2 cylinder

C.　pushing up from under one end

D.　standing on the inflating cylinder and pulling on the straps on the underside of the raft

KEY:　D

[5072]If an inflatable liferaft is to be released manually，where should the operating cord be attached before throwing the raft overboard?

A.　Do not attach the cord to anything but throw it overboard with the raft container.

B.　Attach the cord to a fixed object on the ship.

C.　You should stand on the cord.

D.　Attach the cord to the special pad eye on the raft davit launcher.

KEY:　B

[5073]If the hydrostatic release mechanism for an inflatable liferaft is not periodically serviced and becomes inoperative，it will NOT ______.

A.　set the water lights on immersion

B.　release the dye-marker from the liferaft

C.　free the liferaft from the vessel

D.　break the seal on the carbon dioxide cylinder

KEY:　C

[5074]If the liferaft capsizes，all personnel should leave the raft and ______.

A.　climb onto the bottom

B.　swim away from the raft

C.　right the raft using the righting strap

D.　inflate the righting bag

KEY:　C

[5075]If you are forced to abandon ship in a liferaft，your course of action should be to ______.

A.　remain in the immediate vicinity

B.　head for the nearest land

C.　head for the closest sea-lanes

D.　let the persons in the boat vote on what to do

KEY:　A

[5076]If you find an inflatable liferaft container with steel bands around the case，you should ______.

A.　tell the Master

B.　leave the bands in place

C.　tell the Mate

D.　remove the bands yourself

KEY:　B

[5077]If your vessel is equipped with inflatable liferafts，how should they be maintained?

A.　Have your crew check them annually.

B.　They do not need any maintenance.

C.　Have them sent ashore to an approved maintenance facility annually.

D.　Have them serviced by the shipyard annually.

KEY:　C

[5078]In evacuation from a ship，an individual without the option of a survival craft or liferaft should enter the water on the leeward side，except when ______.

A.　there is burning oil on the water

B.　there is a rescue craft in the area

C.　water temperature is below 40F

D.　a rigid survival craft is in the area

KEY:　A

[5079]Inflatable liferafts are provided with ______.

A.　a Very pistol

B.　a towing connection

C.　a portable radio

D.　canned milk

KEY:　B

[5080]Inflatable liferafts shall be serviced at an approved servicing facility every 12 months or not later than the next vessel inspection for certification. However，the total elapsed time between servicing cannot exceed ______.

A.　12 months

B.　15 months

C.　17 months

D.　18 months

KEY:　C

[5081]On inflatable liferafts，the operating cord should be renewed by ______.

A.　cutting the old line off and renewing same

B.　an approved servicing facility ashore

C.　opening the case and replacing the entire line

D.　one of the ship's officers

KEY:　B

[5082]Personnel boarding a davit-launched liferaft from a ship should be checked to assure they are not in possession of or wearing ______.

A.　oil stained shoes or clothing

B.　sharp objects that may puncture or damage the liferaft

C.　matches or cigarette lighters

D.　drugs or paraphernalia not approved by a medical doctor

KEY:　B

[5083]Prior to boarding from a vessel，a davit-launched liferaft should be well ventilated of excess ______.

A.　hydrocarbon gas

B.　carbon monoxide gas

C.　carbon dioxide gas

D.　freon gas

KEY:　C

[5084]Prior to entering a davit-launched liferaft，you should make sure that ______.

A.　the liferaft is well ventilated of excess carbon dioxide

B.　all personnel have removed their life preservers to facilitate boarding

C.　the door flap has been cut away and stowed inside the raft

D.　All of the above

KEY:　A

[5085]Signaling devices required on inflatable liferafts include a(n) ______.

A.　Very pistol

B.　orange smoke signal

C.　air horn

D.　lantern

KEY:　B

[5086]Signaling devices which are required on inflatable liferafts include ______.

A.　a rocket shoulder rifle

B.　an oil lantern

C.　red flares

D.　an air horn

KEY:　C

[5087]The air spaces in the floor of an inflatable liferaft will provide protection against ______.

A.　asphyxiation from CO2

B.　loss of air in the sides of the raft

C.　rough seas

D.　cold water temperatures

KEY:　D

[5088]The bosun has thrown the liferaft into the water before abandoning the vessel. The operating cord ______.

A.　serves as a sea painter

B.　detaches from the liferaft automatically

C.　is used to rig the boarding ladder

D.　is cut immediately as it is of no further use

KEY:　A

[5089]The capacity of any liferaft on board a vessel can be determined by ______.

A.　examining the Certificate of Inspection

B.　examining the plate on the outside of the raft container

C.　referring to the station bill

D.　referring to the shipping articles

KEY:　B

[5090]The hydrostatic release on the inflatable liferafts on a fishing vessel must be ______.

A.　replaced annually

B.　tested monthly

C.　serviced annually

D.　overhauled quarterly

KEY:　C

[5091]The inside light in an inflatable liferaft is turned on ______.

A.　automatically as the liferaft inflates

B.　with a switch near the boarding handle

C.　at night because the light has a photosensitive switch

D.　by screwing the bulb in after the raft is inflated

KEY:　A

[5092]The lights on the outside of the canopy on an inflatable liferaft operate ______.

A.　by turning the globe clockwise

B.　by a switch at each light

C.　by a light sensor

D.　automatically when the raft is inflated

KEY:　D

[5093]The limit switches on a ships survival-craft winch system ______.

A.　stop the winch just before the craft reaches the final stowage position

B.　limit the amount of cable on the drum

C.　limit the ascent rate

D.　stop the winch in case the craft weighs too much

KEY:　A

[5094]The most important reason for taking anti-seasickness pills as soon as possible after entering a liferaft is to ______.

A.　assist in sleeping

B.　reduce appetite by decreasing nausea

C.　prevent loss of body moisture by vomiting

D.　prevent impaired judgement due to motion-induced deliriousness

KEY:　C

[5095]The most important thing to remember when launching an inflatable liferaft by hand is to ______.

A.　open the CO2 inflation valve

B.　open the raft container

C.　ensure that the operating cord is secured to the vessel

D.　inflate the raft on the vessel，then lower it over the side

KEY:　C

[5096]The operating cord on an inflatable liferaft also serves as a ______.

A.　lifeline

B.　painter

C.　drogue

D.　marker

KEY:　B

[5097]The person assigned to command a lifeboat or inflatable liferaft on a ship shall have a list of the persons assigned to the lifeboat or liferaft. The list shall include each person's ______.

A.　home address

B.　duties

C.　home phone number

D.　nearest relative's address

KEY:　B

[5098]The worst atmospheric condition for dispersion of H2S is ______.

A.　heavy rain

B.　gusty winds with rain

C.　nearly calm，clear nights or early morning

D.　full sun with high winds

KEY:　C

[5099]To inflate a davit launched liferaft you ______.

A.　initially connect the compressed air nozzle to the inflation tube

B.　pull the inflation lanyard

C.　connect the hand pump to the inflation tube and pump it up

D.　connect the helium nozzle to the inflation tube

KEY:　B

[5100]To launch a liferaft by hand you should ______.

A.　cut the casing bands，throw the liferaft over the side，and it will then inflate

B.　detach the operating cord，throw the raft over the side，and it will then inflate

C.　cut the casing bands，throw the raft over the side，and pull the operating cord

D.　throw the liferaft over the side and pull the operating cord

KEY:　D

19.6 海上求生

[5101]Your ship is sinking rapidly. A container containing an inflatable liferaft has bobbed to the surface upon functioning of the hydrostatic release. Which action should you take? ______.

A.　Cut the painter line so it will not pull the liferaft container down.

B.　Swim away from the container so you will not be in danger as it goes down.

C.　Take no action because the painter will cause the liferaft to inflate and open the container.

D.　Manually open the container and inflate the liferaft with the hand pump.

KEY:　C

[5102]When in command of a lifeboat under oars，the command TOSS OARS means to ______.

A.　lift oars to vertical position，trim blades fore and aft with handles resting on the thwarts

B.　complete the stroke，come to Oars，raise the oars smartly to the vertical，rest handles on footings and trim blades fore and aft

C.　place oars in row locks directly from the boated position or from Stand by oars position

D.　complete the stroke (when rowing in ahead motion)，raise the oars with crook of elbow to about 30 degrees，swing blades forward

KEY:　B

[5103]A certificated lifeboatman assigned to command the lifeboat should ______.

A.　be the first individual to board the craft

B.　drain the hydraulic pressure before lowering the craft

C.　have a list of the persons assigned to the lifeboat

D.　All of the above

KEY:　C

[5104]A fully loaded motor-propelled lifeboat must be capable of attaining a speed of at least ______.

A.　3 knots in smooth water

B.　6 knots in smooth water

C.　3 knots in rough water

D.　6 knots in rough water

KEY:　B

[5105]A lifeboat is weight tested by lowering to near the waterand loading to capacity with weight evenly distributed. Itis then lowered into the water and released. This test mustbe done at least once every ______.

A.　6 months

B.　12 months

C.　18 months

D.　24 months

KEY:　D

[5106]A self-righting survival craft will return to an upright position provided that all personnel ______.

A.　are seated with seat belts on and doors shut

B.　are seated with seat belts on and doors open

C.　are to shift to one side to right it

D.　escape from the craft

KEY:　A

[5107]According to the Code of Federal Regulations，on vessels other than river ferryboats and river vessels，how are periodic lifeboat weight tests required to be conducted?

A.　The lifeboat is lowered to near the water，loaded with the allowed capacity，lowered into the water until afloat，then released from the falls.

B.　The lifeboat is loaded with the allowed capacity，lowered into the water until afloat，then released from the falls.

C.　The lifeboat is lowered into the water until afloat，loaded with the allowed capacity，then released from the falls.

D.　The lifeboat is lowered to near the water，loaded with the allowed capacity，then released from the falls.

KEY:　A

[5108]After conducting an abandonment drill，the Master or person in charge of a ship shall log ______.

A.　the names of crew members who participated in the drill

B.　the length of time that each motor propelled lifeboat was operated in the drill

C.　the length of time the lifeboat was in the water

D.　the time it took to lower the boat

KEY:　A

[5109]All inflatable liferafts have ______.

A.　safety straps from the overhead

B.　built in seats

C.　releasing hooks at each end

D.　water stabilizing pockets

KEY:　D

[5110]Aluminum lifeboats are subject to damage by electrolytic corrosion (the aluminum being eaten away). In working around boats of aluminum you must be very careful ______.

A.　to keep the boats covered at all times

B.　not to leave steel or iron tools lying in or near these boats

C.　to keep an electric charge on the hull at all times

D.　to rinse these boats regularly with salt water

KEY:　B

[5111]Before hydraulic starting of an engine on a covered lifeboat，what need NOT be checked?

A.　Fuel supply line valve

B.　Pressure registered on the accumulator gauge

C.　Cold-spark voltage readings test lamp

D.　Engine stop control

KEY:　C

[5112]For each person it is certified to carry，a lifeboat on an oceangoing passenger vessel must be provided with all of the following EXCEPT ______.

A.　3 litres of water

B.　1 unit of provisions

C.　1 seasickness kit

D.　1 life preserver

KEY:　D

[5113]Generally，when lifting an inflatable liferaft back aboard ship you would use the ______.

A.　towing bridle

B.　main weather cover

C.　external lifelines

D.　righting strap

KEY:　A

[5114]How should the lifeboat sea painter be rigged?

A.　Spliced into the ring on the stem post

B.　Secured to the inboard side of a forward thwart and led inboard of the falls

C.　Secured by a toggle to the stem post and led outboard of the falls

D.　Secured by a toggle around the outboard side of a forward thwart

KEY:　B

[5115]If the steersman of your lifeboat gives the command Way enough，you should ______.

A.　complete the stroke，hold your oar out from the boat and level with the water

B.　dip the blade of your oar into the water and leave it there

C.　lift your oar to a vertical position

D.　complete the stroke，raise your oar slightly，swing it forward，and place it in the boat

KEY:　D

[5116]If water is rising in the bilge of a lifeboat，you should FIRST ______.

A.　abandon the survival craft

B.　check for cracks in the hull

C.　shift all personnel to the stern

D.　check the bilge drain plug

KEY:　D

[5117]If you are forced to abandon ship in a lifeboat，you should ______.

A.　remain in the immediate vicinity

B.　head for the nearest land

C.　head for the closest sea-lanes

D.　vote on what to do，so all hands will have a part in the decision

KEY:　A

[5118]In evacuation from a ship，an individual without the option of a survival craft or liferaft should enter the water on the leeward side，except when ______.

A.　there is burning oil on the water

B.　there is a rescue craft in the area

C.　water temperature is below 40F

D.　a rigid survival craft is in the area

KEY:　A

[5119]In launching a lifeboat，when should the tricing pendants be released? ______.

A.　Before the boat is lowered from the stowage position

B.　As soon as the boat-fall blocks clear the davit head

C.　After the limit switch is activated

D.　After all people have been embarked

KEY:　D

[5120]Most covered lifeboats will right themselves after capsizing if the ______.

A.　passengers are strapped to their seats

B.　fuel tanks are not less than half full

C.　sea anchor is deployed to windward

D.　lower ballast tanks are filled with water

KEY:　A

[5121]Most lifeboats are equipped with ______.

A.　unbalanced rudders

B.　balanced rudders

C.　contraguide rudders

D.　straight rudders

KEY:　A

[5122]On a passenger vessel，the vessel's name must appear on ______.

A.　ring life buoy

B.　lifeboats

C.　lifeboat oars

D.　All of the above

KEY:　D

[5123]On open lifeboats，the purpose of the wire stretched between the davit heads is to ______.

A.　keep the movement of the davits at the same speed

B.　keep the davits from slipping when they are in the stowed position

C.　prevent vibration during lowering of the boat

D.　support the manropes

KEY:　D

[5124]Prior to lowering the lifeboat，the most important item to check is the ______.

A.　oars

B.　sail

C.　boat plug

D.　life preservers

KEY:　C

[5125]PULL THE STRINGS AROUND YOUR WAIST AND TIE IN FRONT. This description is likely used in __________.

A.　fire fighting operations

B.　salvage operations

C.　cargo operations

D.　manoeuvering operations

KEY:　B

[5126]Stretchers are fitted in lifeboats to provide a ______.

A.　place for people to lie down

B.　means for rigging the sail

C.　place for rowers to brace their feet

D.　suitable means for water to drain below the footings

KEY:　C

[5127]The functioning of the radiotelegraph installations for motor lifeboats and/or the portable radio apparatus for survival craft，if provided，______ with the provisions of the regulations.

A.　provided

B.　supplied

C.　complied

D.　divided

KEY:　C

[5128]The knife on an inflatable liferaft will always be located ______.

A.　in one of the equipment bags

B.　in a special pocket near the forward entrance

C.　on a cord hanging from the canopy

D.　in a pocket on the first aid kit

KEY:　B

[5129]The lifeboats and liferafts were ______ with the provisions of the regulations annexed to the Convention.

A.　put on board

B.　installed

C.　fitted

D.　equipped

KEY:　D

[5130]The Master of a cargo or tanker shall be responsible that each lifeboat，except those free-fall launched，is lowered to the water with crew and maneuvered at least once every ______.

A.　week

B.　month

C.　three months

D.　year

KEY:　C

[5131]The Master or person in charge on a ship shall insure that line throwing equipment is not operated ______.

A.　during a rain storm

B.　in an explosive atmosphere

C.　near a lifeboat station

D.　by other than senior rig personnel

KEY:　B

[5132]The number of flashlights required as lifeboat equipment on a seagoing barge is ______.

A.　none

B.　one

C.　two

D.　three

KEY:　A

[5133]The operating cord on an inflatable liferaft should be renewed by ______.

A.　removing the top half of the shell，cutting the line at its source，and renewing completely

B.　cutting the line where it enters the case and replacing that portion

C.　leaving the original line and tying another one to it so the two lines will take the strain

D.　an approved servicing facility ashore

KEY:　D

[5134]The order to enter the lifeboats will be given from the __________.

A.　bridge

B.　forward station

C.　aft station

D.　any hold in which the stevedores are working

KEY:　A

[5135]The signal given to commence lowering the lifeboats is ______.

A.　3 short blasts of the ship's whistle

B.　1 short blast of the ship's whistle

C.　3 long blasts of the ship's whistle

D.　1 long blast of the ship's whistle

KEY:　B

[5136]The water pockets located on the underside of inflatable liferafts ______.

A.　stow rainwater; these 4 spaces do not take up valuable space

B.　act as stabilizers by filling with sea water as soon as the raft is inflated and upright

C.　hold the freshwater required by regulation to be provided in the raft when packed

D.　None of the above

KEY:　B

[5137]Two individuals authorized to serve as a lifeboatman must be assigned to any lifeboat that has a capacity of more than ______.

A.　10 persons

B.　20 persons

C.　30 persons

D.　40 persons

KEY:　D

[5138]Under normal conditions a liferaft is released from itscradle by ______.

A.　cutting the restraining strap

B.　unscrewing the turnbuckle on the back of the cradle

C.　lifting one end of the raft

D.　pushing the plunger on the center of the hydrostatic release

KEY:　D

[5139]What is NOT a function of the air supply of a covered lifeboat?

A.　Provides air for engine combustion

B.　Pressurizes water spray system

C.　Provides air for passenger respiration

D.　Prevents smoke and other noxious fumes from entering craft

KEY:　B

[5140]What is one of the FIRST things you would do on boarding an inflatable liferaft?

A.　Open equipment pack.

B.　Post a lookout.

C.　Issue anti-seasickness medicine.

D.　Pick up other survivors.

KEY:　D

[5141]What is placed on the under side of an inflatable liferaft to help prevent it from being skidded by the wind or overturned?

A.　Water pockets

B.　A keel

C.　Strikes

D.　Sea anchor

KEY:　A

[5142]What should be used to steer an open lifeboat if the rudder becomes lost or damaged?

A.　Sea anchor

B.　Steering oar

C.　Spare rudder

D.　Dagger board

KEY:　B

[5143]When a man who has fallen overboard is being picked up by a lifeboat，the boat should approach with the wind ______.

A.　astern and the victim just off the bow

B.　ahead and the victim just off the bow

C.　just off the bow and the victim to windward

D.　just off the bow and the victim to leeward

KEY:　D

[5144]When landing a lifeboat through heavy surf with a strong current running parallel to the beach (from right to left when facing from seaward) the recommended procedure is to______.

A.　approach while coming to the left to take advantage of the current

B.　drop an anchor outside the surf line，then pay out anchor line over the bow while the seas carry the boat toward the beach

C.　approach slow enough so that the boat can be brought around to meet breaking seas on the bow

D.　rig a drogue with tripping line over the bow，back ashore with drogue tripped between breakers

KEY:　D

[5145]When launching a lifeboat，the tricing pennants should be released ______.

A.　before the boat is lowered from the stowed position

B.　as the boat-fall blocks break clear of the davit head

C.　before the boat is lowered from the embarkation level

D.　after the boat is released into the water

KEY:　C

[5146]When should you use distress flares and rockets?

A.　Only when there is a chance of their being seen by rescue vessels

B.　At half-hour intervals

C.　At one-hour intervals

D.　Immediately upon abandoning the vessel

KEY:　A

[5147]When using the rain water collection tubes on a liferaft，the first collection should be ______.

A.　passed around so all can drink

B.　poured overboard because of salt washed off the canopy

C.　saved to be used at a later time

D.　used to boil food

KEY:

[5148]When whistle signals are used for launching lifeboats，one short blast means ______.

A.　use the float-free method only

B.　lower all boats

C.　raise all boats

D.　drill is over，secure all boats

KEY:　B

[5149]Which of following data is NOT painted on the bow of a lifeboat? ______.

A.　Number of persons allowed

B.　Name of the vessel

C.　Weight of the boat

D.　Home port

KEY:　C

[5150]Which operation should be done when launching an inflatable liferaft by hand?

A.　Open the liferaft casing.

B.　Turn the valve on the CO2 cylinder to start inflation.

C.　Make sure the operating cord is secured to the vessel before throwing it over the side.

D.　After inflation，detach operating cord from liferaft.

KEY:　C